3GPP TSG RAN WG2#115-e	R2-2106958

e-Meeting, 9th - 27th August, 2021

3GPP TSG-RAN WG4 #99-e	R4-2108003

# Electronic Meeting, May 19-27, 2021

Title:	Reply LS on simultaneous Rx/Tx capability

Response to:	R2-2102495

Release:	Rel. 15

Work Item:	NR_newRAT

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Valentin Gheorghiu

vgheorgh (at) qti (dot) qualcomm (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN4 would like to thank RAN2 for the reply LS on simultaneous Rx/Tx capability.

RAN4 had further discussion on the simultaneous Rx/Tx capability and concluded that CA capability signaling should be able to indicate support of simultaneous Rx-Tx for all band pairs within a higher order CA combination.

For example, if the UE supports a combination of Band 1 + Band 2 + Band 3 + Band 4, the UE should be able to indicate that it supports simultaneous Rx/Tx between Band 1 and Band 2 but it does not support simultaneous Rx/Tx between Band 3 and Band 4.

The above capability is needed for any TDD-TDD and TDD-FDD inter-band CA, SUL, EN-DC, NE-DC and NR-DC within the same CG or across CGs or both.

# 2	Actions

To 3GPP RAN2

ACTION: To take into account the RAN4’s feedback in the discussion/design of the UE capability signaling.

# 3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #100-e	16-27 August 2021, Electronic