3GPP TSG RAN WG2#115-e	R2-2106968

e-Meeting, 9th - 27th August, 2021

3GPP TSG-SA WG2 Meeting #145E	S2-2105122

Electronic Meeting, May 17 – 28, 2021                                                        (Revision of S2-2104438)

Title:	Response LS on Scheduling Location in Advance to reduce Latency

Response to:	S2-2103735 (R2-2104420) Response LS on Scheduling Location in Advance to reduce Latency

Release:	Rel-17

Work Item:	Enhancement to the 5GC LoCation Services-Phase 2 (5G_eLCS_Ph2)

Source:	SA WG2

To:	RAN WG2

Cc:	RAN WG1, RAN WG3

Contact Person:

#### Name:	Ming Ai

E-mail Address:	aiming (at) catt (dot) cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA2 thanks RAN2 for their response LS on Scheduling Location in Advance to reduce Latency. SA2 discussed the questions and agreed the following answers.

1.	Definition of "scheduled location time":

The definition of scheduled location time is not fully clear to RAN2 since the SA2 endorsed CR describes the scheduled location time differently in different parts of the CR and there is a different description of scheduled location time in the SA2 LS itself. RAN2 would like to have a clear definition of scheduled location time first as this may impact what is signalled to UE and/or NG-RAN. Please see below for detailed comments:

In section 4.1c it says "The request includes the scheduled location time T" which is described as the start of location preparation phase. So, time T is the scheduled location time.

Section 4.1c also says "the scheduled location time allows an external LCS Client, AF or the UE to specify a time in the future at which a current location of the UE is to be obtained". However, in Figure 4.1c-1, the time when the LCS client, AF or UE obtains the location is T+t2. The time T in the figure is shown as the time at which the UE or NG-RAN obtains the location measurements.

The SA2 LS description also states "…support scheduling of location of a target UE in advance using a scheduled location time at which location measurements for the target UE would be obtained by the UE (in the case of DL measurements) and/or NG-RAN (in the case of UL measurements)". This seem to align with the time T in Figure 4.1c-1 which show it as a scheduled measurement time.

In section 6.3.1, step 25 describes the scheduled location time as the time at which the LMF must obtain the UE location, not the time at which the LCS client receives the location or the time at which the LMF schedules the measurement.

Answer 1: SA2 agrees the definition of Scheduled Location Time is a future global time (e.g. UTC) at which a UE is to be located. It is used by an LMF to determine the time to trigger positioning procedures, as defined in clause 6.11, TS 23.273. A location estimate returned to an LCS Client for a scheduled location time can be treated by the LCS Client as an estimate of the location of the UE at the scheduled location time.

2.	In section 6.1.2, there is the following editor’s note:

"Editor's note: Feedback from RAN is needed to verify whether location measurements can be scheduled 		to occur at a UE or NG-RAN at a specific scheduled location time."

Please clarify the requirement whether measurements in UE/NG-RAN need to be scheduled at the scheduled location time received from 5GC or at a time before the scheduled location time received from 5GC.

Answer 2: Some companies in SA2 think the Scheduled Location Time should not be sent to NG-RAN and UE. Other companies believe RAN WGs should decide whether it may be useful to send the Scheduled Location Time to NG-RAN and the UE in order to trigger measurements at or close to the scheduled location time.

Question A: in order to get a clear view from RAN WG, SA2 sincerely ask RAN2 to investigate whether Scheduled Location Time could help the reduction of the LCS latency.

3.	Can SA2 clarify the time format used for the scheduled location time T which is provided to an LMF (e.g., UTC, etc.)?

Answer 3: It is SA2’s understanding that the scheduled location time T is a UTC time. However, the actual format of parameter T is under Stage-3’s remit.

4.	For a Deferred 5GC-MT-LR for periodic location events, RAN2's understanding is that a scheduled location time may apply for the first periodic location report only. For each succeeding periodic location report, the

"scheduled location time" is equivalent to the periodic reporting interval. Please confirm whether RAN2's understanding is correct, or whether different location times T can apply for succeeding periodic location reports.

Answer 4: The “scheduled location time” is applied to the first location report only. The periodic reporting interval is controlled by the existing parameter of “the time interval between successive location reports”. This implicitly means that for succeeding periodic location report, the UE would be located at the time determined based on the formula T + N*P, where T = scheduled location time, N = Report Number after the initial report and P = periodic interval.

5.	Clause 4.1c says "A scheduled location time can be used with a 5GC-MT-LR, 5GC-MO-LR or deferred 5GC-MT-LR for periodic or triggered location events."

RAN2 would like to understand how a scheduled location time can be applicable to triggered location events given its sporadic nature.

Answer 5: The scheduled location time applies to periodic location event report. Whether it applies to triggered event type is still under discussion in SA2.

2. Actions:

To RAN WG2 group.

ACTION: 	SA2 kindly asks RAN2 to take the above information into account and provide answers to Question A.

3. Date of Next TSG-SA2 Meetings:

3GPP TSG-SA2 Meeting #146-e		16 - 27 August 2021			Electronic Meeting

3GPP TSG-SA2 Meeting #147-e		18 - 22 October 2021			Electronic Meeting