3GPP TSG-RAN WG2 #116-e	Tdoc R2-2109395

E-meeting, 1st – 12th November 2021

Agenda Item:	6.1.4.3

Source:	OPPO

Title:	Discussion on capability for DAPS

Document for:	Discussion, Decision

Introduction

This contribution is to discuss the capability for DAPS.

Discussion

The current ASN.1 framework allows reporting of DAPS capability in two ways.

One is the inter-frequency DAPS, reported via a per-BC flag

CA-ParametersNR-v1610 ::=           SEQUENCE {

[…]

interFreqDAPS-r16                                 SEQUENCE {

interFreqAsyncDAPS-r16                            ENUMERATED {supported}    OPTIONAL,

interFreqDiffSCS-DAPS-r16                         ENUMERATED {supported}    OPTIONAL,

interFreqMultiUL-TransmissionDAPS-r16             ENUMERATED {supported}    OPTIONAL,

interFreqSemiStaticPowerSharingDAPS-Mode1-r16     ENUMERATED {supported}    OPTIONAL,

interFreqSemiStaticPowerSharingDAPS-Mode2-r16     ENUMERATED {supported}    OPTIONAL,

interFreqDynamicPowerSharingDAPS-r16              ENUMERATED {short, long}  OPTIONAL,

interFreqUL-TransCancellationDAPS-r16             ENUMERATED {supported}    OPTIONAL

}                                                                               OPTIONAL,

[…]

}

The other is the intra-frequency DAPS, reported via per-band-entry flag

FeatureSetDownlink-v1610 ::=   SEQUENCE {

[…]

intraFreqDAPS-r16                  SEQUENCE {

intraFreqDiffSCS-DAPS-r16          ENUMERATED {supported}            OPTIONAL,

intraFreqAsyncDAPS-r16             ENUMERATED {supported}            OPTIONAL

}                                                                        OPTIONAL,

[…]

}

The two are to be used in combination with the DAPS-specific FSC as follows

BandCombination-v1610 ::=          SEQUENCE {

[…]

featureSetCombinationDAPS-r16       FeatureSetCombinationId                OPTIONAL,

[…]

}

Based on 38.306, DAPS specific FSC is mandatory for DAPS capability reporting

| featureSetCombinationDAPS-r16 Indicates the feature set that the UE supports for DAPS handover on the NR band combination by FeatureSetCombinationId. A UE shall include this field if intra-freq or inter-freq DAPS handover is supported for this band combination. If the number of CCs within a band combination is more than two, UE shall support DAPS handover between every CC pair. A feature set including intraFreqDAPS-r16 can only be referred to by featureSetCombinationDAPS-r16, not by featureSetCombination. A feature set without intraFreqDAPS-r16 is only applied to inter-freq DAPS handover if it is referred to by featureSetCombinationDAPS. Both feature sets with and without intraFreqDAPS-r16 can be referred to by the same featureSetCombinationDAPS-r16. | BC | N/A | N/A | N/A |
| --- | --- | --- | --- | --- |

Yet the highlighted part is not crystal clear on the applicability to intra- and inter-frequency handover.

Firstly, for intra-frequency DAPS handover, logically as shown in the two cases by the figure below

Figure 1 Intra-frequency DAPS handover 
(case-1: same BW for source/target cell, case-2: different BW for source/target cell)

Secondly, for inter-frequency DAPS handover, logically as shown in the four cases by the figure below

Figure 2 Inter-frequency DAPS handover (case-3: intra-band and overlapping, case-4: intra-band contiguous, case-5: intra-band non-contiguous, case-6: inter-band)

Since the original design spirit for DAPS capability reporting is to follow the legacy CA capability framework as much as possible, we can divide the 6 cases above into two types

- Type-1: for case-1/2/3, which does not exist in legacy CA, i.e., there is no such case for (partially / fully) overlapping CCs in legacy CA framework;

- Type-2: for case-4/5/6, which exist in legacy CA, i.e., CCs without overlapping;

In the following, we try to clarify the left issues for the two types separately

|  | Overlapping | Intra/Inter-frequency | Same/different BW | Type |
| --- | --- | --- | --- | --- |
| Case-1 | Y | Intra | Same | 1 |
| Case-2 | Y | Intra | Different | 1 |
| Case-3 | Y | Inter | Same or different | 1 |
| Case-4 | N | Inter | Same or different | 2 |
| Case-5 | N | Inter | Same or different | 2 |
| Case-6 | N | Inter | Same or different | 2 |

Type-1 (Case-1/2/3)

Since this type cannot be mimic-ed by the legacy CA framework, the first question is how for gNB to derive the capability of source and target cell.

Considering the clarification in 38.306 below

| supportedBandwidthDL Indicates maximum DL channel bandwidth supported for a given SCS that UE supports within a single CC (and in case of intra-frequency DAPS handover for the source and target cells), which is defined in Table 5.3.5-1 in TS 38.101-1 [2] for FR1 and Table 5.3.5-1 in TS 38.101-2 [3] for FR2. […] | FSPC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |
| supportedBandwidthUL Indicates maximum UL channel bandwidth supported for a given SCS that UE supports within a single CC (and in case of intra-frequency DAPS handover for the source and target cells), which is defined in Table 5.3.5-1 in TS38.101-1 [2] for FR1 and Table 5.3.5-1 in TS 38.101-2 [3] for FR2. […] | FSPC | CY | N/A | N/A |

it seems for case-1/2, i.e., same or different BW intra-frequency DAPS handover, there are two interpretations:

- the capability for source/target cell can be derived based on the capability reported for a single CC entry, i.e., based on a same per-CC FS ID. E.g., proof can be the sentence above.

- The capability for source/target cell can be derived based on the capability reported for a pair of CC entries, i.e., based on a pair per-CC FS ID. E.g., proof can be the sentence above.

RAN2 clarify for intra-frequency DAPS HO, the source/target cell capability is to be derived based on 1) a same per-CC feature-set ID or 2) a pair of per-CC feature-set ID.

Either If P1 is concluded as intra-frequency DAPS is based on a same per-CC FSID, then the description in 306 is to be corrected, i.e., in case only intra-frequency handover is supported, DAPS-related FSC is only to indicate intra-frequency DAPS based on each single per-CC FS ID, but not support inter-frequency DAPS between each per-CC FS ID pair.

| featureSetCombinationDAPS-r16 Indicates the feature set that the UE supports for DAPS handover on the NR band combination by FeatureSetCombinationId. A UE shall include this field if intra-freq or inter-freq DAPS handover is supported for this band combination. If the number of CCs within a band combination is more than two and inter-freq DAPS handover is supported, UE shall support DAPS handover between every CC pair. A feature set including intraFreqDAPS-r16 can only be referred to by featureSetCombinationDAPS-r16, not by featureSetCombination. A feature set without intraFreqDAPS-r16 is only applied to inter-freq DAPS handover if it is referred to by featureSetCombinationDAPS. Both feature sets with and without intraFreqDAPS-r16 can be referred to by the same featureSetCombinationDAPS-r16. | BC | N/A | N/A | N/A |
| --- | --- | --- | --- | --- |

Or if P1 is concluded as intra-frequency DAPS is based on a pair of per-CC FSID within the same band entry, then the description in 306 is also to be corrected, i.e., in case only intra-frequency handover is supported, DAPS-related FSC is only to indicate intra-frequency DAPS based on each CC pair in the same band entry, but not support inter-frequency DAPS between each per-CC FS ID pair across different band entries.

| featureSetCombinationDAPS-r16 Indicates the feature set that the UE supports for DAPS handover on the NR band combination by FeatureSetCombinationId. A UE shall include this field if intra-freq or inter-freq DAPS handover is supported for this band combination. If the number of CCs within a band entry combination is more than two and intra-freq DAPS handover is supported, UE shall support intra-frequency DAPS handover between every CC pair within the same band entry. If the number of CCs within a band combination is more than two and inter-freq DAPS handover is supported, UE shall support inter-frequency DAPS handover between every CC pair. A feature set including intraFreqDAPS-r16 can only be referred to by featureSetCombinationDAPS-r16, not by featureSetCombination. A feature set without intraFreqDAPS-r16 is only applied to inter-freq DAPS handover if it is referred to by featureSetCombinationDAPS. Both feature sets with and without intraFreqDAPS-r16 can be referred to by the same featureSetCombinationDAPS-r16. | BC | N/A | N/A | N/A |
| --- | --- | --- | --- | --- |

Correct the definition of DAPS FSC in TS 38.306 to align with the conclusion on P1.

Furthermore, it is not super clear whether case-3 is supported. And if yes, how to derive the capability for case-3, i.e., whether a single per-CC FS ID should be used for both source/target cells, or two FS ID should be used.

RAN2 clarify whether inter-frequency DAPS HO with overlapping BW between source and target cell is supported or not. If hard to confirm in RAN2, send LS to RAN1/4 to ask. If confirmed to be supported, RAN2 confirm the source/target cell capability in this case is derived based on a pair of per-CC feature-set ID for a same band entry.

After the problem in P1-P3, the left issue is how for the network to configure the BW for source and target cells. Considering

- the legacy BCS restriction are only for 1) Intra-band contiguous CA, mapped to case-4, 2) Intra-band non-contiguous CA, mapped to case-5, and 3) Inter-band CA, mapped to case-6;

- The legacy frequency separation is only for 2) Intra-band non-contiguous CA, mapped to case-5

Seems the straightforward assumption is BCS and frequency-separation is not applicable to case-1/2/3.

RAN2 clarify for intra-frequency DAPS HO, and inter-frequency DAPS HO where the BW of source and target cells are overlapping with each other (if concluded as supported by P3), the legacy reported field of frequency-separation and BCS is NOT applicable.

While for BW-class, due to the 331 clarification below

FeatureSetDownlink field descriptions

featureSetListPerDownlinkCC Indicates which features the UE supports on the individual DL carriers of the feature set (and hence of a band entry that refer to the feature set). The UE shall hence include at least as many FeatureSetDownlinkPerCC-Id in this list as the number of carriers it supports according to the ca-BandwidthClassDL, except if indicating additional functionality by reducing the number of FeatureSetDownlinkPerCC-Id in the feature set (see NOTE 1 in FeatureSetCombination IE description). The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetDownlinkPerCC-Id in this list.

It means even for class-A, there could be more than 1 per-CC FS ID. So

- If the class is higher than A (e.g., B, C..), it is not questionable that the per-CC FS IDs in the band-entry support intra and inter-frequency DAPS, at least in the shape of intra-band continuous manner, and maybe case-3 as well.

- Or if the class is A, not sure if the per-CC FS IDs in the band-entry support intra-frequency DAPS only.

So, if the understanding above is correct, the following change is needed on 38.306, i.e., different changes depending on the conclusion of P1 above. Either

| featureSetCombinationDAPS-r16 Indicates the feature set that the UE supports for DAPS handover on the NR band combination by FeatureSetCombinationId. A UE shall include this field if intra-freq or inter-freq DAPS handover is supported for this band combination. If the number of CCs within a band combination is more than two and inter-freq DAPS handover is supported, UE shall support inter-frequency DAPS handover between every CC pair, except for the CC pair(s) in the same band entry for which bandwidth class is ‘A’. A feature set including intraFreqDAPS-r16 can only be referred to by featureSetCombinationDAPS-r16, not by featureSetCombination. A feature set without intraFreqDAPS-r16 is only applied to inter-freq DAPS handover if it is referred to by featureSetCombinationDAPS. Both feature sets with and without intraFreqDAPS-r16 can be referred to by the same featureSetCombinationDAPS-r16. | BC | N/A | N/A | N/A |
| --- | --- | --- | --- | --- |

Or

| featureSetCombinationDAPS-r16 Indicates the feature set that the UE supports for DAPS handover on the NR band combination by FeatureSetCombinationId. A UE shall include this field if intra-freq or inter-freq DAPS handover is supported for this band combination. If the number of CCs within a band entry combination is more than two and intra-freq DAPS handover is supported, UE shall support intra-frequency DAPS handover between every CC pair within the same band entry. If the number of CCs within a band combination is more than two and inter-freq DAPS handover is supported, UE shall support inter-frequency DAPS handover between every CC pair, except for the CC pair(s) in the same band entry for which bandwidth class is ‘A’. A feature set including intraFreqDAPS-r16 can only be referred to by featureSetCombinationDAPS-r16, not by featureSetCombination. A feature set without intraFreqDAPS-r16 is only applied to inter-freq DAPS handover if it is referred to by featureSetCombinationDAPS. Both feature sets with and without intraFreqDAPS-r16 can be referred to by the same featureSetCombinationDAPS-r16. | BC | N/A | N/A | N/A |
| --- | --- | --- | --- | --- |

RAN2 confirm if the reported BW-class for a band-entry is A, the per-CC feature-set IDs associated with this band entry only support intra-frequency DAPS HO, otherwise support inter-frequency DAPS HO as well (i.e., for BW-class B/C/…).

Correct the definition of DAPS FSC in TS 38.306 to align with the conclusion on P1 and P5.

Type-2 (Case-4/5/6)

For this type, clearly, the source and target cell configuration is to be derived from two per-CC FS IDs.

While, it is not super clear how to derive the BW combination configuration. On the one hand, for case-4/5/6, the legacy tools of BW-class, frequency-separation and BCS restriction is applicable by definition. On the other hand, the BW-class defined per-band-entry, frequency-separation defined per-band (including multiple band-entry), and BCS restriction defined per-BC does not differentiate between

- FSC for DAPS (limited to 2-CC cases) and

- FSC for non-DAPS case, e.g., normal CA (can include more than 2 CCs, either intra- or inter-band) in the same BC entry.

So, the question is for DAPS HO case-4/5/6, for the BW combination configuration, is the reported BW-class, frequency-separation and BCS restriction in the same BC-entry applicable for both 1) and 2).

RAN2 for inter-frequency DAPS HO cases where the BW of source and target cells are not overlapping with each other, the BW-class, frequency-separation and BCS restriction reported in the same BC-entry is applicable to both non-DAPS FSC and DAPS FSC.

Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 clarify for intra-frequency DAPS HO, the source/target cell capability is to be derived based on 1) a same per-CC feature-set ID or 2) a pair of per-CC feature-set ID.

Proposal 2	Correct the definition of DAPS FSC in TS 38.306 to align with the conclusion on P1.

Proposal 3	RAN2 clarify whether inter-frequency DAPS HO with overlapping BW between source and target cell is supported or not. If hard to confirm in RAN2, send LS to RAN1/4 to ask. If confirmed to be supported, RAN2 confirm the source/target cell capability in this case is derived based on a pair of per-CC feature-set ID for a same band entry.

Proposal 4	RAN2 clarify for intra-frequency DAPS HO, and inter-frequency DAPS HO where the BW of source and target cells are overlapping with each other (if concluded as supported by P3), the legacy reported field of frequency-separation and BCS is NOT applicable.

Proposal 5	RAN2 confirm if the reported BW-class for a band-entry is A, the per-CC feature-set IDs associated with this band entry only support intra-frequency DAPS HO, otherwise support inter-frequency DAPS HO as well (i.e., for BW-class B/C/…).

Proposal 6	Correct the definition of DAPS FSC in TS 38.306 to align with the conclusion on P1 and P5.

Proposal 7	RAN2 for inter-frequency DAPS HO cases where the BW of source and target cells are not overlapping with each other, the BW-class, frequency-separation and BCS restriction reported in the same BC-entry is applicable to both non-DAPS FSC and DAPS FSC.