3GPP TSG-RAN WG2 Meeting #116-electronic	 R2-2109440

Online, November 1st – November 12th, 2021

Source: 	vivo

Title:  	Supporting subsequent UL transmission during RA-SDT

Agenda Item:	8.6.4

Document for:	Discussion and Decision

Introduction

In past RAN2 meetings, the following agreements regarding subsequent transmission during RA-SDT procedure had been achieved [1]:

Agreements: - Send an LS to RAN1 to provide overall relevant agreements. Check if the PUCCH resources used for HARQ-ACK during subsequent SDT transmissions (applicable for both RA and CG). Ask if other L1 PHY resources may be needed for subsequent SDT transmission, for example RAN2 thinks we can use the common resources (PDCCH and PUCCH) for RA and ask if we need others. - Add that RAN2 discussed RA-SDT configuration on non-initial BWP. There was a large number of companies supporting and other companies expressed concerns on complexity and paging monitoring. - Ask RAN1 if they have any concerns from their side. - NOTE that RAN2 agreed for CG-SDT we already agreed to dedicated BWP and why we decided to support it. - No new timer (other than the SDT failure detection timer) is introduced to control the PDCCH monitoring during subsequent transmissions in RA-SDT

In this contribution, we would like to provide our understanding of subsequent UL transmission during RA-SDT procedure.

Discussion

In the previous RAN2 meeting, it was confirmed that a separate common search space (CSS) can be configured for the UEs performing RA-SDT, and the UE is allowed to monitor PDCCH on this CSS as long as the SDT failure detection timer is running. Thus, if a UL grant is received after successful contention resolution but before the expiry of SDT failure detection timer, then the UE can perform subsequent transmission. However, according to the current 38.214 specification, it is not sure how can the UE determine the spatial relation. This is because the RA-SDT UE in RRC INACTIVE state might not have a dedicated PUCCH resource. An illustration of this case is given in the following figure.

Figure 1: Subsequent UL transmission

TS 38.214 section 6.1 For PUSCH scheduled by DCI format 0_0 on a cell, the UE shall transmit PUSCH according to the spatial relation, if applicable, corresponding to the dedicated PUCCH resource with the lowest ID within the active UL BWP of the cell, as described in Clause 9.2.1 of [6, TS 38.213]. For PUSCH scheduled by DCI format 0_0 on a cell and if the higher layer parameter enableDefaultBeamPL-ForPUSCH0-0 is set 'enabled', the UE is not configured with PUCCH resources on the active UL BWP and the UE is in RRC connected mode, the UE shall transmit PUSCH according to the spatial relation, if applicable, with a reference to the RS configured with qcl-Type set to 'typeD' corresponding to the QCL assumption of the CORESET with the lowest ID on the active DL BWP of the cell. For PUSCH scheduled by DCI format 0_0 on a cell and if the higher layer parameter enableDefaultBeamPL-ForPUSCH0 is set 'enabled', the UE is configured with PUCCH resources on the active UL BWP where all the PUCCH resource(s) are not configured with any spatial relation and the UE is in RRC connected mode, the UE shall transmit PUSCH according to the spatial relation, if applicable, with a reference to the RS configured with qcl-Type set to 'typeD' corresponding to the QCL assumption of the CORESET with the lowest ID on the active DL BWP of the cell in case CORESET(s) are configured on the cell.

For our concerned case, we think this is a totally new use case for spatial relation management, which, in our memory, has never been discussed before. Therefore, it is worthy to clarify the UE behavior in such a case.

Currently, for Msg3 initial transmission and Msg3 HARQ retransmission, the spatial domain transmission filter is used up to UE implementation. In addition, for PUCCH transmission with HARQ-ACK associated with Msg4 for IDLE/INACTIVE UE, its corresponding spatial domain transmission filter is the same as that of Msg3 initial transmission.

For UE simplicity, we think it can be up to UE implementation to determine the spatial domain transmission filter for subsequent UL transmission during the RA-SDT procedure. If this is acceptable in RAN2, without a doubt, it is necessary to check this with RAN1. Thus, we propose,

Proposal: Spatial setting for a PUSCH during subsequent transmission phase in RA-SDT procedure is not defined (Check this with RAN1).

Conclusions

In this contribution, we have provided our understanding of subsequent UL transmission during RA-SDT procedure. The following proposal is made:

Proposal: Spatial setting for a PUSCH during subsequent transmission phase in RA-SDT procedure is not defined (Check this with RAN1).

- 5. Reference

[1] RAN2#115-e Session Chair Notes (Small data transmission), E-meeting, 16th August - 27th August 2021.