3GPP TSG-RAN WG2 Meeting #116e	   R2-2109687

Online, Nov 1st - Nov 12th, 2021

Source: 			ZTE Corporation, Sanechips

Title: 	Consideration on the emergency service for the SNPN

Agenda item:		8.16.4

Document for: 	Discussion and Decision

- Introduction

About the emergency service for the SNPN, in RAN2#115e meeting, the agreement as below have been achieved:

=>Introduce a new IE/field to indicate the support of IMS emergency service for SNPN. =>eCall over IMS is not supported in SNPNs in Rel-17. =>PWS can be supported in SNPNs in Rel-17.

Meanwhile, a LS was also sent to CT1 and SA2 to “confirm that the R17 SNPN-capable UEs that are not in SNPN Access Mode and R17 Non-SNPN capable UEs cannot camp on an SNPN cell supporting emergency services to obtain emergency services via any SNPN”.

In this paper, we discuss the remaining issues on the emergence service for the SNPN.

- Discussion

In this chapter, we first discuss the UE types and the corresponding allowed network types for the emergency services, then we focus on how to broadcast emergency support indication for the SNPN network.

2.1 UE and the allowed network types for the emergency services

As described in the introduction part, an LS was sent to “confirm that the R17 SNPN-capable UEs that are not in SNPN Access Mode and R17 Non-SNPN capable UEs cannot camp on an SNPN cell supporting emergency services to obtain emergency services via any SNPN”. However, there is no conclusion on whether the UE operating in the SNPN mode can access the PLMN for the emergency service. About this issue, it has specified that “If the MS needs to make an emergency call, the MS supports accessing a PLMN and there is no available SNPN supporting emergency services, the MS shall stop operating in SNPN access mode and attempt to camp on a cell of a PLMN so that emergency calls can be made”. Thus we think the UE operating in the SNPN mode can only access the SNPN for the emergency service.

Proposal 1: UE operating in the SNPN mode can only access the SNPN for the emergency service.

2.2 Emergency support indication for the SNPN network

About the emergency support indication for the SNPN network, in the last meeting, it has been agreed that “Introduce a new IE/field to indicate the support of IMS emergency service for SNPN”. However, it’s still FFS on whether this indication shall be per cell or per SNPN. About this issue, there is also a related CT1 LS[1] as below:

CT1 thanks RAN2 for providing answers to the questions on the lower layers of a UE operating in SNPN access mode in the limited service state. With respect to the following information from RAN2 in R2-2106777, CT1 would like to provide a comment. Therefore, RAN2 would like to inform CT1 that RAN2 does not know yet if AS can also indicate to NAS which SNPNs advertised by the cell support emergency services. In order for a UE, which is in the limited service state and needs to obtain emergency services, to select an SNPN which supports emergency services, if not all SNPNs sharing a cell support emergency services, the UE needs to be aware of which SNPNs support emergency services.

As described in this LS, the AS need to inform UE NAS which SNPNs support emergency services if not all SNPNs sharing a cell support emergency services, based on this requirement, obviously, the new ims-EmergencySupport field shall be defined per SNPN ID.

Proposal 2: Add a new ims-EmergencySupport field per SNPN ID.

Furthermore, in another LS [2], it also clarifies that “It is possible that, subject to regional or national regulatory requirements, an SNPN may support emergency services but not support PWS in a country where PWS is deployed. But there is no need for a UE to prioritize SNPNs supporting PWS over SNPNs not supporting PWS for SNPN selection” Based on this clarification, we can see that the PWS supported or not would have no impact to the cell selection/Reselection.

Proposal 3: PWS supported or not has no impact to the cell selection/Reselection.

- Conclusion

With the above analysis, we have the following proposals:

Proposal 1: UE operating in the SNPN mode can only access the SNPN for the emergency service.

Proposal 2: Add a new ims-EmergencySupport field per SNPN ID.

Proposal 3: PWS supported or not has no impact to the cell selection/Reselection.

- Reference

[1] C1-215046 Reply LS on limited service availability of an SNPN  Nokia To:RAN2  CC:SA1, SA2

[2] S1-213120 Reply LS on support of PWS over SNPN Huawei  To:CT1 CC: SA2, SA3, RAN2, RAN3, SA, CT, RAN