3GPP TSG-RAN WG2 Meeting #116e	   R2-2109689

Online, Nov 1st - 12th, 2021

Source: 			ZTE Corporation, Sanechips

Title: 	Consideration on the remaining issues of switching procedure

Agenda item:		8.3.3

Document for: 	Discussion and Decision

- Introduction

About the network switching procedure, many agreements have been made based on [1]. In this paper we focus on the remaining issues of the switching procedure.

- Discussion

    - 2.1 Preferred RRC-State

FFS reuse preferredRRC-State or some changes on preferredRRC-State for Multi-SIM purpose.

About this FFS, there are two issues need to be clarified:

- Which kind of states (e.g. RRC_IDLE, RRC_INACTIVE, outOfConnected) shall be included for the MUSIM feature?

- Can the legacy preferredRRC-State be reused for the MUSIM?

For the first question, we think one advantage of the AS based solution is that the UE can indicate it prefers to leave with Inactive state, thus, at least the “RRC_IDLE, RRC_INACTIVE” shall be included.

Proposal 1: The preferredRRC-State for the MUSIM shall include “RRC_IDLE, RRC_INACTIVE” at least.

For the second question, from the network side, it would be helpful to know the leaving reason (e.g. MUSIM or power saving) for the KPI statistics, thus maybe a new preferredRRC-State-R17 can be introduced for MUSIM. If the legacy preferredRRC-State was reused for the MUSIM, at least a leaving cause shall be added to indicate that it’s for the MUSIM.

Proposal 2: The network shall be able to know the leaving is for the MUSIM, which can be indicated either by adding a new preferred state element (e.g. preferredRRC-State-R17) for the MUSIM or by adding a leaving cause for the legacy preferredRRC-State.

    - 2.2 Configured Timer

Introduce a new RRC timer for the “configured time”, used for the UE to leave RRC_CONNECTED without a response. FFS if it's possible to configure UE to always wait for the network response (e.g. "infinite" waiting time)

About this FFS issue, if the “infinite” waiting time is allowed, the UE has to wait for the network response. For the abnormal case that the UE failed to detect the network’s reconfiguration message, the UE may be unable to leave the network A. In most cases, the UE determine to leave connected state with network A when the network B has a higher priority service. Then if the infinite waiting time is configured, it may affects the performance of the UE with network B.

Proposal 3: It’s not allowed to set the “configured time” to infinite explicitly or implicitly.

    - 2.3 Early return and other abnormal cases

With the scheduling Gap, the UE may finish the communication with the network B before or after the scheduling gap. For the case that the UE finish the communication with network B before the scheduling gap, the UE may restore the data transmission as soon as possible with network A if early return was supported. Another use case is that the UE upper layer may trigger a higher priority service during the scheduled gap. Thus early return can be supported to improve the UE’s performance.

Proposal 4: Support early return feature for the switching without leaving connected state case.

For the case that the UE finish the communication with network B before the scheduling gap, to keep align with the network side, the UE shall abort the on-going procedure of the network B and back to the network A.

Proposal 5: If the communication with the network B can’t be finished before the gap, the UE shall abort the on-going procedure of the network B and back to the network A.

Furthermore, for the switching without leaving connected state, the UE shall try to finish the procedure on the network B as soon as possible, thus the network B shall know that the UE was at leaving state of the other USIM, then the network B shall not trigger the mobility (e.g. handover, redirection), measurement and DC related procedure.

Proposal 6: The UE shall inform the network B that it was at switching procedure on the other network, then the network B can avoid to trigger the mobility (e.g. handover, redirection), measurement and DC related procedures.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: The preferredRRC-State for the MUSIM shall include “RRC_IDLE, RRC_INACTIVE” at least.

Proposal 2: The network shall be able to know the leaving is for the MUSIM, which can be indicated either by adding a new preferred state element (e.g. preferredRRC-State-R17) for the MUSIM or by adding a leaving cause for the legacy preferredRRC-State.

Proposal 3: It’s not allowed to set the “configured time” to infinite explicitly or implicitly.

Proposal 4: Support early return feature for the switching without leaving connected state case.

Proposal 5: If the communication with the network B can’t be finished before the gap, the UE shall abort the on-going procedure of the network B and back to the network A.

Proposal 6: The UE shall inform the network B that it was at switching procedure on the other network, then the network B can avoid to trigger the mobility (e.g. handover, redirection), measurement and DC related procedures.

- Reference

- R2-2107857	Summary of Switching message details	vivo	discussion