3GPP TSG-RAN WG2 Meeting #116 electronic	           R2-2109763

Nov 1st – Nov 12th, 2021

Agenda item:	8.7.2.1

Source:	China Telecommunication

Title:	Discussion on system information delivery open issues

Document for:	Discussion

- Introduction

RAN2#115-e meeting has the following agreements about system information delivery:

For any SIB that the remote UE requests in on-demand manner, the relay UE can forward the response (i.e. the relay UE does not filter). FFS which SIBs the remote UE could request. FFS whether relay UE can voluntarily forward the SIBs/posSIBs to remote UE without a request. Short message forwarding via introducing a short message field in SCI is not supported. FFS if short message can be indicated by PC5-RRC.

In this paper, we would like to discuss about the remaining open issues as the following.

- Which SIBs the remote UE could request. - Whether relay UE can voluntarily forward the SIBs/posSIBs to remote UE without a request. - Whether/how to define SI acquisition by remote UE before PC5 connection established. - For L2 U2N relay, direct reception of SI via Uu is supported for in-coverage Remote UE.

- Discussion

Which SIBs the remote UE could request.

Remote UE should be able to request any SIB in on-demand manner if it need. And no additional complexity is introduced compared with only one SIB allowed to be requested by remote UE. It is not intended that SIBs are mandatory to be forwarded.

Proposal 1: the remote UE could request any SIB.

Whether relay UE can voluntarily forward the SIBs/posSIBs to remote UE without a request.

Regarding the overall SI forwarding design, it’s better to support SIB forwarding by relay UE voluntarily, similar to system information over Uu. Relay UE can voluntarily forward the SIBs/posSIBs to remote UE without a request in case of that it has got the remote UE’s preference via a previous request for corresponding SIBs/posSIBs. The relay UE can decide whether to broadcast or unicast or groupcast the SIBs/posSIBs based on the collection of remote UEs’ preference for SIB/posSIBs reception. When many remote UEs request the same set of SIBs, it may be more efficient for relay UE to broadcast/groupcast than unicast the requested SIBs to remote UEs one by one.

Proposal 2: relay UE can voluntarily forward the SIBs/posSIBs to remote UE without a request.

Whether/how to define SI acquisition by remote UE before PC5 connection established.

Remote UE need get necessary SIB info related to RRC establishment (initialization) before PC5 connection established. It is possible that remote UE selects a relay UE and finds that it can’t access the relay UE’s serving cell after PC5 connection established, which is inefficient. It is useful especially for out of coverage remote UE.

Therefore, necessary parameters from system information for relay UE selection and efficient initialization of RRC establishment should be allowed to be forwarded to remote UE, such as cell-barring information, TAC and RANAC. The benefit of forwarding them before PC5 connection established is helping remote UE to figure out whether it can access the relay UE’s serving cell, the usage is same as system information over Uu. From this respective, it’s better to convey them by PC5 RRC message. Furthermore, remote UE can use these information to decide whether to camp a relay UE. From this respective, it’s also logical to convey them in discovery message. Besides, both the two ways can be used to reach remote UE before PC5 connection established. Thus we think these information can be conveyed in discovery message or broadcast PC5 RRC message. We slightly prefer to broadcast PC5 RRC message.

Proposal 3: necessary parameters from system information can be forwarded by relay UE to remote UE before PC5 connection established. These information can be conveyed in discovery message or broadcast PC5 RRC message.

For L2 U2N relay, direct reception of SI via Uu is supported for in-coverage Remote UE.

In LTE D2D, the in-coverage remote UE is allowed to obtain system information via Uu. It is better to have the flexibility to allow the in-coverage remote UE to obtain system information via Uu or relay UE. In addition, we also think that which MIB/SIBs the in-coverage remote UE can obtain via Uu directly should not be restricted.

Proposal 4: direct reception of system information via Uu is supported for the in-coverage Remote UE.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: the remote UE could request any SIB.

Proposal 2: relay UE can voluntarily forward the SIBs/posSIBs to remote UE without a request.

Proposal 3: necessary parameters from system information can be forwarded by relay UE to remote UE before PC5 connection established. These information can be conveyed in discovery message or broadcast PC5 RRC message.

Proposal 4: direct reception of system information via Uu is supported for the in-coverage Remote UE.

4	References

RAN2-115-e-Positioning-Relay-2021-08-27-0330.

R2-2107756, Summary of [Post114-e][605][Relay] SI and paging forwarding, RAN2#115e, vivo.