3GPP TSG-RAN WG2 Meeting #116e                                    R2-2109897

Online, 1st – 12th November, 2021

Source: 			ZTE Corporation, Sanechips

Title: 	Identification, access and camping restrictions for RedCap UE

Agenda item:		8.12.3.1

Document for: 	Discussion and Decision

- Introduction

In last RAN2 meeting, following agreements are reached regarding early identification and access restrictions [1]:

Agreements:

- Msg1 identification which can be configured to be enabled/disabled can be specified from RAN2 point of view.

- Solution for early identification for 2-step RACH will be specified.

- Specify separate indications in SIB1 for barring RedCap UEs with 1 Rx chain and 2 Rx chains.

- Specify a RedCap specific IFRI in SIB1.

Agreements via email - from offline 104:

- IFRI for RedCap UEs in SIB1 is common for UEs with 1 Rx or 2 Rx branches.

- If RedCap-specific IFRI is absent from broadcast SI, the UE considers the cell does not support RedCap.

Agreements online:

- A Msg3 early identification based on dedicated LCID is supported (if SA3 confirms there is no problem)

Agreements:

- RedCap UE applies the existing cellBarred field in MIB

This contribution provides further consideration on remaining issues on early identification, access and camping restrictions.

- Discussion

  - Early identification

Per RAN1 agreement[2], separate PRACH resource and PRACH preamble partitioning can be used for early indication both for shared initial UL BWP and separate initial UL BWP (if configured). The details on how to configure separate PRACH and PRACH partitioning is still unclear. In addition, RAN1 concluded that it is up to RAN2 to decide whether there is RA-RNTI overlapping issue and how to address RA-RNTI overlapping issue in early identification in Msg1.

In RAN2, a separate RACH identification and partitioning session will discuss a common PRACH preamble partition solution for various working items, including RedCap, coverage enhancement and small data transmission, etc. We can wait for the conclusion of the joint session before discuss corresponding specification impact in RedCap session.

Observation 1: The RACH resource configuration and selection for RedCap and/or combination of RedCap with other features (e.g. CE, slicing) will be discussed in RACH partitioning session.

Proposal 1: Postpone the discussion on RACH configuration and selection of Msg1 based early identification in RedCap session, and wait for the conclusion from RACH partitioning session.

A remaining issue related to early identification is how to enable/disable early identification in Msg1 and Msg3. According to WID[3], early identification in Msg1/Msg3 should be configurable:

Specify functionality that will enable RedCap UEs to be explicitly identifiable to networks through an early indication in Msg1 and/or Msg3, and Msg A if supported, including the ability for the early indication to be configurable by the network. [RAN2, RAN1]

According to last RAN2 agreements, Msg1 identification which can be configured to be enabled/disabled can be specified from RAN2 point of view. Per our understanding, since separate PRACH resource or PRACH preamble partitioning will be configured to enable Msg1 identification, the presence/absence of configuration itself can be used as the indication of function enable/disable. In other words, if separate PRACH resource or separate PRACH preamble partition is configured, RedCap UE should apply these RedCap specific PRACH configuration.

Proposal 2: Presence/absence of the PRACH configuration for Msg1 based early identification is regarded as enabled/disabled indication, i.e. RedCap UE should apply RedCap specific PRACH configuration if it is configured.

As to early identification in Msg3, the adopted solution is almost for free in respect of signaling overhead. it seems unnecessary to introduce additional indicator to enable/disable this function. On the other side, we are not sure whether there is any concern on exposing UE type during initial access. Thus we suggest RAN2 to discuss whether explicit indicator in system information is needed to enable/disable this function, or Msg3 identification is always applied when Msg1 identification is not enabled.

Proposal 3: RAN2 to decide whether early identification in Msg3 can be enabled/disabled by network, or it is always applied when early identification in Msg1 is not enabled.

  - Access and camping restriction

As agreed in last RAN2 meeting, if RedCap specific IFRI is absent from system information, the UE considers the cell does not support RedCap.

Agreements via email - from offline 104:

- IFRI for RedCap UEs in SIB1 is common for UEs with 1 Rx or 2 Rx branches.

- If RedCap-specific IFRI is absent from broadcast SI, the UE considers the cell does not support RedCap.

However, since RedCap specific IFRI is absent, it is unclear whether the IFRI in MIB should be applied by RedCap in this case. This issue is raised in RRC running CR discussion. We think there are two options:

Option 1: RedCap UE regards the whole frequency are barred / not support RedCap, i.e. IFRI in MIB is not applied.

Option 2: RedCap UE applies the IFRI in MIB, assuming there may be intra-frequency cell supporting RedCap.

With option 1, if there is a cell does not support RedCap (cannot broadcast RedCap specific IFRI), RedCap UE will consider all the neighbour cells on this frequency are not supporting RedCap. This may prevent network from upgrading gradually and force operator to upgrade gNBs to support RedCap at the same time. But the problem may not be so serious if RedCap is deployed for a specific area (e.g. industrial sensors).

Option 2 seems more flexible compared with option 1. But the risk of option 2 is that if the whole frequency does not support RedCap, and IFRI in MIB is set to ‘allowed’ (usual case), then RedCap UE will search neibhour cells again and again, which wastes its power.

So between Option 1 and Option 2, we slightly prefer Option 1 for saving UE’s power.

Proposal 4: If RedCap specific IFRI is absent in SIB1, the RedCap will not perform intra-frequency reselection on the frequency (e.g. IFRI in MIB is not applied).

Considering there are frequency/cell are not accessible for RedCap UE due to network capability or RRM policy, it is beneficial to indicate whether a neighbor frequency/cell support RedCap UE to camp. With this indication, RedCap UE can avoid unnecessary RRM measurement and overhead for camping attempts on the neighbor frequency/cell which does not support RedCap.

Some companies argues there is no indication whether neighbor cell is barred. However, we think it will be usual that a whole frequency does not support RedCap access in real deployment. This is different to bar a single cell temporary.

And we don’t think the support of RedCap access is expected to change frequently as some companies concerned. Even if a neighbor cell may change RedCap cell barring dynamically according to cell load, it is unnecessary to be reflected in this indication, i.e. the indication does not indicate whether a neighbor freq/cell is barred for RedCap.

Proposal 5: For cell reselection, network can indicate in system information whether a neighbour frequency/cell supports RedCap access.

In some deployment, it is beneficial to have RedCap UE and non-RedCap UE to camp on different frequencies. For example, in connected industrial and video Surveillance use case, such separation can avoid interference from each other. And due to poor coverage of RedCap, it is necessary to direct RedCap UEs e.g. towards lower frequencies for better coverage. To achieve this, we prefer that NW can configure different cell reselection priority for neighbor frequency/cell.

Proposal 6: For a neighbor frequency/cell, network can configure different cell reselection priorities for RedCap UEs and non-RedCap UEs.

- Conclusion

Observation 1: The RACH resource configuration and selection for RedCap and/or combination of RedCap with other features (e.g. CE, slicing) will be discussed in RACH partitioning session.

Proposal 1: Postpone the discussion on RACH configuration and selection of Msg1 based early identification in RedCap session, and wait for the conclusion from RACH partitioning session.

Proposal 2: Presence/absence of the PRACH configuration for Msg1 based early identification is regarded as enabled/disabled indication, i.e. RedCap UE should apply RedCap specific PRACH configuration if it is configured.

Proposal 3: RAN2 to decide whether early identification in Msg3 can be enabled/disabled by network, or it is always applied when early identification in Msg1 is not enabled.

Proposal 4: If RedCap specific IFRI is absent in SIB1, the RedCap will not perform intra-frequency reselection on the frequency (e.g. IFRI in MIB is not applied).

Proposal 5: For cell reselection, network can indicate in system information whether a neighbour frequency/cell supports RedCap access.

Proposal 6: For a neighbor frequency/cell, network can configure different priorities for RedCap UEs and non-RedCap UEs.

- Reference

- RAN2#115e chairman notes

- R2-2109325, LS on RAN1 agreements on RAN2-led features for RedCap

- RP-210918, Revised WID on support of reduced capability NR devices