3GPP TSG-RAN WG2 Meeting #116 electronic	R2-2110241

Electronic Meeting, Nov 1 – 12, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | draft | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solving the issue for random access on multiCarrier in NB-IoT |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh3 |  |  |  |  |  |  | Date: |  |  | 2021-11-1 |  |
| Category: | F |  |  |  |  |  |  | Release: |  |  | Rel-16 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |  |
| Reason for change: |  |  | The anchor carriers are deployed with inter frequency to reduce interference among cells with the same frequency in real NB-IoT network. If multiple UEs initiate service request simultaneously and the utilization rate of the anchor carrier is quite high, such as 70%, the non-anchor carriers should be deployed to solve the concurrent access problem with intra frequency between other cells and the downlink narrowband reference-signal EPRE of the non-anchor carriers is generally lower relative to the downlink narrowband reference-signal EPRE of the anchor carrier to reduce the intervene between other cells using the same frequency. There is the overlapping area that the coverage level of the non-anchor carrier is worse than the coverage level of the anchor carrier and the UEs may fail to access the non-anchor carrier or try more times to access the non-anchor carrier with the nprach resource (such as number of nprach repetitions) based on the current coverage level and select the carrier with worse coverage level by the current selection probability for anchor nprach resource in the overlapping area. In the current specification, random access of the overlapping area has not been defined. This contribution mainly discusses on the random access issues on the multi-carrier in NB-IoT. |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  | - Modify the 6.7.3.2 to add the RSRP Threshold of the non-anchor carriers in the IE NPRACH-ConfigSIB-NB - Modify the 6.7.3.1 to add the selection probability for the anchor carrier NPRACH resource when the coverage level on the anchor carrier is different from that on the non-anchor carrier. Impact analysis Impacted functionality: Random Access Inter-operability: - The coverage level on the non-anchor carriers can be determined by the RSRP Threshold of the non-anchor - The UEs can selet the carrier by the new selection probability when the coverage level is different between the anchor carrier and the non-anchor carriers. |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  | UEs may fail to access the non-anchor carrier or try more times to access the non-anchor carrier and select the carrier with worse coverage level by the current selection probability for anchor nprach resource when the UEs camp on the non-anchor carrier and the coverage level of the non-anchor carrier is worse than the anchor carrier. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  | 6.7.3.1, 6.7.3.2 |  |  |  |  |  |  |  |  |  |
|  |  |  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |

First Change

#### 6.7.3.1 NB-IoT System information blocks

/*Partially omitted*/

#### –	SystemInformationBlockType22-NB

The IE SystemInformationBlockType22-NB contains radio resource configuration for paging and random access procedure on non-anchor carriers.

SystemInformationBlockType22-NB information element

-- ASN1START

SystemInformationBlockType22-NB-r14 ::=	SEQUENCE {

dl-ConfigList-r14					DL-ConfigCommonList-NB-r14	OPTIONAL,	-- Need OR

ul-ConfigList-r14					UL-ConfigCommonList-NB-r14	OPTIONAL,	-- Need OR

pagingWeightAnchor-r14				PagingWeight-NB-r14			OPTIONAL,	-- Cond pcch-config

nprach-ProbabilityAnchorList-r14	NPRACH-ProbabilityAnchorList-NB-r14	OPTIONAL,	-- Cond nprach-config

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

...,

[[	mixedOperationModeConfig-r15	SEQUENCE {

dl-ConfigListMixed-r15			DL-ConfigCommonList-NB-r14	OPTIONAL,	-- Cond dl-ConfigList

ul-ConfigListMixed-r15			UL-ConfigCommonList-NB-r14	OPTIONAL,	-- Cond ul-ConfigList

pagingDistribution-r15			ENUMERATED {true}			OPTIONAL,	-- Need OR

nprach-Distribution-r15			ENUMERATED {true}			OPTIONAL	-- Need OR

}																OPTIONAL,	-- Need OR

ul-ConfigList-r15				UL-ConfigCommonListTDD-NB-r15	OPTIONAL	-- Cond TDD

]],

[[

nprach-ProbabilityAnchorDiffList-r16   NPRACH-ProbabilityAnchorDiffList-NB-r16	OPTIONAL -- Cond nprach-config

]]

}

DL-ConfigCommonList-NB-r14 ::=		SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF

DL-ConfigCommon-NB-r14

UL-ConfigCommonList-NB-r14 ::=		SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF

UL-ConfigCommon-NB-r14

UL-ConfigCommonListTDD-NB-r15 ::=	SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF

UL-ConfigCommonTDD-NB-r15

DL-ConfigCommon-NB-r14 ::=			SEQUENCE {

dl-CarrierConfig-r14				DL-CarrierConfigCommon-NB-r14,

pcch-Config-r14					PCCH-Config-NB-r14			OPTIONAL, -- Need OR

...,

[[	wus-Config-r15					WUS-ConfigPerCarrier-NB-r15		OPTIONAL	-- Cond WUS

]],

[[	gwus-Config-r16					WUS-ConfigPerCarrier-NB-r15		OPTIONAL	-- Cond GWUS

]]

}

PCCH-Config-NB-r14 ::=				SEQUENCE {

npdcch-NumRepetitionPaging-r14		ENUMERATED {

r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1} OPTIONAL, -- Need OP

pagingWeight-r14						PagingWeight-NB-r14	DEFAULT w1,

...

}

PagingWeight-NB-r14	::=			ENUMERATED {w1, w2, w3, w4, w5, w6, w7, w8,

w9, w10, w11, w12, w13, w14, w15, w16}

UL-ConfigCommon-NB-r14 ::=			SEQUENCE {

ul-CarrierFreq-r14					CarrierFreq-NB-r13,

nprach-ParametersList-r14			NPRACH-ParametersList-NB-r14	OPTIONAL, -- Need OR

...,

[[	nprach-ParametersListEDT-r15	NPRACH-ParametersList-NB-r14	OPTIONAL -- Cond EDT

]]

}

UL-ConfigCommonTDD-NB-r15 ::=		SEQUENCE {

tdd-UL-DL-AlignmentOffset-r15		TDD-UL-DL-AlignmentOffset-NB-r15,

nprach-ParametersListTDD-r15		NPRACH-ParametersListTDD-NB-r15	OPTIONAL, -- Need OR

...

}

NPRACH-ProbabilityAnchorList-NB-r14 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF

NPRACH-ProbabilityAnchor-NB-r14

NPRACH-ProbabilityAnchorDiffList-NB-r16 ::=

SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF

NPRACH-ProbabilityAnchorDiff-NB-r16

NPRACH-ProbabilityAnchor-NB-r14 ::=		SEQUENCE {

nprach-ProbabilityAnchor-r14			ENUMERATED {

zero, oneSixteenth, oneFifteenth, oneFourteenth,

oneThirteenth, oneTwelfth, oneEleventh, oneTenth,

oneNinth, oneEighth, oneSeventh, oneSixth,

oneFifth, oneFourth, oneThird, oneHalf}

OPTIONAL	-- Need OP

}

NPRACH-ProbabilityAnchorDiff-NB-r16::=		SEQUENCE {

nprach-ProbabilityAnchorDiff-r16			ENUMERATED{

zero, oneSixteenth, oneFifteenth, oneFourteenth,

oneThirteenth, oneTwelfth, oneEleventh, oneTenth,

oneNinth, oneEighth, oneSeventh, oneSixth,

oneFifth, oneFourth, oneThird, oneHalf}

OPTIONAL	-- Need OP

-- ASN1STOP

SystemInformationBlockType22-NB field descriptions

dl-CarrierConfig For FDD: Provides the configuration of the DL non-anchor carrier. For TDD: Provides the configuration of the non-anchor carrier.

dl-ConfigList, dl-ConfigListMixed For FDD: List of DL non-anchor carriers and associated configuration that can be used for paging and/or random access. E-UTRAN configures DL non-anchor carriers operating in mixed operation mode only in dl-ConfigListMixed and only a UE that supports mixed operation mode uses the carriers in dl-ConfigListMixed. A given carrier is either signalled in the dl-ConfigList or in dl-ConfigListMixed. If dl-ConfigListMixed is present and at least one of the carriers in dl-ConfigListMixed is configured for paging: - If pagingDistribution is present, the UE supporting mixed operation mode creates a combined list of DL carriers for paging by appending dl-ConfigListMixed to the dl-ConfigList while maintaining the order among dl-ConfigList and dl-ConfigListMixed; the total number of signalled DL non-anchor carriers cannot be more than maxNonAnchorCarriers-NB-r14. - If pagingDistribution is absent, the UE supporting mixed operation mode uses the list of DL carriers for paging provided in dl-ConfigListMixed and considers pagingWeightAnchor being set to w0, i.e. the anchor carrier is not used. Otherwise, the pagingDistribution field is not applicable and the UE shall ignore the value. For TDD: List of non-anchor carriers and associated configuration that can be used for paging and/or random access.

gwus-Config For FDD: Carrier specific GWUS Configuration. If both gwus-Config and wus-Config are present for the carrier, E-UTRAN configures the same value for both fields.

mixedOperationModeConfig For FDD: Provides the configuration of DL and UL non-anchor carriers that can be used for paging and random access by a UE that supports mixed operation mode. For TDD: This parameter is absent.

npdcch-NumRepetitionPaging Maximum number of repetitions for NPDCCH common search space (CSS) for paging, see TS 36.213 [23], clause 16.6. If the field is absent, the value of npdcch-NumRepetitionPaging configured in SystemInformationBlockType2-NB in IE pcch-Config applies.

nprach-Distribution Indicates which UL carriers a UE supporting mixed operation mode uses for random access as defined in description of ul-ConfigList, ul-ConfigListMixed.

nprach-ParametersList, nprach-ParametersList-EDT Configure NPRACH parameters for each NPRACH resource on one non-anchor UL carrier. Up to three NPRACH resources can be configured on one non-anchor UL carrier. Each NPRACH resource is associated with a different number of NPRACH repetitions. NPRACH resources in nprach-ParametersListEDT are used to initiate EDT. Each NPRACH resource is associated with a maximum TBS signalled in the corresponding entry of edt-TBS-InfoList in SystemInformationBlockType2-NB. E-UTRAN includes the same number of entries, and listed in the same order, as in nprach-ParametersList in SystemInformationBlockType2-NB.

nprach-ParametersListTDD For TDD: Configure NPRACH parameters for each NPRACH resource on one non-anchor UL carrier. Up to three NPRACH resources can be configured on one non-anchor UL carrier. Each NPRACH resource is associated with a different number of NPRACH repetitions. E-UTRAN includes the same number of entries in nprach-ParametersListTDD, and listed in the same order, as in nprach-ParametersListTDD in SystemInformationBlockType2-NB..

nprach-ProbabilityAnchor Configure the selection probability for the anchor carrier NPRACH resource, see TS 36.321 [6]. Value zero corresponds to a probability of 0, oneSixteenth corresponds to the probability of 1/16, oneFifteenth corresponds to the probability of 1/15, and so on. If the field is absent, the selection probability of the anchor carrier NPRACH resource is 1. All non-anchor carriers NPRACH resources have equal probability between them. If there is no NPRACH resource defined on the anchor carrier for one repetition level in nprach-ParametersList-EDT, (respectively nprach-ParametersListFmt2, nprach-ParametersListFmt2-EDT), the UE shall use the value 'zero' and ignore the signalled value of nprach-ProbabilityAnchor for this repetition level for the NPRACH resources defined by nprach-ParametersList-EDT (respectively nprach-ParametersListFmt2, nprach-ParametersListFmt2-EDT).

NPRACH-ProbabilityAnchorDiff Configure the selection probability for the anchor carrier NPRACH resource when the Coverage level on the anchor carrier is different from that on the non-anchor carrier, see TS 36.321 [6]. Value zero corresponds to a probability of 0, oneSixteenth corresponds to the probability of 1/16, oneFifteenth corresponds to the probability of 1/15, and so on. If the field is absent, the selection probability of the anchor carrier NPRACH resource is 1.

nprach-ProbabilityAnchorList Configures the selection probability for each NPRACH resource on the anchor carrier. E-UTRAN includes the same number of entries, and listed in the same order, as in nprach-ParametersList in SystemInformationBlockType2-NB.

pagingDistribution Indicates which DL carriers a UE supporting mixed operation mode monitors for paging as defined in description of dl-ConfigList, dl-ConfigListMixed.

pagingWeight Weight of the non-anchor paging carrier for uneven paging load distribution across the carriers. Value w1 corresponds to a relative weight of 1, w2 corresponds to a relative weight of 2, and so on. The paging load for a carrier 'i' is equal to w(i)/W where i is equal to 0 for the anchor carrier and equal to the index of the carrier in the dl-ConfigList / dl-ConfigListMixed for a non-anchor carrier, W is the sum of the weights of all paging carriers. To avoid correlation between paging carrier and paging occasion, the weights should be assigned such that: nB * W <= 16384.

pagingWeightAnchor Weight of the anchor carrier for uneven paging load distribution across the carriers. Value w1 corresponds to a relative weight of 1, w2 corresponds to a relative weight of 2, and so on. If the field is absent, the (default) value of w0 is applied, i.e. the anchor carrier is not used for paging.

pcch-Config Configure the PCCH parameters for the non-anchor DL carrier.

tdd-UL-DL-AlignmentOffset Indicates the offset between the UL carrier frequency center with respect to DL carrier frequency center for the non-anchor carrier.

ul-CarrierFreq For FDD: UL carrier frequency of the non-anchor carrier as defined in TS 36.101 [42], clause 5.7.3F. For TDD: This field is absent and the uplink carrier frequency is same as the downlink frequency.

ul-ConfigList, ul-ConfigListMixed For FDD: List of UL non-anchor carriers and associated configuration that can be used for random access. E-UTRAN configures UL non-anchor carriers operating in mixed operation mode only in ul-ConfigListMixed and only a UE that supports mixed operation mode uses the carriers in ul-ConfigListMixed. A given carrier is either signalled in the ul-ConfigList or in ul-ConfigListMixed. If ul-ConfigListMixed is present and at least one of the carriers in ul-ConfigListMixed is configured for random access: - If nprach-Distribution is present, the UE supporting mixed operation mode creates a combined list of UL carriers for random access by appending ul-ConfigListMixed to the ul-ConfigList while maintaining the order among both ul-ConfigList and ul-ConfigListMixed; the total number of signalled UL non-anchor carriers cannot be more than maxNonAnchorCarriers-NB-r14. - If nprach-Distribution is absent, the UE supporting mixed operation mode uses the list of UL carriers for random access provided in ul-ConfigListMixed and considers nprach-ProbabiliyAnchor being set to zero for each NPRACH resource, i.e. the anchor carrier is not used for random access. Otherwise, the nprach-Distribution field is not applicable and the UE shall ignore the value. For TDD: E-UTRAN configures ul-ConfigList-r15 and includes the same number of entries as in dl-ConfigList. The UL carrier frequency of the non-anchor carrier is same as the DL carrier frequency.

wus-Config For FDD: Carrier specific WUS Configuration.

| Conditional presence | Explanation |
| --- | --- |
| dl-ConfigList | This field is optionally present, Need OR, if the field dl-ConfigList is present. Otherwise the field is not present. |
| EDT | The field is optionally present, Need OR, if edt-Parameters in SystemInformationBlockType2-NB is present; otherwise the field is not present and the UE shall delete any existing value for this field. |
| GWUS | This field is optionally present, Need OR, if gwus-Config-r16 is present in SystemInformationBlockType2-NB. Otherwise the field is not present. |
| pcch-config | This field is optionally present, Need OP, if the field dl-ConfigList is present and at least one of the carriers in dl-ConfigList is configured for paging. Otherwise the field is not present and only the anchor carrier is used for paging. |
| nprach-config | This field is mandatory present, if the field ul-ConfigList is present and at least one of the carriers in ul-ConfigList is configured for random access. Otherwise the field is not present and only the anchor carrier is used for random access. |
| TDD | This field is optionally present, Need OR, for TDD. Otherwise the field is not present. |
| ul-ConfigList | This field is optionally present, Need OR, if the field ul-ConfigList is present. Otherwise the field is not present. |
| WUS | This field is mandatory present, if the field wus-Config is present in SystemInformationBlockType2-NB. Otherwise the field is not present, Need OR. |

Second change

6.7.3.2 NB-IoT Radio resource control information elements

/*Partially omitted*/

–	NPRACH-ConfigSIB-NB

The IE NPRACH-ConfigSIB-NB is used to specify the NPRACH configuration for the anchor and non-anchor carriers.

NPRACH-ConfigSIB-NB information elements

-- ASN1START

NPRACH-ConfigSIB-NB-r13 ::=			SEQUENCE {

nprach-CP-Length-r13				ENUMERATED {us66dot7, us266dot7},

rsrp-ThresholdsPrachInfoList-r13	RSRP-ThresholdsNPRACH-InfoList-NB-r13	OPTIONAL,	-- Need OR

nprach-ParametersList-r13		NPRACH-ParametersList-NB-r13

}

NPRACH-ConfigSIB-NB-v1330 ::=		SEQUENCE {

nprach-ParametersList-v1330			NPRACH-ParametersList-NB-v1330

}

NPRACH-ConfigSIB-NB-v1450 ::=		SEQUENCE {

maxNumPreambleAttemptCE-r14			ENUMERATED {n3, n4, n5, n6, n7, n8, n10, spare1}

}

NPRACH-ConfigSIB-NB-v1530 ::=		SEQUENCE {

tdd-Parameters-r15					SEQUENCE {

nprach-PreambleFormat-r15			ENUMERATED {

fmt0, fmt1, fmt2, fmt0-a, fmt1-a},

dummy								ENUMERATED {

n1, n2, n4, n8, n16, n32, n64, n128,

n256, n512, n1024},

nprach-ParametersListTDD-r15		NPRACH-ParametersListTDD-NB-r15

}	OPTIONAL,		-- Cond TDD

fmt2-Parameters-r15					SEQUENCE {

nprach-ParametersListFmt2-r15		NPRACH-ParametersListFmt2-NB-r15 OPTIONAL,	-- Need OR

nprach-ParametersListFmt2EDT-r15	NPRACH-ParametersListFmt2-NB-r15 OPTIONAL	-- Cond EDT2

}	OPTIONAL,		-- Need OR

edt-Parameters-r15					SEQUENCE {

edt-SmallTBS-Subset-r15				ENUMERATED {true}				OPTIONAL,	-- Need OR

edt-TBS-InfoList-r15				EDT-TBS-InfoList-NB-r15,

nprach-ParametersListEDT-r15		NPRACH-ParametersList-NB-r14	OPTIONAL	-- Need OR

}	OPTIONAL		-- Cond EDT1

}

NPRACH-ConfigSIB-NB-v1550 ::=		SEQUENCE {

tdd-Parameters-v1550				SEQUENCE {

nprach-ParametersListTDD-v1550		NPRACH-ParametersListTDD-NB-v1550

}

}

NPRACH-ConfigSIB-NB-v16xy ::=       SEQUENCE {

rsrp-ThresholdsPrachNonAnchorInfoList-r16	RSRP-ThresholdsNPRACH-NonAnchorInfoList-NB-r16	OPTIONAL	-- Need OR

}

NPRACH-ParametersList-NB-r13 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-r13

NPRACH-ParametersList-NB-v1330 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-v1330

NPRACH-Parameters-NB-r13::=			SEQUENCE {

nprach-Periodicity-r13					ENUMERATED {ms40, ms80, ms160, ms240,

ms320, ms640, ms1280, ms2560},

nprach-StartTime-r13					ENUMERATED {ms8, ms16, ms32, ms64,

ms128, ms256, ms512, ms1024},

nprach-SubcarrierOffset-r13				ENUMERATED {n0, n12, n24, n36, n2, n18, n34, spare1},

nprach-NumSubcarriers-r13				ENUMERATED {n12, n24, n36, n48},

nprach-SubcarrierMSG3-RangeStart-r13	ENUMERATED {zero, oneThird, twoThird, one},

maxNumPreambleAttemptCE-r13				ENUMERATED {n3, n4, n5, n6, n7, n8, n10, spare1},

numRepetitionsPerPreambleAttempt-r13	ENUMERATED {n1, n2, n4, n8, n16, n32, n64, n128},

npdcch-NumRepetitions-RA-r13			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1},

npdcch-StartSF-CSS-RA-r13				ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64},

npdcch-Offset-RA-r13					ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

}

NPRACH-Parameters-NB-v1330 ::=		SEQUENCE {

nprach-NumCBRA-StartSubcarriers-r13		ENUMERATED {n8, n10, n11, n12, n20, n22, n23, n24,

n32, n34, n35, n36, n40, n44, n46, n48}

}

NPRACH-ParametersList-NB-r14 ::=		SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF

NPRACH-Parameters-NB-r14

NPRACH-Parameters-NB-r14 ::=			SEQUENCE {

nprach-Parameters-r14					SEQUENCE {

nprach-Periodicity-r14					ENUMERATED {ms40, ms80, ms160, ms240,

ms320, ms640, ms1280, ms2560}

OPTIONAL,	-- NEED OP

nprach-StartTime-r14					ENUMERATED {ms8, ms16, ms32, ms64,

ms128, ms256, ms512, ms1024}

OPTIONAL,	-- NEED OP

nprach-SubcarrierOffset-r14				ENUMERATED {n0, n12, n24, n36, n2, n18, n34, spare1}

OPTIONAL,	-- NEED OP

nprach-NumSubcarriers-r14				ENUMERATED {n12, n24, n36, n48}

OPTIONAL,	-- NEED OP

nprach-SubcarrierMSG3-RangeStart-r14	ENUMERATED {zero, oneThird, twoThird, one}

OPTIONAL,	-- NEED OP

npdcch-NumRepetitions-RA-r14			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1}

OPTIONAL,	-- NEED OP

npdcch-StartSF-CSS-RA-r14				ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64}

OPTIONAL,	-- NEED OP

npdcch-Offset-RA-r14					ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

OPTIONAL,	-- NEED OP

nprach-NumCBRA-StartSubcarriers-r14		ENUMERATED {n8, n10, n11, n12, n20, n22, n23, n24,

n32, n34, n35, n36, n40, n44, n46, n48}

OPTIONAL,	-- NEED OP

npdcch-CarrierIndex-r14					INTEGER (1..maxNonAnchorCarriers-NB-r14)

OPTIONAL,	-- Need OP

...

}	OPTIONAL	-- Need OR

}

NPRACH-ParametersListTDD-NB-r15 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF

NPRACH-ParametersTDD-NB-r15

NPRACH-ParametersTDD-NB-r15 ::=		SEQUENCE {

nprach-Parameters-r15					SEQUENCE {

nprach-Periodicity-r15					ENUMERATED {ms80, ms160, ms320, ms640,

ms1280, ms2560, ms5120, ms10240}

OPTIONAL,	-- NEED OP

nprach-StartTime-r15					ENUMERATED {ms10, ms20, ms40, ms80,

ms160, ms320, ms640, ms1280,

ms2560, ms5120, spare6, spare5,

spare4, spare3, spare2, spare1}

OPTIONAL,	-- NEED OP

nprach-SubcarrierOffset-r15				ENUMERATED {n0, n12, n24, n36, n2, n18, n34, spare1}

OPTIONAL,	-- NEED OP

nprach-NumSubcarriers-r15				ENUMERATED {n12, n24, n36, n48}

OPTIONAL,	-- NEED OP

nprach-SubcarrierMSG3-RangeStart-r15	ENUMERATED {zero, oneThird, twoThird, one}

OPTIONAL,	-- NEED OP

npdcch-NumRepetitions-RA-r15			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1}

OPTIONAL,	-- NEED OP

npdcch-StartSF-CSS-RA-r15				ENUMERATED {v4, v8, v16, v32, v48, v64, v96, v128}

OPTIONAL,	-- NEED OP

npdcch-Offset-RA-r15					ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

OPTIONAL,	-- NEED OP

nprach-NumCBRA-StartSubcarriers-r15		ENUMERATED {n8, n10, n11, n12, n20, n22, n23, n24,

n32, n34, n35, n36, n40, n44, n46, n48}

OPTIONAL,	-- NEED OP

...

}	OPTIONAL	-- Need OR

}

NPRACH-ParametersListTDD-NB-v1550 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF

NPRACH-ParametersTDD-NB-v1550

NPRACH-ParametersTDD-NB-v1550 ::=	SEQUENCE {

maxNumPreambleAttemptCE-v1550			ENUMERATED {n3, n4, n5, n6, n7, n8, n10, spare1},

numRepetitionsPerPreambleAttempt-v1550	ENUMERATED {n1, n2, n4, n8, n16, n32, n64, n128,

n256, n512, n1024}

}

NPRACH-ParametersListFmt2-NB-r15 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-ParametersFmt2-NB-r15

NPRACH-ParametersFmt2-NB-r15 ::=		SEQUENCE {

nprach-Parameters-r15					SEQUENCE {

nprach-Periodicity-r15					ENUMERATED {ms40, ms80, ms160, ms320,

ms640, ms1280, ms2560, ms5120}

OPTIONAL,	-- NEED OP

nprach-StartTime-r15					ENUMERATED {ms8, ms16, ms32, ms64,

ms128, ms256, ms512, ms1024}

OPTIONAL,	-- NEED OP

nprach-SubcarrierOffset-r15				ENUMERATED {n0, n36, n72, n108, n6, n54, n102, n42,

n78, n90, n12, n24, n48, n84, n60, n18}

OPTIONAL,	-- NEED OP

nprach-NumSubcarriers-r15				ENUMERATED {n36, n72, n108, n144}

OPTIONAL,	-- NEED OP

nprach-SubcarrierMSG3-RangeStart-r15	ENUMERATED {zero, oneThird, twoThird, one}

OPTIONAL,	-- NEED OP

npdcch-NumRepetitions-RA-r15			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1}

OPTIONAL,	-- NEED OP

npdcch-StartSF-CSS-RA-r15				ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64}

OPTIONAL,	-- NEED OP

npdcch-Offset-RA-r15					ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

OPTIONAL,	-- NEED OP

nprach-NumCBRA-StartSubcarriers-r15		ENUMERATED {

n24, n30, n33, n36, n60, n66, n69, n72,

n96, n102, n105, n108, n120, n132, n138, n144}

OPTIONAL,	-- NEED OP

npdcch-CarrierIndex-r15					INTEGER (1..maxNonAnchorCarriers-NB-r14)

OPTIONAL,	-- Need OP

...

}	OPTIONAL	-- Need OR

} RSRP-Range

RSRP-ThresholdsNPRACH-InfoList-NB-r13 ::= SEQUENCE (SIZE(1..2)) OF RSRP-Range

EDT-TBS-InfoList-NB-r15 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF EDT-TBS-NB-r15

EDT-TBS-NB-r15 ::=	SEQUENCE {

edt-SmallTBS-Enabled-r15		BOOLEAN,

edt-TBS-r15						ENUMERATED {b328, b408, b504, b584, b680, b808, b936, b1000}

}

RSRP-ThresholdsNPRACH-NonAnchorInfoList-NB-r16 ::= SEQUENCE (SIZE(1..2)) OF RSRP-Range

-- ASN1STOP

NPRACH-ConfigSIB-NB field descriptions

dummy This field is not used in the specification. If received it shall be ignored by the UE.

edt-SmallTBS-Enabled Value TRUE indicates UE performing EDT is allowed to select TBS smaller than edt-TBS for Msg3 according to the corresponding NPRACH resource, as specified in TS 36.213 [23].

edt-SmallTBS-Subset Presence indicates only two of the TBS values can be used according to edt-TBS corresponding to the NPRACH resource, as specified in TS 36.213 [23]. When the field is not present, any of the TBS values according to edt-TBS corresponding to the NPRACH resource can be used. This field is applicable for a NPRACH resource only when edt-SmallTBS-Enabled is included for the corresponding NPRACH resource.

edt-TBS Largest TBS for Msg3 for a NPRACH resource applicable to a UE performing EDT. Value in bits. Value b328 corresponds to 328 bits, value b408 corresponds to 408 bits and so on. See TS 36.213 [23].

maxNumPreambleAttemptCE Maximum number of preamble transmission attempts per NPRACH resource. See TS 36.321 [6]. If the UE supports enhanced random access power control and maxNumPreambleAttemptCE-r14 is included, the UE shall use maxNumPreambleAttemptCE-r14 instead of maxNumPreambleAttemptCE-r13 for the first entry in nprach-ParametersList. maxNumPreambleAttemptCE-r13 applies to FDD and maxNumPreambleAttemptCE-v1550 applies to TDD.

npdcch-CarrierIndex For FDD: Index of the carrier in the list of DL non anchor carriers. The first entry in the list has index '1', the second entry has index '2' and so on. If the UE supports mixed operation mode and dl-ConfigListMixed is present in systemInformationBlockType22-NB, the UE creates a combined list of DL carriers for random access by appending dl-ConfigListMixed to the dl-ConfigList while maintaining the order among both dl-ConfigList and dl-ConfigListMixed; only the first maxNonAnchorCarriers-NB-r14 DL non-anchor carriers in the concatenated list can be used for random access. If the field is absent in the entry in nprach-ParametersListEDT in SystemInformationBlockType22-NB, the value of npdcch-CarrierIndex in the corresponding entry of nprach-ParametersList applies, if present. If the field is absent in an entry in nprach-ParametersListFmt2EDT in SystemInformationBlockType23-NB, the value of npdcch-CarrierIndex in the corresponding entry of nprach-ParametersListFmt2 applies, if present. Otherwise, the DL anchor carrier is used. For TDD: This parameter is absent and the same carrier is used in uplink and downlink.

npdcch-NumRepetitions-RA Maximum number of repetitions for NPDCCH common search space (CSS) for RAR, Msg3 retransmission and Msg4, see TS 36.213 [23], clause 16.6. See NOTE.

npdcch-Offset-RA Fractional period offset of starting subframe for NPDCCH common search space (CSS Type 2), see TS 36.213 [23], clause 16.6. See NOTE.

npdcch-StartSF-CSS-RA Starting subframe configuration for NPDCCH common search space (CSS), including RAR, Msg3 retransmission, and Msg4, see TS 36.213 [23], clause 16.6. See NOTE.

nprach-CP-Length Cyclic prefix length for NPRACH transmission (TCP), see TS 36.211 [21], clause 10.1.6. Value us66dot7 corresponds to 66.7 microseconds and value us266dot7 corresponds to 266.7 microseconds. If the UE uses a NPRACH resource for preamble format 2, the UE ignores the value signalled in nprach-CP-Length and considers the value to be 800 microseconds.

nprach-NumCBRA-StartSubcarriers The number of start subcarriers from which a UE can randomly select a start subcarrier as specified in TS 36.321 [6]. If nprach-Config-v1330 is not included in SystemInformationBlockType2-NB, the UE sets the value of nprach-NumCBRA-StartSubcarriers-r13 to the value signalled by nprach-NumSubcarriers-r13 for the corresponding NPRACH resource. The start subcarrier indices that the UE is allowed to randomly select from, are given by: nprach-SubcarrierOffset + [0, nprach-NumCBRA-StartSubcarriers - 1]. See NOTE.

nprach-NumSubcarriers Number of sub-carriers in a NPRACH resource, see TS 36.211 [21], clause 10.1.6. In number of subcarriers. See NOTE.

nprach-ParametersList, nprach-ParametersListEDT Configures NPRACH parameters for each NPRACH resource. Up to three PRACH resources can be configured in nprach-ParametersList in a cell. Each NPRACH resource is associated with a different number of NPRACH repetitions. E-UTRAN includes the same number of entries, and listed in the same order for nprach-ParametersListEDT, as in nprach-ParametersList in SystemInformationBlockType2-NB. The NPRACH resources in nprach-ParametersListEDT are used to initiate EDT. Each NPRACH resource is associated with a TBS signalled in the corresponding entry of edt-TBS-InfoList. For TDD: The UE shall use nprach-ParametersListTDD and ignore nprach-ParametersList.

nprach-ParametersListTDD For TDD: Configure NPRACH parameters for each NPRACH. Up to three NPRACH resources can be configured in a cell. Each NPRACH resource is associated with a different number of NPRACH repetitions.

nprach-ParametersListFmt2, nprach-ParametersListFmt2EDT Configures NPRACH parameters for each NPRACH resource format 2. Up to three NPRACH resources can be configured on one carrier. Each NPRACH resource is associated with a different number of NPRACH repetitions. E-UTRAN includes the same number of entries, and listed in the same order, as in nprach-ParametersList in SystemInformationBlockType2-NB. The NPRACH resources in nprach-ParametersListFmt2EDT are used to initiate EDT. Each NPRACH resource is associated with a TBS signalled in the corresponding entry of edt-TBS-InfoList. E-UTRAN configures the NPRACH resources format 2 so that they do not overlap in time domain with the NPRACH resources configured in nprach-ParametersList and nprach-ParametersListEDT. If there is no NPRACH resource in nprach-ParametersListFmt2 (respectively nprach-ParametersListFmt2EDT) on any UL carrier for one NPRACH repetition level, the UE uses the NPRACH resources in nprach-ParametersList (respectively nprach-ParametersListEDT) for this NPRACH repetition level. Otherwise, the UE uses only NPRACH resources in nprach-ParametersListFmt2 (respectively nprach-ParametersListFmt2EDT).

nprach-Periodicity Periodicity of a NPRACH resource, see TS 36.211 [21], clause10.1.6. Unit in millisecond. See NOTE.

nprach-PreambleFormat TDD: TDD preamble format, see TS 36.211 [21]. clause 10.1.6, Value fmt0 corresponds to preamble format 0, value fmt1 corresponds to preamble format 1 and so on.

nprach-StartTime Start time of the NPRACH resource in one period, see TS 36.211 [21], clause 10.1.6. Unit in millisecond. See NOTE.

nprach-SubcarrierOffset Frequency location of the NPRACH resource, see TS 36.211 [21], clause 10.1.6. In number of subcarriers, offset from sub-carrier 0. See NOTE.

nprach-SubcarrierMSG3-RangeStart Fraction for calculating the starting subcarrier index of the range reserved for indication of UE support for multi-tone Msg3 transmission, within the NPRACH resource, see TS 36.211 [21], clause 10.1.6. Multi-tone Msg3 transmission is not supported for {32, 64, 128} repetitions of NPRACH. For at least one of the NPRACH resources with the number of NPRACH repetitions other than {32, 64, 128}, the value of nprach-SubcarrierMSG3-RangeStart should not be 0. If nprach-SubcarrierMSG3-RangeStart is equal to zero, no start subcarrier index for the single-tone Msg3 NPRACH is allocated and the start subcarrier indexes for the multi-tone Msg3 NPRACH partition are given by nprach-SubcarrierOffset + [0, nprach-NumCBRA-StartSubcarriers - 1]. If nprach-SubcarrierMSG3-RangeStart is equal to oneThird or twoThird, the start subcarrier indexes for the two partitions are given by: nprach-SubcarrierOffset + [0, FLOOR (nprach-NumCBRA-StartSubcarriers * nprach-SubcarrierMSG3-RangeStart) -1] for the single-tone Msg3 NPRACH partition; nprach-SubcarrierOffset + [FLOOR (nprach-NumCBRA-StartSubcarriers * nprach-SubcarrierMSG3-RangeStart), nprach-NumCBRA-StartSubcarriers - 1] for the multi-tone Msg3 NPRACH partition; If nprach-SubcarrierMSG3-RangeStart is equal to one, the start subcarrier indexes for the single-tone Msg3 NPRACH are given by nprach-SubcarrierOffset + [0, nprach-NumCBRA-StartSubcarriers - 1] and no start subcarrier index for the multi-tone Msg3 NPRACH partition is allocated. See NOTE.

numRepetitionsPerPreambleAttempt Number of NPRACH repetitions per attempt for each NPRACH resource, See TS 36.211 [21], clause 10.1.6. numRepetitionsPerPreambleAttempt-r13 applies to FDD and numRepetitionsPerPreambleAttempt-v1550 applies to TDD.

rsrp-ThresholdsPrachInfoList The criterion for UEs to select a NPRACH resource. Up to 2 RSRP threshold values can be signalled. The first element corresponds to RSRP threshold 1, the second element corresponds to RSRP threshold 2. See TS 36.321 [6]. If absent, there is only one NPRACH resource. A UE that supports powerClassNB-14dBm-r14 shall correct the RSRP threshold values before applying them as follows: RSRP threshold = Signalled RSRP threshold - min{0, (14-min(23, P-Max))} where P-Max: is the value of p-Max field in SystemInformationBlockType1-NB.

rsrp-ThresholdsPrachNonAnchorInfoList The criterion for UEs to select a NPRACH resource on the Non-Anchor carriers. Up to 2 RSRP threshold values can be signalled. The first element corresponds to RSRP threshold 1, the second element corresponds to RSRP threshold 2. See TS 36.321 [6]. If absent, there is only one NPRACH resource. A UE that supports powerClassNB-14dBm-r14 shall correct the RSRP threshold values before applying them as follows: RSRP threshold = Signalled RSRP threshold - min{0, (14-min(23, P-Max))} where P-Max: is the value of p-Max field in SystemInformationBlockType1-NB.

NOTE:

-	If the field is absent in an entry of nprach-ParametersList in SystemInformationBlockType22-NB, the value of the same field in the corresponding entry of nprach-ParametersList in SystemInformationBlockType2-NB applies.

-	If the field is absent in the entry in nprach-ParametersListEDT, the value of the same field in the corresponding entry of nprach-ParametersList on the same UL carrier applies, if present. Otherwise, the value of the same field in the corresponding entry of nprach-ParametersList in SystemInformationBlockType2-NB applies.

-	If the field is absent in an entry of nprach-ParametersListTDD in SystemInformationBlockType22-NB, the value of the same field in the corresponding entry of nprach-ParametersListTDD in SystemInformationBlockType2-NB applies. The field is mandatory present in nprach-ParametersListTDD in SystemInformationBlockType2-NB.

-	If the field is absent in an entry of nprach-ParametersListFmt2 in SystemInformationBlockType23-NB, the value of the same field, if present, in the corresponding entry of nprach-ParametersListFmt2 in SystemInformationBlockType2-NB applies. Otherwise the value of the same field, if present, in the corresponding entry of the first occurence of nprach-ParametersListFmt2 in the non anchor carrier list applies. Otherwise, the value of the same field in the corresponding entry of nprach-ParametersList in SystemInformationBlockType2-NB applies.

-	If the field is absent in an entry of nprach-ParametersListFmt2 in SystemInformationBlockType2-NB, the value of the same field in the corresponding entry of nprach-ParametersList in SystemInformationBlockType2-NB applies.

-	If the field is absent in an entry of nprach-ParametersListFmt2EDT in SystemInformationBlockType23-NB, the value of the same field, if present, in the corresponding entry of nprach-ParametersListFmt2 on the same UL carrier applies. Otherwise, the value of the same field, if present, in the corresponding entry of nprach-ParametersListFmt2 in SystemInformationBlockType2-NB applies. Otherwise the value of the same field, if present, in the corresponding entry of the first occurence of nprach-ParametersListFmt2 in the non anchor carrier list applies. Otherwise, the value of the same field in the corresponding entry of nprach-ParametersList in SystemInformationBlockType2-NB applies.

-	If the field is absent in an entry of nprach-ParametersListFmt2EDT in SystemInformationBlockType2-NB, the value of the same field, if present, in the corresponding entry of nprach-ParametersListFmt2 in SystemInformationBlockType2-NB applies. Otherwise the value of the same field in the corresponding entry of nprach-ParametersList in SystemInformationBlockType2-NB applies.

| Conditional presence | Explanation |
| --- | --- |
| EDT1 | The field is mandatory present if cp-EDT, cp-EDT-5GC, up-EDT or up-EDT-5GC in SystemInformationBlockType2-NB is present; otherwise the field is not present and the UE shall delete any existing value for this field. |
| EDT2 | The field is optionally present, Need OR, if edt-Parameters is present; otherwise the field is not present and the UE shall delete any existing value for this field. |
| TDD | This field is mandatory present for TDD; otherwise the field is not present and the UE shall delete any existing value for this field. |

End of Change