3GPP TSG RAN WG2 Meeting #115 electronic	R2-2110249

Online,  1 – 12 November, 2021

Agenda Item:	8.11.3

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	UE Positioning in RRC_INACTIVE mode.

Document for:	Discussion & Decision

# Introduction

The revised WID [1] for positioning enhancements in Rel. 17, lists the following as objective:

- Specify methods, measurements, signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions, including [RAN2, RAN1, RAN3,RAN4]: - DL NR positioning methods and RAT-independent positioning methods - Support of UE positioning measurements for UEs in RRC_INACTIVE state - Reporting of positioning measurement or location estimate performed in RRC_INACTIVE when the UE is in RRC_INACTIVE state Note: this work will be coordinated with the SDT WI. - As 2nd priority: - UL and DL+UL NR positioning methods - Support of gNB positioning measurements for UEs in RRC_INACTIVE state

In RAN2#114-bis-e [2], we reached the following agreements:

Support pre-configuration of assistance data to the UE at least in an LPP session.  Details of how to enable this are FFS (e.g. what additional functionality beyond deferred location procedure might be needed).

The LPP Request Location Information message can serve as an indication to the UE to utilize the pre-configured AD.  FFS additional conditions/validity criteria for using the pre-configured AD.

In RAN2#115e [3], we reached the following agreements:

- gNB can configure the UE with periodic SRS (assuming periodic SRS is supported in RRC_INACTIVE) by RRCRelease with suspendConfig at least when periodic event is configured for deferred MT-LR.  Other cases can be further discussed.

- Regarding the validity conditions/criteria associated with pre-configured assistance data, consider at least the following options:

  - Option A: Based on a validity area (e.g. a list of cells)

  - Option B: Based on a (configured) validity timer or a numerical limit on number of times it is utilized

  - Option C: Based on explicit modification or release from the LMF/NG-RAN

  - Option D: Based on the UE’s current location and/or the time

In this contribution, we provide our views on the preconfigured assistance data or preconfigured configuration for uplink and downlink respectively.

# Support for pre-configured assistance data

Preconfigured assistance data should greatly reduce signaling overhead and support mobility for a UE in RRC_INACTIVE state. The network does not necessarily know the exact location of the UE in case of cell-reselection mobility, and the assistance data provided by the network may be not optimal at the current location of the UE. If the UE is provided additional assistance data beyond what the UE is expected to measure at the current location, and the applicable assistance data could be based on measurement (RSRP) or logical information (e.g. the camped cell).

In RAN2#115, we agreed that the validity conditions / criteria can be associated with the assistance data, where the validity condition could be at least one of the following:

- Based on a validity area (e.g. a list of cells) (Option A)

- Based on a (configured) validity timer or a numerical limit on number of times it is utilized (Option B)

- Based on explicit modification or release from the LMF/NG-RAN (Option C)

- Based on the UE’s current location and/or the time (Option D)

Figure 1: Depiction of UE Mobility in RRC_INACTIVE state.

As an example, in the above example, the first assistance data could be assigned a validity area of cells {1, 2, 6, 3} and the second assistance data set could be assigned validity area of cells {4, 5}. Then the UE can select one out of the two assistance data sets for measurement based on where the UE currently finds itself. The UE can store the assistance data, even when the UE is outside the validity area and reuse it when it returns back to the validity area.

Observation 1: Pre-configured assistance data could reduce signaling overhead and latency for measurement for RRC_INACTIVE UEs.

Proposal 1: Provide multiple sets of pre-configured assistance data to the UE and select the most suitable one based on validity conditions (e.g. validity area, validity time).

Proposal 2: The assistance data shall be valid across positioning sessions.

# Impact of mobility on assistance data for measuring DL-PRS

A Rel. 16 UE is provided with assistance data for measuring DL-PRS which has a maximum of 4 frequency layers, 64 TRPs per frequency layer, 2 resource sets per TRP and 64 DL-PRS resources per resource set as depicted in Figure 2.  The ASN.1 Syntax for NR-DL-TDOA-ProvideAssistanceData-r16 message is depicted in Figure 1. It contains fields NR-DL-PRS-AssistanceData-r16 consisting of configuration of DL-PRS resources, the IE NR-SelectedDL-PRS-IndexList-r16 signalling the UE the downselection of the DL-PRS resources for a UE,  the NR-PositionCalculationAssistance-r16 providing location and TRD of the TRPs and NR-DL-TDOA-Error-r16 providing causes of DL-TDOA error.

Figure 2: Structure of assistance data for obtaining DL-PRS configuration in Rel. 16

-- ASN1START NR-DL-TDOA-ProvideAssistanceData-r16 ::= SEQUENCE { nr-DL-PRS-AssistanceData-r16 NR-DL-PRS-AssistanceData-r16 OPTIONAL, -- Need ON nr-SelectedDL-PRS-IndexList-r16 NR-SelectedDL-PRS-IndexList-r16 OPTIONAL, -- Need ON nr-PositionCalculationAssistance-r16 NR-PositionCalculationAssistance-r16 OPTIONAL, -- Cond UEB nr-DL-TDOA-Error-r16 NR-DL-TDOA-Error-r16 OPTIONAL, -- Need ON ... } -- ASN1STOP

Figure 3: ASN.1 Syntax for NR-DL-TDOA-ProvideAssistanceData Message

In Figure 3, we depict a possible path taken by a UE which has been transitioned to RRC_INACTIVE state. In this example, a UE was in RRC_CONNECTED state when it was in cell 1. Then it transitioned to RRC_INACTIVE state while the positioning session was still underway. It camped on different cells along its path until it reached Cell 5, where due to expiry of T380 timer, it had to do RNA update. In this example, we have only depicted 6 cells. In practice, the number of TRPs within the RNA could be much larger than 64.  Either the size of assistance data delivered to the UE for use in RRC_INACTIVE mode measurement is increased or  DL-PRS assistance data needs to be updated frequently. Second, the NR-SelectedDL-PRS-IndexList-r16, which downselects the DL-PRS, which the UE needs to perform measurements onfor a given method, needs to be updated as the UE moves along the network. Furthermore, the LMF may or may not have the exact knowledge of UE position (especially in UE-based mode). In these conditions, the NR-SelectedDL-PRS-IndexList-r16 may not be the best choice.

This means either the UE needs to be configured with assistance data of larger dimensions and the SelectedDL-PRS-IndexList needs to be signaled to the UE taking into account the coarse UE location at a given time. This requires frequent switching to RRC_CONNECTED state for providing the assistance data to the UE. Alternatively, a different mechanism of handling assistance for measurements in RRC_INACTIVE mode could be specified.

Observation 2: The assistance data provided for a UE may not correspond to the changes occurring during  the RRC_INACTIVE state - in some scenarios this could lead to the situation that the UE is unaware of DL-PRS from the most suitable TRPs for  the current location of the UE.

Observation 3: An optimum selection of NR-SelectedDL-PRS-IndexList may be specific to a particular UE location within a group of cells. Updates may require transition to RRC_CONNECTED state to receive the assistance data.

It may be advantageous to receive redundant assistance data while the UE is in RRC_CONNECTED state  and for the UE to store it. The portion of assistance data that is applicable for the UE at the current location could be derived by the UE based on configuration and measurements or signaled to the UE (e.g. using the SDT mechanism).

Proposal 3: Support providing UE with assistance data describing the DL-PRS available within a larger area (e.g. list of cells), some of which may be outside the hearability region of the UE at its current location (i.e. redundant assistance data). The UE shall select the AD based on its coarse location.

The following could be some options that could be used by the network to deliver a set of configurations  of DL-PRS during RRC_CONNECTED or using posSIBs. Some of the sets are not applicable at the UE’s current camped cell, but as the UE moves along, the applicable set can be updated using SDT.  The benefit of this approach is that a larger set of AD is delivered during RRC_CONNECTED when signaling capacity is less of an issue.  During RRC_INACTIVE, the configuration of assistance data applicable to the camped cell can be triggered by signaling (for example, indicator pointing to the assistance data) via the SDT mechanism.

Option 1: The assistance data for positioning in RRC_INACTIVE is larger and at least contains assistance data pertaining to more than 64 TRPs (in one frequency layer), to reflect the fact that the UE can roam within RNA without informing the network.

FFS: Signalling the downselection of the resources where the UE needs to perform measurements (e.g. by updating nr-SelectedDL-PRS-IndexList-r16 using the SDT mechanism).

Option 2: The structure of assistance data (AD) for positioning in RRC_INACTIVE is same as that for RRC_CONNECTED, but multiple instants of assistance data are provided to the UE.

FFS: How one of the AD instants are selected by the UE (e.g. depending on the cell-ID of the camped cell or based on measurements) from multiple ADs.

FFS: Whether such assistance data may be hierarchically structured (e.g. a RNA containing assistance data (sets) corresponding to several positioning system areas, each of the sets then contain multiple AD corresponding to smaller areas (e.g. group of cells).

Option 3: An update to AD is provided to the UE using the small data transmission mechanism in the corresponding downlink transmission after the uplink SDT meachanism is established.

FFS: Selection of the fields that need to be updated?

Proposal 4:  The field NR-DL-PRS-AssistanceData-r16, shall be applicable to a larger area (e.g. list of cells) and the UE shall be provided several sets of NR-SelectedDL-PRS-IndexList-r16 applicable to a smaller area (e.g. cell). The UE selects one of the available NR-SelectedDL-PRS-IndexList-r16 based on the validity area.

# Configuration and Triggering mechanism for positioning SRS

In Rel. 16, the positioning SRS configuration was provided to the UE by the serving cell and could be assumed to be orthogonalised among the UEs served by the cell. Furthermore, SRS transmission would be discontinued when transiting to the RRC_INACTIVE or upon handover.

The ability to transmit the SRS in RRC_INACTIVE mode requires that the configuration provided to the UE by the serving cell can be used within a validity area, e.g. a positioning system information area, if frequent state transitions to RRC_CONNECTED are to be reduced. This requires coordination of SRS configuration provided to different UEs at least among the cells within a validity area. One mechanism to reduce state transitions would be to configure the UE with several configurations of SRS and select one of them, depending on the current coarse location of the UE (e.g. the camped cell).

There are multiple options how the configuration could be delivered to the UE

Option 1: A set of parameters common to all UEs within a certain area (e.g. positioning system information area) for transmitting SRS in RRC_INACTIVE mode, supplemented by UE-specific parameters.

FFS:  Mechanism for delivering the common parameters (e.g. via broadcast or via unicast during RRC_CONNECTED)

FFS:  Mechanism for delivering / deriving the UE-specific parameters

Option 2: Providing RRC_INACTIVE configurations as a part of RRCSuspend message.

FFS: Full/Partial configuration and size of it.

Option 3: Continuing using the configuration provided during RRC_CONNECTED, as long as the UE is within certain area (e.g. a group of cells) and as long as an expiry timer is running.

A key point to consider is how the configuration shall be updated while the UE is in RRC_INACTIVE state. Especially if UL-only positioning methods (e.g. UL-TDOA) are being used, then the delivery of configuration may need frequent transition to RRC_CONNECTED.

Observation 4:  The SRS configuration needs to be coordinated among cells within the area where the UE can be expected to transmit SRS while it in RRC_INACTIVE state.

Proposal 5: A list of cells where the SRS configuration is valid in RRC_INACTIVE state shall be signaled to the UE.

Proposal 6: The SRS configuration shall be divided into common and UE-specific for RRC_INACTIVE. The common configuration shall be coordinated among multiple cells.

Especially for MT-LR, it becomes important that the UE in RRC_INACTIVE has a means to trigger positioning resources. If the configuration is split into common and UE-specific parts, then the reception of UE-specific part could be the implicit request to start transmitting the periodic SRS.

Proposal 7: For periodic SRS, common SRS configuration may be provided once and receiving the UE-specific part via SDT in RRC_INACTIVE shall trigger the periodic SRS.

Similar to the DL case, the UE can receive several configuration of SRS with different validity scope and the UE can trigger one or the other configuration based on its coarse location.

Proposal 8: UE shall receive several configurations, each configuration has a validity scope and the UE shall be configured to select an applicable configuration corresponding to its coarse location or based on measurement.

Proposal 9: The transmission of positioning SRS in RRC_INACTIVE mode shall be stopped if the UE moves away from validity area (consisting a list of cells) or exceeds the validity timer.

The SRS configuration shall be associated with a DL-RS measurement (e.g. DL-PRS). The SRS configuration may be triggered or stopped based on RSRP measurement of an associated DL-RS.

Proposal 10: The positioning SRS in RRC_INACTIVE mode shall be triggered to stop if the RSRP on an associated DL-RS (SSB or PRS) falls below a certain threshold or rises above a certain threshold configured by the network.

# Conclusions

In this contribution, we have made the following observations:

Observation 1: Pre-configured assistance data could reduce signaling overhead and latency for measurement for RRC_INACTIVE UEs.

Observation 2: The assistance data provided for a UE may not correspond to the changes occurring during  the RRC_INACTIVE state - in some scenarios this could lead to the situation that the UE is unaware of DL-PRS from the most suitable TRPs for  the current location of the UE.

Observation 3: An optimum selection of NR-SelectedDL-PRS-IndexList may be specific to a particular UE location within a group of cells. Updates may require transition to RRC_CONNECTED state to receive the assistance data.

Observation 4:  The SRS configuration needs to be coordinated among cells within the area where the UE can be expected to transmit SRS while it in RRC_INACTIVE state.

Based on the above observations, we make the following proposals:

Proposal 1: Provide multiple sets of pre-configured assistance data to the UE and select the most suitable one based on validity conditions (e.g. validity area, validity time).

Proposal 2: The assistance data shall be valid across positioning sessions.

Proposal 3: Support providing UE with assistance data describing the DL-PRS available within a larger area (e.g. list of cells), some of which may be outside the hearability region of the UE at its current location (i.e. redundant assistance data). The UE shall select the AD based on its coarse location.

Proposal 4:  The field NR-DL-PRS-AssistanceData-r16, shall be applicable to a larger area (e.g. list of cells) and the UE shall be provided several sets of NR-SelectedDL-PRS-IndexList-r16 applicable to a smaller area (e.g. cell). The UE selects one of the available NR-SelectedDL-PRS-IndexList-r16 based on the validity area.

Proposal 5: A list of cells where the SRS configuration is valid in RRC_INACTIVE state shall be signaled to the UE.

Proposal 6: The SRS configuration shall be divided into common and UE-specific for RRC_INACTIVE. The common configuration shall be coordinated among multiple cells.

Proposal 7: For periodic SRS, common SRS configuration may be provided once and receiving the UE-specific part via SDT in RRC_INACTIVE shall trigger the periodic SRS.

Proposal 8: UE shall receive several configurations, each configuration has a validity scope and the UE shall be configured to select an applicable configuration corresponding to its coarse location or based on measurement.

Proposal 9: The transmission of positioning SRS in RRC_INACTIVE mode shall be stopped if the UE moves away from validity area (consisting a list of cells) or exceeds the validity timer.

Proposal 10: The positioning SRS in RRC_INACTIVE mode shall be triggered to stop if the RSRP on an associated DL-RS (SSB or PRS) falls below a certain threshold or rises above a certain threshold configured by the network.

# References

| [1] | RP-210903, "Revised WID: NR Positioning Enhancements," 3GPP, Online, 2021-03. |
| --- | --- |
| [2] | 3GPP RAN WG2#114-e, "Chairman's Notes on Positioning and Relay Session," 3GPP, Online, 2021-05. |
| [3] | 3GPP RAN WG2#115-e, "Chairman's Notes on Positioning and Relay Session," 3GPP, Online, 2021-08. |
| [4] | 3GPP TR 38.857, "Technical Specification Group Radio Access NEtwork; Study on NR Positioning Enhancements," 3GPP, Sophia Antipolis, 2021-03. |