3GPP TSG RAN WG2 #116-e                                           R2-2110261

Electronic Meeting, November 1 – 12, 2021

Source: 	CMCC

Title: 	Support of emergency services for SNPN

Agenda Item:	8.16.4

Document for:	Discussion and decision

Introduction

In RAN2#115-e [1], the following agreements have been reached:

Introduce a new IE/field to indicate the support of IMS emergency service for SNPN. eCall over IMS is not supported in SNPNs in Rel-17. PWS can be supported in SNPNs in Rel-17. Send an LS to ask question related to P2 (new offline)

CT1 sends two reply LS on limited service availability of an SNPN as follows:

R2-2109306/C1-215046 [2]: With respect to the following information from RAN2 in R2-2106777, CT1 would like to provide a comment. Therefore, RAN2 would like to inform CT1 that RAN2 does not know yet if AS can also indicate to NAS which SNPNs advertised by the cell support emergency services. In order for a UE, which is in the limited service state and needs to obtain emergency services, to select an SNPN which supports emergency services, if not all SNPNs sharing a cell support emergency services, the UE needs to be aware of which SNPNs support emergency services.

R2-2109814/C1-216096 [3]: CT1 would like to provide the following feedback based on RAN2’s question in their LS: RAN2 Question: Can CT1/SA2 to confirm that the R17 SNPN-capable UEs that are not in SNPN Access Mode and R17 Non-SNPN capable UEs cannot camp on an SNPN cell supporting emergency services to obtain emergency services via any SNPN. Yes. Based on the requirements specified in TS 24.501 and TS 23.122, CT1 understands that R17 SNPN-capable UEs that are not in SNPN Access Mode and R17 Non-SNPN capable UEs cannot camp on an SNPN cell and cannot obtain emergency services via any SNPN.

This contribution will focus on the remaining issues on support of emergency services for SNPN.

Discussion

2.1 Granularity of broadcasting parameter

According to Reply LS on limited service availability of an SNPN from CT1 (R2-2109306 [2]), the UE needs to be aware of which SNPNs support emergency services if not all SNPNs sharing a cell support emergency services. Therefore, the indication whether the cell supports emergency services over NG-RAN for UEs in limited service state” should be broadcasted per SNPN.

With respect to the following information from RAN2 in R2-2106777, CT1 would like to provide a comment. Therefore, RAN2 would like to inform CT1 that RAN2 does not know yet if AS can also indicate to NAS which SNPNs advertised by the cell support emergency services. In order for a UE, which is in the limited service state and needs to obtain emergency services, to select an SNPN which supports emergency services, if not all SNPNs sharing a cell support emergency services, the UE needs to be aware of which SNPNs support emergency services.

Proposal 1: The new IE/field to indicate “the cell supports Emergency Services over NG-RAN for UEs in limited service state” should be broadcasted per SNPN.

A possible SIB structure that can realize proposal 1 is shown in Figure 1, it allows to include enhanced SNPN information inside Rel-16 IE NPN-IdentityInfoList-r16 of SIB1. Since the new additional IE is in SNPN granularity, it is proposed to add the new IE inside the CellAccessRelatedInfo in SIB1 as specified in the running CR TS 38.331 as below:

CellAccessRelatedInfo ::= SEQUENCE { plmn-IdentityList PLMN-IdentityInfoList, cellReservedForOtherUse ENUMERATED {true} OPTIONAL, -- Need R ..., [[ cellReservedForFutureUse-r16 ENUMERATED {true} OPTIONAL, -- Need R npn-IdentityInfoList-r16 NPN-IdentityInfoList-r16 OPTIONAL -- Need R ]] [[ snpn-AccessInfoList-r17 SEQUENCE (SIZE (1..maxNPN-r16)) OF SNPN-AccessInfo-r17 OPTIONAL -- Need R ]] } SNPN-AccessInfo-r17 ::= SEQUENCE { extCH-Supported-r17 ENUMERATED {true} OPTIONAL, -- Need R extCH-WithoutConfigAllowed-r17 ENUMERATED {true} OPTIONAL, -- Need R onboardingEnabled-r17 ENUMERATED {true} OPTIONAL, -- Need R ims-EmergencySupport-r17 ENUMERATED {true} OPTIONAL -- Need R }

Figure 1. The structure of SIB1 for supporting emergency services for SNPN

Based on the above discussion, we have the following proposal to support of emergency services for SNPN in RAN2 specifications:

Proposal 2: It is proposed to add the new IE/field inside snpn-AccessInfo-r17 of SIB1.

2.2 Availability of emergency services

In RAN2#113-e and RAN2#115-e meeting, we discussed the issue whether the following kinds of UEs in Rel-17 and Rel-16 and the Rel-15 can be able to camp on SNPN cell to obtain emergency service:

Case 1: UEs in SNPN Access Mode

Case 2: SNPN-capable UEs that are not in SNPN Access Mode

Case 3: Non-SNPN capable UEs

- For Rel-17 UEs

For a Rel-17 UE in SNPN Access Mode, it was agreed that it can camp on an acceptable SNPN cell supporting emergency services to obtain emergency services in RAN2#113-e [4].

For a Rel-17 SNPN-capable UE not in SNPN Access Mode and R17 non-SNPN capable UEs, according to reply LS from CT1 (R2-2109814 [3]), they cannot camp on an SNPN cell and cannot obtain emergency services via any SNPN.

CT1 would like to provide the following feedback based on RAN2’s question in their LS: RAN2 Question: Can CT1/SA2 to confirm that the R17 SNPN-capable UEs that are not in SNPN Access Mode and R17 Non-SNPN capable UEs cannot camp on an SNPN cell supporting emergency services to obtain emergency services via any SNPN. Yes. Based on the requirements specified in TS 24.501 and TS 23.122, CT1 understands that R17 SNPN-capable UEs that are not in SNPN Access Mode and R17 Non-SNPN capable UEs cannot camp on an SNPN cell and cannot obtain emergency services via any SNPN.

Proposal 3: R17 SNPN-capable UEs that are not in SNPN Access Mode and R17 non-SNPN capable UEs cannot camp on an SNPN cell supporting emergency services to obtain emergency services.

- For Rel-16 UEs

Emergency services over SNPN have been explicitly excluded for R16 in both TS 38.300[5] and TS 23.501[6]. Our understanding is that Rel-16 UEs in SNPN Access Mode doesn’t support emergency service feature, so Rel-16 UEs in SNPN Access Mode would not initiate emergency service in SNPN cell.

For a Rel-16 SNPN-capable UE not in SNPN Access Mode and non-SNPN capable UE, we think it is similar to the case b) and c) of Rel-17. And according to the agreements in RAN2#107bis meeting “In a SNPN-only cell, access attempts for emergency services by Rel-16 UEs without support for SNPNs is not allowed”, we think they cannot camp on an acceptable SNPN cell supporting emergency services to obtain emergency services.

In addition, Rel-16 UEs would not comprehend the added new Rel-17 IE for indicating the support of emergency services for SNPN, thus they would not know about which SNPN cell supports the emergency services.

Proposal 4: All kinds of Rel-16 UEs cannot camp on an acceptable SNPN cell supporting emergency services to obtain emergency services.

- For Rel-15 UEs

SNPN feature was introduced in Rel-16, and Rel-15 UEs natively can’t support SNPN feature. Moreover, according the agreement in RAN2#107bis-e, “Access attempts by Rel-15 UEs for emergency services on SNPN-only cell are not allowed”, we think that Rel-15 UEs cannot camp on an SNPN cell supporting emergency services to obtain emergency services.

Proposal 5: Rel-15 UEs cannot camp on an SNPN cell supporting emergency services to obtain emergency services.

Conclusions

The observations and proposals for SNPNs made in this contribution are summarized below.

Proposal 1: The indication “the cell supports Emergency Services over NG-RAN for UEs in limited service state” should be broadcasted per SNPN.

Proposal 2: It is proposed to add a new IE inside snpn-AccessInfo-r17 of SIB1.

Proposal 3: R17 SNPN-capable UEs that are not in SNPN Access Mode and R17 non-SNPN capable UEs cannot camp on an acceptable SNPN cell supporting emergency services to obtain emergency services.

Proposal 4: All kinds of Rel-16 UEs cannot camp on an acceptable SNPN cell supporting emergency services to obtain emergency services.

Proposal 5: Rel-15 UEs cannot camp on an SNPN cell supporting emergency services to obtain emergency services.

References

[1] Report of 3GPP TSG RAN WG2 meeting #115-e, Online, 9 - 27 August, 2021

[2] R2-2109306 Reply LS on limited service availability of an SNPN (C1-215046)

[3] R2-2109814 Reply LS on limited service availability of an SNPN (C1-216096)

[4] Report of 3GPP TSG RAN WG2 meeting #113-e, Online, 25 January - 5 February, 2021

[5] 3GPP TS 38.300, NR; NR and NG-RAN Overall Description; Stage 2 (Release 16)

[6] 3GPP TS 23.501, System Architecture for the 5G System; Stage 2 (Release 16)