3GPP TSG-RAN WG2 Meeting #116 electronic	R2-2110342

Online, November 1-12, 2021

Agenda Item:	8.17.3

Source:	Ericsson

Title:	RAN2 aspects for BFR, BFD and RLM for mTRP operation

Document for:	Discussion, Decision

# 1	Introduction

An update WID for feMIMO can be found in [1] and the objectives are copied below with track changes for convenience. The highlighted objectives are seen most relevant for mTRP and especially BFR/BFD operation for mTRP perspective:

The work item aims to specify the further enhancements identified for NR MIMO. The detailed objectives are as follows:

- Extend specification support in the following areas [RAN1]

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1:

  - Identify and specify features to facilitate more efficient (lower latency and overhead) DL/UL beam management for intra-cell and inter-cell scenarios to support higher UE speed and/or a larger number of configured TCI states:

    - Common beam for data and control transmission/reception for DL and UL, especially for intra-band CA

    - Unified TCI framework for DL and UL beam indication

    - Enhancement on signaling mechanisms for the above features to improve latency and efficiency with more usage of dynamic control signaling (as opposed to RRC)

    - For inter-cell beam management, a UE can transmit to or receive from only a single cell (i.e. serving cell does not change when beam selection is done). This includes L1-only measurement/reporting (i.e. no L3 impact) and beam indication associated with cell(s) with any Physical Cell ID(s)

      - The beam indication is based on Rel-17 unified TCI framework

      - The same beam measurement/reporting mechanism will be reused for inter-cell mTRP

      - This work shall only consider intra-DU and intra-frequency cases

  - Identify and specify features to facilitate UL beam selection for UEs equipped with multiple panels, considering UL coverage loss mitigation due to MPE, based on UL beam indication with the unified TCI framework for UL fast panel selection

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2:

  - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline

  - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception based on Rel-15/16 TCI framework

  - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception

  - Enhancement to support HST-SFN deployment scenario:

    - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission

    - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

- Enhancement on SRS, targeting both FR1 and FR2:

  - Identify and specify enhancements on aperiodic SRS triggering to facilitate more flexible triggering and/or DCI overhead/usage reduction

  - Specify SRS switching for up to 8 antennas (e.g., xTyR, x = {1, 2, 4} and y = {6, 8})

  - Evaluate and, if needed, specify the following mechanism(s) to enhance SRS capacity and/or coverage: SRS time bundling, increased SRS repetition, partial sounding across frequency

- Enhancement on CSI measurement and reporting:

  - Evaluate and, if needed, specify CSI reporting for DL multi-TRP and/or multi-panel transmission to enable more dynamic channel/interference hypotheses for NCJT, targeting both FR1 and FR2

  - Evaluate and, if needed, specify Type II port selection codebook enhancement (based on Rel.15/16 Type II port selection) where information related to angle(s) and delay(s) are estimated at the gNB based on SRS by utilizing DL/UL reciprocity of angle and delay, and the remaining DL CSI is reported by the UE, mainly targeting FDD FR1 to achieve better trade-off among UE complexity, performance and reporting overhead

- Investigate if the requirements on link recovery procedure is suitable for FR2 serving cells [RAN4]

- Specify higher layer support of enhancements listed above, at least including [RAN2]

  - For inter-cell beam management: MAC (if any) and RRC enhancements (including signaling, measurement configuration and TCI state switching) assuming no impact to serving cell (i.e. serving cell does not change when beam selection is done)

- Specify signaling between CU and DU to enable inter-cell beam management if any [RAN3]

- Specify core requirements associated with the items specified by RAN1, at least including [RAN4]

  - UE requirements for inter-cell beam management

In RAN2#115 the following agreements were made:

MAC entity maintains separate beamFailureDetectionTimer and BFI_COUNTER for each BFD-RS set of a serving cell configured with multiple BFD-RS sets.

beamFailureDetectionTimer and beamFailureInstanceMaxCount configuration is configured independently for each TRP of serving cell.

If the MAC entity receives beam failure instance indication for a BFD-RS set of a serving cell, it shall perform the following:

- - (re-)start beamFailureDetectionTimer corresponding to that BFD-RS set of the serving cell;

- - increment BFI_COUNTER corresponding to that BFD-RS set of the serving cell by 1.

- - If BFI_COUNTER >= beamFailureInstanceMaxCount corresponding to that BFD-RS set of the serving cell:

- - trigger a BFR for the BFD-RS set of the Serving Cell;

- For the case of both intra cell and inter cell:

BFD-RS set ID is included in BFR MAC CE to identify the failed TRP.

- For the case of intra cell (FFS for inter cell).

If beam failure is detected on both TRPs (i.e. BFD-RS sets) of an SCell, BFR is triggered for that SCell.

- - FFS whether UE transmits a) legacy BFR MAC CE or b) new BFR MAC CE indicating both failed TRPs as well as the beam failure recovery information for both TRPs.

If beam failure is detected on both TRPs (i.e. BFD-RS sets) of SpCell, random access procedure is initiated on SpCell.

- - FFS whether UE transmits a) legacy BFR MAC CE or b) new BFR MAC CE indicating both failed TRPs as well as the beam failure recovery information for both TRPs.

FFS what is meant in detail by “beam failure is detected on both TRPs”

In this contribution, we provide our view on RAN2 aspects for RLM and BFR/BFD for intra/intercell mTRP. We list (hopefully all)RAN1 agreements related to BFR/BFD operation for mTRP in the appendix.

# RLM for intercell mTRP

RAN1 is providing reply for LS RAN2 [9] sent out in last meeting. Response can be found in [10].

Main takeaways of the LS response are:

- Single HARQ entity for intercell BM and mTRP

- One physical layer configuration for TRPs with different PCIs, only TCI has association to a particular PCI

- Any cell PCell/SPCell/SCell can be configured with intra or intercell mTRP or BM

- UE is always able to receive CD-SSB as well as SI (and short messages) from serving cell TR

  - This is in somewhat contradicting with “It is correct understanding that simultaneous Rx in DL is not supported for inter-cell BM but supported for inter-cell mTRP”

- TCI state switching signaling framework follows that of previous releases and Rel-17 mTRP(MAC CE+DCI)

- Difference between mTRP and BM is that only BM adopts the Rel-17 TCI state framework where UL spatialdirection is renamed as TCI state and it is possible to configure UE such that UL TCI state and DL TCI state are switched in one go.

As the responses indicate UE is assumed to be able to receive the CD-SSB, it is suggested that RLM is not impacted in Rel-17 feMIMO.

RLM is not impacted in Rel-17.

# mTRP BFD

## BFD resource configuration

Beam Failure Detection (BFD) and Beam Failure Recovery (BFR) are features introduced in NR since Rel-15. For the purpose of BFD, the network configures the UE with BFD reference signals (SSB, CSI-RS or both SSB/CSI-RS resources) and the UE declares beam failure when the number of beam failure instance indications from the physical layer reaches a configured threshold before a configured timer expires. SSB-based BFD is based on the SSB associated to the initial DL BWP and can only be configured for the initial DL BWPs and for DL BWPs containing the SSB associated to the initial DL BWP. For other DL BWPs, Beam Failure Detection can only be performed based on CSI-RS.

The RAN2#115 agreements concerning RRC configuration are as follows:

MAC entity maintains separate beamFailureDetectionTimer and BFI_COUNTER for each BFD-RS set of a serving cell configured with multiple BFD-RS sets.

beamFailureDetectionTimer and beamFailureInstanceMaxCount configuration is configured independently for each TRP of serving cell.

Both BFD and RLM resources are configured in IE RadioLinkMonitoring where one list exist per serving cell. As the agreement says there would be now two BFD-RS sets per serving cell RAN2 should discuss how the change is done. Note that currently RAN2 might not have all information available as how mTRP operation is linked with BM operation and how intercell mTRP is going to be modeled in RRC. However, it is good to start comparing options. A simple option is to add the “setID” in the resource and then add additional counter and timer under main IE. Possible downside is that extension is done in a list element which costs some bits per list element. List size here is 10.

If RAN1 decides to increase the number or BFD resources a new list is anyway needed. Thus, another option would be to implement separately new BFD sets separate from RLM/legacy BFD resource configuration. Yet another option is to give separate BFD set only for the additional TRP. We present here three TPs to show examples of possible implementation.

-----------------------------------------------start TP1-----------------------------------------------------------------

#### –	RadioLinkMonitoringConfig

The IE RadioLinkMonitoringConfig is used to configure radio link monitoring for detection of beam- and/or cell radio link failure. See also TS 38.321 [3], clause 5.1.1.

RadioLinkMonitoringConfig information element

-- ASN1START

-- TAG-RADIOLINKMONITORINGCONFIG-START

RadioLinkMonitoringConfig ::=       SEQUENCE {

failureDetectionResourcesToAddModList   SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS

OPTIONAL, -- Need N

failureDetectionResourcesToReleaseList  SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS-Id

OPTIONAL, -- Need N

beamFailureInstanceMaxCount             ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10}                          OPTIONAL, -- Need R

beamFailureDetectionTimer               ENUMERATED {pbfd1, pbfd2, pbfd3, pbfd4, pbfd5, pbfd6, pbfd8, pbfd10}  OPTIONAL, -- Need R

...,

[[

beamFailureInstanceMaxCount2             ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10}                          OPTIONAL, -- Need R

beamFailureDetectionTimer2               ENUMERATED {pbfd1, pbfd2, pbfd3, pbfd4, pbfd5, pbfd6, pbfd8, pbfd10}  OPTIONAL -- Need R

]]

}

RadioLinkMonitoringRS ::=           SEQUENCE {

radioLinkMonitoringRS-Id            RadioLinkMonitoringRS-Id,

purpose                             ENUMERATED {beamFailure, rlf, both},

detectionResource                   CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

},

... ,

[[

BFDRSSetId             INTEGER {0..1}                          OPTIONAL -- Need R

]]

}

-- TAG-RADIOLINKMONITORINGCONFIG-STOP

-- ASN1STOP

RadioLinkMonitoringConfig field descriptions

BFRRSSetId ID for BFD resource when UE is configured with mTRP BFD operation. When this ID is configured for any radioLinkMonitoringRS for the UE, only the radioLinkMonitoringRS with this ID configured can be set to purpose bfd or both.

beamFailureDetectionTimer, beamFailureDetectionTimer2 Timer for beam failure detection (see TS 38.321 [3], clause 5.17). See also the BeamFailureRecoveryConfig IE. Value in number of "Qout,LR reporting periods of Beam Failure Detection" Reference Signal (see TS 38.213 [13], clause 6). Value pbfd1 corresponds to 1 Qout,LR reporting period of Beam Failure Detection Reference Signal, value pbfd2 corresponds to 2 Qout,LR reporting periods of Beam Failure Detection Reference Signal and so on.

beamFailureInstanceMaxCount, beamFailureInstanceMaxCount2 This field determines after how many beam failure events the UE triggers beam failure recovery (see TS 38.321 [3], clause 5.17). Value n1 corresponds to 1 beam failure instance, value n2 corresponds to 2 beam failure instances and so on.

failureDetectionResourcesToAddModList A list of reference signals for detecting beam failure and/or cell level radio link failure (RLF). The limits of the reference signals that the network can configure are specified in TS 38.213 [13], table 5-1. The network configures at most two detectionResources per BWP for the purpose beamFailure or both. If no RSs are provided for the purpose of beam failure detection, the UE performs beam monitoring based on the activated TCI-State for PDCCH as described in TS 38.213 [13], clause 6. If no RSs are provided in this list for the purpose of RLF detection, the UE performs Cell-RLM based on the activated TCI-State of PDCCH as described in TS 38.213 [13], clause 5. The network ensures that the UE has a suitable set of reference signals for performing cell-RLM.

RadioLinkMonitoringRS field descriptions

detectionResource A reference signal that the UE shall use for radio link monitoring or beam failure detection (depending on the indicated purpose). Only periodic 1-port CSI-RS can be configured on SCell for beam failure detection purpose.

purpose Determines whether the UE shall monitor the associated reference signal for the purpose of cell- and/or beam failure detection. For SCell, network only configures the value to beamFailure.

-----------------------------------------------end TP1-----------------------------------------------------------------

-----------------------------------------------start TP2-----------------------------------------------------------------

#### –	RadioLinkMonitoringConfig

The IE RadioLinkMonitoringConfig is used to configure radio link monitoring for detection of beam- and/or cell radio link failure. See also TS 38.321 [3], clause 5.1.1.

RadioLinkMonitoringConfig information element

-- ASN1START

-- TAG-RADIOLINKMONITORINGCONFIG-START

RadioLinkMonitoringConfig ::=       SEQUENCE {

failureDetectionResourcesToAddModList   SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS

OPTIONAL, -- Need N

failureDetectionResourcesToReleaseList  SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS-Id

OPTIONAL, -- Need N

beamFailureInstanceMaxCount             ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10}                          OPTIONAL, -- Need R

beamFailureDetectionTimer               ENUMERATED {pbfd1, pbfd2, pbfd3, pbfd4, pbfd5, pbfd6, pbfd8, pbfd10}  OPTIONAL, -- Need R

...,

[[

bfdSet                      BeamFailureDetectionSet     OPTIONAL,   --Need R

bfdSet2                      BeamFailureDetectionSet     OPTIONAL   --Need R

]]

}

RadioLinkMonitoringRS ::=           SEQUENCE {

radioLinkMonitoringRS-Id            RadioLinkMonitoringRS-Id,

purpose                             ENUMERATED {beamFailure, rlf, both},

detectionResource                   CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

},

...

}

BeamFailureDetectionSet  ::=     SEQUENCE {

BFDRSSetId             INTEGER {0..1}                          OPTIONAL, -- Need R

bfdResourcesToAddModList   SEQUENCE (SIZE(1..maxNrofBFDResourcePerSet)) OF RadioLinkMonitoringRS

OPTIONAL, -- Need N

bfdResourcesToReleaseList  SEQUENCE (SIZE(1..maxNrofBFDResources)) OF RadioLinkMonitoringRS-Id

OPTIONAL, -- Need N

beamFailureInstanceMaxCount             ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10}                          OPTIONAL, -- Need R

beamFailureDetectionTimer               ENUMERATED {pbfd1, pbfd2, pbfd3, pbfd4, pbfd5, pbfd6, pbfd8, pbfd10}  OPTIONAL, -- Need R

...

}

-- TAG-RADIOLINKMONITORINGCONFIG-STOP

-- ASN1STOP

RadioLinkMonitoringConfig field descriptions

bfdSet, bfdSet2 BeamfailureDetectionResource set for the UE for mTRP BFD operation, see [RAN1 reference].

beamFailureDetectionTimer Timer for beam failure detection (see TS 38.321 [3], clause 5.17). See also the BeamFailureRecoveryConfig IE. Value in number of "Qout,LR reporting periods of Beam Failure Detection" Reference Signal (see TS 38.213 [13], clause 6). Value pbfd1 corresponds to 1 Qout,LR reporting period of Beam Failure Detection Reference Signal, value pbfd2 corresponds to 2 Qout,LR reporting periods of Beam Failure Detection Reference Signal and so on.

beamFailureInstanceMaxCount This field determines after how many beam failure events the UE triggers beam failure recovery (see TS 38.321 [3], clause 5.17). Value n1 corresponds to 1 beam failure instance, value n2 corresponds to 2 beam failure instances and so on.

failureDetectionResourcesToAddModList A list of reference signals for detecting beam failure and/or cell level radio link failure (RLF). The limits of the reference signals that the network can configure are specified in TS 38.213 [13], table 5-1. The network configures at most two detectionResources per BWP for the purpose beamFailure or both. If no RSs are provided for the purpose of beam failure detection, the UE performs beam monitoring based on the activated TCI-State for PDCCH as described in TS 38.213 [13], clause 6. If no RSs are provided in this list for the purpose of RLF detection, the UE performs Cell-RLM based on the activated TCI-State of PDCCH as described in TS 38.213 [13], clause 5. The network ensures that the UE has a suitable set of reference signals for performing cell-RLM.

RadioLinkMonitoringRS field descriptions

detectionResource A reference signal that the UE shall use for radio link monitoring or beam failure detection (depending on the indicated purpose). Only periodic 1-port CSI-RS can be configured on SCell for beam failure detection purpose.

purpose Determines whether the UE shall monitor the associated reference signal for the purpose of cell- and/or beam failure detection. For SCell, network only configures the value to beamFailure.

-----------------------------------------------end TP2-----------------------------------------------------------------

-----------------------------------------------start TP3-----------------------------------------------------------------

#### –	RadioLinkMonitoringConfig

The IE RadioLinkMonitoringConfig is used to configure radio link monitoring for detection of beam- and/or cell radio link failure. See also TS 38.321 [3], clause 5.1.1.

RadioLinkMonitoringConfig information element

-- ASN1START

-- TAG-RADIOLINKMONITORINGCONFIG-START

RadioLinkMonitoringConfig ::=       SEQUENCE {

failureDetectionResourcesToAddModList   SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS

OPTIONAL, -- Need N

failureDetectionResourcesToReleaseList  SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS-Id

OPTIONAL, -- Need N

beamFailureInstanceMaxCount             ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10}                          OPTIONAL, -- Need R

beamFailureDetectionTimer               ENUMERATED {pbfd1, pbfd2, pbfd3, pbfd4, pbfd5, pbfd6, pbfd8, pbfd10}  OPTIONAL, -- Need R

... ,

[[

bfdSetSecondTRP                      BeamFailureDetectionSet     OPTIONAL   --Need R

]]

}

RadioLinkMonitoringRS ::=           SEQUENCE {

radioLinkMonitoringRS-Id            RadioLinkMonitoringRS-Id,

purpose                             ENUMERATED {beamFailure, rlf, both},

detectionResource                   CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

},

...

}

BeamFailureDetectionSet  ::=     SEQUENCE {

BFDRSSetId             INTEGER {1}                          OPTIONAL, -- Need R

bfdResourcesToAddModList   SEQUENCE (SIZE(1..maxNrofBFDResourcePerSet)) OF RadioLinkMonitoringRS

OPTIONAL, -- Need N

bfdResourcesToReleaseList  SEQUENCE (SIZE(1..maxNrofBFDResources)) OF RadioLinkMonitoringRS-Id

OPTIONAL, -- Need N

beamFailureInstanceMaxCount             ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10}                          OPTIONAL, -- Need R

beamFailureDetectionTimer               ENUMERATED {pbfd1, pbfd2, pbfd3, pbfd4, pbfd5, pbfd6, pbfd8, pbfd10}  OPTIONAL, -- Need R

...

}

-- TAG-RADIOLINKMONITORINGCONFIG-STOP

-- ASN1STOP

RadioLinkMonitoringConfig field descriptions

bfdSetSecondTRP BeamfailureDetectionResource set for the UE for mTRP BFD operation, see [RAN1 reference].

beamFailureDetectionTimer Timer for beam failure detection (see TS 38.321 [3], clause 5.17). See also the BeamFailureRecoveryConfig IE. Value in number of "Qout,LR reporting periods of Beam Failure Detection" Reference Signal (see TS 38.213 [13], clause 6). Value pbfd1 corresponds to 1 Qout,LR reporting period of Beam Failure Detection Reference Signal, value pbfd2 corresponds to 2 Qout,LR reporting periods of Beam Failure Detection Reference Signal and so on.

beamFailureInstanceMaxCount This field determines after how many beam failure events the UE triggers beam failure recovery (see TS 38.321 [3], clause 5.17). Value n1 corresponds to 1 beam failure instance, value n2 corresponds to 2 beam failure instances and so on.

failureDetectionResourcesToAddModList A list of reference signals for detecting beam failure and/or cell level radio link failure (RLF). The limits of the reference signals that the network can configure are specified in TS 38.213 [13], table 5-1. The network configures at most two detectionResources per BWP for the purpose beamFailure or both. If no RSs are provided for the purpose of beam failure detection, the UE performs beam monitoring based on the activated TCI-State for PDCCH as described in TS 38.213 [13], clause 6. If no RSs are provided in this list for the purpose of RLF detection, the UE performs Cell-RLM based on the activated TCI-State of PDCCH as described in TS 38.213 [13], clause 5. The network ensures that the UE has a suitable set of reference signals for performing cell-RLM.

RadioLinkMonitoringRS field descriptions

detectionResource A reference signal that the UE shall use for radio link monitoring or beam failure detection (depending on the indicated purpose). Only periodic 1-port CSI-RS can be configured on SCell for beam failure detection purpose.

purpose Determines whether the UE shall monitor the associated reference signal for the purpose of cell- and/or beam failure detection. For SCell, network only configures the value to beamFailure.

-----------------------------------------------end TP3-----------------------------------------------------------------

The third option has benefits that it keeps the RLM in original TRP which may be associated to the original PCI while the second TRP may be associated to additional TRP. This way it is clear that RLM stays within the original PCI.

RAN2 to agree to configure BFD sets separately for both TRPs as shown in TP3 and adopted in Running CR in R2-2110666.

## UE actions for beam failure detection

The UE actions related to Beam Failure Detection (BFD) are mainly specified in MAC specifications (TS 38.321). If the MAC entity is not configured with any SCell, there is one DL-SCH, one UL-SCH, and one RACH per MAC entity.

Beam failure is detected by counting beam failure instance (BFI) indication(s) from the lower layers to the MAC entity. If beamFailureRecoveryConfig is reconfigured by upper layers during an ongoing Random Access procedure for beam failure recovery for SpCell, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure using the new configuration.

The following UE variables are used for the beam failure detection procedure:

-	BFI_COUNTER (per Serving Cell): counter for beam failure instance indication which is initially set to 0.

The MAC entity shall for each Serving Cell configured for beam failure detection:

1>	if beam failure instance indication has been received from lower layers:

2>	start or restart the beamFailureDetectionTimer;

2>	increment BFI_COUNTER by 1;

2>	if BFI_COUNTER >= beamFailureInstanceMaxCount:

3>	if the Serving Cell is SCell:

4>	trigger a BFR for this Serving Cell;

3>	else:

4>	initiate a Random Access procedure (see clause 5.1) on the SpCell.

1>	if the beamFailureDetectionTimer expires; or

1>	if beamFailureDetectionTimer, beamFailureInstanceMaxCount, or any of the reference signals used for beam failure detection is reconfigured by upper layers associated with this Serving Cell:

2>	set BFI_COUNTER to 0.

1>	if the Serving Cell is SpCell and the Random Access procedure initiated for SpCell beam failure recovery is successfully completed (see clause 5.1):

2>	set BFI_COUNTER to 0;

2>	stop the beamFailureRecoveryTimer, if configured;

2>	consider the Beam Failure Recovery procedure successfully completed.

1>	else if the Serving Cell is SCell, and a PDCCH addressed to C-RNTI indicating uplink grant for a new transmission is received for the HARQ process used for the transmission of the BFR MAC CE or Truncated BFR MAC CE which contains beam failure recovery information of this Serving Cell; or

1>	if the SCell is deactivated as specified in clause 5.9:

2>	set BFI_COUNTER to 0;

2>	consider the Beam Failure Recovery procedure successfully completed and cancel all the triggered BFRs for this Serving Cell.

Parameters for BFR (e.g. candidate beam list to be selected upon BFR and/or threshold for beam selection, etc.) are configured in the BeamFailureRecoveryConfig Information Element (IE) via RRC

The MAC entity shall:

1>	if the Beam Failure Recovery procedure determines that at least one BFR has been triggered and not cancelled:

2>	if UL-SCH resources are available for a new transmission and if the UL-SCH resources can accommodate the BFR MAC CE plus its subheader as a result of LCP:

3>	instruct the Multiplexing and Assembly procedure to generate the BFR MAC CE.

2>	else if UL-SCH resources are available for a new transmission and if the UL-SCH resources can accommodate the Truncated BFR MAC CE plus its subheader as a result of LCP:

3>	instruct the Multiplexing and Assembly procedure to generate the Truncated BFR MAC CE.

2>	else:

3>	trigger the SR for SCell beam failure recovery for each SCell for which BFR has been triggered and not cancelled.

All BFRs triggered prior to MAC PDU assembly for beam failure recovery for an SCell shall be cancelled when a MAC PDU is transmitted and this PDU includes a BFR MAC CE or Truncated BFR MAC CE which contains beam failure information of that SCell.

In RAN2#115 RAN2 was discussing what happens when BFD is triggered in both TRPs. Further, RAN2 was discussing what does it actually mean if BFD is triggered in both TRPs as in there would be one TRP at the time triggering the BFD.

- For the case of intra cell (FFS for inter cell).

If beam failure is detected on both TRPs (i.e. BFD-RS sets) of an SCell, BFR is triggered for that SCell.

- - FFS whether UE transmits a) legacy BFR MAC CE or b) new BFR MAC CE indicating both failed TRPs as well as the beam failure recovery information for both TRPs.

If beam failure is detected on both TRPs (i.e. BFD-RS sets) of SpCell, random access procedure is initiated on SpCell.

- - FFS whether UE transmits a) legacy BFR MAC CE or b) new BFR MAC CE indicating both failed TRPs as well as the beam failure recovery information for both TRPs.

FFS what is meant in detail by “beam failure is detected on both TRPs”

One interpretation is that if BFD happened in one TRP and that is followed by BFD triggering in another TRP before any recovery due to first BFD has been finished, or any before recovery has been started. For Scell it is about transmitting a BFR procedure and for SPCell it is about random access procedure. Thus we propose the following. Note that for discussing SR transmission the configuration of PUCCH SR resources is needed first.

For SCells, when BF is detected on TRP and UE had already detected a BF on a TRP but has not finished the BFR procedure, cancel the ongoing BFR procedure and restart BFR procedure and send either legacy MAC CE or new BFD MAC CE indicating failure on both TRPs.

For SpCells, when BFD is detected on TRP and UE had already detected a BFD on a TRP but has not started random access procedure, UE should start the random access procedure.

For SpCells, when BF is detected on TRP and UE had already detected a BF on a TRP but has started random access procedure, UE should continue the random access procedure.

Beam failure recovery resource configuration up to two PUCCH RS resources per cell group. Further RAN1 is discussing about possible association of these resources to a TRP. RAN2 should discuss whether this impacts RAN2 specifications. This especially if the impact is in MAC specification as RRC specification impact is likely more straightforward to see from RAN1 parameter excel.

RAN2 should discuss possible impact on PUCCH-RS resource configuration for mTRP BFR.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RLM is not impacted in Rel-17.

Proposal 2	RAN2 to agree to configure BFD sets separately for both TRPs as shown in TP3 and adopted in Running CR in R2-2110666.

Proposal 3	For SCells, when BF is detected on TRP and UE had already detected a BF on a TRP but has not finished the BFR procedure, cancel the ongoing BFR procedure and restart BFR procedure and send either legacy MAC CE or new BFD MAC CE indicating failure on both TRPs.

Proposal 4	For SpCells, when BFD is detected on TRP and UE had already detected a BFD on a TRP but has not started random access procedure, UE should start the random access procedure.

Proposal 5	For SpCells, when BF is detected on TRP and UE had already detected a BF on a TRP but has started random access procedure, UE should continue the random access procedure.

Proposal 6	RAN2 should discuss possible impact on PUCCH-RS resource configuration for mTRP BFR.

# References

RP-211586, “Revised WID: Further enhancements on MIMO for NR,” 3GPP TSG RAN #92e, June 2021.

Agreement

- Evaluate enhancement to enable per-TRP based beam failure recovery starting with Rel-15/16 BFR as the baseline.

- Consider following potential enhancement aspects to enable per-TRP based beam failure recovery

  - Issue 1: TRP-specific BFD

  - Issue 2: TRP-specific new candidate beam identification

  - Issue 3: TRP-specific BFRQ

  - Issue 4: gNB response enhancement

  - Issue 5: UE behavior on QCL/spatial relation assumption/UL power control for DL and UL channels/RSs after receiving gNB response

Agreement

Study Rel.17 enhancements on beam management for multi-TRPs with following priority

- High priority:

  - Beam measurement/reporting enhancement

  - Beam failure recovery for multi-TRP

- Low priority

  - Simultaneous reception of same type of channel/RS with different QCL-TypeD

  - Simultaneous reception of different type of channel/RS with different QCL-TypeD

Agreement

- For M-TRP beam failure detection, support independent BFD-RS configuration per-TRP, where each TRP is associated with a BFD-RS set.

  - FFS: The number of BFD RSs per BFD-RS set, the number of BFD-RS sets, and number of BFD RSs across all BFD-RS sets per DL BWP

  - Support at least one of explicit and implicit BFD-RS configuration

    - With explicit BFD-RS configuration, each BFD-RS set is explicitly configured

      - FFS: Further study QCL relationship between BFD-RS and CORESET

    - FFS: How to determine implicit BFD-RS configuration, if supported

- For M-TRP new beam identification

  - Support independent configuration of new beam identification RS (NBI-RS) set per TRP if NBI-RS set per TRP is configured

    - FFS: detail on association of BFD-RS and NBI-RS

    - Support the same new beam identification and configuration criteria as Rel.16, including  L1-RSRP, threshold

Agreement

Support TRP-specific BFD counter and timer in the MAC procedure

- The term TRP is used only for the purposes of discussions in RAN1 and whether/how to capture this is FFS

Agreement

- Support a BFRQ framework based on Rel.16 SCell BFR BFRQ

  - In RAN1#104-e, select one from the following options

    - Option 1: Up to one dedicated PUCCH-SR resource in a cell group

      - A cell group refers to either MCG, SCG, or PUCCH cell group

      - FFS: number of spatial filters associated with the PUCCH-SR resources

      - FFS: How the SR configuration is done

    - Option 2:  Up to two (or more) dedicated PUCCH-SR resources in a cell group

      - A cell group refers to either MCG, SCG, or PUCCH cell group

      - FFS: whether each PUCCH-SR resource is restricted to be associated to one spatial filter

      - FFS: How the SR configuration is done

  - FFS: Whether no dedicated PUCCH-SR resource can be supported in addition to Option 1 or Option 2

- Study whether and how to provide the following information in BFRQ MAC-CE

  - Index information of failed TRP(s)

  - CC index (if applicable)

  - New candidate beam index (if found)

  - Indication whether new beam(s) is found

  - FFS: whether/how to incorporate multi-TRP failure

Agreement

For M-TRP BFR

Support 1-to-1 association between each BFD-RS set and an NBI-RS set

- FFS: Association details

Agreement

BFRQ response

- Support at least the same gNB response as in Rel.16 SCell BFR (i.e. DCI with toggled NDI scheduling a same HARQ process ID as the PUSCH carrying BFRQ MAC-CE)

Agreement

For BFRQ of M-TRP BFR

- Option 3: Up to two dedicated PUCCH-SR resources in a cell group

- FFS: Whether PUCCH-SR for SCell can be reused for M-TRP

- Support BFRQ MAC-CE that can convey information of failed CC indices, one new candidate beam for the failed TRP/CC (if found), and whether new candidate beam is found

  - Support at least indication of a single TRP failure

    - FFS: whether/what information of failed TRP(s) is conveyed in the MAC-CE

    - FFS: whether/how to support  indication of more than one TRP failure, corresponding BFR procedure, and applicable cell type (SCell vs. SpCell)

- FFS: UE behavior when TRP failure status is different across cells

- FFS: Whether PUCCH SR resource can be configured with 2 spatial relations

Agreement

- Support simultaneous configuration of cell-specific BFR and TRP-specific BFR in different CCs.

- FFS: whether cell-specific and TRP-specific BFR can be configured in the same CC.

Agreement

- Support S-DCI and M-DCI in TRP-specific BFR in Rel.17

  - S-DCI is low priority, M-DCI is high priority

  - Unified design for S-DCI and M-DCI should not be precluded due to the prioritization

Agreement

On BFD-RS of TRP-specific BFR

- BFD-RS resource number:

  - The total number of RSs in two BFR-RS sets per DL BWP is a UE capability

  - On the maximum number of RS per BFD-RS set, down-select from the following two alternatives in RAN1#105-e

    - Alt1: max value is 2

    - Alt2: max value is a UE capability, including possible candidate value of 1

Agreement

Adopt the following beam failure detection criteria for each BFD-RS set

- The physical layer in the UE assesses the radio link quality per BFD-RS set and indicates the BFD-RS set index to higher layers every X ms, if the hypothetical PDCCH BLER of all BFD-RS in the corresponding set of BFD-RS is higher than a threshold

  - X is max{minimal periodicity of BFD RS in the set, 2ms}

Agreement

A UE configured with TRP-specific BFR can be configured with 1 PUCCH-SR resource in a cell group

- NOTE: it has been agreed in RAN1#104-e that a UE can be configured with up to 2 PUCCH-SR resources in a cell group

Agreement

For the TRP specific BFR, for a UE configured with two PUCCH-SR resources in a cell group when beam failure is detected in a one or more CCs in one or more of BFD-RS sets configured in one or more of CCs,

- Down select one of the following PUCCH-SR resource selection rules when SR is triggered (or their combinations) for the study, without precluding other alternatives, in RAN1#105-e

  - Alt-1: PUCCH-SR resource associated with other/non-failed BFD-RS set, association details FFS

  - Alt-2: PUCCH-SR resource associated with failed BFD-RS set, association details FFS

  - Alt-3: Leave it up to UE implementation

- Note: PUCCH-SR resource is PUCCH resource carrying SR

- FFS: Whether two PUCCH-SR resources are under the same or different SR resource configuration or SR configuration (eventual decision may or may not happen in RAN1)

Agreement

If enhanced SFN PDCCH transmission scheme (scheme 1 or TRP-based pre-compensation) is configured and two TCI states are activated for at least one CORESET, support the following configuration of RS for BFD

- Down-select one alternative for implicit configuration

  - Alt 1-2: RS of CORESETs with both single and two TCI states are used

  - Alt 1-3: RS of CORESETs with only two TCI states are used

- Down-select one alternative for explicit configuration

  - Alt 2-1: Support defining CSI-RS resource or SSB pairs as BFD RS

    - FFS other details

  - Alt 2-2: Reuse the existing Rel-15/Rel-16 approach for BFD RS configuration

- Note: down-selection can be done separately for Rel-15/16 cell specific BFR and Rel-17 TRP-specific BFR, Rel-17 TRP-specific BFR to be discussed under AI 8.1.2.3

103:

  - Intra-frequency scenario:

    - The SSBs of non-serving cells have the same center frequency and SCS as the SSBs of the serving cell

    - An SSB of a non-serving cell is associated with a PCI different from the PCI of the serving cell

    - FFS: Support for inter-frequency scenario

104:

Agreement

For inter-cell MTRP operation, further discuss following options and down select in RAN1#104bis-e

- Option1: Indicate/associate non-serving cell PCI in the TCI state

  - FFS other non-serving cell information

Agreement

Agree on scheme1

- Scheme1: PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI

- FFS: whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- FFS: whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB

104bis:

Agreement

- For intercell MTRP operation, 1 additional PCI different from the serving cell PCI is supported per CC

  - The additional PCI is the one associated with one or more TCI states that are activated for [CSI-RS for CSI]/PDSCH/PDCCH, per CC.

  - Applicable at least for non-cross carrier QCL indication

    - FFS: Cross carrier scheduling QCL indication

- RAN1 to decide on the maximum number of PCIs different from the serving cell PCI per CC and/or across all CCs that can be RRC-configured for multi-DCI based inter-cell multi-TRP

- Above should be specified by reusing R15 QCL rules as concluded in RAN1#104-e

Conclusion

Configuration of CSI-RS for mobility as QCL source for intercell MTRP operation is not supported from Rel-17 specifcation point of view

Agreement

For intercell MTRP operation, downselect one or more of the following alternatives in RAN1#105-e

- Alt1: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with only one CORESETPoolIndex

- Alt2: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with more than one CORESETPoolIndex

- Alt3: one PCI associated with TCI states for [PDSCH]/PDCCH via QCL relationship without association with CORESETPoolIndex

Note: This agreement is not related to the down-selection of one of the 5 options from RAN1#104-e

Note: Above should be specified by reusing Rel-15/Rel-16 QCL rules as concluded in RAN1#104-e