3GPP TSG RAN WG2 Meeting #116-e                	R2-2110353

# Online meeting, 1-12 November 2021

Agenda item:		8.9.3

Source:			Sony

Title:		Discussion on dedicated signaling of TRS/CSI-RS configuration

Document for:	 	Discussion and decision

# Introduction

A Work Item on UE power saving enhancements was agreed at RAN plenary #86 [1]. One of the objectives in the WID is the following;

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #100 meeting - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1] - NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required

Related to the objective 1b above, RAN1 made an agreement in RAN1#103-e meeting on the high level principle of signalling method for the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE (s) in Rel-17 UE power saving enhancements [2].

Agreements:

- SIB signaling provides the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s).

  - Up to RAN2 to decide which SIB is to be used.

  - Whether or not to additionally support other high-layer signaling methods (e.g., dedicated RRC, RRC release message, etc.) is up to RAN2

The agreement is summarized in an LS with an action for RAN2 to take the signalling method for the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s) into account in the corresponding RAN2 study.

At the RAN2#113e meeting, the following agreements were made [3];

[041] On signalling providing the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s): - SIB signalling is the baseline; - Other dedicated high-layer signalling methods (e.g., dedicated RRC, RRC release message, etc.) can be additionally considered with justification. It is assumed they do not work alone. [041] RAN2 will down select from the following options on SIB signalling providing the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s): - Option 2: Existing SIB, other than SIB1; - Option 3: New SIB type, e.g. SIB-x;

At RAN2#113bis-e [4], and RAN2#114-e [5], no discussions were held related to TRS/CSI-RS.

At RAN2#115e [6], the following agreements were made.

The TRS/CSI-RS configuration is provided in a new SIB. RAN2 assumes that TRS/CSI-RS configurations are broadcasted. Potential addition of dedicated signalling can be discussed in a later meeting based on company contributions. The legacy SI update procedure is used for changing TRS/CSI-RS configurations. Postpone the topic about TRS/CSI-RS availability until a later meeting when RAN1 also has progressed. On demand SI should be possible for the SIB with TRS/CSI-RS information. Postpone the discussion on segmentation of the new SIB until RAN1 has sent the list of the parameters and a potential structure. Postpone the discussion on splitting the TRS/CSI-RS information to a common and RS-specific part until RAN1 has sent the list of the parameters and a potential structure.

In this contribution, we discuss the dedicated signaling method for the TRS/CSI-RS configuration for idle/inactive mode UEs.

# Discussion on dedicated signalling

The SIB for TRS/CSI-RS configuration is categorized as on-demand SI in which it will be sent to the UE once the gNB has received SI request. This is particularly useful for a UE which moves to a new cell. Once the UE enters a new cell and still operating in idle mode, the UE can request the system information carrying TRS/CSI-RS configuration.

There is also another scenario where a connected mode UE in a cell changes its state to idle/inactive mode. This particular UE has received TRS/CSI-RS configuration and uses it when the UE is in connected mode. It is expected that the UE may re-use the same TRS/CSI-RS configuration in idle/inactive state. Hence, it may not be efficient if this particular UE needs to acquire SI containing an identical TRS/CSI-RS configuration for idle/inactive.

A more compact information can be indicated to that UE or even just an indication to re-use the TRS configuration used when in Connected mode

Observation 1 – A UE that moves from connected to idle/inactive mode can in principle re-use the TRS/CSI-RS that has been used during connected mode.

A UE that was in connected mode (therefore has acquired TRS/CSI-RS for connected mode) may not necessarily acquire the TRS/CSI-RS by requesting OSI containing TRS/CSI-RS configuration. This is particularly the case where the UE in idle mode can continue to use the same TRS/CSI-RS configuration that it had when it was in connected mode. There can be, for example, a flag in a dedicated RRC or RRC release message indicating that once the UE in idle/inactive mode, the UE can re-use the same TRS/CSI-RS. Here, we basically propose that the TRS/CSI-RS configuration is signaled in a dedicated RRC/RRC release message.

According to the WID, idle/inactive mode UEs can re-use TRS/CSI-RS occasion(s) available in connected mode (of other UEs). The TRS/CSI-RS configuration for connected mode UEs is provided via UE specific RRC messaging. Hence, in practice, two or more connected mode UEs can receive different TRS/CSI-RS configurations or basically the gNB transmits multiple TRS/CSI-RS with different configurations. It would be beneficial if idle/inactive mode UE know the existence of the multiple TRS/CSI-RS configurations.

Proposal 1: Support TRS/CSI-RS indication and possible alternative TRS/CSI-RS configuration in dedicated RRC/RRC release message.

# Conclusion

In this contribution, the following observations and proposals are made.

Observation 1 – A UE that moves from connected to idle/inactive mode can in principle re-use the TRS/CSI-RS that has been used during connected mode.

Proposal 1: Support TRS/CSI-RS indication and possible alternative TRS/CSI-RS configuration in dedicated RRC/RRC release message.

# Reference

- RP-193239, New WID: UE Power Saving Enhancements.

- R1-2009848 - LS on signalling method for TRS/CSI-RS occasion(s) for idle/inactive UE(s)

- Minutes RAN2#113e meeting

- Minutes RAN2#113bis-e meeting

- Minutes RAN2#114e meeting

- Minutes RAN2#115e meeting