3GPP TSG-RAN2 Meeting #116-e	R2-2110409

eMeeting, 1st – 12th November, 2021

Agenda Item:	8.1.2.2 Scheduling and power saving

Source: 	Ericsson

Title:  	Aspects on Multicast Inactivity

Document for:	Discussion and Decision

# Introduction

The following topics related to inactivity with multicast transmissions are discussed in this contribution:

- Multicast session deactivation

- Short DRX cycle and PTM DRX

- DRX command MAC CE and PTM DRX

# Background

RAN2 reached the following agreements concerning of multicast session activation/deactivation to the UE:

[037] If the UE which joined the multicast session is in RR CONNECTED state when the session is started, the gNB sends RRC Reconfiguration message with relevant MBS configuration to the UE and there is no need for separate session start notification for this UE. FFS for session activation.

There is Support to have group notification for multicast for MBS supporting nodes (e.g. paging)

For non-supporting nodes, using MBS session ID will not work as it would impact non-MBS nodes. Unicast paging would work.

Use PCCH for Multicast activation notification (also for MBS supporting nodes).

Will not support PTM deactivation/activation beyond RRC reconfiguration acc to first agreement above (and whatever R1 decides).

If Data Inactivity timer is configured, data monitoring is applied both for unicast and MBS multicast (i.e. both PTM and PTP data) (but not MBS broadcast)

NAS is expected to inform UE about multicast session release (e.g. to stop monitoring for multicast session activation).

RAN2 reached the following agreements for multicast DRX:

FFS whether For PTM transmission of NR MBS, DRX scheme is independent of DRX for unicast transmission, e.g. supported on a per G-RNTI basis

FFS whether For PTP transmission, DRX operation for unicast transmission is reused.

For multicast PTM transmission, Multicast DRX pattern is configured on a per G-RNTI basis (i.e. independent of legacy UE-specific DRX for unicast transmission).

Legacy UE-specific DRX pattern for unicast is reused for PTP transmission of NR MBS, which means the UE specific DRX pattern are for both unicast services and the MBS PTP bearer of UE

Multicast long DRX support is baseline for PTM. FFS whether to support optional short DRX or not.

The Multicast Long DRX operation has to support the following parameters which are  similar to the UE-specific DRX for unicast, where the last two parameters are needed if the HARQ- feedback is enabled:

  - drx-onDurationTimerPTM, drx-InactivityTimerPTM, drx-LongCycleStartOffsetPTM, drx-SlotOffsetPTM, drx-HARQ-RTT-TimerDLPTM , drx-RetransmissionTimerDLPTM

For NR Broadcast, the DRX pattern is configured per G-RNTI.

For NR Broadcast, DRX configuration includes: drx-onDurationTimerPTM, drx-SlotOffsetPTM, drx-InactivityTimerPTM, drx-CycleStartOffsetPTM.

# Discussion

## Multicast session deactivation

MBS session state

The possible states of a multicast session are described in TS 23.247, and depicted in the figure below:

Figure 4.3-4 from TS 23.247: Multicast session states and state transitions in NG-RAN

Multicast data transmission only take place when the multicast session is active and only to UEs which joined the session. Session activation and deactivation is triggered by the 5GC. After the session has been deactivated there is no multicast transmission, until the session is activated again. UEs in CM-IDLE state and CM-CONNECTED with RRC Inactive state that joined the multicast session are notified via (group) paging.

Inactivity duration

Dependent on the duration of inactivity there are different functions and features available to use. For example with increasing inactivity duration:

- PDCCH monitoring adaptation (Rel-16 DCP and Rel-17 PDCCH skip and searchspace switching)

- Connected mode DRX (incl DRX command MAC CE)

- RRCReconfiguration (e.g. DRX, MRB, ...)

- RRCRelease (e.g. after multicast/unicast inactivity)

There are potential scalability issues with the last two options when the multicast group is very large. RAN2 agreed to down prioritize multicast reception in idle/inactive, and PRACH enhancement with group paging, i.e. there can be scalability issues to handle large groups of multicast UEs in connected mode in Rel-17:

Observation 1. There can be scalability issues to handle large groups of multicast UEs in Rel-17.

The user plane of a PDU session can be activated/deactivated, and this functionality is also inherited for multicast session (TS 29.502). It is assumed that the session inactivity timer that is used in the CN to trigger a deactivation request is based on CN implementation, similar as the inactivity timer in RAN is based on RAN implementation. But CN deactivation only considers inactivity for a single multicast session, while RAN considers data inactivity for a UE including both unicast and multicast and potentially multiple concurrent sessions.

Observation 2: The CN and RAN inactivity timers are implementation dependent with a different scope.

When the RAN receives a deactivation signal from the CN, it is (only) clear that CN inactivity timer (unknown value) has expired. In our view it is not obvious that the RAN should release the multicast UEs when the session is deactivated (assuming there is no unicast traffic either). Potentially the CN may deactivate more agressively compared to the RAN (releasing the UE), because resources can be re-established in the CN more quickly compared to RAN.

At any time after deactivation the CN may send an activation signal and start data transmission again. Reception of a deactivation signal in RAN is no guarantee that future activation signal is delayed for a minimum time:

Observation 3: Soon after deactivation the CN may activate the session again and start transmitting multicast data which may cause UEs that are released to lose data.

When UEs are released upon session deactivation then the UEs need to paged to return to connected mode when the session is activated again. The main delay component for the UEs to return to connected mode is the DRX cycle that is used in idle/inactive mode. When released to inactive mode the RAN could configure a smaller RAN paging cycle to reduce this delay. For UEs in RRC_IDLE mode this is not possible, i.e. there is a cell specific defaultPagingCycle in SIB1 which is used by all UEs in the cell.  The main delay component for a UE to return from idle/inactive mode to connected mode when the session is activated (again) is the DRX cycle that is used in idle/inactive mode, also for small groups.

But when the multicast group is very large there could be additional delay due to congestion during random access. There is the possiblity to usea Backoff Indicator in the RAR response, to spread/delay access attempts. But when the group of UEs that has to return to connected mode is very large, and congestion during random access delays the return, the latency will not be controlled nor limited. The latency will depend on the size of the group, even when new random access enhancements are agreed to regulate the random access. Furthermore when the large group consists solely of prioritized users (e.g. MCPTT) then there is also no solace in trying to block or reject lower priority users. With large multicast groups, there is a risk that the UE misses multicast data when the UE is released and the session is activated (again):

Observation 4: The gNB can decide not to release a multicast UE in connected mode to avoid potential latency issues and loss of data for the UE to return to connected mode when the session is activated again and data transmission is re-started.

When the session is in active state, then it is reasonable to assume in RAN that there will be multicast data to transmit. Furthermore there is PDCCH monitoring adaptation and DRX for short data inactivity while in connected mode:

Observation 5: Short data inactivity during an active multicast session is handled by connected mode DRX.

The gNB actions when the multicast session is deactivated should be left to gNB implementation. The gNB has different means to make an educated decision what to do (and what not to do):

- QoS (including 5QI) info including latency and reliability requirements (e.g. sessions with tight latency requirements are kept in connected mode and DRX is not reconfigured).

- gNB knows which and how many UEs in connected mode have joined the multicast session based on the multicast info in the UE context.

- gNB can configure a short RAN paging cycle if the multicast UE is released to Inactive mode.

- gNB can reconfigure the connected mode MBS DRX to be more power efficient (e.g. with a longer DRX cycle and/or shorter drx-InActivityTimer, wakeup signalling, etc.).

The gNB actions (e.g. release, reconfigure or no action) when the multicast session is deactivated should be left to RAN implementation:

Proposal 1: gNB actions when a multicast session is deactivated are left to gNB implementation.

RAN2 agreed to not support PTM deactivation/activation beyond RRC reconfiguration. In section 3.3 the use of DRX command when the session is deactivated is discussed further.

## Short DRX cycle and PTM DRX

Fallback into Short DRX before Long DRX is entered can be considered a trade-off between latency and UE power saving, i.e. a longer Long DRX can be configured when the UE first quickly drops into a Short DRX cycle, and if there indeed no more traffic, the UE can safely drop into a (long) Long DRX cycle.

Support for Short DRX cycle is optional with explicit UE capability signalling for unicast in NR. For inter-operability all the UEs in the MBS group would need to support short DRX cycle. Potentially short DRX can be used for voice during a talk burst, while long DRX is used during a silence period. For inter-operability reasons all UEs in the group would need to support short DRX. If some UEs in the PTM group are in short DRX while others are in long DRX, then all UEs in the group wake-up at the same time, but some wake-up more frequent then others:

Proposal 2: Support short DRX with PTM DRX provided that Short DRX support is mandatory for UE supporting multicast.

## DRX command MAC CE and PTM DRX

When the UE receives the DRX command MAC CE, then the UE stops the drx-onDurationTimer and drx-InactivityTimer, i.e. the UE stops monitoring the PDCCH with G-RNTI. The MAC CE signalling may fail, i.e. in a large group there could be one or two UEs that missed the command. However the negative effect for those (few) UEs is that they have to continue monitor the PDCCH with G-RNTI until the timers expire, i.e. consume more power, but such mismatch between UE and network does not create inter-operability issues, i.e. all the UEs in the group are still reachable during the Active Time.

The DRX command can be used when the gNB knows there is no more data for some time. When the CN indicates that the session is deactivated the gNB knows that there is no more multicast data until the session is activated again, i.e. this could be an opportunity to use the DRX command and put the UEs immediately to sleep:

Proposal 3: When the UE receives a DRX command MAC CE with DCI scrambled with G-RNTI then the UE stops drx-onDurationTimerPTM and drx-InactivityTimerPTM timer for that G-RNTI.

We do not think that it is likely that two sessions are deactivated at the same time, i.e. we assume that these events are not correlated.

# Summary

RAN2 is kindly asked to discuss inactivity with multicast:

Observation 1. There can be scalability issues to handle large groups of multicast UEs in connected mode in Rel-17.

Observation 2: The CN and RAN inactivity timers are implementation dependent with a different scope.

Observation 3: Soon after deactivation the CN may activate the session again and start transmitting multicast data which may cause UEs that are released to lose data.

Observation 4: The gNB can decide not to release a multicast UE in connected mode to avoid potential latency issues and loss of data for the UE to return to connected mode when the session is activated again and data transmission is re-started.

Observation 5: Short data inactivity during an active multicast session is handled by connected mode DRX.

Proposal 1: gNB actions when a multicast session is deactivated are left to gNB implementation.

Proposal 2: Support short DRX with PTM DRX provided that Short DRX support is mandatory for UE supporting multicast.

Proposal 3: When the UE receives a DRX command MAC CE with DCI scrambled with G-RNTI then the UE stops drx-onDurationTimerPTM and drx-InactivityTimerPTM timer for that G-RNTI.

# References

- R2-2110319, [Post115-e][092][MBS] Remaining User plane issues, Report, Lenovo, RAN2#116-e (draft)

- R2-2108926, 38.321 running CR for NR MBS, running CR 38.331, OPPO, RAN2#115-e