3GPP TSG-RAN WG2 Meeting#116-e		       					       R2-2110467

Electronic, 1st - 12th November 2021

Source: 			ZTE corporation, Sanechips

Title: 	UE location report and TAC in NTN

Agenda item:		8.10.3.1

Document for: 	Discussion and Decision

- Introduction

There has been discussion on the UE location report and TAC in NTN in the past meetings. In this paper, we share some further analysis on some stage 3 details.

- Discussion

  - UE location report

    - Coarse location report in Msg5

- Issue 1: ASN.1 for the coarse location

The following location coordinate types have been defined for the location report for MDT [1] [2].

CommonLocationInfo-r16 ::= SEQUENCE {

gnss-TOD-msec-r16          OCTET STRING     OPTIONAL,

locationTimestamp-r16      OCTET STRING     OPTIONAL,

locationCoordinate-r16     OCTET STRING     OPTIONAL,

locationError-r16          OCTET STRING     OPTIONAL,

locationSource-r16         OCTET STRING     OPTIONAL,

velocityEstimate-r16       OCTET STRING     OPTIONAL

}

LocationCoordinateTypes ::= SEQUENCE {

ellipsoidPoint											BOOLEAN,

ellipsoidPointWithUncertaintyCircle						BOOLEAN,

ellipsoidPointWithUncertaintyEllipse					BOOLEAN,

polygon													BOOLEAN,

ellipsoidPointWithAltitude								BOOLEAN,

ellipsoidPointWithAltitudeAndUncertaintyEllipsoid		BOOLEAN,

ellipsoidArc											BOOLEAN,

...,

[[

highAccuracyEllipsoidPointWithUncertaintyEllipse-r15

BOOLEAN		OPTIONAL, -- Need ON

highAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15

BOOLEAN		OPTIONAL  -- Need ON

]]

}

Figure 1: Location coordinate types

We understand the coarse location report is mainly used for CN selection and construction of cell ID mapped to geographical area in RAN3, thus an ellipsoid point, i.e. figure 1-(1), would be sufficient.

Proposal 1: An ellipsoid point with longitude and latitude should be introduced in Msg5 for coarse UE location report.

-- ASN1START

Ellipsoid-Point ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607)		-- 24 bit field

}

-- ASN1STOP

In the existing ASN.1 as copied above, the co-ordinates of an ellipsoid point are coded with an uncertainty of less than 3 metres. The latitude is coded with 24 bits: 1 bit of sign and a number between 0 and 223-1 coded in binary on 23 bits. The relation between the coded number N and the range of (absolute) latitudes X it encodes is the following (X in degrees):

except for N=223-1, for which the range is extended to include N+1.

The longitude, expressed in the range -180°, +180°, is coded as a number between -223 and 223-1, coded in 2's complement binary on 24 bits. The relation between the coded number N and the range of longitude X it encodes is the following (X in degrees):

As agreed in RAN#114e and RAN2#115e that the coarse GNSS coordinates with ~2km accuracy will be reported in Msg5, the following ASN.1 can be considered:

-- ASN1START

Ellipsoid-Point ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8191),			-- 13 bit field

degreesLongitude			INTEGER (-8192..8191)		-- 14 bit field

}

-- ASN1STOP

The latitude is coded with 14 bits: 1 bit of sign and a number between 0 and 213-1 coded in binary of 13 bits. The relation between the coded number N and the range of (absolute) latitudes X it encodes is the following (X in degrees):

except for N=213-1, for which the range is extended to include N+1.

The longitude, expressed in the range -180°, +180°, is coded as a number between -213 and 213-1, coded in 2's complement binary on 14 bits. The relation between the coded number N and the range of longitude X it encodes is the following (X in degrees):

Proposal 2: The following ASN.1 is taken as a baseline for the coarse location report in Msg5:

-- ASN1START

Ellipsoid-Point ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8191),			-- 13 bit field

degreesLongitude			INTEGER (-8192..8191)		-- 14 bit field

}

-- ASN1STOP

- Issue 2: Report condition of the coarse location via Msg5

Although UE with GNSS capability has been assumed for NTN in Rel-17, UE may not have the location information available all the time, thus we understand UE should include the coarse location information in Msg5 if it is available at UE side.

Proposal 3: UE should include the coarse location information in Msg5 if it is available at UE side.

    - GNSS based location report in connected mode

Issue 1: Procedure for the GNSS based location report

The following agreements have been made in RAN#115e on the GNSS based location report in connected mode:

After AS security is established, gNB can obtain a GNSS-based location information from the UE using existing signalling method, i.e., by configuring includeCommonLocationInfo in the corresponding reportConfig.

Event triggered-based UE location reporting are configured by gNB to obtain UE location update of mobile UEs in RRC_CONNECTED.

Periodic location reporting can also be configured by gNB to obtain UE location update of mobile UEs in RRC_CONNECTED. RAN2 discuss whether it is part of existing periodic measurement report configuration or a new configuration for periodic reporting of UE location.

In the existing location report for immediate MDT, the measurement report is extended to carry location information. Thus, the location report for MDT via measurement report can either be event triggered or periodical. In addition, the following agreement has also been made for location triggered measurement report:

Specify that measurement reports can be configured to be piggybacked with location report when location based event triggers it.

With the location based event introduced, we understand measurement report carrying location information would cover all the possible triggers of location report in NTN thus prefer to reuse this procedure.

Issue 2: Whether to allow UE location report without RRM measurement results?

Considering that some fields are mandatory present in measResults, e.g. measResultServingMOList, UE location report via measurement report message is not possible. In addition, we understand there is no harm for having at least the RRM measurement results for the serving cell because there would be no additional delay if the RRM based event is configured appropriately. Thus, we understand there is no need to support UE location report without RRM measurement results and this is not a strong motivation to introduce new procedure only for UE location acquisition.

Proposal 4: The existing procedure for location report is reused in NTN, which includes:

- Configuration

  - reportConfig->includeCommonLocationInfo-r16(EventTrigger/PeriodicalReport)

  - OtherConfig -> obtainCommonLocation-r16

- Report: MeasurementReport message -> measResults -> locationInfo-r16

  - TAC

    - Introduction of NTN TAC list

On the introduction of NTN TAC list, the following options can be considered:

- Option 1: Reuse the existing TAC field, i.e. trackingAreaCode, and the TACs per PLMN for a NTN cell would be: trackingAreaCode + trackingAreaCodeList-NTN

| TN cell supports PCell functionality |  | TN cell only supports PSCell/SCell functionality |  | NTN cell supports PCell functionality |  | NTN cell only supports PSCell/SCell functionality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| trackingAreaCode | √ | trackingAreaCode | × | trackingAreaCode | √ | trackingAreaCode | × |
| trackingAreaCodeList-NTN | × | trackingAreaCodeList-NTN | × | trackingAreaCodeList-NTN | √ | trackingAreaCodeList-NTN | × |

- Option 2: Introduce new TAC list for NTN without using the existing field. The TACs per PLMN for a NTN cell would be: trackingAreaCodeList-NTN

| TN cell supports PCell functionality |  | TN cell only supports PSCell/SCell functionality |  | NTN cell supports PCell functionality |  | NTN cell only supports PSCell/SCell functionality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| trackingAreaCode | √ | trackingAreaCode | × | trackingAreaCode | × | trackingAreaCode | × |
| trackingAreaCodeList-NTN | × | trackingAreaCodeList-NTN | × | trackingAreaCodeList-NTN | √ | trackingAreaCodeList-NTN | × |

The main difference between option 1 and 2 is for the case when a NTN cell supports PCell functionality and the behaviour of legacy UEs would be different.

- In option 1, both the existing TAC field and the additional TAC list for NTN will be broadcast. Legacy UE, e.g. R15 and R16 UEs, would consider this NTN cell as a PCell and try to get access to it.

- In option 2, the existing TAC field will be absent while the additional TAC list for NTN will be broadcast. Legacy UE, who can only decode the existing TAC field, would consider this cell as a cell supporting only PSCell/SCell functionality and will not try to access this cell in a standalone way.

Thus, we understand whether we go for option 1 or option 2 is closely related to whether we would like to allow legacy UEs to access a NTN cell. If the answer is “Yes”, then we need to go for option 1; Otherwise, we should go for option 2.

Considering that access to a NTN cell requires some operation at UE side, e.g. pre-compensation, we understand the legacy UE not supporting such operation would not be able to access a NTN cell successfully, so we prefer not to allow such access so that legacy UE will not give a try and bear the pain of failure. Thus, option 2 is preferred from our perspective.

Proposal 5a: A new TAC list, allowing more than one TACs per PLMN, should be introduced for NTN.

Proposal 5b: For a NTN cell supports PCell functionality (per PLMN), the existing trackingAreaCode will be absent while all the TAC(s) will be broadcast in the new TAC list.

Proposal 5c: For a NTN cell only supports PSCell/SCell functionalit (per PLMN), neither the existing trackingAreaCode nor the new TAC list will be broadcast.

    - TAC in ULI

There has been discussion on how to include the TAC in ULI when more than one TACs are broadcast to try to answer RAN3’ question on this without reaching consensus as one candidate solution requires input from CT1. The following solutions have been mentioned:

- Option 1: gNB selects one from the broadcast TACs based on the reported UE coarse location in Msg5

- Option 2: UE NAS layer select one TAC from the broadcast TAC(s) based on its location and report to NW in Msg5, which will be included in ULI later

- Option 3: gNB include all the broadcast TACs in the ULI

We understand option 1 requires gNB to perform the mapping between UE location and TAC and would only be applicable for the case when UE coarse location is reported via Msg5. Option 2, is applicable for the case when CT1 confirms that UE would select TAC based on its location. Option 3 would apply for all the cases and if CN wants the UE location to decide where to page UE, the LCS procedure can be triggered and CN can map UE location into TAC as the mapping relation between TAC and geographical area would be available at least at CN.

With above consideration, we slightly prefer option 3 as it is applicable for all the cases, including the case UE does not have location information available and does not require gNB to store the mapping relationship between TAC and geographical area and perform the mapping.

Proposal 6: All the broadcast TACs per PLMN should be included in the ULI and sent to CN.

- Conclusion and proposals

With the above analysis, we have the following conclusions and proposals:

Proposal 1: An ellipsoid point with longitude and latitude should be introduced in Msg5 for coarse UE location report.

Proposal 2: The following ASN.1 is taken as a baseline for the coarse location report in Msg5:

-- ASN1START

Ellipsoid-Point ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8191),			-- 13 bit field

degreesLongitude			INTEGER (-8192..8191)		-- 14 bit field

}

-- ASN1STOP

Proposal 3: UE should include the coarse location information in Msg5 if it is available at UE side.

Proposal 4: The existing procedure for location report is reused in NTN, which includes:

- Configuration

  - reportConfig->includeCommonLocationInfo-r16(EventTrigger/PeriodicalReport)

  - OtherConfig -> obtainCommonLocation-r16

- Report: MeasurementReport message -> measResults -> locationInfo-r16

Proposal 5a: A new TAC list, allowing more than one TACs per PLMN, should be introduced for NTN.

Proposal 5b: For a NTN cell supports PCell functionality (per PLMN), the existing trackingAreaCode will be absent while all the TAC(s) will be broadcast in the new TAC list.

Proposal 5c: For a NTN cell only supports PSCell/SCell functionalit (per PLMN), neither the existing trackingAreaCode nor the new TAC list will be broadcast.

Proposal 6: All the broadcast TACs per PLMN should be included in the ULI and sent to CN.

- References

- TS 38.331

- TS 37.355