3GPP TSG-RAN2 #116-e	R2-2111068

# Electronic meeting, November 01 – November 12, 2021

Agenda item:		‎6.1.1

Source:	CATT, Huawei, HiSilicon

Title: 	Discussion on downlink unmapped QoS flows

Document for:	Information

1.	Introduction

In [1], RAN3 ask RAN2 to discuss whether it is possible to use the default DRB to deliver downlink packets for a QoS flow without ‎association to any DRB yet. This issue is discussed in the contribution. In one accompanying contribution [2] a draft reply LS is also provided for discussion.

2.	Discussions

Background

As background, the issue concerns A.3‎	New QoS Flow with Explicit RRC Signalling in TS 38.300. In particular in section A.3 in the TS, an example is provided message flow when explicit RRC signalling is used for a new QoS flow. In the example, the gNB receives from UPF a first downlink packet associated with a QFI, for which the QoS parameters are already ‎known from the PDU session establishment, but for which there is no association to any DRB yet in AS.‎ The following steps are listed in the TS, i.e.,

0. PDU session and DRB(s) have been already established. 1. gNB receives a downlink packet with a new QFI from UPF. 2. gNB decides to send the new QoS flow over an existing DRB using explicit RRC signalling for updating the AS mapping rules. 3. gNB sends an RRCReconfiguration message to UE with the new QFI to DRB mapping rule. gNB may also decide to update the DRB configuration if required to meet the QoS requirements for the new QoS Flow. 4. UE updates the QFI to DRB mapping rules and configuration (if received). 5. UE sends an RRCReconfigurationComplete message to gNB. 6. User Plane Data for the new QoS flow can then be exchanged between UE and gNB over the DRB according to the updated mapping rules and between UPF and gNB over the tunnel for the PDU session.

The main question in the LS [1] is on how to trigger Step 2 and/or 3 since it is the gNB-CU-UP to receive the first DL packets from the UPF while it ‎is the gNB-CU-CP to decide the QoS flow to DRB mapping and to send the RRCReconfiguration message. ‎According to the LS, there is consensus in RAN3 that the gNB-CU-UP should notify the gNB-CU-CP when such case happens‎.

So as requested, RAN2 should discuss whether/how gNB-CU-UP delivers DL packets to UE before the new QFI to DRB mapping is configured for the QoS flow.

Discussions

According the SDAP specification, default DRB is used for uplink, see for example section 5.2.1, 5.3.1, and 5.3.2 of TS 37.324.

On the other hand, one may ask what the real specification/UE impact is if in this case the default DRB is also used to transfer the DL packets. In our understanding this is achievable based on gNB’s implementation. More specifically if gNB decides to deliver the DL packets to the UE before new QFI is configured to a DRB, it just set the SDAP header (RDI, QFI, etc.) properly. Accordingly, the UE upon reception of an SDAP data PDU just retrieves the SDAP SDU and deliver it with the QFI value to high layer, even though the QFI value may not have been configured with DRB mapping. There seems to be no impact to the procedures in section 5.2.1, 5.3.1, and 5.3.2 of TS 37.324.

Based on these we come to the following proposal.

Proposal 1	RAN2 discuss and confirm that gNB implementation allows delivery of DL packets to UE (via default DRB or via dedicated DRB) before the new QFI to DRB mapping is configured for the QoS flow. RAN2 reply to RAN3’s LS accordingly.

A draft LS can be found in [2].

4.	Conclusions

This contribution discusses the issue raised by RAN3 LS [1]. The following is proposed.

Proposal 1	RAN2 discuss and confirm that gNB implementation allows delivery of DL packets to UE (via default DRB or via dedicated DRB) before the new QFI to DRB mapping is configured for the QoS flow. RAN2 reply to RAN3’s LS accordingly.

A draft LS is provided in [2] for discussion and approval.

References

[1] R2-2109344/R3-214453, LS on downlink unmapped QoS flows ‎

[2] R2-2111069‎, Draft reply LS on downlink unmapped QoS flows ‎