3GPP TSG-RAN2 Meeting #116-e	R2-2111619

# Online, 1-12 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | CRNum | rev | RevNum | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running 38300 CR for RedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2021-11-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This is a draft of the running RedCap CR for 38.300. To be updated as the work progresses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Agreements and if/how they have been captured: RAN2#116:. RAN2 will not further discuss L2 buffer size reduction for RedCap UEs in Rel-17 (this does not prevent future discussion in future releases); No impact. In MAC perspective, a RedCap UE uses Msg1 early identification whenever transmitting preamble for CBRA, as long as the Msg1 early identification is configured for RedCap by NW.; No impact. For Msg1 early identification, RAN2 confirm both dedicated ROs and dedicated PRACH preamble can be supported from signalling point of view; Captured. For RedCap, Msg1 early identification is enabled/disabled implicitly by the presence of dedicate RACH configuration for Msg1 early identification.; No impact. At least the dedicated LCID (i.e. the Msg3 early identification solution) can be supported for MsgA early identification. It is up to RAN1 on the need of dedicated preamble and/or dedicated PUSCH resource configuration.; No impact. Do not support the RedCap specific UAC parameters.; No impact. In MAC perspective, RedCap UE uses the dedicated LCID for Msg3 early identification, when the Msg3 includes the CCCH data. FFS on whether it requires no other precondition, or precondition as “when Msg1 early identification is not configured”, or precondition as “when Msg3 early identification is enabled by NW”.; No impact. Two reserved LCIDs are used for CCCH and CCCH1 cases respectively for Msg3 early identification; No impact. The max eDRX cycle length for RRC Inactive is 10.24s in Rel-17; No impact. PO determination for non-overlapping CN/RN case is applicable to eDRX; No impact. When IDLE eDRX and INACTIVE eDRX are configured and both cycles are no longer than 10.24s, PO is determined by IDLE eDRX.; No impact. When IDLE eDRX is configured and is no longer than 10.24s, INACITVE eDRX cycle is not configured, PO is determined by IDLE eDRX.; No impact. During CN PTW when IDLE eDRX is configured and longer than 10.24s, and INACTIVE eDRX is configured, PO is determined by the shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer.; No impact. During CN PTW when IDLE eDRX is configure and is longer than 10.24s, INACTIVE eDRX cycle is not configured, PO is determined by the shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer.; No impact. eDRX supporting UEs are assumed to also support the UE capability on PO determination for non overlapping CN/RN case (Further discuss on the reporting of eDRX capability); No impact. The below working agreement is now changed to an agreement. When IDLE eDRX cycle is longer than 10.24s, CN PTW_start calculation formula defined in LTE is re-used as the baseline, as below. FFS whether CN PTW_start position could be configurable by network and in case which node decides the N value. Note: this formula would be revisited if INACTIVE eDRX cycle can be above 10.24s PTW_start denotes the first radio frame of the PH that is part of the PTW and has SFN satisfying the following equation: SFN = 1024/N* ieDRX, where ieDRX = floor(UE_ID_H /TeDRX,H) mod N FFS N = 4 or 8, FFS if N can take other values; No impact. The same LTE hashed UE_ID calculation is used for UE_ID_H for NR.; No impact. eDRX feature can be supported by non RedCap UEs.; No impact. A UE in idle mode requests eDRX configuration via NAS signalling. FFS if capability signalling in RAN, as part of the UE capability message, is also needed.; Partly captured, FFS removed. eDRX support is optional for the RedCap UE.; No impact. the UE_ID for eDRX is defined by 5G-S-TMSI mod 4096.; No impact. the eDRX acquisition period is the maximum configurable value of the eDRX cycle; No impact. No eDRX specific on-demand SI enhancements are considered for Rel-17; No impact. For the eDRX PTW start calculation, agree to N=8. No signalling needed to CN.; No impact. The eDRX acquisition period is the same for IDLE and INACTIVE.; No impact. A) For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is not configured, T is determined by the shortest of RAN paging cycle and IDLE eDRX cycle. B) For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is not configured, outside CN PTW, T is determined by RAN paging cycle.; No impact. UE is not allowed to relax its RRM measurements if both stationarity criterion and R17 not-at-cell-edge criterion are configured but UE meets only the R17 not-at-cell-edge criterion.; No impact. UE reports to network when it no longer meets relaxation criteria.; Captured. No additional signaling is introduced for network to tell UE whether and which criteria for RRM relaxation is considered satisfied when leaving RRC_CONNECTED state.; No impact. No need for UE to send UE Assistance Information to request network configuring it with relaxation criteria.; No impact. UE does not report its history/state of RRM relaxation when transitioning from RRC Idle/Inactive to RRC Connected.; No impact. Relaxation criteria for UEs in RRC Connected are configured by only dedicated signaling.; No impact. RAN2 assume that the existing RRM measurement framework can be used as baseline for enabling and disabling RRM relaxations for UEs in RRC Connected. Other methods can be considered too based on relaxation methods agreed by RAN4.; No impact. RAN2 understands that no prohibit timer is needed, if legacy measurement reporting framework is reused by UE to report its relaxation status; No impact. The granularity of RRM measurement relaxations (i.e. whether it should be specified per beam, per cell or per frequency) should be handled by RAN4; No impact. RAN2#115:. 1. The number of DRBs supported by RedCap UEs is less than legacy value (which is 16). There will be a single mandatory value (FFS if 4 or 8). FFS if it will be possible to have an optional capability; No impact. 2. “RRC processing delay” is not relaxed for RedCap UE; No impact. 3. PDCP/RLC AM 12 bits SN is mandatory for RedCap UE, and PDCP/RLC AM 18bits SN is optional supported by RedCap UE; FFS on how to capture this in specification; No impact. 4. NE-DC, and (NG)EN-DC are not supported by RedCap UE; FFS on how to capture it in the specification; Captured. 5. DAPS and CAPC related capabilities are not applicable for RedCap UE; [8/20] FFS on CHO. FFS on how to capture this in the specification;; Captured. 1. Maximum 8 DRBs is mandatory supported by RedCap UEs.; No impact. 2. From RAN2 perspective, inter RAT mobility related capabilities are applicable for RedCap UE;; Captured. 3. From RAN2 perspective, measurement related capabilities are applicable for RedCap UE;; No impact. 4. From RAN2 perspective, URLLC related capabilities are applicable for RedCap UE except those affected by CA/DC;; Partly captured. 5. From RAN2 perspective, IAB related capabilities are not applicable for RedCap UE, i.e. the RedCap UE is not expected to act as IAB node;; Captured. 6. Do not introduce capability signalling on the supported Rx number for RedCap UE since the number of Rx branches for RedCap is implicitly indicated by the corresponding capability parameter maxNumberMIMO-LayersPDSCH in the existing UE capability framework;; No impact. 1. Msg1 identification which can be configured to be enabled/disabled can be specified from RAN2 point of view.; Captured. 2. Solution for early identification for 2-step RACH will be specified.; No impact. 3. Specify separate indications in SIB1 for barring RedCap UEs with 1 Rx chain and 2 Rx chains.; Captured. 4. Specify a RedCap specific IFRI in SIB1.; Captured. 1. IFRI for RedCap UEs in SIB1 is common for UEs with 1 Rx or 2 Rx branches.; No impact. 2. If RedCap-specific IFRI is absent from broadcast SI, the UE considers the cell does not support RedCap.; Captured. 1. A Msg3 early identification based on dedicated LCID is supported (if SA3 confirms there is no problem); Captured. 1. RedCap UE applies the existing cellBarred field in MIB; No impact. 1. When IDLE eDRX cycle is longer than 10.24s, PH calculation formula defined in LTE is re-used, i.e. PH_CN: H-SFN mod TeDRX,_CN,H= (UE_ID_H mod TeDRX_CN,H) - where TeDRX_CN,H is equal to IDLE eDRX cycle.; No impact. 2. When IDLE eDRX cycle is longer than 10.24s, CN PTW_end calculation formula defined in LTE is re-used, i.e. PTW_end is radio frame satisfying SFN = (PTW_start + L*100 - 1) mod 1024, - where L is PTW length configured by upper layers; No impact. 3. For RRC_IDLE UE, when eDRX cycle is no longer than 10.24s, T is determined by IDLE eDRX cycle. When eDRX cycle is longer than 10.24s, during the CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, and default paging cycle.; No impact. 4. For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and Inactive eDRX cycle is not configured, during CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, RAN paging cycle and default paging cycle.; No impact. 5. For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and Inactive eDRX cycle is no longer than 10.24s, outside CN PTW, T is determined by INACTIVE eDRX cycle.; No impact. 1. RAN2 considers the configuration as an invalid case, where INACTIVE eDRX cycle is configured but IDLE eDRX cycle is not configured. FFS whether to capture this restriction in RAN2 spec.; No impact. 2. RAN2 considers the configuration as invalid case, where INACTIVE eDRX cycle is longer than IDLE eDRX cycle. FFS whether to capture this restriction in RAN2 spec.; No impact. 3. The maximum PTW length is 40.96s when IDLE eDRX cycle is longer than 10.24s.; No impact. 4. The minimum PTW length is 1.28s and the step length/granularity of PTW length is 1.28 when IDLE eDRX cycle is longer than 10.24s.; No impact. 5. Introduce an additional new IE for INACTIVE eDRX to contain all values of INACTIVE eDRX cycles (also include values >10.24, if agreed in future).; No impact. 6. For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is no longer than 10.24s, T is determined by the shortest of IDLE eDRX cycle and INACTIVE eDRX cycle.; No impact. 7. For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is no longer than 10.24s, during CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, INACTIVE eDRX cycle and default paging cycle.; No impact. 8. eDRX feature is optional for any UE (including RedCap and non-RedCap UEs).; No impact. 1. eDRX is optional for any gNB (either supporting RedCap or not), which means it is up to gNB implementation whether to support eDRX; No impact. Working Assumption: 2. When IDLE eDRX cycle is longer than 10.24s, CN PTW_start calculation formula defined in LTE is re-used as the baseline, as below. FFS whether CN PTW_start position could be configurable by network and in case which node decides the N value. Note: this formula would be revisited if INACTIVE eDRX cycle can be above 10.24s PTW_start denotes the first radio frame of the PH that is part of the PTW and has SFN satisfying the following equation: SFN = 1024/N* ieDRX, where ieDRX = floor(UE_ID_H /TeDRX,H) mod N FFS N = 4 or 8, FFS if N can take other values; No impact. 1. For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is not configured, FFS which option below is adopted for paging monitoring: Option 1: T is determined by the shortest of RAN paging cycle, IDLE eDRX cycle, and default paging cycle. Option 2: T is determined by the shortest of RAN paging cycle and IDLE eDRX cycle.; No impact. 2. For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is not configured, outside CN PTW, FFS which option below is adopted for paging monitoring: Option 1: T is determined by the shortest of RAN paging cycle and default paging cycle. Option 2: T is determined by RAN paging cycle.; No impact. - Do not introduce nor reuse not-at-cell-edge threshold for R17 RRC_CONNECTED UEs.; No impact. 1. Do not introduce beam change based criterion in Rel-17.; No impact. 2. The network provides the configuration of stationarity criterion to the UE via dedicated signalling (e.g. RRCReconfiguration message) in RRC_CONNECTED.; No impact. 3. Send LS to RAN4 to inform RAN2 conclusions for RRM relaxation.; No impact. 4. The LS to RAN4 includes the agreed RAN2 conclusions and “For RRC_IDLE/INACTIVE, RAN4 is asked to study and define corresponding R17 RRM relaxation method” .; No impact. 1. Introduce separate Rel-17 not-at-cell-edge threshold, and the new threshold is only associated with Rel-17 stationary criterion (if configured). If configured with a not-at-cell-edge criterion, the R17 stationary criterion can only be configured together with the R17 not-at-cell-edge criterion, not with the R16 one; No impact. RAN2#114:. Working assumption: 1. Extend UE-NR-Capability using NCE to capture RedCap capabilities; No impact. 2. We will continue the discussion on which capability are applicable to RedCap UE (FFS if we need to have an exhaustive check); No impact. 3. At least for early identification there will be only one RedCap UE (no need to define separate RedCap UE types for FR1 and FR2); Captured. 4. It is up to the network how to prevent RedCap UEs from using radio capabilities not intended for RedCap UEs (no specification impact is foreseen at least in RAN2. FFS whether something is needed from SA2/CT1); Captured. 1. RAN2 Working Assumption: by default, all non-RedCap UE capabilities are applicable for RedCap UE, and therefore only for non-RedCap capabilities that are not appliable for RedCap UE, we clarify in the definitions for parameters in TS38.306, the value or feature is not applicable for RedCap UE; No impact. 2. We will have an email discussion until the next meeting to discuss which higher layer capabilities are not applicable for RedCap UEs (it could result in a draft 38.306 CR) and how to reflect the handling of RedCap specific capabilities (e.g. Maximum BW, Max Rx, MIMO-Layer, 256QAM, CA/DC, HD-FDD, etc); No impact. 3. The network needs to know if the UE is a RedCap UE or not in order to at least correctly identify the set of mandatory features (i.e. baseline capabilities) that the UE supports, including Handover case; No impact. 4. The network needs to unambiguously know whether the UE is a RedCap or a non-RedCap UE from its reported UE capability information.; No impact. 1. SIB1 (not MIB) indicates cell barring for 1 Rx branch and 2 Rx branches separately for RedCap UEs. Further details of the solution are FFS; Captured. 2. The cell barring for RedCap UE is per cell (not per PLMN).; No impact. 3. RedCap UE supports the Intra Frequency Reselection Indicator.; No impact. 4. Either Msg1 and/or Msg3 early identification will be supported; No impact. 1. There is no need to support Rx branches specific early identification from RAN2 perceptive (final decision up to RAN1).; No impact. 2. Send LS to ask RAN3 to consider the coordination between gNBs on whether a neighbour/target gNB supports RedCap UEs, if needed, to avoid handover RedCap to a target cell that it can’t access. We can come back in the next meeting with discussions on other restrictions, e.g. related to number of RX; No impact. 1. Lower bound for eDRX configuration in RRC_IDLE and RRC_INACTIVE is 2.56 seconds. Inform SA2/CT1 and check if there is any concern.; No impact. 2. It is up to RAN to configure the length for PTW for RAN paging, the RAN PTW length can be different from the CN PTW length.; Partly captured. 3. When RAN and CN paging coincide in the same PH, the PTW starting locations are the same. FFS how to calculate the PTW starting location so that it is the same for RAN and CN PTW.; No impact. 1. Continue in the next meeting the discussion on how UE is expected to monitor RAN and CN PTW, e.g. whether UE in RRC_INACTIVE monitors for only RAN PTW or both CN and RAN PTW when they overlap; No impact. 1. An RSRP/RSRQ based stationarity criterion (Working Assumption: the same as in idle/inactive) can be configured for UEs in RRC Connected. If the criterion is met, this is reported to the network (FFS how/when). It is FFS whether, based on this, besides possibly reconfiguring RRM measurements (up to network implementation), the network can enable RRM measurement relaxation (FFS whether same method as in Idle/Inactive); Partly captured. 1. Subscription based relaxation criteria will not be considered in Rel-17 RRM relaxation; No impact. 1. Reuse R16 low mobility criterion, as part or whole of Rel-17 stationary criterion in RRC_IDLE/INACTIVE. When NW configures both Rel-17 stationary criterion and Rel-16 low mobility criterion, NW configures different Rel-17 thresholds (i.e., SSearchDeltaP_stationary/TSearchDeltaP_stationary) from Rel-16 (SSearchDeltaP / TSearchDeltaP). How to configure the criterion (e.g. more stringent) is left to NW implementation (i.e. no specification impact to RAN2).; No impact. 2. Postpone the following discussion until RAN4 defines RRM relaxation method for Rel-17: When NW configures both R16/R17 relaxation criteria and the UE fulfills both, UE performs: - Option 1) UE performs Rel-17 RRM relaxation method - Option 2) It is up to UE implementation to select either Rel-16 or Rel-17 relaxation operation; No impact. 1. Working Assumption: If beam-level criterion is adopted for Rel-17 stationary criterion in RRC_IDLE/INACTIVE, it is configured separately with Rel-16 low mobility criterion reused; No impact. 2. When NW configures Rel-17 RRM relaxation for RRC_IDLE/INACTIVE, Rel-17 stationary criterion is mandatory, and Rel-17 not-at-cell-edge criterion is optional configuration. FFS whether the same applies to RRC Connected; No impact. 3. Continue discussion on Rel-17 not-at-cell-edge criterion in RRC_IDLE/INACTIVE within two options: - Option 1) Reuse Rel-16 not-at-cell-edge criterion with the same thresholds (i.e., SSearchThresholdP / SSearchThresholdQ) - Option 2) Reuse Rel-16 not-at-cell-edge criterion with the different thresholds; No impact. RAN2#113bis. 1. RAN decides and configures eDRX via RRC for RRC_INACTIVE (FFS on the need and details of coordination with the CN); Captured. 2. At least for eDRX cycle, the configurations of the eDRX for RRC_IDLE and RRC_INACTIVE can be different (FFS for PTW, e.g. length and starting point, when eDRX cycles are longer than 10.24s); No impact. 1. RAN2 assumes that CN provides necessary assistance information on eDRX config. for RRC_IDLE to RAN (e.g. reusing eDRX config. defined in “CN Assistance Information for RRC INACTIVE IE” for E-UTRA/5GC).; No impact.. 2. eDRX feature, including the related parameters (i.e. PH, PTW. H-SFN) and corresponding paging operation defined for E-UTRA/5GC is used as baseline to enable eDRX >10.24sec for both RRC_IDLE and RRC_INACTIVE in NR/5GC; Captured. 3. RAN2 confirms that CN paging and RAN paging use the same paging frame offset and first PDCCH monitoring occasion in PO, which are configured by RAN without involvement of CN.; No impact. 4. RAN2 confirms that SI modification mechanism from LTE is used as a baseline for SI change (other than ETWS and CMAS), i.e. by using an eDRX acquisition period and a flag to indicate SI modification for eDRX in Short Message (e.g. systemInfoModification-eDRX); No impact. 1. Assuming there will be a stationary property based on subscription (which is FFS), we will not restrict to this and will continue to assume that a UE can use some RSRP/RSRQ based criteria (FFS whether reuse R16 thresholds or new ones. FFS also on the use of a beam based criteria); No impact. At least for RRC idle/inactive, a measurement-based R17 stationarity criterion can be configured separately from R16 low-mobility criterion for R17 UEs supporting the feature. FFS how the configuration is provided. FFS whether this stationarity criterion is based on: - the same algorithm used in R16 low-mobility criterion but with its own specific set of thresholds; and/or - a combination of R16 low-mobility criterion and/or beam-change based criterion. Exact details of beam change criterion are FFS.; Partly captured. 1. Network can configure R17 stationarity criterion/criteria together with a not-at-cell-edge criterion, to trigger RRM relaxations in RRC Idle/Inactive for R17 UEs supporting the feature. FFS whether the R16 not-at-cell-edge thresholds can be reused or separate R17 not-at-cell-edge thresholds are needed.; Partly captured. |
| Consequences if not approved: |  | RedCap is not supported in 38.300 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | TBD |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS TODO CR TODO |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be lifted to the latest version of the specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the initial version of running CR for 38.300 for RedCap WI. |  |  |  |  |  |  |  |  |

First Modified Subclause

# 3	Abbreviations and Definitions

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCH	Broadcast Channel

BH	Backhaul

BL	Bandwidth reduced Low complexity

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CAG	Closed Access Group

CAPC	Channel Access Priority Class

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFRA	Contention Free Random Access

CHO	Conditional Handover

CIoT	Cellular Internet of Things

CLI	Cross Link interference

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

CP	Cyclic Prefix

CPC	Conditional PSCell Change

DAG	Directed Acyclic Graph

DAPS	Dual Active Protocol Stack

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DL-AoD	Downlink Angle-of-Departure

DL-SCH	Downlink Shared Channel

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell-ID (positioning method)

EHC	Ethernet Header Compression

ETWS	Earthquake and Tsunami Warning System

FS	Feature Set

GFBR	Guaranteed Flow Bit Rate

HRNN	Human-Readable Network Name

H-SFN	Hyper System Frame Number

IAB	Integrated Access and Backhaul

IFRI	Intra Frequency Reselection Indication

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

LDPC	Low Density Parity Check

MDBV	Maximum Data Burst Volume

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MPE	Maximum Permissible Exposure

MT	Mobile Termination

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

NB-IoT	Narrow Band Internet of Things

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

P-MPR	Power Management Maximum Power Reduction

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated NPN

PO	Paging Occasion

PH	Paging Hyperframe

PRB	Physical Resource Block

PTW	Paging Time Window

PRACH	Physical Random Access Channel

PRG	Precoding Resource block Group

PS-RNTI	Power Saving RNTI

PSS	Primary Synchronisation Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QPSK	Quadrature Phase Shift Keying

RA	Random Access

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RIM	Remote Interference Management

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

SCS	SubCarrier Spacing

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SFI-RNTI	Slot Format Indication RNTI

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SMC	Security Mode Command

SMF	Session Management Function

S-NSSAI	Single Network Slice Selection Assistance Information

SNPN	Stand-alone Non-Public Network

SNPN ID	Stand-alone Non-Public Network Identity

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRS	Sounding Reference Signal

SRVCC	Single Radio Voice Call Continuity

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TPC	Transmit Power Control

TRP	Transmit/Receive Point

UCI	Uplink Control Information

UL-AoA	Uplink Angles of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

V2X	Vehicle-to-Everything

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

Second Modified Subclause

## 3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

BH RLC channel: an RLC channel between two nodes, which is used to transport backhaul packets.

CAG Cell: a PLMN cell broadcasting at least one Closed Access Group identity.

CAG Member Cell: for a UE, a CAG cell broadcasting the identity of the selected PLMN, registered PLMN or equivalent PLMN, and for that PLMN, a CAG identifier belonging to the Allowed CAG list of the UE for that PLMN.

CAG-only cell: a CAG cell that is only available for normal service for CAG UEs.

Cell-Defining SSB: an SSB with an RMSI associated.

Child node: IAB-DU's and IAB-donor-DU's next hop neighbour node; the child node is also an IAB-node.

Conditional Handover (CHO): a handover procedure that is executed only when execution condition(s) are met.

CORESET#0: the control resource set for at least SIB1 scheduling, can be configured either via MIB or via dedicated RRC signalling.

DAPS Handover: a handover procedure that maintains the source gNB connection after reception of RRC message for handover and until releasing the source cell after successful random access to the target gNB.

Downstream: Direction toward child node or UE in IAB-topology.

Early Data Forwarding: data forwarding that is initiated before the UE executes the handover.

gNB: node providing NR user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

IAB-donor: gNB that provides network access to UEs via a network of backhaul and access links.

IAB-donor-CU: as defined in TS 38.401 [4].

IAB-donor-DU: as defined in TS 38.401 [4].

IAB-DU: gNB-DU functionality supported by the IAB-node to terminate the NR access interface to UEs and next-hop IAB-nodes, and to terminate the F1 protocol to the gNB-CU functionality, as defined in TS 38.401 [4], on the IAB-donor.

IAB-MT: IAB-node function that terminates the Uu interface to the parent node using the procedures and behaviours specified for UEs unless stated otherwise. IAB-MT function used in 38-series of 3GPP Specifications corresponds to IAB-UE function defined in TS 23.501 [3].

IAB-node: RAN node that supports NR access links to UEs and NR backhaul links to parent nodes and child nodes. The IAB-node does not support backhauling via LTE.

Intra-system Handover: Handover that does not involve a CN change (EPC or 5GC).

Inter-system Handover: Handover that involves a CN change (EPC or 5GC).

Late Data Forwarding: data forwarding that is initiated after the source NG-RAN node knows that the UE has successfully accessed a target NG-RAN node.

MSG1: preamble transmission of the random access procedure for 4-step random access (RA) type.

MSG3: first scheduled transmission of the random access procedure.

MSGA: preamble and payload transmissions of the random access procedure for 2-step RA type.

MSGB: response to MSGA in the 2-step random access procedure. MSGB may consist of response(s) for contention resolution, fallback indication(s), and backoff indication.

Multi-hop backhauling: Using a chain of NR backhaul links between an IAB-node and an IAB-donor.

ng-eNB: node providing E-UTRA user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

NG-C: control plane interface between NG-RAN and 5GC.

NG-U: user plane interface between NG-RAN and 5GC.

NG-RAN node: either a gNB or an ng-eNB.

Non-CAG Cell: a PLMN cell which does not broadcast any Closed Access Group identity.

NR backhaul link: NR link used for backhauling between an IAB-node and an IAB-donor, and between IAB-nodes in case of a multi-hop backhauling.

NR sidelink communication: AS functionality enabling at least V2X communication as defined in TS 23.287 [40], between two or more nearby UEs, using NR technology but not traversing any network node.

Numerology: corresponds to one subcarrier spacing in the frequency domain. By scaling a reference subcarrier spacing by an integer N, different numerologies can be defined.

Parent node: IAB-MT's next hop neighbour node; the parent node can be IAB-node or IAB-donor-DU

PLMN Cell: a cell of the PLMN.

RedCap UE: The UE with reduced capabilities as specified in TS 38.306 [11] sub-clause 4.2.x.x.

SNPN Access Mode: mode of operation whereby a UE only accesses SNPNs.

SNPN-only cell: a cell that is only available for normal service for SNPN subscribers.

SNPN Identity: the identity of Stand-alone NPN defined by the pair (PLMN ID, NID).

Transmit/Receive Point: Part of the gNB transmitting and receiving radio signals to/from UE according to physical layer properties and parameters inherent to that element.

Upstream: Direction toward parent node in IAB-topology.

V2X sidelink communication: AS functionality enabling V2X communication as defined in TS 23.285 [41], between nearby UEs, using E-UTRA technology but not traversing any network node.

Xn: network interface between NG-RAN nodes.

Last Modified Subclause

## 16.x	Support of Reduced Capability (RedCap) NR devices

### 16.x.1	Introduction

## A RedCap UE has reduced capabilities with the intention to have lower complexity with respect to non-RedCap UEs. It is mandatory for a RedCap UE to support 20MHz maximum UE channel bandwidth in FR1 and 100MHz in FR2.

### 16.x.2	Capabilities

NR-CA, MR-DC, DAPS, CPC and IAB related capabilities are not supported by RedCap UEs, as defined together with other limitations in TS 38.306 [11]. It is up to the network to prevent RedCap UEs from using radio capabilities not intended for RedCap UEs.

### 16.x.3	Identification, access and camping restrictions

Early identification of RedCap UEs can be made via both MSG1 (PRACH occasion or PRACH preamble) and/or MSG3 (LCID). For early identification via MSG1, RedCap specific random access configuration may be provided.

RedCap UEs with 1 Rx branch and 2 Rx branches can be barred separately via system information.

A RedCap specific IFRI (Intra Freq Reselection Indication) can be provided in SIB1. When absent, the cell is considered as not supported for RedCap UEs.

### 16.x.4	RRM relaxations

RRM relaxation is enabled and disabled by the network. In RRC_IDLE and RRC_INACTIVE a RedCap UE is allowed to relax neighbour cells RRM measurements when the RSRP/RSRQ stationary criterion is met or when both stationary criterion and not-at-cell-edge criterion are met. Network may configure RSRP/RSRQ based stationary criterion for a UE in RRC_CONNECTED and the UE shall report when the stationarity criterion is met or no longer met.

### x.x.x	Extended DRX for RRC_IDLE and RRC_INACTIVE

When extended DRX (eDRX) is used, the following applies:

-	For RRC_INACTIVE, eDRX configuration is decided and configured by NG-RAN.

-	For RRC_IDLE, eDRX is configured by upper layers.

-	For RRC_IDLE, the DRX cycle can be extended up to 10485.76 seconds (2.91 hours) while for RRC_INACTIVE, the maximum value of the DRX cycle is 10.24 seconds;

-	The hyper SFN (H-SFN) is broadcast by the cell and increments by one when the SFN wraps around;

-	Paging Hyperframe (PH) refers to the H-SFN in which the UE starts monitoring paging DRX during a Paging Time Window (PTW) used in CM-IDLE. The PH is determined based on a formula that is known by the AMF, UE and NG-RAN;

-	H-SFN, PH and PTW are used if the eDRX cycle is greater than 10.24 seconds;

-	When the eDRX cycle is longer than the system information modification period, the UE verifies that stored system information remains valid before establishing an RRC connection.