3GPP TSG-RAN WG2 Meeting #116-e                                                 R2-2111xxx

E-Meeting: 1 – 12 Nov 2021

Agenda item:	9.1.2

Source:	Qualcomm Incorporated

Title:	draft Report of [AT116e][303][NBIOT/eMTC] RLF measurements (Qualcomm)

Document for:	Discussion and Decision

# Introduction

RAN2 initiated the following offline:

- [AT114-e][303][NB-IoT/eMTC] RLF measurements  (Qualcomm)

Scope: Conclude the FFS on RLF measurements.

Intended outcome: Report in R2-2111393

Deadline: Monday 8 Nov 12:00 UTC

Agreements - NW signals two separate thresholds for intra- and inter-frequency measurements. - The values of s-SearchDeltaP and TSearchDeltaP may be different in RRC_CONNECTED and RRC_IDLE, they are signalled in a separate set of parameters. - s-SearchDeltaP has the same value range as the existing RRC_IDLE parameter - FFS how to define TSearchDeltaP - FFS how to specify the state change - FFS whether NW can disable / what happens when the IEs are absent - [FFS] An indication that the UE starts measurement is not introduced. - No enhancement is introduced to have a shorter T310 timer for mobile UEs supporting connected mode measurement.

# References

|  | R2-2109913 | Discussion on connected mode measurement in NB-IoT | E/// |
| --- | --- | --- | --- |
|  | R2-2110109 | Remaining FFSs of RLF measurements | ZTE |
|  | R2-2110147 | Network assistance for Re-establishment enhancement | Nokia |
|  | R2-2110474 | Relaxed monitoring in RRC connected mode | HW |
|  | R2-2110476 | Summary of [301] RLF measurements (Huawei) | HW |
|  | R2-2110693 | Consideration on open issues for neighbour cell measurement in RRC connected state | QC |
|  | TS 36.304 | User Equipment (UE) procedures in idle mode | 3GPP |

# Discussion

## How to define TSearchDeltaP

For RRC_IDLE mode, TSearchDeltaP = max (5min, eDRX cycle length) [7], and for RRC_CONNECTED state companies have proposed the following:

- Separate TSearchDeltaP should be introduced as configurable by SIB3-NB. [1]

- TSearchDeltaP for relaxed monitoring in connected mode is signalled in system information as enumerated values in multiple of seconds, e.g. 5s, 10s, 20s, 40s. [4]

- TSearchDeltaP can be fixed to 60 sec for RRC connected state. [6]

For RRC_IDLE mode TSearchDeltaP is a fixed value 5mins (in case of DRX and eDRX< 5min) or same as the eDRX  to avoid the need for UE to wake-up more frequently than eDRX cycle.

In RRC_CONNECTED state, UE typically makes serving cell measurements on the UE specific search space (NPDCCH) and such measurement occasions only happen when UE is receiving NPDCCH, not when UE is receiving NPDSCH.

Question 1.0: For RRC_CONNECTED state, should TSearchDeltaP be configurable via SIBs or fixed in specification?

| Company | Fixed/Configurable | Justification for your answer. |
| --- | --- | --- |
| Huawei, HiSilicon | Configurable | We have a preference for configurable as this allows tuning by the NW. It is different from RRC_IDLE where we have long time line and where the value can be set in a conservative way. |
| Qualcomm | Configurable | Same view as HW. |
| Ericsson | Configurable | Agree with Huawei |

Question 1.1: If your answer to Question 1 is Fixed, then propose a value?

| Company | Fixed Value |  |
| --- | --- | --- |

Question 1.2: If your answer to Question 1 is Configurable then what value range?

| Company | Value range | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | 10 – 60 s | we do not need many values, probably 4 would be sufficient |
| Qualcomm | 10 – 60s | We think it is better to have the configuration as multiples of NPDCCH periods because NPDCCH period will control the measurement opportunities. |
| Ericsson | FFS for now |  |

Question 1.3: If your answer to Question 1 is Configurable then should there be a default value?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | if we have only a few values (e.g. 2-3 bits), no need for a default value |
| Qualcomm | No | If the value for TSearchDeltaP is not configured then this means relaxed neighbour cell monitoring for RRC_CONNECTED state is not enable. |
| Ericsson | FFS for now |  |

## How to specify the state change

Upon entering RRC_CONNECTED state, there are various aspects to consider for relaxed neighbour cell monitoring, such as:

What is the relaxed neighbour cell monitoring state upon entering RRC_CONNECTED?

Upon entering RRC_CONNECTED state, the serving sell NRSRP could be quite different from the idle mode NRSRP. Therefore, in this case how is the reference NRSRP (SrxlevRef) is set upon entering RRC_CONNECTED state?

Should there be a limit on the maximum duration for which UE can be in relaxed neighbour cell monitoring state  in RRC_CONNECTED state?

When to switch from RRC_IDLE state TSearchDeltaP to RRC_CONNECTED state TSearchDeltaP?

The following proposal has been proposed in the reference documents:

- When entering RRC_CONNECTED, the UE re-initialises the evaluation of the relaxed monitoring criterion with the new parameters. The relaxed monitoring status in RRC_IDLE is used as the initial state. [4]

Question 2.0: What is the relaxed neighbour cell monitoring state upon entering RRC_CONNECTED?

Same as Idle means:

- if UE was in relaxed neighbour cell monitoring state in RRC_IDLE then UE considers itself to be in relaxed neighbour cell monitoring state in RRC_CONNECTED state until serving cell measurements degrade by RRC_CONNECTED state s-SearchDeltaP.

- If UE was not in relaxed monitoring state in RRC_IDLE then UE considers itself to be not in relaxed neighbour cell monitoring state in RRC_CONNECTED state until serving cell measurements do not degrade by RRC_CONNECTED state s-SearchDeltaP over a period of RRC_CONNECTED state TSearchDeltaP.

Relaxed means:

- UE considers itself to be in relaxed neighbour cell monitoring state in RRC_CONNECTED state until serving cell measurements degrade by RRC_CONNECTED state s-SearchDeltaP.

Non-Relaxed means:

- UE considers itself to be in not-relaxed neighbour cell monitoring state in RRC_CONNECTED state until  UE determines it can enter relaxed serving state based on the RRC_CONNECTED state values of s-SearchDeltaP & TSearchDeltaP.

While UE is in non-relaxed neighbour cell monitoring state, UE may perform neighbour cell measurements based on the configured threshold for triggering neighbour cell measurements.

| Company | Same as Idle/ Relaxed/ Non-Relaxed | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Same as Idle |  |
| Qualcomm | Same as Idle |  |
| Ericsson | Preferred same as idle, but would like to keep it FFS for this meeting | We are introducing several new timers for connected mode measurement to identify new neighbor cell quickly but then we also say UE follows relaxed neighbor cell monitoring. It is good to understand the impacts/interaction any contradiction etc? |

The following proposals have been proposed in the reference documents:

- When entering RRC_CONNECTED, the UE re-initialises the evaluation of the relaxed monitoring criterion with the new parameters. [4]

- The dedicated downlink carrier specific nrs-PowerOffsetNonAnchor is applied to translate measurement on non-anchor dedicated carrier to anchor carrier equivalent. [6]

- Set reference level to the measurements made on the dedicated carrier. [6]

If the UE continues with the RRC_IDLE mode relaxed neighbour cell measurement state upon entering RRC_CONNECTED state, then some way the possible discontinuity in the serving cell measurement needs to be considered to ensure UE does not flip the relaxed neighbour cell monitoring state. If, on the other hand, the relaxed neighbour cell monitoring state is reset to one of relaxed or not relaxed then reference level can be set according to RRC_CONNECTED mode serving cell measurements and no need to consider whether there is discontinuity in NRSRP.

Question 2.1: If upon entering RRC_CONNECTED state UE always assumes it is in relaxed monitoring state or UE always assumes it is in a non-relaxed monitoring state then reference level (SrxlevRef) is set to the measurement done on the USS (i.e., RRC_IDLE mode reference level is ignored in RRC_CONNECTED state)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Qualcomm | Yes |  |
| Ericsson | FFS |  |

Question 2.2: If upon entering RRC_CONNECTED state UE continues with the same relaxed neighbour cell monitoring state as in RRC_IDLE state then how is the discontinuity in the NRSRP between RRC_IDLE and RRC_CONNECTED state to be handled? Possible solutions are:

Option 1:	UE converts the RRC_IDLE state SrxlevRef to the RRC_CONNECTED state SrxlevRef by applying the nrs-PowerOffsetNonAnchor.

Option 2:	UE sets SrxlevRef to the first NRSRP measurement done on the USS in RRC_CONNECTED state.

Option 3:	Other

| Company | Option 1/ Option 2/ Option 3 | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | option 2 | no strong opinion, option 1 is fine as well |
| Qualcomm | Option 1 |  |
| Ericsson | FFS |  |

For RRC_IDLE state the maximum duration a UE can remain in relaxed neighbour cell monitoring state is 24 hours [7]. Rapporteurs’ view is that upon leaving RRC_CONNECTED state, the relaxed neighobur cell monitoring state is no longer valid as the idle mode requirements will then apply. This means if there is no limit, then UE can remain in relaxed monitoring state as long as the UE is in RRC_CONNECTED state.

Question 2.3: Should there be a limit for how long UE in RRC_CONNECTED state can remain in relaxed neighbour cell monitoring state?

| Company | Yes/No | If the answer is Yes, then also propose a value for this duration. |
| --- | --- | --- |
| Huawei, HiSilicon | No | The limit of 24 hours in RRC_IDLE is so UE can see change in the network topology. There is no such need in RRC_CONNECTED |
| Qualcomm | No | RRC_CONNECTED state is not expected to last very long and UE does not take into account change in system information that may happen while UE is in RRC_CONNECTED state. For this reason no need to limit the duration for relaxed neighbour cell monitoring state. |
| Ericsson | FFS |  |

In RRC_IDLE state, TSearchDeltaP is started when:

- UE selects or reselects a cell,

- SrxlevRef is set/updated

While TSearchDeltaP is running UE performs neighbour cell measurements.

Upon entering RRC_CONNECTED state, the SrxlevRef needs to be set according to the measurements performed on the USS hence it is effectively ‘updated’ by the first measurement upon entering RRC_CONNECTED state, which means TSearchDeltaP will be started, and which means neighbour cell measurements may be triggered. This is not a desirable behaviour. Therefore, if the UE was in relaxed monitoring state in RRC_IDLE and UE continues with this state in RRC_CONNECTED then while the SrxlevRef can be adjusted to take account of the discontinuity in the RRC_IDLE and RRC_CONNECTED state measurements but TSearchDeltaP should not be started.

Question 2.4: If UE considers itself to be in relaxed neighbour cell monitoring state upon entering RRC_CONNECTED state and the first measurement from USS is used to set/adjust SrxlevRef then TSearchDeltaP should not be started?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | yes | Otherwise, UE will always perform measurements upon entering RRC_Connected |
| Qualcomm | Yes | This is independent of whether Option 1 or Option 2 is selected from Question 2.2. |
| Ericsson | FFS |  |

## Whether NW can disable / what happens when the IEs are absent

The following proposals have been proposed in the reference documents:

Proposal 2: Relaxed monitoring in RRC_CONNECTED is enabled by the explicit signalling of the two configuration parameters, TSearchDeltaP and SSearchDeltaP. [4]

Proposal 2: RAN2 to further discuss enabling/disabling of the variance criteria in broadcast signalling. [5]

Proposal 3: If SSearchDeltaP for RRC connected state not configured, then UE uses SSearchDeltaP value configured for RRC idle. [6]

Question 3.1: If SSearchDeltaP for RRC_CONNECTED state is not broadcasted then should UE use the SSearchDeltaP (if configured) for RRC_IDLE mode?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | NA | It is a lot easier to broadcast always (2 bits) |
| Qualcomm | Depends on conclusion for TSearchDeltaP. | Given RAN2 has agreed to have same range for SSearchDeltaP for RRC_CONNECTED state as for RRC_IDLE and this range requires 2-bits, it is not necessary to add 1 more bit to make this parameter optional. Ultimately, the final decision can depend on what is agreed for TSearchDeltaP. That is, if RAN2 agrees TSearchDeltaP must be explicitly signalled for RRC_CONNECTED state then it can be mandate that both either both SSearchDeltaP & TSearchDeltaP are provided, otherwise none in which case relaxed neighbour cell monitoring would not be enabled. |
| Ericsson | NA | Same view as Huawei |

If RAN2 agrees to provide value for TSearchDeltaP (see §3.1) for RRC_CONNECTED state in SIBs and network does not broadcast a value for TSearchDeltaP and there is no default value defined for TSearchDeltaP then it is reasonable to assume relaxed neighbour cell monitoring in RRC_CONNECTED state is disabled and UE, based on serving cell measurement and configured threshold, may perform neighbour cell measurements.

Question 3.2: In the absence of any default value, do you agree relaxed neighbour cell monitoring in RRC_CONNECTED state is enabled if TSearchDeltaP for RRC_CONNECTED state is broadcasted?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Agree with comments | Our understanding of the online discussion is that relaxed monitoring is always enabled if connected mode measurements are enabled. we are fine with this. In that case, the two parameters SSearchDeltaP and TSearchDeltaP are mandatory present if connected mode measurements are enabled |
| Qualcomm | Agee | Also agree with HW comment. |
| Ericsson | FFS |  |

If, for RRC_CONNECTED state, RAN2 agrees to have a default or specification defined value for TSearchDeltaP (see §3.1) then should there be an explicit indication to enable relaxed neighbour cell monitoring in RRC_CONNECTED state?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | Sew answer to Q2.3 We think the signalling can be very simple. Connected mode measurements are configured by a structure including: - NRSRP threshold for intra-frequency measurement, Mandatory - NRSRP threshold for inter-frequency measurement, Optional, if absent same as intra - TsearchDeltaP, mandatory - SsearchDeltaP, mandatory |
| Qualcomm | No | See answer to Q3.1. |
| Ericsson | No |  |

## [FFS] An indication that the UE starts measurement is not introduced.

This topic has been discussed online and offline and many companies do not think there is anything additional to discuss at this meeting. Rapporteur proposes to postpone conclusion of this FFS till RAN4 had time to discuss the inter-frequency measurements requirements for RRC_CONNECTED state. The current FFS imposes no specification changes at this point hence this keeping the FFS open for now does not impede RAN2 progress of this objective in the WID.

Question 4.0: Do you agree to postpone conclusion on UE indication?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | yes |  |
| Qualcomm | Yes |  |
| Ericsson | Yes |  |

## Any other suggestions

| Company | Suggestion/Comments |
| --- | --- |

# Conclusion

TBD

# Participants

| Company | Name | e-mail address |
| --- | --- | --- |
| Huawei, HiSilicon | Odile Rollinger | odile.rollinger@huawei.com |
| Qualcomm | Mungal Dhanda | mdhanda@qti.qualcomm.com |
| Ericsson | Ritesh Shreevastav | Ritesh.shreevastav@ericsson.com |