3GPP TSG RAN WG2 Meeting #116-e	                                        R2-2111406

- Electronic, 1st – 12th November 2021

Agenda item:		4.2

Source:			Lenovo

Title:		[AT116-e][401][eMTC R15] Scheduling restrictions of positioning SI messages (Lenovo)

Document for:	 	Discussion and decision

# Introduction

This report summarizes the email discussion below that took place during RAN2#116-e meeting:

- [AT116-e][401][eMTC R15] Scheduling restrictions of positioning SI messages (Lenovo)

Scope: Check whether the intention is agreeable and there is sufficient support
in principle; collect initial comments regarding the wording etc.

Intended outcome: Report in R2-2111406

Deadline: Wednesday 2021-11-03 12:00 UTC

# Reference

The following documents are treated in this email discussion:

[1]	R2-2109514	Summary of discussion on missing scheduling restrictions of positioning SI messages for eMTC	Lenovo, Motorola Mobility	discussion	Rel-15	LCS_LTE_acc_enh-Core

[2]	R2-2109515	Addition of scheduling restrictions of positioning SI messages for eMTC	Lenovo, Motorola Mobility	CR	Rel-15	36.331	15.15.0	4691	-	F	LCS_LTE_acc_enh-Core	R2-2107261

[3]	R2-2109516	Addition of scheduling restrictions of positioning SI messages for eMTC	Lenovo, Motorola Mobility	CR	Rel-16	36.331	16.6.0	4692	-	A	LCS_LTE_acc_enh-Core	R2-2107262

# Contact information

| Company | Contact Name, Email |
| --- | --- |
| Qualcomm | mdhanda@qti.qualcomm.com |
| Huawei, HiSilicon | Odile Rollinger, odile.rollinger@huawei.com |
| Lenovo | Hyung-Nam Choi, hchoi5@lenovo.com |
| ZTE | Lu Ting, lu.ting@zte.com.cn |

# Discussion

## Issue on missing scheduling restrictions of positioning SI messages for eMTC

In [1] a summary of discussion on the issue of missing scheduling restrictions of positioning SI messages for eMTC is provided which took place in the Positioning session at RAN2#115-e:

- If positioning assistance data are broadcast to BL/CE UEs which are camped on an LTE cell that supports this type of UEs then majority of companies confirmed that

- Positioning SIBs (SystemInformationBlockPos) in PosSystemInformation-r15-IEs are transmitted in the BR version of SI message (SystemInformation-BR-r13).

- Scheduling information of the positioning SI messages (posSchedulingInfoList-r15) is sent in the BR version of SIB1 (SystemInformationBlockType1-BR-r13).

- Scheduling restrictions given by schedulingInfoList-BR-r13 are currently missing for scheduling of positioning SI messages to BL/CE UEs.

- In order to fix this issue, the solution to introduce a new field posSchedulingInfoList-BR-r15 (containing the missing scheduling restrictions for scheduling of positioning SI messages to BL/CE UEs) in SIB1-BR was agreed. However, on the actual changes a slight majority of companies preferred to have them reviewed in the eMTC session. Therefore, the corresponding CRs to TS 36.331 for introducing the solution were only endorsed.

Question 1: Do companies have comments on the outcome of discussion from the Positioning session on the issue of missing scheduling restrictions of positioning SI messages for BL/CE UEs?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Qualcomm | No |  |
| Huawei, HiSilicon | No |  |
| ZTE | No |  |

Rapporteur’s summary:

- There are no comments on the outcome of discussion from the Positioning session on the issue of missing scheduling restrictions of positioning SI messages for BL/CE UEs.

Proposal 1: The issue of missing scheduling restrictions of positioning SI messages for BL/CE UEs is confirmed.

## CRs for adding the missing scheduling restrictions of positioning SI messages for eMTC

The endorsed CRs [2], [3] introduce the agreed solution for adding the missing scheduling restrictions of positioning SI messages for BL/CE UEs). In [1] some clarifications to the proposed changes in the CRs are given:

- New field posSchedulingInfoList-BR-r15 has been introduced in the late NCE branch of SIB1 as Rel-15 NCE of field bandwidthReducedAccessRelatedInfo. This was done since in Rel-16 the regular NCE branch of SIB1 is already taken by SystemInformationBlockType1-v1610-IEs.

- New field posSchedulingInfoList-BR-r15 indicates the list of additional scheduling information (in terms of si-Narrowband-r13 and si-TBS-r13) of positioning SI messages for BL UEs and UEs in CE. It includes the same number of entries, and listed in the same order, as in posSchedulingInfoList-r15.

- New condition “SI-Pos-BR” on the presence of field posSchedulingInfoList-BR-r15 has been introduced to clarify that the field is mandatory present if posSchedulingInfoList-r15 is included in SIB1-BR. Otherwise the field is not present.

Question 2: Do companies agree on the changes in the CRs or are further changes needed?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Qualcomm | No | The intent of the changes is correct but ASN.1 can be more compact if element in bandwidthReducedAccessRelatedInfo-v15xy us made mandatory as follows: SystemInformationBlockType1-v15xy-IEs ::= SEQUENCE { bandwidthReducedAccessRelatedInfo-v15xy SEQUENCE { posSchedulingInfoList-BR-r15 PosSchedulingInfoList-BR-r15 OPTIONAL -- Cond SI-Pos-BR } OPTIONAL, -- Cond BW-reduced nonCriticalExtension SEQUENCE {} OPTIONAL } Delete description of SI-Pos-BR from the table. Update field description as follows: posSchedulingInfoList-BR Indicates additional scheduling information of positioning SI messages for BL UEs and UEs in CE. E-UTRAN always includes this field if posSchedulingInfoList-r15 is included in SystemInformationBlockType1-BR, and iIt includes the same number of entries, and listed in the same order, as in posSchedulingInfoList-r15. Draft revised CR with these changes uploaded to the folder. [Lenovo] We are fine with the proposed changes. |
| Huawei, HiSilicon |  | We agree with the intent but we have comments. 1. we probably need the sentence “the CR is considered mandatory to support broadcast of positioning assistance data” on the coversheet [Lenovo] We are fine to add this sentence on the coversheet. 2. a New IE type PosSchedulingInfoList-BR-r15 is introduced but its definition is identical to SchedulingInfoList-BR-r13, so no new type is needed PosSchedulingInfoList-BR-r15 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo-BR-r13 SchedulingInfoList-BR-r13 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo-BR-r13 [Lenovo] We are fine to reuse the existing SchedulingInfoList-BR-r13 type. 3. w.r.t to Q comment above, we would rather remove the condition BW-reduced (which is obvious as this is an extension of bandwidthReducedAccessRelatedInfo which is conditional to BW-reduced) and replace it by the condition SI-Pos-BR [Lenovo] We prefer to adopt QC’s proposal and to keep the condition BW-reduced. This is to avoid any potential misconfiguration, i.e. when the legacy bandwidthReducedAccessRelatedInfo is absent but the extension bandwidthReducedAccessRelatedInfo-v15xy is present. |
| ZTE |  | We agree with the intent and generally fine with all the HW’s suggestions. For the last one, we have no strong view now but still slightly think BW-reduced condition is not so needed. |

Rapporteur’s summary:

- All companies agree on the intent of the changes in the CRs but suggest some modifications.

- Referring to the comments made, rapporteur suggests to agree on the following modifications:

  - Add the sentence “The CR is considered mandatory to support broadcast of positioning assistance data for BL UEs and UEs in CE.” on the coversheet.

  - Remove the condition “SI-Pos-BR” for the presence of field posSchedulingInfoList-BR-r15.

  - Reuse the existing IE SchedulingInfoList-BR-r13 for field posSchedulingInfoList-BR-r15.

  - Add the sentence “E-UTRAN always includes this field if posSchedulingInfoList-r15 is included in SystemInformationBlockType1-BR” in the description of posSchedulingInfoList-BR.

- Regarding the comment to remove the condition “BW-reduced” for the presence of bandwidthReducedAccessRelatedInfo-v15xy, rapporteur suggests to keep it. It may be obvious that the extended field bandwidthReducedAccessRelatedInfo-v15xy should be absent when the legacy field bandwidthReducedAccessRelatedInfo-r13 is absent but from ASN.1 signaling point of view it is recommended to specify it clearly.

Proposal 2: The CRs in R2-2109515 and R2-2109516 will be revised and agreed with the following modifications:

- Add the sentence “The CR is considered mandatory to support broadcast of positioning assistance data for BL UEs and UEs in CE.” on the coversheet.

- Remove the condition “SI-Pos-BR” for the presence of field posSchedulingInfoList-BR-r15.

- Reuse the existing IE SchedulingInfoList-BR-r13 for field posSchedulingInfoList-BR-r15.

- Add the sentence “E-UTRAN always includes this field if posSchedulingInfoList-r15 is included in SystemInformationBlockType1-BR” in the description of posSchedulingInfoList-BR.

# Conclusion

Based on company’s feedback the following proposals are made:

Proposal 1: The issue of missing scheduling restrictions of positioning SI messages for BL/CE UEs is confirmed.

Proposal 2: The CRs in R2-2109515 and R2-2109516 will be revised and agreed with the following modifications:

- Add the sentence “The CR is considered mandatory to support broadcast of positioning assistance data for BL UEs and UEs in CE.” on the coversheet.

- Remove the condition “SI-Pos-BR” for the presence of field posSchedulingInfoList-BR-r15.

- Reuse the existing IE SchedulingInfoList-BR-r13 for field posSchedulingInfoList-BR-r15.

- Add the sentence “E-UTRAN always includes this field if posSchedulingInfoList-r15 is included in SystemInformationBlockType1-BR” in the description of posSchedulingInfoList-BR.