3GPP TSG-RAN WG2 Meeting #117e                                                      R2-2202935

Online, 21 Feb–3 March 2022

Source:	Qualcomm Incorporated

Title:	Support of MDT and QoE alignment

Agenda Item:	8.14 NR QoE

Document for:	Discussion and Decision

# Introduction & Background

RAN3 has made some agreements on MDT-QoE alignment and have sent the following LS [3]. This contribution discusses RAN2 aspects to support MDT-QoE alignment.

1. Overall description:

To enable time alignment of the QoE and Immediate MDT measurements, RAN3 has agreed that an NG-RAN node is required to receive from the UE a QoE measurement session start indication and a session end indication from the application and via RRC signalling.

In the case that MDT and QoE are configured simultaneously to NG-RAN, the NG-RAN node can configure the UE with an immediate MDT configuration upon receiving the QoE measurement session start indication, to enable the MDT and QoE measurements to run in parallel. Before receiving the measurement session start indication, the corresponding immediate MDT configuration is not configured to UE.

Further, the session start/end indication can enable the NG-RAN node to add the session start time and session end time in the QoE report sent to MCE, for the alignment of an ongoing Immediate MDT and a QoE measurement started later.

The relevant agreements from RAN3#114-e and RAN3#114bis-e meetings:

To enable time alignment between an already ongoing Immediate MDT and a QoE measurement started later, the start time and end time of the QoE measurement, in addition to the Trace Reference and Trace Recording Session ID, needs to be added to the QoE measurement report at the NG-RAN node

UE assisted solution can be used for MDT-QoE alignment. UE can indicate to NG-RAN via a flag whether a QoE measurement session started/ended. If the NG-RAN knows there is an MDT configuration associated with a QoE configuration (e.g., upon receiving NG-RAN Trace ID in the QoE configuration from OAM),

- NG-RAN can configure the UE with that associated MDT configuration upon receiving the QoE measurement session start indication from the UE

- NG-RAN can deactivate the associated MDT configuration upon receiving the QoE measurement session end indication from the UE

2. Actions:

To 3GPP RAN2

ACTION:

- RAN3 respectfully asks RAN2 to take the above information into consideration and specify in RRC the QoE measurement session start/end indication from the UE to the NG-RAN node.

# Discussion

RAN3 agreed that UE needs to report QoE measurement session start or end indication to assist RAN to configure MDT measurements in a timely manner. In order to support QoE-MDT alignment, RAN2 needs to discuss how the UE should send session start or end indication to RAN.

To enable time alignment between an already ongoing Immediate MDT and a QoE measurement started later, the start time and end time of the QoE measurement, in addition to the Trace Reference and Trace Recording Session ID, needs to be added to the QoE measurement report at the NG-RAN node

UE assisted solution can be used for MDT-QoE alignment. UE can indicate to NG-RAN via a flag whether a QoE measurement session started/ended. If the NG-RAN knows there is an MDT configuration associated with a QoE configuration (e.g., upon receiving NG-RAN Trace ID in the QoE configuration from OAM),

- NG-RAN can configure the UE with that associated MDT configuration upon receiving the QoE measurement session start indication from the UE

- NG-RAN can deactivate the associated MDT configuration upon receiving the QoE measurement session end indication from the UE

Session start or end indication is to be used for gNB to configure MDT in real time, and also provide star time and end time of the QoE measurement added to QoE measurement report at gNB, so it should be taken as higher priority than QoE reporting. Existing UEAssistanceInformation message is used to transmit UE assistance information to assit gNB configuration, so it is reasonable to reuse UEAssistanceInformation message to transmit session start or end indication.

- Use UEAssistanceInformation on SRB1 to send session start or end indication.

In order to save signalling overhead on Uu (i.e., to avoid UE to send session start or end indication for every QoE configuration), NG-RAN node should be able to  configure which QoE configurations require MDT-QoE alignment, based on received information from MCE as RAN3 agreed as highlighted below. In this way, UE only need to send session start or end indication for those QoE configurations which require MDT-QoE alignment.

UE assisted solution can be used for MDT-QoE alignment. UE can indicate to NG-RAN via a flag whether a QoE measurement session started/ended. If the NG-RAN knows there is an MDT configuration associated with a QoE configuration (e.g., upon receiving NG-RAN Trace ID in the QoE configuration from OAM),

- NG-RAN can configure the UE with that associated MDT configuration upon receiving the QoE measurement session start indication from the UE

- NG-RAN can deactivate the associated MDT configuration upon receiving the QoE measurement session end indication from the UE

- RAN can indicate to UE which QoE configurations require MDT-QoE alignment, and UE only needs to consider these QoE configurations to send session start or end indication.

A UE could be configured with multiple QoE configurations, e.g. 16 QoE configurations andfor each QoE configuration, there could be multiple QoE sessions running simultaneously. If UE indicates the session start or end indication whenever each QoE session is started or ended, this will result in a large amount of signalling overhead.

Figure 1: Session start and end time

Takie the above figure as an example where both QoE conf #1 and QoE conf #2 needs MDT alignment .When session #1 is started at t1 for QoE configuration #1, UE should send session start indication to gNB, and gNB should provides MDT configuration to UE; at t2, session #2 for QoE configuration #1 is started, UE does not need to send session start indication to gNB; at t3, session #1 for QoE configuration #1 is terminated, UE should not send session end indication to gNB since there is session #2 running. Similarly, UE should not send session indication at t4, t5. At t6, when all sessions are terminated, UE sends session end indication to gNB.

- UE does not send redundant session start indication to gNB; UE does not send session end indication if there is an ongoing session for a QoE configuration requiring MDT-QoE alignment. Details are FFS.

# Conclusion

This contribution discusses how UE sends session start or end indication for QoE adn MDT alignment, and has the following proposals.

- Use UEAssistanceInformation on SRB1 to send session start or end indication.

- RAN can indicate to UE which QoE configurations require MDT-QoE alignment, and UE only needs to consider these QoE configurations to send session start or end indication.

- UE does not send redundant session start indication to gNB; UE does not send session end indication if there is an ongoing session for a QoE configuration requiring MDT-QoE alignment. Details are FFS.

# Reference

[1]    RP-210913: “New WID on NR QoE management and optimizations for diverse services”

[2]    TS 38.306, NR; User Equipment (UE) radio access capabilities

[3]    R2-2202128 LS on QoE Measurement Session Start and Measurement Session End Indication from the UE (R3-221243; contact: Ericsson)	RAN3