3GPP TSG-RAN WGX #117 electronic	R2-2203068

21st February – 3rd March 2022

Online

Source:                    	DENSO CORPORATION

Title:  	TRS/CSI-RS configuration in RRC_CONNECTED

Document for:        	Discussion and decision

Agenda Item:         	8.9.5	Other

- Introduction

A TRS/CSI-RS configuration is supposed to be provided for UE in RRC_IDLE/INACTIVE via a new SIB specific to TRS/CSI-RS. This paper discusses how the UE handles the TRS/CSI-RS configuration 1) upon the UE transits to RRC_CONNECTED, and 2) upon the UE receives it via the new SIB while in RRC_CONNECTED.

- Discussion

The TRS/CSI-RS configuration acquired by system information is intended to use while the UE is in RRC_IDLE/INACTIVE. Once the UE enters RRC_CONNECTED, the specification has supported the mechanism since Rel-15 that the TRS/CSI-RS configuration specific to the UE can be configured by the network via the dedicated signalling, in accordance with the UE capability. Although the network may configure the same TRS/CSI-RS configuration to the UE as in the TRS/CSI-RS SIB for the idle/connected mode, from the configuration point of view, UE-specific configurations are different from cell-specific configurations. It has to be clarified whether the UE continue to use the TRS\CSI-RS configuration acquired by system information, even after the UE enters RRC_CONNECTED. If it is used even in RRC_CONNECTED and the network also configures the dedicated TRS/CSI-RS configuration to the UE, it is not clear how those two configurations can co-exist. A simple approach is to use only the dedicated TRS/CSI-RS configuration in the connected mode, as defined since Rel-15. The TRS/CSI-RS configuration via the system information is release upon entering RRC_CONNECTED. The following is proposed.

Proposal 1:	The TRS/CSI-RS configuration acquired by system information whilst the UE is in RRC_IDLE/INACTIVE is released upon entering RRC_IDLE.

Whilst the UE is in RRC_CONNECTED, the UE may receive the TRS/CSI-RS SIB, e.g. system information change. If the TRS/CSI-RS configuration broadcast in the TRS/CSI-RS SIB is not used for the UE in RRC_CONNECTED. It makes sense that the UE ignores the broadcast TRS/CSI-RS configuration. The following is proposed.

Proposal 2:	The UE in RRC_CONNECTED ignores the TRS/CSI-RS configuration if acquired via the system information.

- Summary and proposal

This paper discussed the handling of TRS/CSI-RS configuration acquired via the system information, whilst the UE is in RRC_CONNECTED. In summary, the followings were proposed.

Proposal 1:	The TRS/CSI-RS configuration acquired by system information whilst the UE is in RRC_IDLE/INACTIVE is released upon entering RRC_IDLE.

Proposal 2:	The UE in RRC_CONNECTED ignores the TRS/CSI-RS configuration if acquired via the system information.

- References

[1] TS 38.331 v16.7.0, “NR; Radio Resource Control (RRC) protocol specification,”