3GPP TSG-RAN WG2 Meeting #117 electronic	R2-2203088

e-Meeting, 21st February – 3rd March 2022

Source:	vivo

Title:          Discussion on latency enhancement

Agenda Item:	8.14.3.2

Document for:	Discussion and Decision

- Introduction

In RAN2#116bis meeting, RAN2 discussed the spec impact of pre-configuration and made the following agreements. However, the format of ‘validity area’ was not discussed in detail, which was captured as ‘area ID’ in the running CR [1].

Agreements:

Proposal 3a (modified): 	 Pre-configured DL-PRS assistance data can be associated with a "validity area" at least in LPP.  FFS on details and whether it would be included in RRC broadcast.

Proposal 3:	Pre-configured DL-PRS assistance data can consist of multiple instances, where each instance is applicable to a different area within the network. FFS on additional specification impacts and whether this can already be supported with the agreement made that pre-configured DL-PRS assistance data can be associated with a "validity area".  Single instance of AD is not excluded; FFS if there would be signalling for multiple area IDs in the same instance.  Signalling details can be discussed in the LPP running CR discussion.

Besides, RAN2 discussed the pre-configured measurement gap and PRS processing window based on RAN1 LS [2][3] and made the following agreements. However, the overall procedures are not concluded, especially the activation/deactivation from LMF.

Pre-configured measurement gap

Agreements:

Proposal 5a:	A new UL MAC CE for positioning measurement gap activation and deactivation request is introduced.

Proposal 5b:	The new UL MAC CE for positioning measurement gap activation and deactivation request includes at least the ID of the pre-configured positioning measurement gap configuration for which the activation/deactivation is requested. Other parameter are FFS.

Proposal 5c (modified):	A new DL MAC CE for positioning measurement gap activation and deactivation command is introduced for positioning latency reduction.

Proposal 5d:	The new DL MAC CE for positioning measurement gap activation and deactivation command includes at least the ID of the pre-configured positioning measurement gap configuration which has been configured/activated by the gNB. Other parameter are FFS.

PRS processing window

Agreements:

Proposal 7:	The PRS processing window configuration is provided via RRCReconfiguration message. Whether PRS processing window configuration is provided per BWP or not is up to RAN1 to decide.

Proposal 8:	A new DL MAC CE for PRS Processing Window activation and deactivation command is introduced.

Proposal 9:	The new DL MAC CE for PRS Processing Window activation and deactivation command includes at least the ID of the pre-configured PRS Processing Window configuration, at least in the case when multiple PRS Processing Windows can be configured.

In this contribution, we further discuss the mechanisms to enhance the positioning latency, including:

-	The validity area in pre-configured assistance data

-	Unified procedure of pre-configured measurement gap and PRS processing window

- Discussion

  - Pre-configured assistance data

To support mobility, the pre-configured assistance data may include a list of PRS configurations with the different validity areas. For instance, in Figure 1, the pre-configured PRS configuration (i.e. list 1) of TRP 1~3 are valid when UE is camping on TRP 1/2, and the pre-configured PRS configuration (i.e. list 2) of TRP 3~5 are valid when UE is camping on TRP 3/4/5. In this case, when the target UE is served by TRP1, the PRS configuration in list 1 is valid for the target UE to measure. After the target HO to TRP4 due to mobility, the UE shall measure PRS configuration in list 2. With this, the configuration of PRS of different areas can be pre-configured to the target UE at once, hence the overall latency is further reduced.

Figure 1 pre-configured assistance data with a list of available PRS configurations

Due to the priority rule in RAN1, the same pre-configured PRS resources associated with different validity areas may be in different orders as the priority in different areas can be different. Taking TRP3 as an example, the PRS resources 1&2 are higher priority compared with PRS resources 3&4 in list 1 while are a lower priority in list 2.

TS38.214 Within a positioning frequency layer, the DL PRS resources are sorted in the decreasing order of priority for measurement to be performed by the UE, with the reference indicated by nr-DL-PRS-ReferenceInfo being the highest priority for measurement, and the following priority is assumed: - Up to 64 dl-PRS-IDs of the frequency layer are sorted according to priority; - Up to 2 DL PRS resource sets per dl-PRS-ID of the frequency layer are sorted according to priority.

Based on the above analysis, the pre-configured PRS resources can be:

-- ASN1START NR-DL-TDOA-ProvideAssistanceData-r16 ::= SEQUENCE { nr-DL-PRS-AssistanceData-r16 NR-DL-PRS-AssistanceData-r16 OPTIONAL, -- Need ON nr-DL-Preconfigured-PRS-AssistanceData-r17 SEQUENCE (SIZE (1..nrMaxPreconfiguration-r17)) OF NR-DL-Preconfigured-PRS-AssistanceDataPerArea-r17 OPTIONAL, -- Need ON nr-SelectedDL-PRS-IndexList-r16 NR-SelectedDL-PRS-IndexList-r16 OPTIONAL, -- Need ON nr-PositionCalculationAssistance-r16 NR-PositionCalculationAssistance-r16 OPTIONAL, -- Cond UEB nr-DL-TDOA-Error-r16 NR-DL-TDOA-Error-r16 OPTIONAL, -- Need ON ... } NR-DL-Preconfigured-PRS-AssistanceDataPerArea-r17 ::= SEQUENCE { validityCellList-r17 SEQUENCE (SIZE (1..nrMaxValidCell)) OF NCGI-r15, expirationTime-r15 UTCTime, nr-DL-PRS-AssistanceData-r16 NR-DL-PRS-AssistanceData-r16 } -- ASN1STOP

Proposal 1: The validity area associated with each pre-configured assistance data can be a list of cells that the target UE may camp on.

When the UE moves to an area where no pre-configured assistance data is valid, the UE shall stop using the pre-configured assistance data for positioning and request new assistance data from LMF.

Proposal 2: The UE shall request new assistance data when no pre-configured assistance data is valid.

Meanwhile, taking the pos SIB as a reference, the pre-configured assistance data shall be associated with a validity timer.

AssistanceDataSIBelement-r15 ::= SEQUENCE { valueTag-r15 INTEGER (0..63) OPTIONAL, -- Need OP expirationTime-r15 UTCTime OPTIONAL, -- Need OP cipheringKeyData-r15 CipheringKeyData-r15 OPTIONAL, -- Need OP segmentationInfo-r15 SegmentationInfo-r15 OPTIONAL, -- Need OP assistanceDataElement-r15 OCTET STRING, ... }

The intention to introduce the validity timer includes:

- The UE should not hold the pre-configuration all the time in case it arrives at another area where the PRS ID (ranges from 0 to 255) is the same while the PRS configuration is different. In this case, the LMF may correlate the measurements report with incorrect TRPs.

- The gNB may modify or stop the PRS transmission at a future time due to RAN overload or indicated by the OAM, which means the pre-configured PRS configuration is valid in a specific duration and the UE is not expected to measure the invalid PRS configuration.

Thus, the UE shall release the pre-configured assistance data when the validity timer expires.

Proposal 3: The pre-configured assistance data shall associate with a validity timer.

Proposal 4: The UE shall release the pre-configured assistance data when the validity timer expires.

  - Unified procedure of pre-configured measurement gap and PRS processing window

RAN1 introduced the pre-configured measurement gap and PRS processing window to further reduce the latency of PRS measurement,

- The gNB may pre-configure the MG to the UE, and the LMF or UE may request to activate them via NRPPa message or MAC CE.

- The gNB may pre-configure the PRS processing window to the UE, and UE can process PRS and receive data (e.g., PDCCH/PDSCH/CSI-RS reception) in a configured PRS processing window after the LMF activate them via NRPPa message.

The related agreements made by RAN1 are as follows:

pre-configured measurement gap Agreement Support the following options (in the agreement made in RAN1#106-e) for a new mechanism of MG activation request for the purpose of positioning. - Option 2: by UE (via UCI or UL MAC CE) - Select only one of UCI and UL MAC CE in RAN1#106bis-e - Option 1: by LMF (via an NRPPa message) - Note: This is transparent to the UE Agreement Preconfiguration of MG(s) in RRC is supported from RAN1 perspective. - Each MG in the preconfiguration is associated with an ID - The information in the UL MAC CE for MG activation request by the UE can be one ID associated with the preconfiguration of the MG Send an LS to RAN2 and RAN3

PRS processing window Agreement The following options are supported subject to UE capability for priority handling of PRS when PRS measurement is outside MG. - Option 1: UE may indicates support of two priority states. - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 2: UE may indicate support of three priority states - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 3: UE may indicate support of single priority state - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS Note: SSB is a separate issue. Agreement At least the following parameters for PRS processing window from the gNB to the UE are supported. - Starting slot - Periodicity - Duration/length - Cell and SCS information associated with the above parameters Discuss during the maintenance phase on the necessity of other parameters including but not limited to - Processing type (associated with the corresponding UE capability 1A/1B/2) - Band/CC-ID as needed depending on each scenario on which the PRS processing window is applied - The above cell and SCS information to determine where/when the PRS processing window is applied Note: Indication of processing type does not suggest UE indication of multiple capabilities among (1A/1B/2) is already supported, which is a separate discussion. Note: Some of the parameters above may not be mandatory for a PRS processing window Agreement The priority of PRS for UE supporting two priority states and three priority states can at least be indicated in RRC. Agreement For capability 1A as per working assumption made in RAN1#106-e, the DL signalings/channels in a per UE fashion (i.e. both across NR & LTE) inside the PRS processing window are dropped if the DL PRS is determined to be higher priority. For capability 1B as per working assumption made in RAN1#106-e, only the DL signalings/channels from a certain band inside the PRS processing window are dropped if the DL PRS is determined to be higher priority. Working assumption: Subject to UE capability, support PRS measurement outside the MG, within a PRS processing window, and UE measurement inside the active DL BWP with PRS having the same numerology as the active DL BWP. - Inside the PRS processing window, subject to the UE determining that DL PRS to be higher priority, support the following UE capabilities: - Capability 1: PRS prioritization over all other DL signals/channels in all symbols inside the window. - Cap. 1A: The DL signals/channels from all DL CCs (per UE) are affected. - Cap. 1B: Only the DL signals/channels from a certain band/CC are affected. - FFS: band or CC - Capability 2: PRS prioritization over other DL signals/channels only in the PRS symbols inside the window - A UE shall be able to declare a PRS processing capability outside MG. - FFS: Details of capability signalling (e.g., per UE or per band, etc.). Agreement PRS processing window request to the gNB by the LMF is supported from RAN1 perspective. - It is up to RAN3 to design the necessary information to be transferred in the NRPPa message. - Note: It is up to gNB to determine the usage of measurement gap or PRS processing window - Include it in the LS to RAN2 and RAN3. Agreement For PRS processing window configuration and indication, at least the following mechanism is supported - RRC (pre-)configuration for PRS processing window configuration and DL MAC CE activation for PRS processing window, respectively. Include it in the LS to RAN2 and request RAN2 to decide whether DL MAC CE is feasible for this indication.

The overall procedures were captured in [4] while not concluded, and there remain some open issues:

How does the gNB determine the pre-configured measurement gap?

The gNB is not aware of which UE will perform DL-PRS measurement, thus the pre-MG will not properly be allocated. In the RRC location measurement indication procedure, the UE sends some assistance data to assist the gNB to determine the corresponding matched gap configuration. Based on the same consideration, the LMF can send some essential information to the gNB to let the serving gNB knows in advance the time/frequency domain characteristics of PRS to be measured. And then the pre-configured MG configuration can be generated by gNB appropriately.

Besides, to reduce the latency, the pre-configuration needs to occur before the location request arrives at UE. Otherwise, no latency reduction can be achieved.

Observation 1: The pre-configured measurement gaps need to be sent to the UE before the location request arrives at the UE.

How does the LMF send the pre-configured measurement gap activation request?

The LMF is not aware of whether UE is configured with the pre-MG in the current procedure in [4]. In our view, at least the following options can be considered for LMF to activate the pre-MG.

- Option 1: The ID associated with the pre-MG, the precondition is that the pre-MG shall be sent to LMF after the pre-MG is sent to target UE.

- Option 2: UE associated assistance data in the LocationRequestInformation (i.e., positioning requirement, latency, Bandwidth that is needed to meet accuracy requirement)

- Option 3: UE associated assistance data similar to LocationMeasurementInfo (Rel-16 RRC MG request includes PRS time/frequency characteristics)

In the last RAN3 meeting, option 3 was agreed to send the assistance data similar to LocationMeasurementInfo to the gNB to let the serving gNB determine the activation.

The coordination of pre-configured measurement gap and PRS processing window.

In our understanding, the PRS processing window is similar to the measurement gap. The main difference is that the UE still needs to receive the data transmission in the PRS processing window. Therefore, the detail schemes for the pre-configured gap are also applied to PRS priority. The LMF may send the common assistance data and activation request to the gNB and it is up to gNB implementation to determine the pre-configuration and activation of the measurement gap and/or PRS processing window.

Based on the above analysis, we suppose that the overall procedure is as below:

Figure 2 unified pre-configured MG/processing window procedure

1. The LMF sends the assistance information to gNB via NRPPa message to help with the pre-configured MG/PRS processing window configuration. The NRPPa message can be non-UE associated.

2-4. The gNB determines the pre-configured MG or/and PRS processing window configuration based on the assistance data and configures it to the UE. The pre-configuration is configured to the UE with potential positioning requirements and left to gNB implementation.

5. The Location Request arrives at the LMF. The LMF determines the PRS configuration and sends the PRS configuration to UE.

6. The LMF sends an NRPPa message to gNB including the PRS measurement characteristics. The NRPPa message is UE associated.

7. Based on the PRS configuration, the UE may send MAC CE to gNB to request activating the pre-configured MG or PRS processing window.

8. Based on Step 6 or Step 7, the gNB determines to configure a new measurement gap/PRS processing window or activate the pre-configured measurement gap/PRS processing window by the implementation.

9. The gNB then activates the MG/PRS processing window via DL MAC CE, or configures a new measurement gap/PRS processing window via RRC Reconfiguration message.

For the assistance information to determine the pre-configured MG in step 1, the gap parameters of pre-configuration need to try to align with the potential PRS configuration as possible, e.g., try to let potential PRS fall into gap duration. The LMF has each TRP’s PRS configuration information and determines the PRS configuration according to positioning QoS. The LMF can send the PRS configurations of related TRP to gNB in advance to assist the gNB to determine the pre-configuration parameters. The PRS configuration includes the time/frequency characteristics of PRS, i.e., periodicity/offset for time-domain per frequency layer.

In Step 6, the LMF can send the UE-associated PRS measurement characteristics and the QoS of positioning to the gNB to activate the pre-configured MG.

However, RAN3 introduced a UE-associated procedure to exchange the assistance data in step 1 [5], which implies that the pre-MG is configured to specific UE after LMF receives the location request for the UE. In this case, we are wondering how the latency is reduced and propose to ask RAN1 to confirm.

Proposal 5: LS to RAN1 to confirm the pre-MG is configured to specific UE after LMF receives the location request for the UE.

Besides, RAN3 has not decided to reuse a unified procedure with pre-MG or introduce a new NRPPa class 2 procedure for the LMF-initiated PRS Processing Window activation request.

As the NRPPa is in RAN3 scope, we propose:

Proposal 6: LS to RAN3 to specify the unified NRPPa procedure including the assistance data from LMF to gNB to help with the activation/deactivation of pre-configured MG and PRS processing window.

- the required QoS of positioning, e.g., response time

- the suggested start time for measurement

For the pre-configured MG in step 3, in our understanding, similar to the normal measurement gap, a list of pre-configured MGs could be added to MeasGapConfig and each pre-configured MG is associated with an ID. For example:

MeasGapConfig ::=                   SEQUENCE {

gapFR2                   SetupRelease { GapConfig }            OPTIONAL,   -- Need M

...,

[[

gapFR1                   SetupRelease { GapConfig }         OPTIONAL,   -- Need M

gapUE                    SetupRelease { GapConfig }          OPTIONAL    -- Need M

]],

[[

preconfiguredGap         SetupRelease { PreconfiguredGapList }         OPTIONAL   -- Need M

]]

}

PreconfiguredGapList ::=  SEQUENCE (SIZE (1..maxNrofPreconfiguredGap)) OF PreconfiguredGapConfig

PreconfiguredGapConfig ::=                       SEQUENCE {

preconfiguredGapID                  INTEGER (1.. maxNrofPreconfiguredGap),

preconfiguredGapState               ENUMERATED {activated}             OPTIONAL,

preconfiguredGap     ::=      CHOICE {

gapFR2                     GapConfig,

gapFR1                     GapConfig,

gapUE                      GapConfig,

res                        NULL

}

}

For LMF initiated MG requests, none of the pre-configured GAPs may be suited for the PRS measurement characteristics in Step 6. In this case, the gNB shall be able to modify the pre-MG configuration. To reduce the time delay, the initial status of a new gap should be activated. Similarly, a pre-configured PRS processing window can have an initial status. Therefore, we suggest:

Proposal 7: Introduce an initial status of each pre-configured MG and PRS processing window, i.e., activated or not.

As to the LMF initiated MG request, the UE is not aware of the procedure and may request for another MG before the UE gets the MG configuration. To avoid duplicated MG request from both LMF and UE, the LMF shall indicate to UE whether the LMF will help to request MG when LMF send the LPP RequestLocationInformation message to the UE. If the LMF indicates that the LMF will not help to request MG, the UE may request the MG by UL MAC CE or RRC message in implementation.

Proposal 8: The LMF should indicate whether the LMF will help to request MG when LMF sends the LPP RequestLocationInformation message to the UE.

If the gap pre-configuration scheme is introduced for the location feature, one question is whether the scheme is restricted to only the scenario, i.e. whether the UE can only send the UL MAC CE to activate the pre-configured gap after receiving the RequestLocationInformation message? Can the UE request to activate the pre-configured gap for other purposes, e.g., RRM measurement?

Proposal 9: RAN2 to discuss whether the UL MAC CE activation of the pre-configured gap is used only for positioning scenario, or also for other purposes, e.g., RRM measurement.

- Conclusion

pre-configured assistance data

Proposal 1: The validity area associated with each pre-configured assistance data can be a list of cells that the target UE may camp on.

Proposal 2: The UE shall request new assistance data when no pre-configured assistance data is valid.

Proposal 3: The pre-configured assistance data shall associate with a validity timer.

Proposal 4: The UE shall release the pre-configured assistance data when the validity timer expires.

pre-configured measurement gap and PRS processing window

Observation 1: The pre-configured measurement gaps need to be sent to the UE before the location request arrives at the UE.

Proposal 5: LS to RAN1 to confirm the pre-MG is configured to specific UE after LMF receives the location request for the UE.

Proposal 6: LS to RAN3 to specify the unified NRPPa procedure including the assistance data from LMF to gNB to help with the activation/deactivation of pre-configured MG and PRS processing window.

- the required QoS of positioning, e.g., response time

- the suggested start time for measurement

Proposal 7: Introduce an initial status of each pre-configured MG and PRS processing window, i.e., activated or not.

Proposal 8: The LMF should indicate whether the LMF will help to request MG when LMF sends the LPP RequestLocationInformation message to the UE.

Proposal 9: RAN2 to discuss whether the UL MAC CE activation of the pre-configured gap is used only for positioning scenario, or also for other purposes, e.g., RRM measurement.

- Reference

- R2-2201723	Running LPP CR for NR positioning enhancements	Qualcomm

- R2-2200074	LS on latency improvement for PRS measurement with MG

- R2-2200089	LS on PRS processing window

- R2-2201815	Running 38.305 CR for Positioning WI on RAT dependent positioning methods

- R3-221558		Introduction of NR Positioning enhancements to NRPPa