3GPP TSG-RAN2 Meeting #117-e	R2-2203254

eMeeting, 21 February - 3 March 2022

Agenda Item:	8.9.5 Other

Source: 	Ericsson

Title:  	TRS exposure

Document for:	Discussion and Decision

# Introduction

The following topics are discussed in this contribution:

- SIB issues

- Dedicated signalling

- On-demand TRS SIB

- UE capability signalling

- Configuration of L1 availability

# Background

RAN1

The RAN 1 related work is finished and now is only at the maintenance stage.

RAN2

The following agreement were made in the last meeting:

- The number of bits N in the bitmap used for L1 availability indication is derived implicitly from the number of different values of indBitID. There is no need for an explicit parameter.

- RAN2 confirm TRS/CSI-RS can be applied to eDRX UEs.

- Confirm that there will be no particular mechanism for availability indication based on SIB (beyond the presence of the RS configuration)

- A UE which acquired SIB-X with a TRS/CSI-RS configuration but didn’t yet receive an associated L1-based availability indication considers the configured TRS/CSI-RS as FFS: “unavailable” or “available”.

- R2 doesn't send an LS to R1 on SIB segmentation

- [055] Indicating the TRS/CSI-RS availability in Idle/Inactive when releasing the UE to Idle/Inactive in the RRCRelease message is not pursued.

- [055] RAN2 follows RAN1 agreement that if TRS resource is configured in SIB, L1 based availability indication is always enabled based on that configuration.

- [055] RAN2 waits for RAN1 to finalize the contents of SIB-X before finalizing aspects on SIB-X sizing, segmentation etc

# Discussion

## SIB issues

SIB segmentation can be useful for TRS exposure, however it is not necessary. The NW has the option to configure even the maximum number of TRS resource sets i.e., 64 sets such that SIB segmentation is not necessary. Furthermore, additional optimization can be considered in ASN1 to reduce the need for segmentation. We should not forget that segmentation leads to a large additional overhead on the NW, therefore, a reasonable NW design would avoid that by proper configuration.

## On-demand TRS SIB

It is not obvious when to stop broadcasting the new SIB with the TRS information, i.e. the UE that requested the SIB may have moved out of the cell, while other UEs moved into the cell which are using the feature. There is the possibility to switch the SIB off after some time, and check if a UE reacts to this by requesting the SIB to be turned on again, or the SIB is switched off during off-peak hours to minimize the possible impact on UEs:

Proposal 1: Stop broadcasting the new (on-demand) SIB is left to NW implementation.

## UE capability signalling

When the UE does not signal a capability related a power saving feature, the network does not know if UEs are using the (potentially large and segmented) SIB that is being broadcasted in all cells of the network/tracking area, or whether UEs use the L1 indications sent in Paging PDCCH.

In case of optional features without capability signalling, i.e. there is no inter-operability issue when the UE does not configure/use the feature, the network may waste resources (e.g. large SIBs and L1 availability indication) without benefits.

Introduction of explicit capability signalling does not completely resolve this issue, i.e. the UE may support the feature, but no use it, or only use under certain conditions, however, still it is worth for the NW to know the capabilities of the UEs to at least make sure if transmitted it can be used by some UEs.

Proposal 2: UE capability of using TRS in idle/inactive mode is signaled explicitly.

## Configuration of L1 based availability

It is simple to assume that if configuration of TRS is provided to idle/inactive UEs, at least one approach of Paging DCI or PEI is used to indicate the availability. Nevertheless, it should be up to the NW to decide to indicate the availability in one of them or both. For example, the reserved bits in paging DCI may be needed for other applications, and thus TRS availability cannot be indicated there but it can in PEI, or other way around.

Proposal 3: It is up to the NW to configure TRS availability indication only in Paging DCI or PEI or in both.

# Summary

RAN2 is kindly asked to discuss:

Proposal 1: Stop broadcasting the new (on-demand) SIB is left to NW implementation.

Proposal 2: UE capability of using TRS in idle/inactive mode is signaled explicitly.

Proposal 3: It is up to the NW to configure TRS availability indication only in Paging DCI or PEI or in both.