3GPP TSG-RAN WG2 #117	R2-220XXX

- Online, February 21 – March 3, 2022

Agenda item:	8.17.4.2

Source:	Samsung

Title:	Summary of [AT117-e] [016] [feMIMO] MAC (Samsung)

Document for:	Discussion & Decision

# Introduction

This document is intended to progress below offline discussion.

- [AT117-e][016][feMIMO] MAC (Samsung)

Scope: Take into account on-line. Make further progress based on non-resolved parts of R2-2203709. Take into account new LS from RAN1 when/if it becomes available, to the extent reasonable. Update MAC CR. (This discussion will also continue as a post discussion for the CR). Determine agreeable parts, identify discussion points if any.

Intended outcome: Report, revised MAC CR (CR might not be needed for CB).

Deadline: In time for online CB W2 Wednesday

- Phase I discussion

It includes the non-resolved parts of R2-2203709.

Deadline for Phase I discussion: W2 Monday (28th Feb, 1200 UTC)

- Phase II discussion

Further issues will be triggered after receiving RAN1 LS replies regarding the below RAN2 LSes:

- LS on MPE information signalling (R2-2111600)

- LS on feMIMO RRC parameters (R2-2202002)

- LS on Enhanced TCI state indication for UE-specific PDCCH MAC CE (R2-2201950)

Deadline for Phase II discussion: In time for online CB W2 Wednesday

The following delegates participated in the discussion:

| Company | Contact Name | Email |
| --- | --- | --- |
| ASUSTeK | Xinra Kung | Xinra_Kung@asus.com |
| LGE | Hanul Lee | hanul.lee@lge.com |
| Samsung | Anil Agiwal (BFD/BFR) Seungri Jin | anilag@samsung.com seungri.jin@samsung.com |
| OPPO | Youxin ZhongdaDu | youxin@oppo.com duzhongda@oppo.com |
| Fujitsu | Meiyi Jia | jiameiyi@fujitsu.com |
| Apple | Fangli | fangli_xu@apple.com |
| CATT | Erlin Zeng | erlin.zeng@catt.cn |
| Xiaomi | Yumin Wu | wuyumin@xiaomi.com |
| Qualcomm | Ruiming Zheng | rzheng@qti.qualcomm.com |
| Sharp | Hidekazu Tsuboi | Tsuboi.hidekazu@sharp.co.jp |

# Phase I Discussion

## Multi TRP Beam Failure Detection and Recovery

### Enhanced BFR MAC CE Format Aspects

#### Indication of whether one or both TRPs are failed

RAN2#115e Agreement

- BFD-RS set ID is included in BFR MAC CE to identify the failed TRP

RAN2#116e Agreement

- New BFR MAC CE including beam failure recovery information of both failed TRPs is transmitted when beam failure is detected for both TRPs of SCell.

- The Following pieces of information are included in enhanced BFR MAC CE for M-TRP BFR

  - Info 1: For the Identity of serving cell of failed TRP, Ci/SP fields are included.

  - Info 2: For indicating whether candidate beam is available or not for a failed TRP of serving cell, AC field is included.

  - Info 3: Candidate beam (if available) for a failed TRP is indicated by including the Candidate RS ID field.

- Both single octet bitmap (7 Ci bits and 1 SP bit) and 4 octet bitmap (31 Ci bits and 1 SP bit) formats are supported for enhanced BFR MAC CE.

- Both truncated and non-truncated enhanced BFR MAC CE are supported.

RAN2#116bis-e Agreement

- For the enhancement BFR MAC CE design, it is FFS with:

- •	Two sets of serving cell bitmap (Option 2)

- •	A bitmap in addition to serving cell bitmap (Option 3)

| Source | Proposal |
| --- | --- |
| Samsung [1] | Proposal: Include a Sj bitmap in addition to serving cell Ci bitmap, which indicates per failed Serving Cell configured with mTRP BFD/BFR whether one or both of the TRPs associated with the Serving Cell failed. The R bit of the AC/Candidate RS ID octet indicates the failed TRP ID (i.e. BFD-RS set ID) Proposal: One octet Sj bitmap is included in MAC CE with 7 Ci bits and 1 SP bit. Up to 4 octets Sj bitmap are included in MAC CE with 31 Ci bits and 1 SP bit. Proposal: Sj corresponds to jth failed serving cell in increasing order of Serving Cell Index. Serving Cell index of SpCell is assumed to be zero. |
| Vivo [2] | Proposal: Include two sets of serving cell bitmap (i.e. option 2) for the enhanced BFR MAC CE design. |
| Fujitsu [3] | Proposal: For the enhancement BFR MAC CE design, a bitmap in addition to serving cell bitmap (Option 3) applies. |
| Oppo [4] | Proposal: For the enhancement BFR MAC CE design, a bitmap in addition to serving cell bitmap (Option 3) is used as baseline. |
| Apple [5] | Proposal: Agree the Enhanced BFR MAC CE format as indicated in Figure-1. Figure 1 |
| Intel [7] | Proposal : Enhanced BFR MAC CE includes AC field and include two sets of serving cell bitmap in MAC CE (i.e. Option 2). |
| Qualcomm [12] | Proposal : The Rel-17 mTRP BFR MAC CE selects option 2 by using two sets of TRP bitmap at least for the eight-CC case. |
| CATT [16] | Proposal: RAN2 to consider option 2 (Two sets of serving cell bitmap introduced in the BFR MAC CE) as baseline to design the detailed format of the enhanced BFR MAC CE. |
| ZTE [17] | Proposal: Two sets of serving cell bitmap are supported for enhanced BFR MAC CE. Proposal: For the sequence of presence of the octets containing AC field and Candidate RS field, the first one or two octets are associated with the SpCell. Then the subsequent octets are present in ascending order based servingcellIndex in the first set of serving cell bitmap, and then the left octets are present in ascending order of the servingcellIndex in the second set of serving cell bitmap. |
| Huawei [21] | Proposal: Two bitmaps are needed in the enhanced BFR MAC CE. One bitmap indicates serving cells which occurred beam failure, and the other bitmap indicates the number of failed TRPs per failed serving cell. |
| Nokia [24] | Proposal: The first bitmap in the New BFR MAC CE indicates the failure status of the Serving Cell (with one BFD-RS set configured) or the failure status of the BFD-RS set#0 of the Serving Cell (with two BFD-RS sets configured). Proposal: The second bitmap indicates the failure status of the BFD-RS set#1 of the Serving Cells with two BFD-RS sets configured. Proposal: The size of the second bitmap in the New BFR MAC CE is based on the number of Serving Cells configured with two BFD-RS sets (ie., it can be smaller than the first bitmap). Proposal: For each failed BFD-RS set, the UE indicates if the candidate beam evaluation was completed or not upon the building of the New BFR MAC CE. The R bit of the AC/R/Candidate RS ID octet can be used for the purpose. |

Q1: Which one of the following options do you agree for enhanced BFR MAC CE format?

Option 1:

- Include two sets of serving cell bitmap in MAC CE. The first set of serving cell bitmap indicates the failure information associated with the first BFD-RS set and the second set of serving cell bitmap indicates the failure information associated with the second BFD-RS set.

- Beam failure recovery information of BFD-RS set does not include TRP ID (i.e. BFD-RS set ID)

Option 2:

- Include a bitmap in addition to previously agreed serving cell bitmap which indicates per failed Serving Cell configured with mTRP BFD/BFR whether one or both of the TRPs associated with the Serving Cell failed.

- Beam failure recovery information of BFD-RS set includes TRP ID (i.e. BFD-RS set ID) as previously agreed.

| Company | Option 1/Option 2 | Comments (if any) |
| --- | --- | --- |
| ASUSTeK | - Option 2 | A first bitmap indicates failed Serving Cells and include an additional bitmap to indicate one or both of the TRPs failed. |
| LGE | - Option 1 | Option 1 does not require the additional TRP ID field and is simple. |
| Samsung | - Option 2 | In our view both option works. Option 2 is aligned with previous RAN2 Agreements. - Previously we have agreed that BFD-RS set ID is included in MAC CE to identify TRP ID. In option 2, TRP ID is included in beam failure recovery information octet (AC, BFD RS set ID, candidate RS ID). In option 1, TRP ID is not included in MAC CE - Previously we have agreed that Ci bitmap/SP is included to identify the serving cell for which beam failure is detected for at least one TRP ID. Option 2 does not change this. In option 1, first Ci bitmap/SP definition needs to be changed. In option 1, bit in first Ci bitmap/SP bit indicates whether beam failure is detected or not for TRP 0 of serving cell. Option 2 can have less overhead in case there is no serving cell for which both TRPs have failed. In option 1, second bitmap needs to be always included when there is failure of only second TRP of one or more serving cell (s). In option 1, one R bit is unused as TRP ID is not included in beam failure recovery information octet, however this does not reduce any overhead. |
| OPPO | - Option 2 | Option 2 requires less signalling overhead. |
| Fujitsu | - Option 2 | We think that Option 2 is clear and requires less bytes. Option 1 has something to clarify. For example, how to provide the bitmaps in case that some serving cells are configured with multiple TRPs while some serving cells only support cell level beam failure detection/recovery. |
| Intel | - Either option is ok. | Both options are working and there seems no big difference. Option 1 indicates BFD RS set ID implicitly, while Option 2 indicates explicitly. Regarding option 2, do we have any additional bit to omit the second bit for the case where all serving cells have only one TRP failed? [Samsung] No, there is no additional bit. I have updated the comment. |
| Apple | - Option 2 | Option 2 has less signaling overhead when the number of the serving cell with failed TRP info is less than the total number. For example, If UE is configured with 32 serving cells, but only <8 serving cells are required to indicate the failed TRP info, in Option2, the overhead is only 1-octet, but in Option 1, the overhead is 4-octet. |
| CATT | - Option 1 | We agree both options work. Our preference is for Option 1, as a) Option 1 does not need the BFD-RS set ID, and thus the reserved R bit could be used for future extension. b) in case cases, Option 2 is less efficient, e.g., 2 bits “01” or “10” are simply used for Option 1 to indicate that which TRP fails and which not for the case when only one out of two TRPs fails, but for Option 2 this is not possible, some further indication is needed. |
| Xiaomi | - Option 2 | We have no strong view, but slightly prefer Option 2 due to the less signalling overhead. |
| Qualcomm | - Option 1 | Option 1 is simpler, and we think both options are aligned with previous RAN2 agreements. In Option 1, the second set of serving cell bitmap can be optimized to indicate the failure status of the second TRP only if the mTRP is configured in one serving cell. Option 2 requires indicating the BFD-RS set ID in the beam failure recovery information octet which is not good for the truncation case and hence is not preferred. |
| Sharp | - Option 2 | Option 2 requires less overhead. |

Q2: If option 1 is agreed in Q1, which one of the following options do you agree for second serving cell bitmap?

- Option a: The size of the second serving cell bitmap is based on the number of Serving Cells configured with two BFD-RS sets (i.e., it can be smaller than the first bitmap).

- Option b: The size of the second serving cell bitmap is same as the size of the first serving cell bitmap

| Company | Option a/Option b | Comments (if any) |
| --- | --- | --- |
| LGE | - Option b | Option b is easy to understand and simple. |
| Samsung | - Option b |  |
| CATT | - Option b, with comments | Both options acceptable to us. For simplicity, Option b could be used. For the case of when a single octet bitmap is used (i.e., greatest ServCellIndex less than 8), it seems no much room for singaling saving, but for four octets case, Option b might save a few octets depending on the real cases. |
| Qualcomm | - Option a |  |

Q3: If option 1 is agreed in Q1, do you agree that R bit in beam failure recovery information octet of failed BFD-RS set can indicate if the candidate beam evaluation was completed or not upon the building of the New BFR MAC CE?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| LGE | - No | We prefer to follow the legacy behaviour, i.e., if the evaluation of the candidate beams has not been completed, Ci field set to 0 and the octet containing AC field is not included. |
| Samsung | - No | Same view as LGE |
| CATT | - No | Save view as LGE. |
| Qualcomm | - No | Follow the legacy behaviour. |

Q4: If option 2 is agreed in Q1, which one of the following options do you agree for the bitmap in addition to serving cell bitmap?

- Option a: The size of the bitmap is based on the number of Serving Cells configured with two BFD-RS sets (i.e., it can be smaller than serving cell bitmap).

- Option b: The size of the bitmap is same as the size of the serving cell bitmap

- Option c: The size of the bitmap is based on the number of failed Serving Cells configured with two BFD-RS sets

| Company | Option a/b/c | Comments (if any) |
| --- | --- | --- |
| ASUSTeK | - Option b | Fixed size bitmap to serving cell bitmap is simpler for mapping and can avoid decoding error if mTRP configuration is asynchronous between UE and gNB. |
| LGE | - Option b | Option b is easy to understand and simple. |
| Samsung | - Option b or c | Option c is preferable to reduce overhead |
| OPPO | - Option c |  |
| Fujitsu | - Option c | If Option c is supported, the bitmap will not be included in the Enhanced Truncated BFR MAC CE when 4-octet serving cell bitmap is truncated to 1-octet bitmap. |
| Apple | - Option c | Option c has less signalling overhead considering the max number of serving cell with 2 BFD-RS sets could be 32. |
| Xiaomi | - Option c |  |
| Qualcomm | - Option c |  |
| Sharp | - Option c | Option c can reduce the overhead. |

#### LCID/eLCID in MAC Subheader of Enhanced BFR MAC CE

| Source | Proposal |
| --- | --- |
| Samsung [1] | Proposal: LCID is used for Enhanced BFR MAC CE with one octet Ci and truncated Enhanced BFR MAC CE with one octet Ci. Proposal: eLCID is used for Enhanced BFR MAC CE with four octets Ci and truncated Enhanced BFR MAC CE with four octets Ci. |
| Huawei [21] | Proposal: Use eLCID for the 1-octet Ci enhanced BFR MAC CE and for the 1-octet Ci truncated enhanced BFR MAC CE. If companies are not ready to make such a decision, this can be discussed in A.I. 8.0.4. |

Q5: Which one of the following options do you agree?

Option 1:  Use LCID for Enhanced BFR MAC CE with one octet Ci and truncated Enhanced BFR MAC CE with one octet Ci.

Option 2:  Use eLCID for Enhanced BFR MAC CE with one octet Ci and truncated Enhanced BFR MAC CE with one octet Ci.

| Company | Option1/2 | Comments (if any) |
| --- | --- | --- |
| ASUSTeK | - Option 2 |  |
| LGE | - Option 2 | As discussed in the agenda 8.0.4, there are few values left for LCID and the use of LCID should be avoided except the essential case. |
| Samsung | - Option 1 | To minimise overhead of BFR MAC CE transmitted in RA triggered for BFR |
| OPPO | - Option 2 |  |
| Fujitsu | - Option 2 | As agreed in A.I. 8.0.4, coverage limited cases shall use LCID and other cases use eLCID. |
| Apple | - Option 1 | LCID could be used for the truncated BFR MAC CE with one octet Ci. Following the guidance we made in main session, the truncated Enhanced BFR MAC CE may be delivered in the coverage limited cases - Confirm that coverage limited cases shall use LCID, other cases use eLCID |
| CATT | - Option 2 | It is true that we already reached some conclusion in A.I. 8.0.4, so option 2 should the way to go. |
| Xiaomi | - Option 2 |  |
| Qualcomm | - Option 1 |  |
| Sharp | - Option 2 |  |

#### Truncation Aspects

| Source | Proposal |
| --- | --- |
| Samsung [1] | Proposal: TRP level truncation is supported. Proposal: During truncation, TRP level truncation is applied to maximize the number of serving cells for which beam failure recovery information of at least one TRP is included in MAC CE. |
| Vivo [2] | Proposal : In the Truncated Enhanced BFR MAC CE, it can include the BFR info of one TRP if the BFR of both TRPs are triggered. |
| Fujitsu [3] | Proposal: If per TRP truncation is supported, the octets containing TRP BFR information, if present, are included in ascending order based on the ServCellIndex of SCell and BFD-RS ID. RAN2 to discuss the following two options: - Option 1: based on the ServCellIndex first - Option 2: based on BFD-RS ID first |
| Apple [5] | Proposal: In the Truncated Enhanced BFR MAC CE, it can only include the BFR info of one TRP if the BFR of both TRPs are triggered. |
| Qualcomm [12] | Proposal: In the truncated enhanced BFR MAC CE, the second failed TRP field (if any) of each serving cell are truncated after truncating all the beam failure information of the failed TRPs or failed serving cells. |
| LG [15] | Proposal: If the beam failure is detected on both TRP, the truncation of BFR information per TRP is not supported, i.e., the UE does not includes BFR information for both TRP for the Serving Cell if there is not enough bits. Proposal: If the truncation of BFR information per TRP is supported, Truncated Enhanced BFR MAC CE that includes BFR information for a one TRP for a cell first, then includes BFR information for the remaining TRPs for the cell if UL grant remains. |
| CATT [16] | Proposal: The enhanced truncated BFR MAC CE can contain the BFR information for one of both failed TRPs when there are no enough bits for uplink transmission. |
| Huawei [21] | Proposal: If beam failure occurred on both TRPs of a serving cell and the grant size can only accommodate one TRP’s BFR information, the UE truncates the BFR MAC CE and only include one TRP’s BFR information. |
| Nokia [24] | Proposal: The TRP level truncation is supported such that the beam failure information of BFD-RS set#0 is included before the BFD-RS set#1 for each Serving Cell with both BFD-RS sets in failure condition – otherwise, the Rel-16 truncation principle is applied. |

Q6: Do you agree that for TRP level truncation, beam failure recovery information of one TRP is included first before the other TRP for each Serving Cell with both BFD-RS sets in failure condition?

| Company | Option | Comments (if any) |
| --- | --- | --- |
| ASUSTeK | - Yes | Maximize the number of Serving Cell reported before reporting both TRP of each mTRP Serving Cell if TRP level truncation is supported (i.e. report one TRP of each Serving Cell first, report both TRPs if the UL grant can accommodate). |
| LGE | - Yes | We think it would be beneficial to maximize the number of available cell from Carrier Aggregation perspective. |
| Samsung | - Yes |  |
| OPPO |  | We agree that UE can report the beam failure information of one TRP if the UL grant is not able to accommodate the beam failure information of the second TRP when the beam failure is detected on both TRPs. And the beam failure information of TRPs is included in ascending order based on the ServCellIndex of the serving cell. |
| Fujitsu | - Yes |  |
| Intel | - Yes |  |
| Apple | - Yes |  |
| CATT | - Yes | It seems that this scheme aims to inform gNB as many number of failed cells as possible, which helps gNB to recover at least one TRP for a certain serving cell. |
| Xiaomi | - Yes |  |
| Qualcomm | - Yes |  |
| Sharp | - Yes |  |

#### Enhanced BFR MAC CE transmission in Msg3/MsgA of RA triggered for beam failure of both TRPs of SpCell

| Source | Proposal |
| --- | --- |
| Samsung [1] | Proposal: If RA procedure is initiated for beam failure recovery of both TRPs of SpCell, RAN2 to discuss and agree on one of the following - Option 1: - Depending on the size of Msg3/MsgA available after including C-RNTI MAC CE, include BFR MAC CE with beam failure recovery information of both TRPs; or include BFR MAC CE with beam failure recovery information of one TRP or include BFR MAC CE with beam failure recovery information of zero TRPs. - UE uses truncated format with one octet Ci bitmap, if truncated format with 4 octet Ci bitmap format cannot be included. - Option 2 - If RA procedure is initiated for beam failure recovery of both BFD-RS sets of SpCell, UE select preamble group B. |
| Fujitsu [3] | Proposal: RAN2 to discuss the following two options to fit MAC CE for BFR information and its MAC sub-header into 4 bytes: - Option 1: No L bits (at least one byte) in its MAC sub-header by using fixed size of the MAC CE for BFR information - Option 2: Including BFR information for one of TRPs in the MAC CE (and Proposal A is agreed) Proposal A: a single octet serving cell bitmap is used when beam failure is detected on both TRPs of an SpCell and the SpCell is to be indicated in a Truncated BFR MAC CE and the UL-SCH resources available for transmission cannot accommodate the Truncated enhancement BFR MAC CE with the 4 octets serving cell bitmap plus its subheader as a result of LCP. |
| ZTE [17] | Proposal : The following alternative can be taken into account when there is no enough bits for accommodating the enhanced (truncated) BFR MAC CE in which the beam information for both failed TRP of SpCell are present: Only (truncated) enhanced BFR MAC CE with two sets of one octet serving cell bitmap is generated for the RACH based BFR. Proposal: (truncated) Enhanced BFR MAC CE for indicating that both TRPs of SpCell are failed, it only can be sent via Msg.A/Msg.3. |
| Nokia [24] | Proposal: For Msg3/MsgA, at the minimum, the UE indicates the first octets of both bitmaps to convey the failure status of both BFD-RS sets of SpCell to NW. Proposal: With more bytes available in the Msg3/MsgA grant, the UE first includes beam failure information of BFD-RS sets of SpCell and after that the beam failure information about BFD-RS sets of SCells. |

Q7: Do you agree that for the RA procedure initiated for beam failure recovery of both TRPs of SpCell, depending on the size of Msg3/MsgA, UE includes beam failure recovery information of both TRPs or one TRP or zero TRPs of SpCell?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| ASUSTeK | - Yes |  |
| LGE | - No | If UL grant is not enough to accommodate the Enhanced BFR MAC CE, the UE may select preamble Group B. |
| Samsung | - Yes, but | We are also fine with suggestion from LGE |
| OPPO | - Yes |  |
| Fujitsu |  | We agree that the number of TRP(s) whose beam failure recovery information is included depends on the size of Msg3/MsgA. We just have some concerns on the benefit of zero TRP case. Maybe RAN2 can discuss this question after the progress made on the Enhanced BFR MAC CE design. |
| Intel | - See comment | We are ok to send one TRP beam failure recovery information. We wonder what the meaning of zero TRP is. Does it mean that BFR MAC CE is not transmitted? In this case, we are ok. [Samsung Reply]: Zero TRP means that BFR MAC CE is transmitted with only Ci/SP bitmaps(s). Beam failure recovery information octets (AC/Candidate beam ID) is not transmitted. So network can know which serving cell(s) TRP(s) are failed. Regarding LG’s proposal, we are not sure how much it is critical to support given that it requires change in preamble group selection. If we allow sending one TRP, it would be similar to the legacy operation. |
| Apple | - Yes | It’s our understanding that the “zero TRP” case means UE just indicates the Cell endures the failure case in the header but not include the detailed failed TRP info for this serving cell. |
| CATT | - Yes | How to generate the truncated Enhanced BFR MAC CE shall follow the principle agreed in 2.1.1.3. Since per TRP truncation is supported for the enhanced BFR MAC CE, it is reasonable to decide the BFR information depending on the size of Msg3/MsgA. For the case where the enhanced BFR MAC CE includes zero TRP BFR information for SpCell, it also follows legacy scheme for the BFR MAC CE, which is captured in R16 MAC spec as follows: NOTE: The number of the octets containing the AC field in the Truncated BFR MAC CE can be zero. |
| Xiaomi | - Yes |  |
| Qualcomm | - No | Unclear on the zero TRPs case. It may be related to MAC CE format design. OK for both TRPs or one TRP. |
| Sharp | - Yes | We agree limited information is included depending on the size of Msg3/MsgA. |

Q8: Do you agree that for the RA procedure initiated for beam failure recovery of both TRPs of SpCell, UE uses truncated format with one octet Ci bitmap, if truncated format with 4 octet Ci bitmap format cannot be included?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| ASUSTeK | - Yes |  |
| LGE | - No | If UL grant is not enough to accommodate the Enhanced BFR MAC CE, the UE may select preamble Group B. |
| Samsung | - Yes | Same as legacy principle |
| OPPO | - Yes |  |
| Fujitsu | - Yes |  |
| Apple | - Yes |  |
| CATT | - Yes | Following legacy. |
| Xiaomi | - Yes |  |
| Qualcomm | - Yes |  |
| Sharp | - Yes |  |

### RA Cancellation Aspects of Multi TRP BFR

| Source | Proposal |
| --- | --- |
| CATT [16] | Proposal: When the second TRP is detected to be failed during the RACH procedure used for requiring uplink resource to transmit the BFR MAC CE due to the first TRP failure, UE should stop the ongoing RACH and start a new RACH procedure for BFR of the SpCell. |
| Lenovo [6] | Scenario: - Beam failure of the first TRP is detected. - Then, the BFR is triggered and BFR MAC CE is transmitted for first TRP. - The beam failure of the second TRP is detected before the response for BFR MAC CE of the first failed TRP is received - RA is triggered for BFR of both TRPs of SpCell - While the RA procedure is ongoing, response to BFR MAC CE transmitted for first TRP is received. Response means PDCCH addressed to C-RNTI for new transmission for same HARQ process which is used to transmit the BFR MAC CE for first TRP - RA is stopped Proposal 1: If two TRPs are configured to the SpCell, RAN2 needs to discuss whether the UE can monitor the response during RA for BFR in the case that beam failure of both TRP are detected and the BFR MAC CE for the first TRP has been transmitted. Proposal 2: When UE can receive the response to BFR MAC CE during RA for BFR, UE can stop RA for BFR. |

Q9: Do you agree that when the second TRP is detected to be failed during the RACH procedure used for requiring uplink resource to transmit the BFR MAC CE due to the first TRP failure, UE should stop the ongoing RACH and start a new RACH procedure for BFR of the SpCell?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| ASUSTeK | - No | Whether to stop the current RACH and start a new RACH should be UE implementation. If the BFR MAC CE for the first TRP is transmitted in the first RACH, the UE can at least recover the first TRP when the first RACH is complete. |
| LGE | - No | In the current specification, whether to stop the ongoing RA procedure is UE implementation, and this principle should be kept. |
| Samsung | - No | Same view as LGE |
| OPPO |  | Prefer to left it to UE implementation, |
| Fujitsu | - No | It depends on the UE implementation. |
| Intel | - No |  |
| Apple | - No | It could be up to UE implementation. |
| CATT | - Yes | We understand that it is possible to leave this to UE implementation, but on the other hand, this is the case where both TRPs fail, and it is then very likely that the previous RACH procedure will not complete successfully. |
| Xiaomi | - No |  |
| Qualcomm | - No | Similar to the legacy, it can be up to UE implementation. |
| Sharp | - No | It can be left to UE implementation. |

Q10: Do you agree to stop the RA procedure in following scenario?

- Beam failure of the first TRP is detected. Then, the BFR is triggered and BFR MAC CE is transmitted for first TRP. The beam failure of the second TRP is detected before the response for BFR MAC CE of the first failed TRP is received, resulting in RA trigger for BFR of both TRPs of SpCell. While the RA procedure is ongoing, response to BFR MAC CE transmitted for first TRP is received. Note that response means PDCCH addressed to C-RNTI for new transmission for same HARQ process which was used to transmit the BFR MAC CE for first TRP

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| ASUSTeK | - No | [Rapp]: Please update your answer as per the revised question. Thanks rapporteur for the clarification and notification. Regarding the revised question, if the RACH is stopped, the UE may need to request other UL resources to transmit the beam failure information of the second TRP. If the beam failure information for both TRPs is already transmitted, then it might be faster to recover both TRPs via completion of the RACH. In addition, it may not be desirable to change the RACH stopping condition for a corner case. |
| LGE | - No | Once RA procedure for SpCell BFR due to both TRP failure is triggered, the UE stops the RA procedure when the ongoing RA procedure is successfully completed. If the ongoing RA procedure is stopped by one TRP recovery, we think it complicates the UE behaviour and RAN2 need further discussion. |
| Samsung | - No | Same view as LGE |
| OPPO | - No | We see no need to stop the ongoing RACH procedure as the BFR procedure for the second TRP is not completed. |
| Fujitsu | - No |  |
| Apple | - No |  |
| CATT | - No |  |
| Xiaomi | - No |  |
| Qualcomm | - No |  |
| Sharp | - No |  |

### Concurrent beam failure on both BFD-RS sets of SpCell

| Source | Proposal |
| --- | --- |
| Nokia [24] | For SpCell, it shall be noted that when the New BFR MAC CE is transmitted for one of the BFD-RS sets, there may be no candidate beam information indicated to the NW for that BFD-RS set in the MAC CE. In this case, if the Triggered BFRs are cancelled at the completion of BFR procedure upon NW response to the MAC CE (DCI with new UL grant), the UE has no beam to operate in case the second TRP fails at this point in time (or before the TCI states for the first TRP are reconfigured). The UE would transmit BFR SR in vain while it had no beam to receive an UL grant to indicate the New BFR MAC CE for the second TRP failure. In this case, the RA procedure shall be triggered instead since no valid beam for DL for neither of the TRPs exist. On the other hand, if the candidate beam information for the first BFD-RS set was indicated and NW responded with DCI, the failure of the second TRP after this point in time could be indicated with New BFR MAC CE given the new candidate beam indicated for the first failed TRP can be applied for DL reception. Hence, the RA procedure shall be triggered by the failure of the second BFD-RS set until a beam for the first BFD-RS set has been recovered (new TCI state activated) Proposal: For SpCell, the RA procedure for BFR shall be triggered by the failure of the second BFD-RS set until a beam for the first BFD-RS set has been recovered (new TCI state activated). [Rapporteur’s comments]: This was discussed in RAN2#116bise and RAN2 agreed that Beam failure is detected on both TRPs” means that BFR is triggered for a TRP of the serving cell while the BFR for another TRP of same serving cell is not successfully completed |

Q11: Do you agree that, for SpCell, the RA procedure for BFR shall be triggered by the failure of the second BFD-RS set until a beam for the first BFD-RS set has been recovered (new TCI state activated)?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| ASUSTeK | - No | BFR for the first BFD-RS set is cancelled when response of the BFR MAC CE is received. BFR-SR procedure will be triggered for second BFD-RS set if the second TRP failed at this point of time. |
| LGE | - No | Once RA procedure for SpCell BFR due to both TRP failure is triggered, the UE stops the RA procedure when the ongoing RA procedure is successfully completed. |
| Samsung | - No | Was discussed previously and not agreed |
| OPPO | - No |  |
| Fujitsu |  | We see two understandings on this question: 1) the RA procedure is triggered by the failure of the second BFD-RS set and is stopped when a beam for the first BFD-RS set has been recovered: for this understanding, our answer is No since the RA procedure will not be stopped. 2) the RA procedure is triggered if beam failure of the second BFD-RS set is detected and if a beam for the first BFD-RS set has not been recovered: for this understanding, our answer is Yes, but the impact to specifications need more clarifications. |
| Apple | - No | Prefer to keep the BFR recovery procedure same as legacy. |
| CATT | - No |  |
| Xiaomi | - No |  |
| Qualcomm | - No | In the last meeting, we have agreed the concept that BFR is triggered for a TRP while the BFR for another TRP is not successfully completed, it implies beam failure is detected on both TRPs. For SpCell, RACH for BFR will be triggered. |
| Sharp | - No |  |

### BFR MAC CE prioritization

| Source | Proposal |
| --- | --- |
| Samsung [1] | UE can be configured with multiple serving cells. Depending on the configuration, a serving cell (say serving cell Y) can be configured with M-TRP BFD/BFR (i.e. multiple BFD-RS sets are configured) while another serving cell (say serving cell X) can be configured with legacy BFD/BFR (i.e. multiple BFD-RS sets are not configured). If beam failure is detected for serving cell X, serving cell BFR is triggered and legacy BFR MAC CE needs to be transmitted for BFR of SCell X as per R16 procedure. If beam failure is detected for TRP(s) of serving cell Y, BFR for BFD-RS set of serving cell Y is triggered and enhanced BFR MAC CE needs to be transmitted for BFR of BFD-RS set of serving cell Y as per R17 procedure. Depending on the size of available UL grant, both the legacy BFR MAC CE and enhanced BFR MAC CE cannot be always accommodated in same MAC PDU. So some prioritization rule is needed. The issue is whether to prioritize (LCP) legacy BFR MAC CE or enhanced BFR MAC CE if the UL grant size is not enough. Proposal: If SP bit in legacy BFR MAC CE is set to 1, prioritize legacy BFR MAC CE over enhanced BFR MAC CE. If SP bit in enhanced BFR MAC CE is set to 1, prioritize enhanced BFR MAC CE over legacy BFR MAC CE. Proposal: If SP bit is not set to 1 in either MAC CEs, UE can prioritize legacy or enhanced BFR MAC CE which indicates more number of serving cells having beam failure. |

Q12: Do you agree that for the case both legacy BFR MAC CE (as per legacy procedure for SCell BFR) and enhanced BFR MAC CE are triggered (as per R17 procedure for MTRP BFR)

- If SP bit in legacy BFR MAC CE is set to 1, prioritize legacy BFR MAC CE over enhanced BFR MAC CE. If SP bit in enhanced BFR MAC CE is set to 1, prioritize enhanced BFR MAC CE over legacy BFR MAC CE.

- If SP bit is not set to 1 in either MAC CEs, UE can prioritize legacy or enhanced BFR MAC CE which indicates more number of serving cells having beam failure.

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| ASUSTeK | - No | A single enhanced BFR MAC CE should be enough to cover both cases. When beam failure is detected on at least one of the serving cell with multiple TRPs, enhanced BFR MAC CE is used. There will not be a case where both legacy BFR MAC CE and enhanced BFR MAC CE are used at the same time. |
| LGE | - No | We don’t see this optimization as necessary. |
| Samsung | - Yes | M-TRP BFR is configured on per serving cell basis. In case of CA, depending on the configuration, a serving cell (say serving cell Y) can be configured with M-TRP BFD/BFR (i.e. multiple BFD-RS sets are configured) while another serving cell (say serving cell X) can be configured with legacy BFD/BFR (i.e. multiple BFD-RS sets are not configured). If beam failure is detected for serving cell X, serving cell BFR is triggered and legacy BFR MAC CE needs to be transmitted for BFR of SCell X as per R16 procedure. If beam failure is detected for TRP(s) of serving cell Y, BFR for BFD-RS set of serving cell Y is triggered and enhanced BFR MAC CE needs to be transmitted for BFR of BFD-RS set of serving cell Y as per R17 procedure. So there are two options in our view: Option 1: New Enhanced MAC CE is also used for legacy BFR procedure if UE is configured with at least one serving cell with M-TRP BFR. This require some change(s) to legacy procedure. Option 2: Define prioritisation rule between legacy BFR MAC CE and enhanced BFR MAC CE. No change to legacy procedure. The proposal is based on option 2. BFR MAC CE which reports SpCell as failed is prioritised. |
| OPPO | - No | No need to introduce the optimization. |
| Fujitsu | - Partially yes | We agree to prioritize legacy BFR MAC CE over Enhanced BFR MAC CE if SP bit in legacy BFR MAC CE is set to 1. We don't see the need for other prioritizations. |
| Intel | - Comment | We wonder if we need to configure both legacy BFR MAC CE and enhanced BFR MAC CE because enhanced BFR MAC CE can also cover single TRP case. Would it be simpler to configure enhanced BFR MAC CE if there is any serving cell with two BFD-RS set? |
| Apple | - No | We have same understanding as LGE. If one serving cell is configured with two BFD-RS sets, only enhanced BFR MAC CE will be triggered. The UE will not trigger the legacy BFR MAC CE and enhanced BFR MAC CE at the same time. |
| CATT | - No | We do not see strong need, as this is an optimization. |
| Xiaomi | - No | It seems that the additional optimization is not essential. |
| Qualcomm | - No | The enhanced BFR MAC CE should cover all cases. As long as mTRP is configured in one serving cell in CA, the enhanced BFR MAC CE is triggered. |
| Sharp | - No | We don’t see the need of optimization. |

### PUCCH resource prioritization

SR for beam failure recovery of a BFD-RS set (as agreed in RAN2#116bise)

- When the MAC entity has pending SR for beam failure recovery of a BFD-RS set and the MAC entity has one or more PUCCH resources overlapping with PUCCH resource for beam failure recovery of that BFD-RS set for the SR transmission occasion, the MAC entity considers only the PUCCH resource for beam failure recovery of that BFD-RS set as valid

SR for SCell BFR (legacy procedure)

- When the MAC entity has pending SR for SCell beam failure recovery and the MAC entity has one or more PUCCH resources overlapping with PUCCH resource for SCell beam failure recovery for the SR transmission occasion, the MAC entity considers only the PUCCH resource for SCell beam failure recovery as valid.

| Source | Proposal |
| --- | --- |
| Samsung [1] | Proposal: if PUCCH resource for pending SR for SCell beam failure recovery overlaps with PUCCH resource for pending SR for beam failure recovery of BFD-RS set for the SR transmission occasion, discuss and agree on of the following options: Option 1: the MAC entity considers the PUCCH resource for BFR of BFD-RS set of serving cell as valid and the MAC entity considers the PUCCH resource for SCell beam failure recovery as invalid. Option 2: the MAC entity considers the PUCCH resource for BFR of BFD-RS set of serving cell as valid and considers the PUCCH resource for SCell beam failure recovery as invalid, if the pending SR for BFR of BFD-RS set of serving cell is for SpCell. Option 3: the MAC entity considers the PUCCH resource for SCell beam failure recovery as valid and the MAC entity considers the PUCCH resource for BFR of BFD-RS set of serving cell as invalid. |

Q13: Which of the following options do you agree for the case PUCCH resource for pending SR for SCell beam failure recovery overlaps with PUCCH resource for pending SR for beam failure recovery of BFD-RS set for the SR transmission occasion:

- Option 1: the MAC entity considers the PUCCH resource for BFR of BFD-RS set of serving cell as valid and the MAC entity considers the PUCCH resource for SCell beam failure recovery as invalid.

- Option 2: the MAC entity considers the PUCCH resource for BFR of BFD-RS set of serving cell as valid and considers the PUCCH resource for SCell beam failure recovery as invalid, if the pending SR for BFR of BFD-RS set of serving cell is for SpCell.

- Option 3: the MAC entity considers the PUCCH resource for SCell beam failure recovery as valid and the MAC entity considers the PUCCH resource for BFR of BFD-RS set of serving cell as invalid.

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| ASUSTeK |  | Can be up to UE implementation. |
| LGE | - Option 1 | If mTRP is configured, the PUCCH resource for TRP is prioritized over PUCCH resource for serving cell. |
| Samsung | - No strong view | Can follow majority views |
| OPPO |  | We can leave it to UE implementation. |
| Fujitsu |  | We see no need to specify this. If overlapped, it is up UE implementation to send SCell BFR SR or TRP BFR SR. |
| Apple |  | If the SR triggered by SCell BFR is overlapped with the SR triggered by SpCell TRP failure, the SpCell TRP failure case should be prioritized. Otherwise, it could be up to UE implementation. We are also fine to leave all cases up to UE implementation. |
| CATT | - no strong view |  |
| Xiaomi |  | We prefer to leave it to the UE implementation. |
| Qualcomm |  | We prefer to leave it to UE implementation. |
| Sharp |  | It can be up to UE implementation. |

### SR Aspects

| Source | Proposal |
| --- | --- |
| Qualcomm [12] | UE sends SR via one (the first) SR resource when beam failure is detected in one TRP of one SCell and it is possible that UE may detect the beam failure in another TRP of the same or another SCell before UE receives any granted uplink resource for mTRP BFR MAC CE reporting. Then, UE may trigger another SR from the second SR resource configuration. However, this second SR triggering may be redundant and unnecessary. Since the SR prohibit timer is configured within the SR resource configuration, the SR prohibit timer cannot control the other pending SR triggered from another SR configuration. Therefore, a prohibit timer is proposed being shared cross the two SR resource configurations for mTRP BFR. Proposal : A prohibit timer is introduced to avoid the excessive SR triggering in the case that UE may trigger the second SR due to the other BFR triggered in the second TRP of one SCell or SCell BFR while UE hasn’t received the uplink resource requested by the first triggered SR. |
| Nokia [24] | Proposal: For SpCell, only the SR configuration associated with the failed BFD-RS set is considered valid in the SR procedure. - [Rapporteur’s comments]: RAN1 made agreement that: “On the PUCCH-SR resource/SR configurations selection rule when SR is triggered and 2 PUCCH-SR resource/SR configurations are configured, the UE triggers the PUCCH-SR resource/SR configuration that is associated with failed BFD-RS set”. This seems sufficient. Proposal: As in legacy, for SR configuration associated with a BFD-RS set, NW can also associate logical channels or LBT failure recovery. Proposal: NW can associate both BFD-RS sets of the SpCell to the same SR configuration [Rapporteur’s comments]: These are up to network configuration. RRC does not specify any restriction. For example, schedulingRequestID-BFR-SCell Indicates the scheduling request configuration applicable for BFR on SCell, as specified in TS 38.321 [3]. schedulingRequestID-LBT-SCell Indicates the scheduling request configuration applicable for consistent uplink LBT recovery on SCell, as specified in TS 38.321 [3]. |

Q14: Do you agree that a prohibit timer is introduced to avoid the excessive SR triggering in the case that UE may trigger the second SR due to the other BFR triggered in the second TRP of one SCell or SCell BFR while UE hasn’t received the uplink resource requested by the first triggered SR?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| ASUSTeK | - No | Prohibit timer is configured per SR configuration and may not need to handle SR triggering between different SR configurations. |
| LGE | - No | If the two SR is controlled by one prohibit timer, there is no case that the UE transmits two SR using different SR resource respectively, and in this case, there is no reason to use two SR configuration. |
| Samsung | - No | Not essential |
| OPPO | - No | The legacy SR prohibit timer is enough. The SR triggered for the second TRP can implicit indicate NW the beam failure information, which helps for the UL grant allocation. And the pending SR will be cancelled if the BFR MAC CE is transmitted as we agreed in previous meeting. Thus, we see no need for the new prohibit timer. |
| Fujitsu | - No |  |
| Apple | - No |  |
| CATT | - No |  |
| Qualcomm | - Yes | The legacy SR prohibit timer can only control the SR transmission using the resource configured within one SR configuration. UE may already get the UL grant based on the first SR triggering, if there is no new prohibit timer, the redundant SR will be triggered. In addition, it is not necessary to use SR to implicit indicate the beam failure information. BFR MAC CE can provide the failed BFD-RS set ID information anyway. |
| Sharp | - No |  |

## MAC CE impacts and others

### PDCCH repetition impacts on DRX

RAN2#116bis-e Agreement

- If the PDCCH reception includes two PDCCH candidates from corresponding search space sets, start or restart drx-InactivityTimer for this DRX group in the first symbol after the end of the PDCCH candidate that ends later in time. FFS how to capture this agreement in the TS 38.321 whether adding it as a NOTE or adding it in the normative text

- FFS whether to clarify the Active Time when the PDCCH repletion is configured

| Source | Proposal |
| --- | --- |
| vivo [2] | Proposal: A note should be introduced to clarify the reference point of starting a timer when PDCCH repetition if configured. Proposal: There is no need to clarify the Active Time when the PDCCH repletion is configured. |
| Qualcomm [13] | Proposal: The text ‘If the PDCCH reception includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1 in 38.213, start or restart drx-InactivityTimer for this DRX group in the first symbol after the end of the PDCCH candidate that ends later in time’ can be captured as either a NOTE or a normative text. Proposal: If PDCCH is configured with repetition, both PDCCH candidates should be included in the Active Time. Proposal: A Note is introduced to clarify the Active Time for the PDCCH repetition case. Note: If the Active Time for Serving Cells in a DRX group includes a first PDCCH candidate that is linked to a second PDCCH candidate from two corresponding search space sets, as described in clause 10.1 in 38.213, the Active Time for Serving Cells in a DRX group also includes the second PDCCH candidate. |
| LG [15] | Proposal: The reference point of starting drx-InactivityTimer is captured as a NOTE. Proposal: The definition of Active Time is not impacted even if the second PDCCH candidate is outside the monitor window. |

For the issue (FFS point) whether to clarify the Active Time when the PDCCH repletion is configured, proponent company (1/3) explained that the UE behaviour should be clarified because the network may configure the second PDCCH candidate is outside of the DRX monitor window. Meanwhile other companies (2/3) provided the view that this issue can be handled by gNB implementation and it is just an error case, i.e. value of DRX related timers should be set properly by gNB to include the second PDCCH candidate in the active time. In addition, MAC specification has some text to handle similar case as below:

The MAC entity needs not to monitor the PDCCH if it is not a complete PDCCH occasion (e.g. the Active Time starts or ends in the middle of a PDCCH occasion).

Q15: Do you agree adding a Note to clarify the Active Time for the PDCCH repetition case?

Note: If the Active Time for Serving Cells in a DRX group includes a first PDCCH candidate that is linked to a second PDCCH candidate from two corresponding search space sets, as described in clause 10.1 in 38.213, the Active Time for Serving Cells in a DRX group also includes the second PDCCH candidate.

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| ASUSTeK | - No | No need to add a Note as gNB implementation could avoid this issue. |
| LGE | - Yes | It is sufficient to be captured as NOTE. |
| Samsung | - Yes | It is fine to add a NOTE in order for NW to implement correctly. |
| OPPO | - No |  |
| Fujitsu | - No |  |
| Apple | - See comments | It’s necessary to clarify to start or restart drx-InactivityTimer in the first symbol after the end of the PDCCH candidate that ends later (proposal 1 from vivo and from QC). With the clarification on the reference point to start the DRX inactivity timer, it’s no need to clarify the DRX active time, since the two linked PDCCH search space candidate are just within the same slot, and the slot level DRX timer configuration can garantuee both PDCCH candidates are in the active time. |
| CATT | - No |  |
| Xiaomi | - No |  |
| Qualcomm | - Yes | Proponent. It is necessary to define clear UE heavier on how to handle the second PDCCH candidate if it is outside of the DRX monitoring window in the PDCCH repetition case. |
| Sharp | - No |  |

### Unified TCI state activation/deactivation MAC CE

RAN2#116bis-e Agreement

- RAN2 agrees on Separate TCI state lists for joint/DL and UL in PDSCHConfig and UL BWP, respectively, and separate Id pools.

- RAN2 continues discussing MAC CE design for joint and separate TCI state operation as well as the UL/DL BWP association

| Source | Proposal |
| --- | --- |
| OPPO [4] | Proposal: UL BWP ID should be included within TCI state update MAC CE. Proposal: Joint and separate TCI state update should be covered by same MAC CE design. J bit: whether this MAC CE is for joint TCI states or separate TCI states. J=1 for joint TCI states, in this case 2nd OCTED is omitted. J=0 for separate TCI states, in this case UL BWP ID is in 2nd OCTED. D/U bit: indicate whether the TCI state ID in the same OCTET is for downlink or uplink TCI state. It is valid only when J=0, otherwise it is a reserved bit. D/U bit=1 for downlink and 0 for uplink. P bit: when J=1 or J=0 but D/U bit is 1, it is the most significant bit of TCI state ID in the same OCTET. When J=0 and D/U bit indicates a uplink TCI state in the same OCTET , then it indicates whether an immediate preceding downlink TCI state, if any, belongs to the same DCI code point together with the uplink TCI state in this OCTET. Otherwise this bit is reserved bit. |
| Intel [7] | Proposal: UL BWP ID is included in unified TCI state activation/deactivation MAC CE. |
| MediaTek [8] | Proposal: Adopt the MAC CE in Figure 2 for activation/deactivation of joint DL/UL TCI states. Proposal: Adopt the MAC CE in Figure 3 for activation/deactivation of separate DL/UL TCI states. - D/U: This bit indicating whether the TCI state ID in the same octet is for DL (0) or UL (1) - SC (Same Codepoint): This bit exists only in octets containing UL TCI state IDs (D/U=1). If the bit is set (SC=1), the UL TCI state ID belongs to the same codepoint as the (DL) TCI state ID in the previous octet (that is, the codepoint represents a pair of DL and UL TCI states); otherwise, the codepoint represents only a UL TCI state. Proposal: For joint DL/UL TCI, introduce a link between DL and UL BWPs with the same index, and the BWP ID in TCI state activation/deactivation MAC CE applies to both DL and UL. Proposal : For separate DL/UL TCI, choose between (1) Add a field for UL BWP ID, and (2) Introduce link between DL and UL BWP IDs. |
| ASUSTeK [10] | Proposal: Design a new TCI state MAC CE format for UE operating in joint/DL TCI state and another new TCI state MAC CE format for UE operating in separate TCI state. Proposal: RAN2 decides whether to use same or different LCIDs for handling joint and separate TCI state MAC CE formats. Proposal: For a Serving Cell with unpaired spectrum, TCI states indicated by joint/separate TCI state MAC CE are activated for both DL BWP and UL BWP of the same BWP id indicated by the MAC CE. Proposal: For a Serving Cell with paired spectrum, RAN2 selects from one of the options handling UL/DL BWP association for unified TCI state MAC CE - Option 1: unified TCI state MAC CE includes one BWP id. - Option 1a: the UE applies the TCI state activation to DL BWP of the BWP id and all UL BWPs of the UE in the Serving Cell. - Option 1b: the UE applies the TCI state activation to the DL BWP of the BWP id and the active UL BWP. - Option 2: unified TCI state MAC CE includes two BWP ids indicating a DL BWP and a UL BWP. - The UE applies the TCI state activation/deactivation to the indicated DL BWP and indicated UL BWP. - Option 3: unified TCI state MAC CE does not include BWP id. - The UE applies the TCI state activation for all DL and UL BWPs of the UE. Proposal: In joint TCI state MAC CE, each Joint TCI state ID maps to one code point based on its position among all the Joint TCI state IDs in the MAC CE. Proposal: Separate TCI state MAC CE contains an octet indicating each code point has one or two TCI state ids. Each octet contains a bit indicating the corresponding TCI state ID is a DL or UL TCI state. Proposal 7: Adopt Figure 1 and Figure 2 as baseline for designing new unified TCI state MAC CE format for joint and separate TCI state |
| Ericsson [14] | Proposal: RAN2 to agree on the above MAC CE design for unified TCI state update. BWP id UL (1st bit); Serving cell ID; BWP id DL (2 bits). C; Joint/DL TCI state ID (7 bits 128 states). F; BWP id UL (2nd bit); UL TCI state ID (6 bits 64 states). C; Joint/DL TCI state ID (7 bits 128 states). F; BWP id UL (2nd bit); UL TCI state ID (6 bits 64 states). C; Joint/DL TCI state ID (7 bits 128 states). F; BWP id UL (2nd bit); UL TCI state ID (6 bits 64 states). C; Joint/DL TCI state ID (7 bits 128 states). F; BWP id UL (2nd bit); UL TCI state ID (6 bits 64 states). C; Joint/DL TCI state ID (7 bits 128 states). F; BWP id UL (2nd bit); UL TCI state ID (6 bits 64 states). C; DL TCI state ID (7 bits 128 states). F; BWP id UL (2nd bit); UL TCI state ID (6 bits 64 states). C; Joint/DL TCI state ID (7 bits 128 states). F; BWP id UL (2nd bit); UL TCI state ID (6 bits 64 states). C; Joint/DL TCI state ID (7 bits 128 states). F; BWP id UL (2nd bit); UL TCI state ID (6 bits 64 states). C field describes whether octet with UL TCI state ID is present F field describes whether UE should consider the preceding octet as padding or as DL TCI state (only needed for “separate beam indication”). BWP id UL points to the BWP where UL TCI state list is configured |
| ZTE [19] | Proposal: One set of bitmap with the length of one octet is introduced to indicate whether there are two TCI states are present for one codepoint in DCI Proposal: Regarding the octet containing the TCI states identifier, DL TCI state ID is present next to the UL/DL flag if the UL/DL flag indicating DL. While UL BWP ID is present next to the UL/DL flag if the UL/DL flag indicating UL, and then the UL TCI state ID is present next to the UL BWP ID. Proposal: For the joint TCI states activation MAC CE, one octet containing serving cell ID and DL BWP ID followed by at most 8 octets containing TCI state identifier are included. Proposal: Two eLCIDs are needed for unified joint TCI states activation MAC CE and unified joint TCI states activation MAC CE. |
| Huawei [21] | Proposal: Design a general format for TCI state activation MAC CE that can work on both joint TCI mode and separate TCI mode. Proposal: Agree the above TCI state activation MAC CE format for Rel-17 unified TCI framework. (1) For Category A: one UL-only TCI + one DL-only TCI. X = 0 means this octet indicates a UL only TCI state, Y =1 means there is a DL only TCI state which maps to the same TCI codepoint in DCI. (2) For Category B: one UL-only TCI X = 0 means this octet indicates a UL only TCI state, Y =0 means there is not a DL only TCI state which maps to the same TCI codepoint in DCI. (3) For Category C: one DL-only TCI or one joint TCI If the separate TCI mode is used, then X =1 means this octet indicates a DL-only TCI state. If the joint TCI mode is used, then X field can be a reserved bit. |

For detail design for unified TCI state activation/deactivation MAC CE, following issue is remaing to discuss.

- Issue: Whether joint and separate TCI state update are covered by same MAC CE or separate MAC CEs are introduced.

- Option 1: Introduce one MAC CEs covering both Joint TCI state and separate TCI state modes

- Option 2: Introduce two MAC CEs for Joint TCI state and separate TCI state modes, respectively.

For the first issue, companies’ proposals are split with two positions i.e. option 1 (3/6) and option 2 (3/6), so it is difficult to select option without further comments from other companies.

Q16: Which of the following options do you agree for unified TCI state activation/deactivation MAC CE format?

- Option 1: Introduce one MAC CEs covering both Joint TCI state and separate TCI state modes

- Option 2: Introduce two MAC CEs for Joint TCI state and separate TCI state modes, respectively.

| Company | Option 1/Option 2 | Comments (if any) |
| --- | --- | --- |
| ASUSTeK | - Option 2 |  |
| LGE | - Option 1 | We don’t prefer that one MAC CE is interpreted differently depending on the included contents. |
| Samsung | - Option 1 | If one MAC CE design has no critical problem it should be achieved. |
| OPPO | - Option 1 | As agreed during online discussion that UE can be configured with either joint TCI state or separate TCI state, so single MAC CE with single LCID is sufficient since there is no ambiguity. As for the format as we indicate in our paper single format is also feasible. |
| TCL | - Option 2 | We think the separate design can avoid many indication bits and is more compatible. |
| Fujitsu | - Option 2 |  |
| Intel | - Option 1 | There seems no issue to have one MAC CE for both joint and separate TCI state. |
| Apple | - Option 1 | It’s no problem to support both joint and separate TCI state in one MAC CE. |
| CATT | - Option 1 |  |
| Xiaomi | - Option 1 |  |
| Qualcomm | - Option 1 |  |
| Sharp | - Option 1 |  |

### Enhanced PHR for mTRP PUSCH repetition and MPE for Rel-17

RAN2#116-e Agreement

- RAN2 to discuss how to support PHR reporting for mTRP PUSCH repetition, and may address e.g.:

  - New MAC CE design including the function which TRP is applied for PHR reporting

  - How to incorporate the additional MPE information coming in Rel-17 to the new PHR format

  - Whether use legacy parameters (timer, threshold, etc.) or adding TRP specific parameters

  - PHR triggering conditions

RAN2#116bis-e Agreement

- Request the following further information from RAN1: A) How many resources (i.e. SSBRI/CRI ) can be configured in mpe-ResourcePool, and whether the resources are per BWP? B) For mTRP, does UE indicate CORESET pool ID, SRS resource set ID or something else in the mTRP PHR? C) Is the PCMax,f,c needed, and if yes is it included per indicated SSBRI/CRI value, or is it cell-specific?

Please note that all fugures for the suggested MAC CE formats are skipped here, please see the figures in the company contributions.

| Source | Proposal |
| --- | --- |
| Apple [5] | Proposal: The Enhanced PHR MAC CE with two PHs of the same serving cell is introduced for both the single entry format and multiple entry format. Proposal: Both single octet bitmap (7 Ci bits and 1 R bit) and 4 octet bitmap (31 Ci bits and 1 R bit) formats are supported for the Enhanced PHR MAC CE. Proposal: The two PHs together with two Pcmax.f.c are included the Enhanced PHR MAC CE for the serving cell configured with the mTRP PUSCH repetition feature. Proposal: Agree the single entry Enhanced PHR MAC CE as indicated in Figure-2. Proposal: For the multiple entry Enhanced PHR MAC CE, for the serving cell with mTRP repetition configuration, the corresponding entry will follow the single entry enhanced PHR MAC CE format. Proposal: The Enhanced PHR MAC CE (including two PHs) is explicitly configured via RRC configuration. Proposal: The PHR trigger condition of the pathloss change is restricted in the same TRP. Proposal: Postpone the RAN2 discussion on the MPE enhancement feature until more information is provided by RAN1 and RAN4. |
| Intel [7] | Proposal: include N P-MPR values paired with 1 SSBRI/CRI resource ID, where N is configured by RRC signaling (numberofN). Proposal: up to 4 P-MPR value reporting is included for serving cell(s) enabled for P-MPR reporting. Proposal: RAN2 wait for RAN1 response to decide whether the single MAC CE format can be introduced to support both P-MPR reporting and two PHR reporting. |
| ASUSTeK [9] | Proposal: Create new MAC CE for mTRP PHR reporting both TRPs of Cells with mTRP PUSCH repetition in one MAC CE. Proposal: When the UE is configured with twoPHRMode for a CG, the UE decides whether to report one or two PHs for a serving cell of the CG based on if mTRP PUSCH repetition is enabled for the serving cell. Proposal: Order of two PHs for a serving cell is based on the “TRP identity”. (Wait for RAN1 response on the definition of “TRP identity”.) Proposal: PHR MAC CEs for mTRP PUSCH repetition do not explicitly include “TRP identity”. Proposal: MPE could be reported for each TRP in PHR MAC CE for mTRP PUSCH repetition. Proposal: PCMAX is reported for each TRP in PHR MAC CE for mTRP PUSCH repetition. Proposal: A bitmap is included in Multiple Entry PHR MAC CE for mTRP PUSCH repetition, and each bit indicates whether one or two PHs is reported for the corresponding serving cell. Proposal: Adopt Figure 1 and Figure 2 as baseline for designing new PHR MAC CEs for mTRP PUSCH repetition. |
| Samsung [11] | Proposal: RAN2 to agree on separate parameter configuration for per TRP PHR reporting e.g. phr-Tx-PowerFactorChange2, phr-ProhibitTimer2, phr-PeriodicTimer2, phr-Tx-PowerFactorChange2. Proposal: RAN2 to consider per TRP PHR triggering when mTRP PUSCH repetition is configured. Proposal: Both single entry PHR and multiple entry PHR are used for PHR for mTRP PUSCH repetition. Proposal: Introduce a new MAC CE for multi-TRP PHR where one PHR per TRP are reported in a single multi-TRP MAC-CE instance. Proposal: If both MAC CEs for TRP 1 and TRP 2 are pending and UL grant is not large enough to accommodate both the MAC CEs, UE select the one triggered earlier than the other. |
| Qualcomm [13] | Proposal: Two PH value fields and the associated V fields for one serving cell are introduced in the enhanced PHR MAC CE if the serving cell is configured with mTRP PUSCH repetition and twoPHRMode is configured. Proposal: No new TRP specific PHR related parameters are introduced. The legacy PHR related timers and threshold parameters are reused for the enhanced PHR reporting for the mTRP PUSCH repetition case. Proposal: The legacy PHR triggering conditions are reused for supporting enhanced PHR reporting in the mTRP PUSCH repetition case. |
| Ericsson [14] | Proposal: RAN2 to conclude to report PHR values of the TRPs in one MAC CE. Proposal: RAN2 to assume the legacy threshold for MPE reporting also for Rel-17. |
| LG [15] | Proposal: MPE information reporting related issues would be discussed after receiving reply LS from RAN1. Propose: The legacy parameters in PHR-Config is used for mTRP PHR reporting, and the new parameter is introduced which indicates whether mTRP PHR reporting is enabled or not. Propose: Introduce a single new PHR MAC CE for both TRPs, i.e., MAC CE containing PHR for both TRPs. Proposal: The single new PHR MAC CE for both TRPs does not include TRP identifier, i.e., the PH information for a TRP is included in ascending order of TRP ID for the associated serving cell. |
| ZTE [18] | Proposal: For PHR related to mTRP PUSCH repetition, a unified PHR parameter set in the CG can be reused. Proposal: All triggering conditions for triggering legacy PHR can be reused for new PHR with mTRP. Proposal: If a PHR is triggered for one MAC entity, which type of PHR MAC CE (i.e legacy or PHR MAC CE with mTRP) shall be used upon the presence of twoPHRMode. Proposal: For the new RRC parameter twoPHRMode , RAN2 is kindly asked which one of the following interpretation is correct: - If the twoPHRMode is configured for one MAC entity, UE shall calculate two PHRs for all activated serving cells belonging to this MAC entity, otherwise, UE should calculate one PHR for all activated serving cells belonging to this MAC entity. - If the twoPHRMode is configured for one MAC entity, UE shall calculate two PHRs for all activated serving cells for both MAC entities in DC Mode if the UL grant used for sending the PHR MAC CE is received for this MAC entity. Proposal 5: Send an LS to RAN1 to ask for the understanding of the twoPHRMode in proposal 4 which is correct. |
| Nokia [20] | Proposal 2: Use bitmap to indicate how many beams are present for each serving cell. Proposal 3a: If P_CMax,f,c is not included per beam (per serving cell) in the PHR with MPE, define the Rel-17 single-entry PHR with MPE to contain: - Per cell, one octet for beam presence (4 bits) and P-bits (4 bits) - Per cell, one octet for MPE information (for all beams, with 2 bits / beam) - Per beam (if present), one octet per SSBRI/CRI of the beam Proposal 3b: If P_CMax,f,c is not included per beam (per serving cell) in the PHR with MPE, define the Rel-17 multi-entry PHR with MPE to contain: - Per PHR, one octet for bitmap indicating which serving cells have beam information present (8 bits) - Per cell, one octet for beam presence (4 bits) and P-bits (4 bits) - Per cell, one octet for MPE information (for all beams, with 2 bits / beam) - Per beam (if present), one octet per SSBRI/CRI of the beam Proposal 4a: If P_CMax,f,c is included per beam (per serving cell) in the PHR with MPE, define the Rel-17 single-entry PHR with MPE to contain: - Per cell, one octet for beam presence (4 bits, with 4 bits left as R-bits) - Per beam (if present), one octet per cell indicating P-bit, MPE information and P_CMax,f,c - Per beam (if present), one octet per SSBRI/CRI for each beam that is present Proposal 4b: If P_CMax,f,c is included per beam (per serving cell) in the PHR with MPE, define the Rel-17 multi-entry PHR with MPE to contain: - Per PHR, one octet for bitmap indicating which serving cells have beam information present (8 bits) - Per cell, one octet for beam presence (4 bits, with 4 bits left as R-bits) - Per beam (if present), one octet per cell indicating P-bit, MPE information and P_CMax,f,c - Per beam (if present), one octet per SSBRI/CRI for each beam that is present |
| Huawei [22] | Proposal: Before discussing the detailed PHR MAC CE design, RAN2 needs to determine whether Rel-17 MPE changes are applicable to mTRP framework. Proposal: The power headroom information of two TRPs of serving cells can be included in one single Rel-17 PHR MAC CE. Proposal: There is no need to introduce TRP-specific parameters for PHR triggering, i.e., phr-ProhibitTimer, phr-PeriodicTimer are maintained per MAC entity, and phr-Tx-PowerFactorChange applies to each TRP of each serving cell of the MAC entity. |
| InterDigital [23] | Proposal: Modify the triggering procedure to measure pathloss/power factor changes per PL-RS group. Proposal: Introduce a new MAC-CE for multi-TRP PHR where both PHRs are reported in a single multi-TRP MAC-CE instance. Proposal: If PUSCH resource allocation associated to the uplink grant does not have sufficient resources to carry both PHR values, UE reports only one of the computed PHR values. |

In [9], [13] and [18] proposed to clarify when the new PHR MAC CE with mTRP is used. In the current RRC running CR, twoPHRMode is configured in PHR-Config (CellGroup level) and PUSCH repetition for mTRP can be configured by a Serving cell level.

- Proposal in [9]: When the UE is configured with twoPHRMode for a CG, the UE decides whether to report one or two PHs for a serving cell of the CG based on if mTRP PUSCH repetition is enabled for the serving cell.

- Proposal in [13]: Two PH value fields and the associated V fields for one serving cell are introduced in the enhanced PHR MAC CE if the serving cell is configured with mTRP PUSCH repetition and twoPHRMode is configured.

- Proposal in [18]: If a PHR is triggered for one MAC entity, which type of PHR MAC CE (i.e legacy or PHR MAC CE with mTRP) shall be used upon the presence of twoPHRMode.

For more detail, which PHR MAC CE is used should be clarified in case that twoPHRMode is configured in PHR-Config (CellGroup level) but mTRP PUSCH repetition is not activated for some of activated serving cells.

Q17: Do you agree below high-level proposal when the new enhanced PHR MAC CE is used? Or please enhance the below text using better wording if you have suggestion.

Proposal: When the UE is configured with twoPHRMode for a CG, the UE decides whether to report one or two PHs (by the legacy PHR MAC CE or PHR MAC CE with mTRP) for a serving cell of the CG based on if mTRP PUSCH repetition is enabled for the serving cell.

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| ASUSTeK | - See comment | If twoPHRMode is configured for a MAC entity (CG), PHR MAC CE with mTRP is used. In the MAC entity (CG), each serving cell can be operating with either single TRP or mTRP PUSCH. The UE can report one or more PHs for a serving cell in the MAC CE based on if the serving cell is enabled with mTRP PUSCH repetition. We suggest to modify the proposal as follows: Proposal: When the UE is configured with twoPHRMode for a CG, PHR MAC CE with mTRP is used. The UE decides whether to report one or two PHs for a serving cell of the CG based on if mTRP PUSCH repetition is enabled for the serving cell |
| LGE | - Partially Yes | We think “mTRP PUSCH repetition is configured” is desirable than “mTRP PUSCH repetition is enabled” |
| Samsung | - Yes | We are fine with the way of this proposal and variant from other companies above. |
| OPPO | - See comment | It is clear that PHR MAC CE with mTRP is used when twoPHRMode is configured since at least one serving cell will be configured with PUSCH repetition otherwise such configuration doesn’t make sense. But if at least one serving cell in the cell group is not configured with PUSCH repetition or in another NR MAC entity or another MAC entity is LTE, there are two alternatives to report PH: Option1: report both legacy PHR MAC CE and PHR MAC CE with mTRP Option2: report single PHR MAC CE with both per cell PH value and per TRP value From signalling point of view option2 is better. But it means the reported PHR MAC CE contain both legacy part and PH per TRP. Then the question is whether we really need to assign a new LCID for this MAC CE. Our view is that legacy PH can be extended to add extra PH value corresponding to 2nd SRS resource set i.e. PHR MAC CE should be extended. There is no backward compatibility issue the change of the MAC CE format is controlled by RRC configuration in advance. |
| Fujitsu |  | ASUSTeK’s version is fine to us. |
| Intel | - Comment | We tend to agree with Oppo that we should try to design new PHR MAC CE to include both per cell PH value and per TRP value. |
| Apple |  | We are fine with the revised version form ASUSTeK. If twoPHRMode for a CG is configured, UE only reports the Enhanced PHR MAC CE, and 1 PH is included for the cell which is not configured with the 2nd SRS resource set. |
| Xiaomi |  | We are fine with ASUSTek’s revision. |
| Qualcomm | - Yes | LGE’s suggestion is good for us. |
| Sharp |  | We are fine with ASUSTeK’s modification. |

In addition, one company [18] ask for the DC case how UE calculates the PHRs for each MAC entities if the twoPHRMode is configured for one MAC entity. It is required to determine when the legacy PHR MAC CE or the new PHR MAC CE with mTRP is used. For the new RRC parameter twoPHRMode, RAN2 is kindly asked which one of the following interpretation is correct:

- Interpretation 1: If the twoPHRMode is configured for one MAC entity, UE shall calculate two PHRs for all activated serving cells belonging to this MAC entity, otherwise, UE should calculate one PHR for all activated serving cells belonging to this MAC entity.

- Interpretation 2: If the twoPHRMode is configured for one MAC entity, UE shall calculate two PHRs for all activated serving cells for both MAC entities in DC Mode if the UL grant used for sending the PHR MAC CE is received for this MAC entity.

Q18: Which interpretation is correct for the new RRC parameter twoPHRMode for DC case?

| Company | Interpretation 1/ Interpretation 2 | Comments (if any) |
| --- | --- | --- |
| ASUSTeK | - Interpretation 3 | If the twoPHRMode is configured for one MAC entity, UE shall calculate two PHRs for all activated serving cells enabled with mTRP PUSCH repetition belonging to this MAC entity and one PHR for activated serving cells with sTRP PUSCH belonging to this MAC entity. |
| LGE | - Interpretation 1 | "twoPHRMode" is the configuration per Cell Group. If the network want two PHRs from both MAC, "twoPHRMode" is configured for each Cell Group. |
| Samsung | - Interpretation 2 | From the UE perspective, PH values for the other CG can be affected. From our understanding, legacy PHR triggering condition consider “any MAC entity”. |
| OPPO | - none | What if another MAC entity is LTE? And please refer to answer to Q17 |
| Fujitsu | - Interpretation 1 |  |
| Apple | - Interpretation 1 | The configuration is per CG. |
| Xiaomi | - Interpretation 1 |  |
| Qualcomm | - Interpretation 1 |  |
| Sharp | - Interpretation 1 |  |

### SP-SRS resource set activation by MAC CE

RAN1 #106-e meeting Agreement

For antenna switching SRS, support maximum one SRS resource set for periodic SRS and maximum 2 SRS resource sets for semi-persistent SRS.

- Note: the two SP-SRS resource sets are not activated at the same time

- For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS

- Applies for all supported xTyR where y<=8

- For each xTyR antenna switching (except for 4T6R if supported), each periodic or semi-persistent resource set contains y/x resources.

- This feature is UE optional: For UEs that do not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS

| Source | Proposal |
| --- | --- |
| Huawei [22] | Proposal: Upon reception of a MAC CE to activate an SP SRS resource set for antenna switching, the UE considers any previously activated SP SRS resource set for antenna switching as deactivated. |

According to the contribution in [22], RAN1 made agreements that two SP SRS resource sets for antenna switching cannot be activated at the same time. Regarding this agreement, it may useful to simultaneously: deactivate one SP SRS resource set for antenna switching and activate the other one. From rapporteur perspective, this proposal is optimizing the signalling load of this MAC CE to remove the redundant deactivation MAC CE. Without any change of TS 38.321, for antenna switching, the network would need to send one MAC CE, wait until it is received by the UE, and then send another MAC CE.

Q19: Do you agree below proposal?

Proposal: Upon reception of a MAC CE to activate an SP SRS resource set for antenna switching, the UE considers any previously activated SP SRS resource set for antenna switching as deactivated.

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| LGE | - - | No strong view, but needs to check whether there is impact on the physical layer behaviour or not. |
| Samsung | - No strong view | We agree that this proposal help to reduce the redundant MAC CE signalling but it is not essential. |
| OPPO | - No | We don’t think such optimization is needed |
| Fujitsu | - Yes |  |
| Apple | - Yes | Since there is only one active SP-SRS resource at a time, it’s straightforward that the activation of the new SP-SRS resource means the implicit deactivation of the previous resource. |
| Xiaomi | - Yes |  |
| Qualcomm | - No | No need such optimization. |

# Phase II Discussion

Further issues will be triggered after receiving RAN1 LS replies regarding the below RAN2 LSes:

- LS on MPE information signalling (R2-2111600)

- LS on feMIMO RRC parameters (R2-2202002)

- LS on Enhanced TCI state indication for UE-specific PDCCH MAC CE (R2-2201950)

# Conclusion

TBD

# References

- R2-2202288	Multi TRP Beam Failure Detection and Recovery	Samsung Electronics Co., Ltd

- R2-2202320	Discussion on remaining issues on MAC aspects for feMIMO	vivo

- R2-2202349	Remaining issues on beam failure with mTRP	Fujitsu

- R2-2202448	MAC CE design for FeMIMO	OPPO

- R2-2202557	MAC impacts of FeMIMO	Apple

- R2-2202572	BFR for both SpCell and SCell in mTRP	Lenovo, Motorola Mobility

- R2-2202670	Remaining issues on MAC CEs	Intel Corporation

- R2-2202772	MAC CE Design for Unified TCI States Activation Deactivation	MediaTek Inc.

- R2-2202851	Discussion on Power Headroom Reporting for mTRP PUSCH repetition	ASUSTeK

- R2-2202852	Discussion on MAC CE design regarding separate and joint TCI state	ASUSTeK

- R2-2202928	Discussions on PHR enhancements for mTRP PUSCH repetition	Samsung

- R2-2202957	Remaining issues on multi-TRP BFR	Qualcomm Incorporated

- R2-2202958	Remaining issues on MAC and MIMO MAC CEs	Qualcomm Incorporated

- R2-2203044	MAC CE impacts 	Ericsson

- R2-2203093	Remaining issues on MAC	LG Electronics Inc.

- R2-2203104	Discussions on the remaining open Issues of 38.321 Running CR	CATT

- R2-2203246	Consideration on Implementation of BFR for mTRP	ZTE Corporation, Sanechips

- R2-2203247	Further Considerations On New PHR and PHR MAC CE	ZTE Corporation, Sanechips

- R2-2203248	Consideration on the unified TCI State MAC CE for ICBM	ZTE Corporation, Sanechips

- R2-2203269	PHR reporting for FeMIMO	Nokia, Nokia Shanghai Bell

- R2-2203382	FeMIMO MAC open issues	Huawei, HiSilicon

- R2-2203383	SP-SRS resource set activation by MAC CE	Huawei, HiSilicon

- R2-2203426	Discussion on Multi-TRP PHR enhancements	InterDigital

- R2-2203282	Beam failure with mTRP 	Nokia, Nokia Shanghai Bell

- R2-2202927	PUCCH power control for mTRP FR1	Samsung