3GPP TSG-RAN WG2 Meeting #117 electronic	R2-220xxxx

Electronic meeting, Feb 21 – Mar 03, 2022

Agenda item:	8.23.2

Source:	CATT

Title:            Report of [AT117-e][051][UDC] Open Issues and CRs (CATT)

WID/SID:	NR_UDC-Core

Document for:	Discussion and Agreement

- 1	Introduction

This document is for summary of the following discussions:

- [AT117-e][051][UDC] Open Issues and CRs (CATT)

Scope: Ph1 Address the UDC Open Issues aiming to close all, Collect comments on major issues and/or blocking points in the provided CRs if any. Ph2 The discussion will continue W2 aiming for CR agreement (focusing on smaller things).

Intended outcome: Report

Deadline: Ph1 W1 Friday (for On-line CB Monday W2).

The participants are invited to leave their contact information in the following table.

| Company | Contact: Name (E-mail) |
| --- | --- |
| LG | Geumsan Jo (Geumsan.jo@lge.com) |
| Samsung | Donggun Kim (s_dg.kim@samsung.com) Sangkyu Baek (sangkyu.baek@samsung.com) |
| Apple | Ralf Rossbach (rrossbach@apple.com) |
| OPPO | Zhe Fu(fuzhe@OPPO.com) |

- 2	Ph1 – on the open issues

Based on the previous discussions (see Appendix) and company contributions [7]-[16], the following open issues need to be discussed, i.e.,

- Issue 1: FFS on other inter-node coordination, see section 2.1

- Issue2: An Editor note in PDCP CR that is related to drb-ContinueUDC, see section 2.2

- Issue 3: An FFS on UE capability, see section 2.3

- Issue 4: on PDCP PDUs to be discarded in the network side, see section 2.4

## 2.1 FFS on inter-node coordination

The key point to discuss is whether additional inter-node coordination needs to be introduced. In the current 38.331 CR [2], the previous agreement has been specified, i.e., one new parameter (to indicate how many UDC DRBs can SN configure) is introduced in the internode signaling CG-ConfigInfo.

Some company contributions addresses this issue. For example, in [12] and [13], it has been proposed that no more coordination needs to be introduced.

Rapporteur observes that there are no other technical proposals to add more coordination information to this RAN2 meeting.

Companies are invited to share their view on this issue.

Question 1	Do you agree that no additional inter-node coordination between MN and SN is needed on top of what has been specified in the current RRC CR [2]?

| Company | Agree or not | Comments if any |
| --- | --- | --- |
| LG | Agree |  |
| Samsung | Yes |  |
| ZTE | Yes | It needs |
| Apple | Agree |  |
| OPPO | Yes |  |

Conclusion

TBD

## 2.2 Editor note in PDCP CR

Another issue is in the draft 38.323 CR [3], there is an editor note which should be addressed in this meeting also:

Editor Note: FFS whether or how to reflect if drb-ContinueUDC is configured and if the PDCP SDU has been compressed before, UE performs integrity protection and ciphering of PDCP SDU (containing UDC header and UDC data block) using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8

Some company contributions addressed this issue and there seems to be different views, see e.g., [8], [10], [13], [14], and [15].

We observe the following possible options, i.e.,

- Option 1 no changes to the spec and the Editor note is dropped

- Option 2 to clarify “UE should apply integrity protection and ciphering to PDCP SDU containing UDC header and UDC data block if drb-ContinueUDC is configured and if the PDCP SDU has been compressed before” with normative text

--------start of example TPs for option 2

An example is given below for option 2 (see [13])

- perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4 and/or using EHC as specified in the clause 5.12.4; - perform uplink data compression of the PDCP SDU if drb-ContinueUDC is not configured, as specified in the subclause 5.X.4; - perform uplink data compression of the PDCP SDU which has not been compressed before and if drb-ContinueUDC is configured, as specified in the subclause 5.X.4; - If drb-ContinueUDC is configured and if the PDCP SDU has been compressed before: - perform integrity protection and ciphering of the compressed PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8; - else: - perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8; - submit the resulting PDCP Data PDU to lower layer, as specified in clause 5.2.1.

--------end of example TPs for option 2

- Option 3 to clarify “UE should apply integrity protection and ciphering to PDCP SDU containing UDC header and UDC data block if drb-ContinueUDC is configured and if the PDCP SDU has been compressed before” with a Note

--------Start of example TPs for option 3

An example is given below for option 3, see the highlighted part

for AM DRBs whose PDCP entities were not suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below: - perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4 and/or using EHC as specified in the clause 5.12.4; - perform uplink data compression of the PDCP SDU if drb-ContinueUDC is not configured, as specified in the subclause 5.X.4; - perform uplink data compression of the PDCP SDU which has not been compressed before and if drb-ContinueUDC is configured, as specified in the subclause 5.X.4; Note: If drb-ContinueUDC is configured and if the PDCP SDU has been compressed before, UE performs integrity protection and ciphering of PDCP SDU (containing UDC header and UDC data block) using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8 - perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8; - submit the resulting PDCP Data PDU to lower layer, as specified in clause 5.2.1.

--------end of example TPs for option 3

Please note that the above TPs are only for examples, so that companies understand generally what the option means. The detailed wording, if needed, can be refined in Ph2.

Rapporteur observes that the intended UE behaviour should be the same (as the normative text or note is saying), but it is more about whether this is reflected in the CR and if yes how.

Companies are invited to share their view on this issue.

Question 2	Which option do you prefer regarding the handling of the above mentioned editor note in 38.323 CR [3]?

| Company | Option 1/2/3 | Comments if any |
| --- | --- | --- |
| LG | Option 1 | According to the specification, it is obvious that the PDCP entity performs the integrity protection and ciphering to a PDCP SDU considered as the retransmission regardless of whether the PDCP SDU has been compressed or not. for AM DRBs whose PDCP entities were not suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below: … perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8; Thus, the clarification and the specification change are not needed |
| Samsung | Option 2 or 3 | If we do not clarify this, the problem is that UE may use original PDCP SDU (i.e. uncompressed PDCP SDU) other than UDC header and UDC data block (i.e. compressed PDCP SDU) for integrity protection, ciphering, and actual transmission at PDCP re-establishment because UE will not apply UDC to PDCP SDU which has been compressed before if drb-ContinueUDC is configured. Therefore, we need to clarify which one should be used for integrity protection/ciphering for (re)transmission by adding a normative text or a NOTE. [LG] If the PDCP entity has only one SDU buffer, there is no case where the security is not applied to all PDCP SDU because all stored PDCP SDUs are already compressed in order to support the UDC continuity. In this case, there is no reason to capture it in the spec. However, if PDCP entity has two SDU buffers, i.e., one for original PDCP SDU and another for compressed PDCP SDU, in order to support the UDC continuity, we may need to specify something. Considering the above, we think whether the PDCP entity has one SDU buffer or two SDU buffers for supporting UDC continuity should be discussed first. Note that even if we support the ROHC continuity, it is assumed that the PDCP entity has only one SDU buffer. |
| ZTE | See comments | As we discussed in our contribution[15], we think whether to capture anything is upon How UE implementation of the PDCP entity when UDC continuity is configured: - Only one buffer for PDCP SDU，in this case, we do not need to introduce any specification change. First of all, It does not matter the PDCP SDU compressed before is compressed again since anyway the PDCP SDU in the buffer will be associated with a new SN number. - Two buffers for PDCP SDU, one buffer is for saving the PDCP SDU with no operation, the other one is for saving the PDCP SDU with compression. Then the specification change is needed, and notification is needed as well, we can accept both option 2/option 3 |
| Apple | Option 1 or 2 with comments | We slightly prefer using TP1 or TP2 given in [9]. The TPs are close to Option 1 and bear the same functionality as Option 2. All three options 1/2/3 are acceptable to address the problem. We do not think there is a need to specify anything extra about the SDU buffer. How the UE represents or identifies whether an SDU has been compressed before is internal to UE implementation. |
| OPPO | Option 1 | We understand that the EN indicates 1) the PDCP layer does integrity protection and ciphering without performing UDC, 2) The PDCP SDU used should be the one containing the UDC header and UDC data block. For 1) we understand that the current spec is sufficient. No matter which PDCP SDU type it is, the PDCP layer should go through the spec text line by line. If the compression condition is not fulfilled, the PDCP layer does integrity protection and ciphering without performing UDC. Otherwise, the PDCP layer does integrity protection and ciphering after performing UDC. Thus, there is no ambiguity room for clarification. For 2) we understand that it implies that three PDCP packet types should be stored at the transmitting PDCP entity, i.e. the original PDCP SDU, the PDCP SDU containing UDC header /UDC data block and the associated PDCP PDU. The PDCP transmitting entity should maintain three type buffers if UDC is enabled. It is surely complicated UE’s behaviour and requires a higher UE capability. On the other hand, there are other solutions to handle this. For example, the PDCP layer can use the original PDCP SDU and set FU to 0. Accordingly, the smart decompressor can use the original PDCP SDU for buffer synchronization. Another alternative is the UE can pull the buffer status back and perform re-compression from the first unsuccessful delivered PDCP SDU. We think it is better to leave the detailed behaviour to the UE implementation. Last but not the least, we understand RoHC also faces this same issue, but there is no place in the spec to reflect. Thus, we may follow the way RoHC does. |
| Samsung |  | Regarding LG’s comments, it is NOT obvious that the PDCP entity performs the integrity protection and ciphering to “compressed PDCP SDU” or “uncompressed PDCP SDU”. If uncompressed PDCP SDU is used when it have been UDC compressed before and drb-ContinueUDC is configured, then the decompression failure will happen. Regarding OPPO’s comments, other solutions are not valid, e.g. “For example, the PDCP layer can use the original PDCP SDU and set FU to 0. Accordingly, the smart decompressor can use the original PDCP SDU for buffer synchronization. Another alternative is the UE can pull the buffer status back and perform re-compression from the first unsuccessful delivered PDCP SDU.” If the original PDCP SDU is used at PDCP re-establishment when it has been UDC-compressed before and drb-ContinueUDC is configured, the decompression failure will happen because the network expects that UE continues UDC compression buffer. Moreover, as we mentioned in our contribution (R2-2200581), pulling the buffer status back is practically infeasible. [OPPO] If the original PDCP SDU is used, the NW needs to consider more other than the UE continues UDC compression buffer. For example, the NW should know that the packets associated with previous SN (e.g. SN 4, SN =5 is received before PDCP re-establishment) is not compressed and can be used to decompress packets of e.g. SN =5. In addition, would you please share your view on how we currently handle this similar case for RoHC? [Samsung] As you know, the way of UDC compression/decompression is very different from that of ROHC. ROHC can decompress each packet based on ROHC context and thus can handle a packet regardless of whether the packet has been compressed or not and whether the packet is received as compressed PDCP SDU or uncompressed PDCP SDU, which is not sensitive to data loss and is not dependent on the previous packet while UDC decompresses a packet based on UDC buffer updated by the previous UDC compressed packet, i.e. UDC decompression failure always happens if the previous UDC compressed packet is lost or changed from earlier one (e.g. original PDCP SDU is used instead of previous UDC compressed PDCP SDU). In this regard, it is difficult for UDC to handle your scenario unlike ROHC. The critical problem is that UE can pre-process several PDCP PDUs including UDC compression and store them and they have never been transmitted over the air. In this case, the network cannot know whether these PDCP PDUs have been compressed or not. Note that the meaning of continuing UDC compression buffer is that UDC buffer updated by such pre-processed PDCP PDUs is kept, which also means that compressed PDCP SDU should be used for transmission if it has been compressed before because it already updated the UDC compression buffer. In the same manner, the decompression buffer at NW should be updated by such pre-processed PDCP PDUs to avoid desynchronization. Therefore, the original PDCP SDU should not be used at PDCP re-establishment if it has been compressed before and drb-ContinueUDC is configured. [LG] Since we do not see the need of having two SDU buffers only for the UDC continuity, we assume that the PDCP entity has one buffer. Thus, as commented by ZTE, if there is one buffer, we do not need to specify additional note or procedure in the spec. |

Conclusion

TBD

## 2.3 FFS on UE capability

The previous open issue seems to focus on the need of some additional UE capability restriction to apply UDC, in terms of maximum uplink data rate.

Some company contributions addressed this issue, see e.g., [7][9][11] and some others. In the previous discussions, there were also comments that how to specify such data rate limit is unclear. And it seems [7] and [9] have provided some examples. More specifically in [7], proposals as well as TPs to 306/323 CRs are provided regarding how to specify such a uplink data limit. In [9], several options are provided to define a clear boundary of the maximum UDC data rate applicable to NR.

Rapporteur understands that there is strong view from some companies to introduce such uplink data rate limit, but there are also views that this is not necessary.

Companies are invited to share their view on this issue.

Question 3	Do you agree to introduce additional UE capability restriction (i.e., in terms of uplink data rate limit for applying NR UDC)?

| Company | Agree or not | Comments if any |
| --- | --- | --- |
| LG | Disagree | If the UE processing power is not enough, the PDCP entity may not compress a PDCP SDU. Note that the PDCP entity can determine whether to compress the PDCP SDU by itself. Considering that, there is no reason to introduce the UE capability for limitation. |
| Samsung | Yes | It would be good to have a limit for UDC data rate. However, we have some sympathy with the fact that it would be difficult to decide the exact value within limited TUs. Another approach would be to allow UE implementation to skip UDC compression, which needs to be clarified in the specification. |
| ZTE | No | We tend to agree with LG’s analysis. Specification does not limit UE to have to compress data once the UDC is configured. And introducing the capability, NW still have no idea how to use |
| Apple | Yes | By adding a UDC data rate limitation UE implementation does not need to be overly complex (e.g., due to unrealistic UC assumptions) and the network can properly allocate resources. If a UE was to skip compression by setting the FU bit more often during a processing limitation, this not only comes with some overhead, but it might also lead to mismatched assumptions on the network side (the network may not expect such behavior and it’s hard to predict how long it will last). Thus, an indication of a data rate limitation can help both the UE and the network. As mentioned in our paper [9] there are several options for adding a capability for a UDC maximum data rate. Alternatively, the UE could as well inform the network of a temporary capability limitation if it cannot sustain the high data rate with UDC. Another approach is to allow the UE to skip the compression. But it seems not clearly defined in the specification whether the FU field can be applicable in this situation. We agree with Samsung that this would need to be clarified in the specification. |
| OPPO | No | Agree with LG and ZTE |

Conclusion

TBD

## 2.4 Potential issue on PDCP PDUs to be discarded in the network side

One more issue regarding PDCP PDUs to be discarded in the network side was discussed in [14], where it was proposed

Proposal 2. Upon receiving the PDCP Control PDU containing the checksum error indication, the PDCP entity indicates the discard only for the PDCP PDUs that the data block is compressed by UDC to the RLC entity. [14]

The following TP was also provided [14]

--------Start of TPs in [14]

5.X.8	UDC checksum error handling

UDC checksum error notification PDCP control PDU indicates the compression buffer and de-compression buffer are out of synchronization. When receiving the notification, the UE shall trigger UDC buffer reset procedure to resynchonize the compression buffer and indicate to the RLC entities the discard indication for the PDCP data PDUs that the UDC data block is compressed by UDC.

--------end of TPs in [14]

Companies are invited to share their comments on the above issue.

Question 4 	Do you agree with the above mentioned issue on PDCP PDUs to be discarded in the network side and the proposed changes (as in [14])?

| Company | Agree or not | Comments if any |
| --- | --- | --- |
| LG | Agree | In NR, since the PDCP entity in the UE can perform the pre-processing, the PDCP entity in the UE may submit to the RLC entity the PDCP PDUs that the data block is compressed by UDC, before reception of the PDCP Control PDU containing checksum error indication. In this case, these PDCP PDUs will be discarded in the PDCP entity in the network due to out of synchronization between the compression and de-compression buffer even if these PDCP PDUs are successfully transmitted. In other words, during the UDC buffer reset procedure, the PDCP entity in UE can transmit the PDCP PDUs to be discarded in the network side. The transmission of the PDCP PDUs to be discarded in the network side causes the waste of the radio resources. In addition, it causes the delay of the transmission for PDCP PDU for resynchronization. Thus, we think that upon receiving the PDCP Control PDU containing the checksum error indication, the PDCP entity indicates the discard only for the PDCP PDUs that the data block is compressed by UDC to the RLC entity. |
| Samsung | No | This proposal can complicate the network implementation given that it can cause dispersed PDCP SN gaps and the network cannot know whether a PDCP PDU discarded by UE was UDC-compressed before or not, which also causes delay because the network should wait for the expiry of re-ordering timer before UDC decompression. If we remember correctly, RAN2 discussed similar issues in LTE TEI and such proposals were not pursued. Similar optimizations should be avoided. Moreover, we don’t see a big benefit to deviate from LTE UDC operation. [LG] We think that it cannot complicate the network implementation because there is no additional procedure in the network side. Of course, the discarding PDCP PDU in UE side may cause the reordering delay in network side. However, considering that the UDC is not configured for the delay sensitive service because the RLC AM is always used for UDC, the reordering delay in network side is not a big problem. Thus, it would be better to discard the PDCP PDUs to be discarded in the network side. In LTE discussion, the UDC continuity is not supported due to large overhead and small gain. Considering that, we should follow the LTE principle, i.e., UDC-continuity for NR should not be introduced. However, it was agreed that the UDC-continuity is needed only for NR. Considering that we did not follow all LTE principles for UDC, we think this issue can be discussed again in NR. [Samsung] The most important thing is that there would be a trade-off between latency and resource saving. Within limited TUs, we cannot make analysis on that and don’t see the need thereof. In our opinion, the latency issue would be more important and keeping LTE UDC operation would be beneficial given that UDC already takes the benefit of huge resource saving and LTE UDC has been implemented and tested. Regarding UDC continuity, there was a huge support and thus RAN2 are working on it to enable it without any problem. Note that nothing is broken without this enhancement. [LG] Note that the restriction for the data limitation is also a kind of enhancement that is nothing broken with small support.  If we consider the latency, we shall not introduce the UDC for the split bearer. This is because the latency may be increased for split bearers due to the different radio quality of MCG and SCG. However, we agreed that the UDC can be configured to the split bearer. Thus, we think the common understanding is that latency is not critical for UDC. In addition, it should be a baseline that the radio resources should be efficiently used as much as possible regardless of whether there is a benefit in reducing the radio resource by compression or not. Let’s hear other companies' views. |
| ZTE | No strong point of view | It seems an enhancement. We can follow the majority. |
| Apple | No | We also think it complicates the internal operation between L2 sublayers. Besides some PDUs may already be sitting in the LCH queue waiting for the next MAC PDU (or are even being processed in parallel). It might be hard to avoid the discard at the gNB in some cases, but we have the FR bit on the first compressed PDU to ensure a clean start. If a UE nevertheless can avoid sending the packets depending on the situation, it can be done locally based on implementation. [LG] Considering that the discard indication is already used when the discardTimer expires, there is no impact on the internal operation without PDCP. In addition, since the discard indication is already specified in the PDCP specification, a case where some PDUs may already be sitting in the LCH queue waiting for the next MAC PDU can happen in legacy. Thus, since it is already considered in legacy behaviour, it is not a big problem. In addition, if we can handle how to discard internal packet by implementation, we may not specify anything in the specification to discard the PDCP PDU. However, since when to discard the PDCP PDU and SDU is already specified in the specification, we believe that the discard operation should be specified in the spec, i.e., the discard indication should not be up to UE implementation. Note that in order to prevent unnecessary transmission, some enhancement is already specified in the specification, e.g., in PDCP duplication, when the PDCP entity receives the successful delivery confirmation from one RLC entity, the PDCP entity indicates the discard indication to another RLC entity. |
| OPPO | No | We think it is an enhancement and may complicate the implementation. [LG] Could you explain why do you think this complicate the implementation? [OPPO] To discard such PDCP PDU at the UE side may cause the reordering delay in the network side since the PDCP SN gaps exist. Also, it is possible that the associated PDCP PDU has already been delivered to the MAC layer, which makes the discarding behaviour not very useful. Anyway, the FR bit can be used for a clean start, which makes this solution unessential. [LG] Thank you for the explanation. We think the reordering delay is not a critical issue. Since the RLC AM is always used for UDC, the UDC is not configured for the delay-sensitive service. According to the current specification, the UE discards an RLC SDU if any part of RLC SDU is not submitted to the MAC layer when the discard indication is received for the RLC SDU. In our understanding, the RLC entity can submit the RLC PDUs to the MAC entity after receiving the UL grant and most RLC SDUs would be stored in the buffer. Thus, the discarding behaviour would be useful to efficiently use the radio resources. |

## 2.5 Other issues if any

The following question is for other potential issues, if any.

Question 5 	Please specify in the table below if you see any other remaining issues in order to close this WI.

| Company | Please explain the issue if you see any, and please provide your suggestion if any to solve the issue. |
| --- | --- |

Conclusion

TBD

- 3	Ph2 – on the CRs

TBD – will be added based on Ph1 progresses.

- 4	Conclusion

TBD

- 5	References

[1] R2-2203107	Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek Inc., Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung, Apple, Nokia, Nokia Shanghai Bell	CR	Rel-17	38.300	16.8.0	0415	-	B	NR_UDC-Core

[2] R2-2203108	Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek Inc., Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung, Apple, Nokia, Nokia Shanghai Bell	CR	Rel-17	38.331	16.7.0	2927	-	B	NR_UDC-Core

[3] R2-2203109	Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek Inc., Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung, Apple, Nokia, Nokia Shanghai Bell	CR	Rel-17	38.323	16.6.0	0087	-	B	NR_UDC-Core

[4] R2-2203110	Introduction of UE capabilities for NR UDC	CATT, CMCC, Huawei, HiSilicon, MediaTek Inc., Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung, Apple, Nokia, Nokia Shanghai Bell	draftCR	Rel-17	38.306	16.7.0	B	NR_UDC-Core

[5] R2-2203111	Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek Inc., Ericsson, China Unicom, China Telecom, OPPO, Samsung, Apple, Nokia, Nokia Shanghai Bell	CR	Rel-17	37.340	16.8.0	0298	-	B	NR_UDC-Core

[6] R2-2203112	Introduction of UE capabilities for NR UDC	CATT, CMCC, Huawei, HiSilicon, MediaTek Inc., Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung, Apple, Nokia, Nokia Shanghai Bell	draftCR	Rel-17	38.331	16.7.0	B	NR_UDC-Core

[7] R2-2202367	Limit UL data rate for UDC in UE capability	MediaTek Inc., Samsung	discussion	Late

[8] R2-2202442	Consideration on NR UDC	OPPO	discussion	Rel-17	NR_UDC-Core

[9] R2-2202520	UDC constraints and limitations	Apple	discussion	Rel-17	NR_UDC-Core

[10] R2-2202678	Clarification on PDCP SDU for UDC continuity	Samsung Electronics	discussion	NR_UDC-Core

[11] R2-2202961	Remaining issues on NR UDC	Qualcomm Incorporated	discussion	Rel-17	NR_UDC-Core

[12] R2-2203023	Discussion on remaining issues for UDC	Huawei, HiSilicon	discussion	Rel-17	NR_UDC-Core

[13] R2-2203106	Considerations on NR UDC open issues	CATT	discussion	Rel-17	NR_UDC-Core

[14] R2-2203164	Discussion on UDC	LG Electronics Inc.	discussion	NR_UDC-Core

[15] R2-2203249	Furhter Consideration on  UDC in NR	ZTE Corporation,Sanechips	discussion	Rel-17	NR_UDC-Core

- Appendix Agreements for NR UDC

Agreements in R2#116bis-e

On CRs

R2-2201277	Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung	draftCR	Rel-17	38.300	16.8.0	B	NR_UDC-Core

R2-2201278	Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung	draftCR	Rel-17	38.331	16.7.0	B	NR_UDC-Core

R2-2201279	Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung	draftCR	Rel-17	38.323	16.6.0	B	NR_UDC-Core

R2-2201280	Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung	draftCR	Rel-17	38.306	16.7.0	B	NR_UDC-Core

R2-2201281	Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, Samsung	draftCR	Rel-17	37.340	16.8.0	B	NR_UDC-Core

- Can Use these CRs as a baseline for further work (except 37340 CR which may not be needed dependent on further agreements)

On email summary in R2-2200039

- The parts without TBD in Table 1 are assumed to directly follow LTE UDC mechanism.

- UDC is not applied to the SDAP header and SDAP control PDU.

- The UDC header is located after SDAP header in the UDC PDU format.

- UDC is not applied to DAPS in NR.

- NR UDC is not applied to sidelink DRBs.

- With Figure 4.2.2-1, there is no need to further clarify UDC decompression being performed after PDCP re-ordering in the specification.

- UE shall support number of UDC DRBs 2. FFS if we need to support some additional UE capability.

- Continue by email, can include tech proposals from tdocs below (proponents are expected to request), continue on the non-agreed parts, review CRs.

On offline summary R2-2201914

[Change the UE cap FFS into: FFS whether UE data rate limitation with UDC need to be provided as a UE capability.] Chair: The FFS for the UE cap agreement above is removed, and the below is agreed instead.

- FFS whether UE data rate limitation with UDC need to be supported with a UE capability.

- UDC continuity can be configured for the same cases as ROHC continuity

- Assume that P2 and P5 can be supported, CRs for review to next meeting anyway. If issues are found R2 can revert this assumption (at next meeting).

- P2: UDC is supported for non-split bearer type in NR-DC. It is supported that MN sends to SN the maximum number of UDC DRBs that can be configured by SN. FFS if any other coordination is needed.

- P5: Support NR UDC for MR-DC and split bearer type, with the following restrictions

- - Only include NR-DC, NGEN-DC, and NE-DC (i.e., EN-DC is not supported)

- - No enhancements supported for potential data loss for split bearer case.

- Send an LS to RAN3 to inform of NR UDC potential impact to CU-CP/UP splitting scenario. R2 understands that decisions as well as the required specification work are up to RAN3.

Update CRs taking into acct all agreements, review in an offline discussion, tech. endorse if possible.

Post meeting email #053

=> The LS out in R2-2202033 is approved.

=> The report in R2-2202034 is Noted

=> The CRs in R2-2202035, R2-2202036, R2-2202037, R2-2202038, R2-2202039 are Noted as baseline.

Agreements in R2#117-e

TBD