3GPP TSG-RAN WG2 Meeting #117 electronic	R2-2203573

Electronic Meeting, Feb 21 –Mar 03, 2022

Agenda item:	7.3

Source:	CMCC

Title:	Offline discussion on the issue for Random Access on multicarrier for NB-IoT

Document for:	Discussion and Decision

# Introduction

 [AT117-e][306][NBIOT R16] Random access on multiCarrier in NB-IoT (CMCC)

Scope: Discuss issues in R2-2202633 and CRs in R2-2202634 and R2-2202635. Discussion of whether correction is needed, and work on the CRs.

Intended outcome: Report in R2-2203573, and revised CRs (if needed – Tdocs can be allocated if necessary).

Deadline: Thursday 24th February 1200 UTC.

Status: started

Contact list

| Company and Name | Email |
| --- | --- |
| Huawei, HiSilicon: Odile Rollinger | odile.rollinger@huawei.com |
| Nokia : Srinivasan Selvaganapathy | Srinivasan.selvaganapathy@nokia.com |
| Mediatek: Aaron Cai | Aaron.cai@mediatek.com |
| Ericsson: Ritesh Shreevastav | Ritesh.shreevastav@ericsson.com |

# Discussion

Deployment scenario:

In contribution [1], Figure 1 shows a typical deployment scenario in NB-IoT real network. Single-carrier cells are deployed to meet coverage requirements for most scenarios, and multi-carriers cells are deployed for concurrent service scenarios to meet capacity expansion requirements. The anchor carriers are deployed with inter frequency to reduce interference among cells, and it’s generally that the non-anchor carriers in one cell are deployed on the same frequency as the anchor carrier in the neighbour cell.

Figure 1 The deployment in the NB-IoT network

Issue description:

For the downlink, the downlink narrowband reference-signal EPRE (Energy Per Resource Element) of the non-anchor carriers is generally lower relative to the downlink narrowband reference-signal EPRE of the anchor carrier to reduce the interference between the non-anchor carrier and the neighbour cells cells using the same frequency. Due to lower EPRE of non-anchor carrier than EPRE of anchor carrier, shrunken coverage of non-anchor carrier may result to MSG2 failure if npdcch-NumRepetitions-RA-r14 is configured same for anchor carrier and non-anchor carrier.

For the uplink, the background noise is relatively strong in NB-IoT network, and the non-anchor carriers suffer more uplink interference due to the same frequency neighbor cell with uplink service terminals. This may degrade uplink performance, such as MSG 1 failure on the non-anchor carrier.

In the 116-e, RAN2 have confirmed the scenario is valid and the issue exists[2] as follows:

- RAN2 confirm the following scenario is valid, for standalone deployment: EPRE of non-anchor carrier smaller than EPRE of anchor cell. Non-anchor carrier is deployed on the same frequency with anchor carrier of neighbouring cell.

- RAN2 confirm the following issue exists:

    - DL issue: Due to lower EPRE of non-anchor than EPRE of anchor cell, shrunken coverage of non-anchor carrier may results to MSG2 failure.

    - UL issue: Non-anchor carrier suffered more UL interference than anchor carrier for the same cell, due to intra-frequency interference from anchor carrier of neighbouring cell. This may degrade uplink performance, e.g., MSG 1 failure on non-anchor carrier.

- postponed

For the DL issue, a higher NPDCCH repetitions (see npdcch-NumRepetitions-RA-r14 in SIB22-NB) can be configured with a higher value for the non-anchor carrier.

According to the current specification, the UL repetition numbers configured for the anchor carrier also apply to the non-anchor carriers. For the UL issue, increasing the UL repetition numbers configured for the anchor carrier or using smaller RSRP threshold to shrink the coverage of anchor carrier is proposed to solve the stronger uplink interference issue in non-anchor carrier. The solution would also influence the users in anchor carrier.

Q1: Do you agree that the current solutions to address the UL interference in non-anchor carrier, e.g., increasing the UL repetition numbers configured for the anchor carrier or using smaller RSRP threshold to shrink the coverage of anchor carrier, would cause unnecessary UL resources waste for UEs in anchor carrier?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Nokia | Yes | The uplink interference situation is not static and it depends on the uplink load at different cells at the time of uplink access. So any mechanism which statically changes the coverage level based on RSRP will not be optimum for this scenario. |
| MediaTek | Yes | However wet think the cost of current solution is not significant. For increasing the UL repetition numbers for the anchor carrier, as only the resources of Msg1 and Msg2 of random access needs to be increased, the rest of resources for Msg3 and Msg4 of RA, for all the other data transmission and reception in connected mode are dynamically assigned by the network, which should not cause resource waste. For shrinking the coverage of anchor carrier, the UEs in coverage area of shrinking will need to use the NPRACH resource of next CE level, the repetition number would be more than they need. Comparing the solution of increasing the UL repetition number, the wasted resource should be less. |
| Ericsson | Yes |  |

As analyzed in [1], the current implementation solution to address the uplink interference issue is to adjust the RSRP threshold and NPRACH repetitions configuration of the anchor carrier, which will have bad impact on UEs in the anchor carrier. The RSRP threshould and NPRACH repetitions for non-anchor have to keep the same as anchor carrier. So [1] propose the following solution to solve the uplink interference issues in non-anchor carrier.

Q2: To solve the uplink interference issue, do you agree to introduce a new RSRP threshold list for each non-anchor carrier for random access to determine the UE’s CE level on non-anchor carrier. The RSRP threshold list can be configured according the uplink interference.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | yes |  |
| Nokia |  | Only the cell edge users are affected. So the repetition level for last CEL alone to be changed. RSRP Threshold list only changes the threshold for CEL selection. But the issue seems to be for the last CEL only. So change of Repetition level for last coverage level for non-anchor carrier is sufficient. |
| MediaTek | No | The solution 1 (i.e. implementation methods like increasing repetition number or shrinking the coverage) and the solution 2 (i.e. Introduce a new RSRP threshold list for each non-anchor carrier for random access to determine the UE’s CE level on non-anchor carrier) in [1] are exclusive. If the solution 1 is applied, there no need to apply solution 2, and vice versa. For a Rel-14/15 network, only solution 1 can be applied as this method is introduced in Rel-16. For a Rel-16 network, solution 1 can also be applied as it can fix the problem. However, if solution 2 is applied, only Rel-16 UEs can enjoy the benefit. For the large amount of existing legacy UEs(Rel-13/14/15) still suffer from the issue that solution 1/2 tried to fix. Solution 2 is ideal, not suitable for the real world. |
| Ericsson | Yes |  |

- The NPRACH CE level of the users on the non-anchor carrier is determined based on comparison between NRSRP measurement results and the RSRP threshold for the non-anchor carrier. The NRSRP measurement results for the non-anchor carrier can be deduced or measured by the UE on this non-anchor carrier. If the NRSRP measurement results is deduced by the UE, the result is sum of NRSRP measurement results for the anchor carrier and the downlink narrowband reference-signal EPRE offset of the non-anchor carrier relative to the downlink narrowband reference-signal EPRE of the anchor carrier(see nrs-PowerOffsetNonAnchor-r14 in SIB22).

Q3: If you agree with Q2, do you agree that the NRSRP measurement results for the non-anchor carrier can be deduced or measured by the UE on this non-anchor carrier and there is no need to modify current specification?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | yes | We agree that the NRSRP measurement results for the non-anchor carrier can be deduced from the anchor carrier using nrs-PowerOffsetNonAnchor and that there is no need for specification change. Note, this is similar (although the opposite way) to what is done for RACH in connected mode where the UE uses the configured carrier NRSRP measurement to select the PRACH resource. |
| Nokia | NA |  |
| Ericsson | Yes |  |

Q4: If you agree with Q2, which option is preferred if the UE selects a non-anchor carrier and the CE level of the non-anchor carrier is different from that of the anchor carrier?

- Option 4a: the UE shall access to the anchor carrier with the better CE level if the CE level of the non-anchor carrier is worse than that of the anchor carrier.

- Option 4b: if there is NPRACH resources for the determined CE level on this non-anchor carrier, the UE will access to the non-anchor carrier with the corresponding CE level of the non-anchor carrier. Otherwise, the UE will access to the anchor carrier with the better CE level (e.g., the CE level determined with the original RSRP threshold).

- Option 4c: other options.

| Company | Which option is preferred | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | 4c | 4a: this is a possibility but it would be moved all the load to the anchor carrier when it could be distributed. it seems also simpler to exclude the non-anchor carrier before the NPRACH resource selection (see solution 4c) 4b: we think that may break power ramping and have a large impact on MAC specification. 4c: we think the simpler approach is to exclude the carriers with worse CEL than the anchor carrier when building the list of NPRACH resources. Then the selection mechanism is as per legacy. This could be captured with a single sentence in MAC in section 5.1.2. |
| Nokia | 4c | For UE in lower CEL the current algorithm and spec works well. For the last CEL, UE in non-anchor carrier can use higher repetitions to compensate the uplink interference. Alternatively, UE can calculate extension of repetition level for last CEL based on the difference between anchor and non anchor carrier |
| Ericsson | 4b/4c | if we want to properly handle the CE level selection with consideration of interference, then we may also need to take the NRSRQ into account as well. |

The draft 36.331CR and the draft 36.321CR are now available in the draft folder.

Q5: Do you agree with the draft 36.331CR and the draft 36.321CR? Comments are welcome.

| Company | Yes/No for 36.331CR | Yes/No for 36.321CR | Comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon |  |  | Exact details will need to be discussed once the solution is agreed. Below are some initial feedback. 36.331: the new RSRP thresholds should be signalled in SIB22 not SIB2 36.306 we also need a 36.306 CR to capture the support of the new mechanism as optional w/o capability reporting |

# Conclusions

TBD

# Reference

- R2-2202633 Discussion on the issue for Random Access on multicarrier for NB-IoT   CMCC     discussion

- R2-2201970 Report of 3GPP TSG RAN WG2 meeting #116-e, Online  ETSI MCC

[3]	R2-2202634 Solution for random access issue on multiCarrier in NB-IoT   CMCC     draftCR   Rel-16 36.331     16.7.0      F      NB_IOTenh3-Core

[4]	R2-2202635 Solution for random access issue on multiCarrier in NB-IoT    CMCC     draftCR   Rel-16 36.321     16.6.0      F      NB_IOTenh3-Core