3GPP TSG-RAN WG2 #117-e	R2-220xxxx

- Online, February 21 – March 3, 2022

Agenda item:	8.7.1

Source:	Samsung

Title:	[AT117-e][620][Relay] Reply LS to RAN3 on mapping configuration (Samsung)

Document for:	Discussion & Decision

# Introduction

This is to discuss a Reply LS to RAN3 on mapping configuration as below.

- [AT117-e][620][Relay] Reply LS to RAN3 on mapping configuration (Samsung)

Scope: Draft a reply to the LS in R2-2202136.

Intended outcome: Approved LS (preferably without CB)

Deadline:  Thursday 2022-02-24 1200 UTC

# Discussion

According to the current RRC running CR [1], LCID is used to indicate Uu egress RLC channel in “sl-Egress-RLC-Channel-Uu-r17”. To implement this in CU-DU split architecture, the configuration “SL-MappingToAddMod-r17” is generated by gNB-CU and LogicalChannelIdentity is generated by gNB-DU. This requires extra signalling for gNB-CU to receive and interpret LogicalChannelIdentity from gNB-DU and impacts to F1 interface. RAN3 thinks that such extra signalling is not needed if Uu RLC Channel ID is used instead of LogicalChannelIdentity to identify the egress Uu RLC channel and asks the following question in the LS [2].

Is it feasible for RAN2 to change the current running CR by indicating the Uu RLC Channel ID instead of LCID when configuring “sl-Egress-RLC-Channel-Uu-r17”?

As presented in a contribution [3], RAN2 made agreements on a mapping from E2E bearer ID to egress RLC channel ID/LCID during RAN2#116e meeting. So Rapporteur observes that RAN2 can be flexible to use RLC channel ID instead of LCID in the mapping configuration and would like to ask whether RAN2 can change LCID to Uu RLC channel ID in the configuration.

Q1. Do companies agree that Uu RLC channel ID can be used instead of LCID in the “sl-Egress-RLC-Channel-Uu-r17”?

| Company | Yes / No | Comments |
| --- | --- | --- |
| CATT | Yes | In R2-2202822, 18/23 companies agreed to introduce new Uu/PC5 RLC channel configuration. The detail can be up to running-CR to handle. |
| OPPO | Yes | Same view as CATT. And accordingly, we may start LS drafting based on the direction as output of 2822. |
| Qualcomm | Yes | We prefer to use RLC channel ID, i.e., update RAN2 RRC spec which allows RAN3 to reuse the existing F1AP produces. Because we understand RAN3 impact on existing F1AP procedure is large, but RRC spec is not frozen yet. |
| MediaTek | Yes | It can be flexible to use RLC channel ID instead of LCID |
| Ericsson | Yes |  |
| ZTE | Yes | Similarly, for PC5 RLC channel configuration, PC5 RLC channel ID shall be used when configuring “sl-Egress-RLC-Channel-PC5-r17”, instead of “SL-RLC-BearerConfigIndex”. |
| LG | Yes |  |
| vivo | Yes | Just like IAB, we can introduce new RLC channel configuration for the F1AP signaling in L2 U2N relay. |
| Xiaomi | Yes |  |
| Nokia | Yes |  |
| Apple | Yes |  |

Q2. Any additional comment on the reply LS?

| Company | Yes / No | Comments |
| --- | --- | --- |

# Conclusion

# References

[1] R2-2201817, RRC running CR for SL relay, Huawei, HiSilicon

[2] R2-2202136, LS on mapping configuration of sidelink relay (R3-221411; contact: Samsung), RAN3

[3] R2-2202952, Discussion on RAN3 LS on mapping configuration of sidelink relay, Samsung