3GPP TSG-RAN WG2 #117-e	R2-220xxxx

- Online, February 21 – March 3, 2022

Agenda item:	8.7.2.4

Source:	Samsung

Title:	Summary of [Pre117-e][602][Relay] Open issues on relay QoS

Document for:	Discussion & Decision

# Introduction

This is the summary of the pre-meeting discussion on open issues of Relay QoS.

# Discussion

There are 7 identified open issues on QoS in [1]. This discussion will handle O3.01 through O3.05 and O3.07 based on the proposals in the summary of agenda item 8.7.2.4 of RAN2#116bis-e meeting [2]. Please note that O3.08 will be handled by CR Rapporteur.

| Issue Index | Description | Suggested handling | Reason for add/remove this open issue |
| --- | --- | --- | --- |
| O3.01 | [Unhandled issue from Pre-R2#116b summary] FFS on further enhancement of L2 relay QoS to support flow control | Pre117-e-offline | Due to the proposal raised in QoS A.I. summary: Proposal 1. RAN2 to discuss whether to support flow control in L2 U2N Relay. We have the corresponding open issue On the other hand, it is pending CB decision from 619 Proposal 1 (13/17) Control PDU is not supported in neither PC5 SRAP layer nor Uu SRAP layer in this release. |
| O3.02 | [Unhandled issue from Pre-R2#116b summary ]FFS on further enhancement of L2 relay QoS to support pre-emptive BSR | Pre117-e-offline | Due to the proposal raised in QoS A.I. summary Proposal 2. RAN2 to discuss whether to support pre-emptive BSR transmission by a Relay UE to gNB. We have the corresponding open issue |
| O3.03 | [Unhandled issue from Pre-R2#116b summary] FFS on further enhancement of L2 relay QoS to support bit rate recommendation | Pre117-e-offline | Due to the proposal raised in QoS A.I. summary Proposal 3. RAN2 to discuss whether to support the bit rate recommendation procedure. We have the corresponding open issue |
| O3.04 | [Unhandled issue from Pre-R2#116b summary] FFS on further enhancement of L2 relay QoS to support dedicated resources for relay traffic | Pre117-e-offline | Due to the proposal raised in QoS A.I. summary Proposal 7. RAN2 to discuss the need of dedicated resources at Relay UE for relayed traffic. We have the corresponding open issue |
| O3.05 | [Unhandled issue from Pre-R2#116b summary] FFS on QoS information report in SUI for SL discovery. | Pre117-e-offline | Due to the proposal raised in QoS A.I. summary Proposal 5. RAN2 to discuss that UE does not need to report PC5 QoS information in SUI for SL discovery. We have the corresponding open issue |
| O3.07 | [EN from running CR of 38.321] whether to apply PDB restriction when performing MAC PDU transmission | CR rapporteur handled | Due to the following EN in 38.321 running CR: Editor’s Note: FFS the above change is needed, depending on “according to the associated priority” phrase is needed in Rel-16 specification. We have the corresponding open issue. Based on further input from companies, this issue include PDB aspect of discovery message as well. |
| O3.08 | [EN from running CR of 38.323] whether to adopt new code-point for SDU type | CR rapporteur handled | Due to the following EN in 38.323 running CR: Editor’s Note: FFS for ARP (Address Resolution Protocol) e.g., use “010” for ARP, no ROHC for ARP, applicable only for NR sidelink communication for groupcast and broadcast We have the corresponding open issue. |

(1) O3.01 - Whether to support flow control for L2 Relay

We have the following proposals in the summary [2]. Rapporteur thinks that Proposal 1-2 does not have to be discussed in this pre-meeting discussion. RAN2 also made an agreement at RAN2#116bis-e that control PDU is supported in neither PC5 SRAP layer nor Uu SRAP layer in this release.

Proposal 1. RAN2 to discuss whether to support flow control in L2 U2N Relay. Proposal 1-1. If flow control is supported in P1, Relay UE can transmit flow control indication to its Remote UE and gNB for UL/DL transmission over PC5 Proposal 1-2. In P1-1, the flow control indication can be either MAC signalling or via control PDU in the adaptation layer handled in AI 8.7.2.3.

Q1. Do companies support flow control in L2 U2N Relay?

| Company | Yes / No | Comments |
| --- | --- | --- |
| InterDigital | Yes | If we can manage to specify a simple flow control mechanism, it would be preferrable, since this is anyway supported in IAB. |
| CATT | Yes | UL of L2 U2N relay Since only mode 2 is used by remote UE, PC5 UL transmission is controlled by remote UE itself. When the remote UE performs mode 2 resource allocation, according to the current MAC specification, it will only consider the available resource indicated by physical layer, the remaining PDB of SL data, the number of HARQ retransmission and etc. Hence, for UL, it is possible that a lot of data is sent from remote UE to relay UE, but it cannot be sent to gNB since the Uu UL is congested. Hence, UL flow control is necessary for L2 U2N relay. DL of L2 U2N relay There are two cases: - Case 1: Relay UE is configured with mode 1. In this case, both the DL Uu and PC5 transmission are controlled by gNB. gNB is aware whether PC5 link is congested or not, hence gNB implementation can handle the DL flow control if relay UE is configured with mode 1. - Case 2: Relay UE is configured with mode 2. In this case, only the DL Uu scheduling is controlled by gNB, but the DL PC5 scheduling is controlled by relay UE. Hence, it is possible that Uu is not congested but PC5 is congested. In order to avoid it, DL flow control is necessary for L2 U2N relay if relay UE is configured with mode 2. |
| Ericsson | yes | We see FC as a useful feature, we can support it in this release with limited spec changes. |
| MediaTek | Yes | We agree with CATT. |
| Philips | Yes | This is especially critical for situations when the Relay UE is experiencing congestion either on Uu or on SL. If there is congestion on Uu the Remote UEs should be informed so that they adapt their traffic. Likewise, if there is congestion on PC5 the network should be aware and adjust DL scheduling on Uu accordingly. What it has been specified for IAB can be used as a basis. |
| Apple | Yes | We share the same view as InterDigital that a simple flow control mechanism can be supported in this release. |
| Qualcomm | No | - From technique perspective, because Rel-17 supports only single hop, we think flow control can be achieved by relay UE and gNB implementation. For example, the below solutions can be considered: - gNB or relay UE can limit number of remote UE connections based on the load and channel quality of the Uu and PC5 links. - In single hop relay, the traffic load of remote UEs can be roughly estimated based on SL BSR - It is conflicted with below agreement in last RAN2 meeting: Proposal 1 (modified) Control PDU is supported in neither PC5 SRAP layer (13/19) nor Uu SRAP layer (14/19) in this release. This agreement means we can’t specify similar flow control solution in IAB. - We still don’t know what is a “simple flow control mechanism”. We are not sure how an on-fly solution can be specified in last Ran2 meeting of Rel-17. |
| Sharp | Yes | We share the same view with CATT. |

Q2. Do companies support that Relay UE can transmit flow control indication to its Remote UE and gNB for UL/DL transmission over PC5?

| Company | Yes / No | Comments |
| --- | --- | --- |
| InterDigital | Yes | Both directions makes sense for flow control. |
| CATT | Yes | For UL, relay UE should send flow control indication to the remote UE if the UL Uu buffer size of logical channels used for relay link is above the configured threshold. For DL, relay UE should send flow control indication to gNB if the DL PC5 buffer size is above the configured threshold at least when relay UE is in mode 2. |
| Ericsson | yes | We see FC as a useful feature, we can support it in this release with limited spec changes. |
| MediaTek | Yes | Same as Q1. |
| Philips | Yes | It is important that the Relay UE inform the Remote UE of potential congestion situations so that the Remote UE can adapt its traffic. |
| Apple | Yes |  |
| Qualcomm | No | We should respect agreement of last RAN2 meeting: Proposal 1 (modified) Control PDU is supported in neither PC5 SRAP layer (13/19) nor Uu SRAP layer (14/19) in this release. Because SRAP header can’t be used, can proponent explain how to send the indication? |
| Sharp | Yes |  |

(2) O3.02 - Whether to support pre-emptive BSR

We have the following proposal in the summary [2].

Proposal 2. RAN2 to discuss whether to support pre-emptive BSR transmission by a Relay UE to gNB.

Q3. Do companies support pre-emptive BSR transmission by a Relay UE to gNB?

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | No | To shorten the signalling latency, gNB can configure the configured grant towards relay UE, which can achieve the same purpose as pre-emptive BSR and does not require any additional spec effort. Therefore, it seems repetitive to support pre-emptive BSR |
| InterDigital | Yes | If a simple approach can be specified in this release, it would be preferrable. The timing of a configured grant for the relay UE cannot account for the specific timing when the relay will receive data from the remote UE, and so the latency could be large. |
| CATT | Yes | In our understanding, it is hard for gNB to configure configured grant to shorten the signalling latency since gNB does not know the SL data arrival pattern in PC5, it may result resource waste compared with introduction of pre-emptive BSR. If pre-emptive BSR is introduced, it can also help reducing UL scheduling latency: - Relay UE can trigger the pre-emptive BSR once it receives SCI, it does not need to wait for SL data decoding. - If SCI contains resource reservation for further SL transmissions, it can also trigger the pre-emptive BSR before the further SL data transmission. |
| Ericsson | Yes | Agree with Interdigital and CATT |
| MediaTek | Yes |  |
| Philips | Yes | Pre-emptive BSR will reduce the latency of the communication and can simply be triggered upon reception of SCI from the Remote UE. |
| Apple | No | For the idea to use SCI information to trigger the pre-emptive BSR: - If the pre-emptive BSR is used for relaying the data in currently received PSSCH slot, then it will only take a few milliseconds to decode the PSSCH. Relay UE can wait for a few milliseconds to generate a normal BSR. - If the pre-emptive BSR is used for relaying the future traffic reserved in remote UE’s SCI with reservation interval, we think this SCI indication is destination-agnostic, so relay UE cannot simply assume the reserved resource in the next period are for the same SL destination. Even remote UE has reserved the next period for the same SL relay destination, the traffic may not needs to be forwarded to gNB as it may be some local traffic (PC5-RRC/PC5-S). So, this is not a very accurate scheme to be used to report buffer status for end-to-end UL traffic.. |
| Qualcomm | Yes, if same design as IAB | If totally same design as IAB is reused, we can support it because there is no extra specification work. |
| Sharp | Yes |  |

(3) O3.03 - Whether to support recommended bit rate control via Relay UE

We have the following proposal in the summary [2].

Proposal 3. RAN2 to discuss whether to support the bit rate recommendation procedure.

Q4. Do companies support the bit rate recommendation procedure via Relay UE?

| Company | Yes / No | Comments |
| --- | --- | --- |
| InterDigital | No | It seems recommended bit rate is not crucial for a first release and can be considered in later releases. |
| Ericsson | No |  |
| MediaTek | Yes | If gNB knows the desired bit rate of a remote UE, the gNB can reconfigure the Uu logical channel or SL logical channel for this remote UE so as to increase the supported bit rate. Similarly, it is also preferred that gNB can control the bit rate of a remote UE by transmitting the recommended bit rate explicitly to the remote UE. |
| Philips | Yes | We think that Relays should be available not just for best effort type of data but also for voice and video as well as other types of streaming data. For reliable voice and video it is essential to support recommended bit rate. Voice and video are crucial for public safety use cases as well as for anyone who needs to maintain a voice or video call under poor radio conditions. |
| Apple | No | There is no end-to-end relayed MAC CE concept in Sidelink UE-to-NW relay, so, we need to introduce the new SL MAC CE or convey the recommendation or recommendation query in PC5-RRC signalling. However, this will require new signalling in PC5. Also, in Uu hop, we need some ways to differentiate the RBR MAC CE for relay UE and RBR MAC CE for remote UE, given that the SRAP header is not applicable to MAC CE. All of the above would be challenging as the completion deadline of work is near. We’d better to consider this in future release. |
| Qualcomm | No |  |
| Sharp | No |  |

(4) O3.04 - Whether need to specify dedicated resource for relayed traffic

We have the following proposal in the summary [2].

Proposal 7. RAN2 to discuss the need of dedicated resources at Relay UE for relayed traffic.

Q5. Do companies support the dedicated resources at Relay UE for relayed traffic?

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | No | We do not see any particular service requirement for relay traffic compared with normal sidelink traffic. Therefore, there seems lack of motivation to support dedicated resources for relayed traffic. |
| InterDigital | No | Agree with OPPO. |
| CATT | No | Same view as OPPO. |
| Ericsson | No | Agree with OPPO |
| MediaTek | No |  |
| Apple | No |  |
| Sharp | No |  |

(5) O3.05 - Whether need to report PC5 QoS flow in SUI for SL discovery

We have the following proposal in the summary [2].

Proposal 5. RAN2 to discuss that UE does not need to report PC5 QoS information in SUI for SL discovery.

Q6. Do companies support that UE does not need to report PC5 QoS information in SUI for SL discovery?

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | Agree (not need to report) |  |
| InterDigital | Yes |  |
| CATT | Yes |  |
| Ericsson | Yes |  |
| MediaTek | Yes |  |
| Apple | Yes (no need) |  |
| Qualcomm | Yes |  |
| Sharp | Yes |  |

(6) O3.07 - Whether to apply PDB restriction for SL SRBs including SRB4 when performing MAC PDU transmission

We have the following proposal in the summary [2].

Proposal 6. RAN2 to discuss whether PDB for SL discovery can be determined by TX UE implementation.

The original proposal is about SL SRB4 but it is observed that this discussion is applied for any SL SRBs. So Rapporteur would like to ask a discussion first whether to apply PDB restriction for any SL SRBs. Based on the company inputs on the need of PDB requirement, RAN2 may discuss further how to apply PDB restriction for SL SRBs.

Q7. Do companies support to apply PDB restriction for any SL SRBs when performing MAC PDU transmission?

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | No with comment | Since both relay/remote UE will not report QoS info towards gNB for discovery, as we supported in the previous Q. Therefore, there is no need to apply PDB restriction for SL-SRB4. For other SL-SRBs, we assume it is out of the scope of relay discussion and prefer to discuss them in Rel-16 CR discussion. |
| InterDigital | No | We do not see the need to implement such restriction for any SRB at this stage as it is not supported in Rel16. |
| CATT | No | Same view as OPPO and InterDigital. |
| Ericsson | No | Agree Interdigital and CATT, and OPPO |
| MediaTek | No |  |
| Apple | No | We think this has been left to UE implementation in Rel-16. We are not sure Rel-17 SRB4 needs to be treated differently. |
| Qualcomm | No |  |
| Sharp | No |  |

# Conclusion

In summary, the following are proposed:

# References

[1] R2-2201721, Remaining Open issue list of R17 Sidelink Relay Wi, OPPO

[2] R2-2201659, Summary of agenda item 8.7.2.4 (QoS), Samsung