- 3GPP TSG RAN WG2 Meeting #118e      	                         R2-220xxxx
E-meeting, 9th – 20th May 2022

Agenda item:	5.1.2

Source:	ZTE Corporation (Rapporteur)

Title:	Report of [AT118-e][013][NR1516] Stage-2 (ZTE)

Document for:	Discussion and Decision

# Introduction

This document summarizes the following offline discussion.

- [AT118-e][013][NR1516] Stage-2 (ZTE)

Scope: Treat R2-2205923, R2-2205924, R2-2206110, R2-2206111, R2-2205978, R2-2205979, R2-2205990
Ph1 Determine agreeable parts, Ph2 for agreeable parts agree CRs (offline agreement, CB online only if necessary).

Intended outcome: Report, Agreed CRs

Deadline: Schedule 1

# Contact information

| Company | Name and email address |
| --- | --- |
| ZTE Corporation (rapporteur) | Sergio Parolari (sergio.parolari@zte.com.cn) |
| Nokia | amaanat.ali@nokia.com |

# Discussion

## SCell activation

Question 1: Do companies agree with the Rel-16/17 37.340 CRs in [1] and [2] on Correction for SCell activation? (if there are concerns with (parts of) the CR, please justify)

| Company | Yes or No | Comments |
| --- | --- | --- |
| Nokia | No | We understand the intent of the existing text was only to set the context of how the serving cells on the MCG and the SCG can be activated using the MAC CE. So, this was to disallow cross CG activation. Hence there is no ambiguity, and this is not essential to correct. |

## Clean-up CRs

Question 2: Do companies agree with the Rel-16/17 37.340 CRs in [3] and [4] on Rapporteur Clean-up? (if there are concerns with (parts of) the CR, please justify)

| Company | Yes or No | Comments |
| --- | --- | --- |
| Nokia | Yes | We understand the changes are editorial corrections. |

## UL Tx switching

Question 3: Do companies agree with the Rel-16/17 37.340 CRs in [5] and [6] on Support of 1Tx-2Tx UL Tx switching for EN-DC? (if there are concerns with (parts of) the CR, please justify)

| Company | Yes or No | Comments |
| --- | --- | --- |
| Nokia | Netural --> No | Honestly some description in RAN2 Stage 2 is desirable for this feature but the place where this is made is incorrect and does not belong there. However, one may argue there are many other RAN1 features which don't have a clear RAN2 stage 2 description and maybe one can start collecting those all but these are also found from RAN1 specifications. |

Question 4: Do companies agree with the 38.300 CR in [7] on Support of UL Tx switching for inter-band UL CA and SUL? (if there are concerns with (parts of) the CR, please justify)

| Company | Yes or No | Comments |
| --- | --- | --- |
| Nokia | Netural --> No | Honestly some description in RAN2 Stage 2 is desirable for this feature but the place where this is made is incorrect and does not belong there. However, one may argue there are many other RAN1 features which don't have a clear RAN2 stage 2 description and maybe one can start collecting those all but these are also found from RAN1 specifications. |

# Conclusion

To be provided.

# References

[1]	R2-2205923	Correction for SCell activation	Huawei, HiSilicon	CR Rel-16	37.340	16.9.0	0323	-	F	LTE_NR_DC_CA_enh-Core

[2]	R2-2205924	Correction for SCell activation	Huawei, HiSilicon	CR Rel-17	37.340	17.0.0	0324	-	F	LTE_NR_DC_CA_enh-Core

[3]	R2-2206110	Rapporteur Clean-up	ZTE Corporation (Rapporteur), Sanechips, Ericsson	CR	Rel-16	37.340	16.9.0	0325	1	F	TEI16, LTE_NR_DC_CA_enh-Core, NR_IAB-Core

[4]	R2-2206111	Rapporteur Clean-up	ZTE Corporation (Rapporteur), Sanechips, Ericsson	CR	Rel-17	37.340	17.0.0	0326	1	F	TEI16, LTE_NR_DC_CA_enh-Core, NR_IAB-Core

[5]	R2-2205978	Support of 1Tx-2Tx UL Tx switching for EN-DC	Huawei, HiSilicon, China Telecom	CR	Rel-16	37.340	16.9.0	0327	-	F	NR_RF_FR1-Core

[6]	R2-2205979	Support of 1Tx-2Tx UL Tx switching for EN-DC	Huawei, HiSilicon, China Telecom	CR	Rel-17	37.340	17.0.0	0328	-	A	NR_RF_FR1-Core

[7]	R2-2205990	Support of UL Tx switching for inter-band UL CA and SUL	Huawei, HiSilicon, China Telecom	CR	Rel-16	38.300	16.8.0	0470	-	F	NR_RF_FR1-Core