- 3GPP TSG RAN WG2 Meeting #118e      	                         R2-220xxxx
E-meeting, 9th – 20th May 2022

Agenda item:	5.1.2

Source:	ZTE Corporation (Rapporteur)

Title:	Report of [AT118-e][013][NR1516] Stage-2 (ZTE)

Document for:	Discussion and Decision

# Introduction

This document summarizes the following offline discussion.

- [AT118-e][013][NR1516] Stage-2 (ZTE)

Scope: Treat R2-2205923, R2-2205924, R2-2206110, R2-2206111, R2-2205978, R2-2205979, R2-2205990
Ph1 Determine agreeable parts, Ph2 for agreeable parts agree CRs (offline agreement, CB online only if necessary).

Intended outcome: Report, Agreed CRs

Deadline: Schedule 1

# Contact information

| Company | Name and email address |
| --- | --- |
| ZTE Corporation (rapporteur) | Sergio Parolari (sergio.parolari@zte.com.cn) |
| Nokia | amaanat.ali@nokia.com |
| Qualcomm Incorporated | mkitazoe@qti.qualcomm.com |
| Samsung | Jaehyuk JANG (jack.jang@samsung.com) |
| vivo | wenjuan.pu@vivo.com |
| LG Electronics | seungjune.yi@lge.com |

# Discussion

## SCell activation

Question 1: Do companies agree with the Rel-16/17 37.340 CRs in [1] and [2] on Correction for SCell activation? (if there are concerns with (parts of) the CR, please justify)

| Company | Yes or No | Comments |
| --- | --- | --- |
| Nokia | No | We understand the intent of the existing text was only to set the context of how the serving cells on the MCG and the SCG can be activated using the MAC CE. So, this was to disallow cross CG activation. Hence there is no ambiguity, and this is not essential to correct. |
| Qualcomm Incorporated | Yes |  |
| Samsung | Yes | Perhaps Nokia is correct on the context, but the current text seems misleading considering the latest status, so it can be improved. |
| vivo | Yes |  |
| LG | No | The point in the legacy text is to disallow cross CG activation/deactivation by MAC CE, while the point in the Huawei CR is that SCell activation/deactivation can be done not only by MAC CE but also by RRC message. Removing “only” from the legacy text may be misleading that cross CG activation/deactivation by MAC CE can be allowed. To resolve all the concerns, we propose followings: In MR-DC, the UE is configured with two MAC entities: one MAC entity for the MCG and one MAC entity for the SCG. The serving cells other than the PCell can be activated/deactivated by RRC or MAC Control Element. For activation/deactivation by MAC Control Element, the serving cells of the MCG other than the PCell can only be activated/deactivated by the MAC Control Element received on MCG, and the serving cells of the SCG other than PSCell can only be activated/ deactivated by the MAC Control Element received on SCG. The MAC entity applies the bitmap for the associated cells of either MCG or SCG. PSCell in SCG is always activated like the PCell (i.e. deactivation timer is not applied to PSCell). With the exception of PUCCH SCell, one deactivation timer is configured per SCell by RRC. |

## Clean-up CRs

Question 2: Do companies agree with the Rel-16/17 37.340 CRs in [3] and [4] on Rapporteur Clean-up? (if there are concerns with (parts of) the CR, please justify)

| Company | Yes or No | Comments |
| --- | --- | --- |
| Nokia | Yes | We understand the changes are editorial corrections. |
| Qualcomm Incorporated | Yes |  |
| Samsung | Yes |  |
| vivo | Yes |  |
| LG | Yes |  |

## UL Tx switching

Question 3: Do companies agree with the Rel-16/17 37.340 CRs in [5] and [6] on Support of 1Tx-2Tx UL Tx switching for EN-DC? (if there are concerns with (parts of) the CR, please justify)

| Company | Yes or No | Comments |
| --- | --- | --- |
| Nokia | Netural --> No | Honestly some description in RAN2 Stage 2 is desirable for this feature but the place where this is made is incorrect and does not belong there. However, one may argue there are many other RAN1 features which don't have a clear RAN2 stage 2 description and maybe one can start collecting those all but these are also found from RAN1 specifications. |
| Qualcomm Incorporated | Yes |  |
| Samsung | Maybe no | We share the view with Nokia in general, and we are not sure whether this addition is made in the right place. In addition, the coversheet anyway needs to be improved (i.e., Impact Analysis is missing, even if it is not critical for this change). |
| vivo | Yes | Fine to add some stage 2 descriptions in 37.340. |
| LG | No | We think 38.300 is proper place to capture UL Tx switching |

Question 4: Do companies agree with the 38.300 CR in [7] on Support of UL Tx switching for inter-band UL CA and SUL? (if there are concerns with (parts of) the CR, please justify)

| Company | Yes or No | Comments |
| --- | --- | --- |
| Nokia | Netural --> No | Honestly some description in RAN2 Stage 2 is desirable for this feature but the place where this is made is incorrect and does not belong there. However, one may argue there are many other RAN1 features which don't have a clear RAN2 stage 2 description and maybe one can start collecting those all but these are also found from RAN1 specifications. |
| Qualcomm Incorporated | Yes |  |
| Samsung | Maybe no | We share the view with Nokia in general, and we are not sure whether this addition is made in the right place. In addition, the coversheet anyway needs to be improved (i.e., Impact Analysis is missing, even if it is not critical for this change). |
| vivo | Yes |  |
| LG | Maybe Yes | We think the intention of the CR is to add the Rel-17 UL Tx switching text to Rel-16. |

# Conclusion

To be provided.

# References

[1]	R2-2205923	Correction for SCell activation	Huawei, HiSilicon	CR Rel-16	37.340	16.9.0	0323	-	F	LTE_NR_DC_CA_enh-Core

[2]	R2-2205924	Correction for SCell activation	Huawei, HiSilicon	CR Rel-17	37.340	17.0.0	0324	-	F	LTE_NR_DC_CA_enh-Core

[3]	R2-2206110	Rapporteur Clean-up	ZTE Corporation (Rapporteur), Sanechips, Ericsson	CR	Rel-16	37.340	16.9.0	0325	1	F	TEI16, LTE_NR_DC_CA_enh-Core, NR_IAB-Core

[4]	R2-2206111	Rapporteur Clean-up	ZTE Corporation (Rapporteur), Sanechips, Ericsson	CR	Rel-17	37.340	17.0.0	0326	1	F	TEI16, LTE_NR_DC_CA_enh-Core, NR_IAB-Core

[5]	R2-2205978	Support of 1Tx-2Tx UL Tx switching for EN-DC	Huawei, HiSilicon, China Telecom	CR	Rel-16	37.340	16.9.0	0327	-	F	NR_RF_FR1-Core

[6]	R2-2205979	Support of 1Tx-2Tx UL Tx switching for EN-DC	Huawei, HiSilicon, China Telecom	CR	Rel-17	37.340	17.0.0	0328	-	A	NR_RF_FR1-Core

[7]	R2-2205990	Support of UL Tx switching for inter-band UL CA and SUL	Huawei, HiSilicon, China Telecom	CR	Rel-16	38.300	16.8.0	0470	-	F	NR_RF_FR1-Core