3GPP TSG-RAN WG2 #118-e	R2-220xxxx

E-meeting, May 2022

Agenda Item:	x.x.x.x

Source:	OPPO

Title:	Summary of [AT118-e][044][NR17] Dual PA (OPPO)

Document for:	Discussion, Decision

# Introduction

This is for the discussion of CR for SRAP

- [AT118-e][044][NR17] Dual PA (OPPO)

Scope: Treat R2-2204501, R2-2204629, R2-2204630, R2-2204631, R2-2205380, R2-2205381, R2-2205382, R2-2205383, R2-2205384, R2-2205516, R2-2205514, R2-2205515

Ph1 Determine agreeable parts, Ph2 agree CRs

Intended outcome: Report, Agreed CRs

Deadline: Schedule 1

# Contact from companies

| Company | Email |
| --- | --- |
| Huawei, HiSilicon | zhaoyang@huawei.com |
| Intel Corporation | seau.s.lim@intel.com |
| Apple | naveen.palle@apple.com |
| Samsung | sy0123.jung@samsung.com |
| Qualcomm Incorporated | mkitazoe@qti.qualcomm.com |
| Nokia, Nokia Shanghai Bell | tero.henttonen@nokia.com |
| CATT | zhangxiangdong@catt.cn |
| MediaTek | morton.lin@mediatek.com |

# Phase-1 Discussion

The requirement by RAN4 is as follows:

Therefore, RAN4 would like to respectfully ask RAN2 to extend the meaning of dualPA-Architecture capability in TS38.306 from Rel-16 if there is no NBC issue. The proposed changes are as below for consideration:

Firstly, for the applicability to R16, R2 need to discuss and conclude on whether it is NBC change.

- In 5516 (Huawei), it is claimed this is NBC since “the current spec allows the UE supporting dualPA-Architecture only report 1 DC locations and thus the change proposed by RAN4 seems NBC”;

- In 4629 (OPPO), it is claimed that this is not NBC since “Essentially, the question is whether there exists a UE implementation of dual-PA + single-LO. Our understanding is no, and R4 preference on reusing the dual-PA capability for dual-LO capability reporting justified that understanding at least from R4 perspective. I.e., if R2 can confirm this understanding, then there would be no inter-operability issue for R16.”

- In 5380 (Nokia), it is proposed to implement the capability since R16 + add new per-FR capability on top of it since “The only possible inter-operability concerns with the RAN4 request are with legacy UEs in updated networks, but even those can be avoided by proper network implementation.”

Q1: Is the reporting of single/dual-LO capability via dualPA-Architecture in R16 NBC or not?

| Company | NBC / BC | Comments |
| --- | --- | --- |
| OPPO | BC | As analyzed in 5380, essentially the Q is if a R16 UE report dualPA yet is single-LO, whether there is a problem for updated network? Firstly, based on internal check, we understand there is no such UE implementation (dualPA yet single-LO, or singlePA yet dual-LO). Secondly, when a UE reporting dualPA/dual-LO, it is not to mandate UE to report two DC-location, i.e., even if the CR starts from R16, we can still follow the R2 agreement that “It is left to UE implementation whether a UE supporting dualPA-Architecture for a BC always reports two DC locations for the BC.” By the two, we understand this is not NBC. |
| Huawei, HiSilicon | TBD | We understand from last RAN2 meeting whether dual PA always reports two DC locations is up to the UE implementation, which allows support of dual PA only indicates one DC location. With this change, the field can only indicate the case that the UE supports dual PA and dual LO, it is unclear whether this means the UE needs to always report two DC locations? This is the main reason that we feel this could be NBC. So better to confirm which is the common understanding: 1) with the changes proposed by RAN4, there is no case that the UE indicating support of dual PA does not report the second DC location, in this case RAN2 previous agreement should be reverted; or 2) with the changes proposed by RAN4, it is still allowed to only report one DC location. |
| Intel | BC | We agree with Nokia’s analysis that there is slight NBC issue but this can be resolved through the DC location reporting. However, we are fine to go with the majority |
| Apple | Technically it is NBC, but we can go with majority to handle this (similar to what Nokia proposed) |  |
| Samsung | See comments | As Huawei commented, the suggested change in RAN4 seems not clear whether the UE supporting dualPA-Architecture is allowed to report only one DC location as it only says that "the field indicates the support of dual PA and dual LO frequencies…". If not allowed, then it actually deviates from previous RAN2 agreement so that it seems NBC from UE side. |
| Qualcomm Incorporated | See comment | We do not see why the RAN4 proposal mandates the dual PA UE to report the second DC location. Agree with Nokia it can be sorted out by network implementation. |
| Nokia, Nokia Shanghai Bell | BC (with caveat) | As we explained in our contribution, the only problem that could occur is when UE would support two DC locations but not indicate dual PA, and network doesn't query two DC locations. This could lead to demodulation performance loss due to unknown DC carrier. Assuming all UEs supporting dual PA supprot also the >=2 DC location reporting, this can be avoided by network requesting the DC locations from the UE. |
| Ericsson | BC | Our understanding is that for UL intra-band CA, if UE supports dual-PA architecture, then it means that the UE supports dual LOs as RAN4 LS suggests. We think this is always true, and so this is backwards compatible per RAN4 LS. What is not clear is the relation between the number of LOs and the number of DC location reporting. We think this should be clarified. |
| ZTE | BC | We agree with OPPO’s analysis |
| CATT | BC | We think the RAN4 version has no restriction on the two DC location reporting. |
| MediaTek | Could be NBC | Such change it anyhow differentiates the definition(requirement) of dual PAs usage between the case CA-ParametersNR and MRDC-Parameters and it could be a NBC change for those who adopted similar dual PA implementation for the target UL CA part in MRDC and NR CA scenarios. |

Secondly, one issue as pointed out by 5380 (Nokia) is whether this R4-required change applicable to the dualPA-Architecture in MRDC-Parameters as well (“it should be noted that the RAN4 description of the UE capability is based on the CA-ParametersNR version of the capability, but the capability also exists for MRDC-Parameters as shown below:”).

Q2: Is the change applicable to dualPA-Architecture in MRDC-Parameters as well?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | No | For the IE in MRDC-Parameters, it is clarified that If this capability is included in an "Intra-band (NG)EN-DC/NE-DC combination supporting both UL and DL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component", this capability applies to the intra-band (NG)EN-DC/NE-DC BC part. After internal check, the single/dual-LO discussion in R4 did not touch the MR-DC part, but just the intra-band CA part. |
| Huawei, HiSilicon | No | Agree with OPPO. |
| Intel |  | Maybe there is a need to check with RAN4 |
| Apple |  | We also agree it’s better to bring this to RAN4 attention. |
| Samsung |  | Tend to agree with OPPO but there seems no harm to check with RAN4. |
| Qualcomm Incorporated | No | OPPO’s explanation is reasonable. |
| Nokia, Nokia Shanghai Bell |  | RAN4 didn't discuss UE capability structure but the issue itself: If this doesn't apply to the MR-DC part, then does it mean that UE might support two LO for intra-band UL CA without DC, but NOT when the same intra-band UL CA is part of MR-DC? If there is doubt, we can also check with RAN4 but it would be better to anyway discuss the RAN2 parts before sending any LSs. |
| ZTE | No |  |
| CATT | No |  |
| MediaTek |  | It was from RAN4 agreement to reuse dualPA-Architecture signaling to help distinguishing MPR requirement for FR1 standalone case so it’s reasonable to have RAN4 views on RF design aspect whether such a dualLO bundling is also applicable for possible UL CA case in intra-band ENDC or not, even though we see the RAN4 majority view that there is no such dualPA+singleLO implementation holds only for NR intra-band UL CA case so far. |

Thirdly, 5380 (Nokia) propose to introduce additional per-FR capability.

Proposal 2: Create a new (optional) UE capability for FR1 dual PA support for CA and MR-DC according to the following definition: " For band combinations with single-band FR1 UL CA, this field indicates the support of dual PAs and dual LO frequencies. If absent in such band combinations, the UE supports single PA and single LO frequency for all the FR1 intra-band ULs in the band combination. For other band combinations, this field is not applicable. UE indicating this capability shall also indicate the dualPA-Architecture for this band combination. "

Proposal 3: Create a new (optional) UE capability for FR2 dual PA support for CA and MR-DC according to the following definition: " For band combinations with single-band FR2 UL CA, this field indicates the support of dual PAs and dual LO frequencies. If absent in such band combinations, the UE supports single PA and single LO frequency for all the FR2 intra-band ULs in the band combination. For other band combinations, this field is not applicable. UE indicating this capability shall also indicate the dualPA-Architecture for this band combination. "

Q3: Do you agree to introduce the additional per-BC-per-FR capability for the single/dual-LO capability reporting?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | No | Since this IE is a per-BC capability and is limited to intra-band CA case For band combinations with single-band with UL CA, this field indicates the support of dual PA… Seems there is no need to further introduce a per-BC-per-FR capability? |
| Huawei, HiSilicon | No | Same understanding as OPPO that the existing capability is already per BC. |
| Intel | No | If it is agreed that the slight interoperability issue can be resolved by network implementation, we do not see to introduce these duplicated capabilities. |
| Apple | Not necessary if majority agree. |  |
| Samsung | No |  |
| Qualcomm Incorporated | No | This is not correct because the dual PA capability is applicable to CA with a single intra-band CA component. |
| Nokia, Nokia Shanghai Bell | See comments | If companies think this is not an issue, we are fine not to introduce these but let's be clear on the consequences: - UE doesn't typically report single-band UL CA band combinations. Those are part of parent band combinations with more bands. So the dual PA capability can be reported as NR CA part of MR-DC band combination. - For FR1-FR2 (CA) band combination, UE may support intra-band CA for both FR1 and FR2. If UE indicates dual PA for that case, does it have to support dual LO for both FR1 and FR2 parts? We understand this would be the case, so this prevents UE from using the capability if it only supports single LO for e.g. FR2. |
| ZTE | See comments | There hasn’t been any CA BC with both FR1 intra-band CA and FR2 intra-band CA. So currently no need to distinguish FR1 and FR2. If for the FBC considering, we think it’s better to extend this capability to be per band per BC |
| MediaTek | No | According to coversheet, such extra signalling could introduce early implementation efforts to both legacy UE and NW even no inter-operability issue is foreseen. No strong view on its necessity. |

Besides, some misc-issues as follows.

5380 (Nokia) raised a question on whether it should be Cat-C CR or Cat-F CR.

Q4-1: Whether the CR should be Cat-C or Cat-F?

| Company | Cat-C/F | Comments |
| --- | --- | --- |
| Intel | Maybe Cat-C | Since it is clarifying the feature further. But no strong view. |
| Nokia, Nokia Shanghai Bell | No strong opinion | Technically this is perhaps Cat-C but since this seems to be the original intention in RAN4 anyway, we are fine with Cat-F as well. |

5514 (Huawei) propose to further clarify the usage for singlePA-TxDirectCurrnet

singlePA-TxDirectCurrent The uplink Tx Direct Current location for the UE which support single PA for this uplink carrier aggregation. For the UEs which support dual PA for this uplink carrier aggregation, this field is for reporting the uplink Tx Direct Current location of the first PA and the uplink Tx Direct Current location of the second PA is reported in secondPA-TxDirectCurrent if any..

Q4-2: Do you agree with the change in 5514?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | This is relevant to Q1, regardless whether the common understanding is 1) or 2), we see it useful to clarify in RAN2 spec to avoid any misunderstanding in the future. |
| Intel | No | We think it is not essential. It is already clear looking at the field description for the secondPA-TxDirectCurrent: The uplink Tx Direct Current location used by the UE with the second PA for the UEs which support dual PA for this uplink carrier aggregation. This field shall be absent for the UplinkTxDirectCurrentTwoCarrier entity where deactivatedCarrier of carrierOneInfo or carrierTwoInfo is set to deactivated. |
| Apple | This can be decided based on Q1 outcome. |  |
| Samsung |  | Depends on Q1 outcome. |
| Qualcomm Incorporated | No | Agree with Intel. |
| Nokia, Nokia Shanghai Bell | No | Agree with Intel this is not essential. Also, what is "second PA"? Does it refer to the PA that has DC location with smaller frequency? Or something else? In general either UE reports 1 or 2 DC locations, and then network uses that information. It's sufficient that the UE is not always required to report 2 DC locations and that is handled by the ASN.1 encoding. |
| Ericsson | No | We understand that the only change by HW is the addition of “if any”. But as indicated above, it has to be clarified that the number of LOs and the number of DC location reporting. |
| ZTE | No |  |
| CATT | No |  |
| MediaTek |  | Agree with other views to wait for Q1 outcome. |

# Phase-1 Summary

So far there is not much support on Q3 and Q4-2, so maybe no need to bring them into Phase-2. Thus we may focus on Q1 (whether R16 CR is feasible) and Q2 (whether it is applicable to dualPA-Architecture in MRDC-Parameters).

For Q2, there seems some preference on checking R4 using LSout. Plus one question from Nokia that “If this doesn't apply to the MR-DC part, then does it mean that UE might support two LO for intra-band UL CA without DC, but NOT when the same intra-band UL CA is part of MR-DC?” My understanding is for the BC in this case, it still has different IEs to report dualPA for the intra-band CA for the NR part (rf-ParametersMRDC /supportedBandCombinationList-v1540/supportedBandCombinationList-v1540/ca-ParametersNR-v1540), and intra-band CA for the MR-DC part (rf-ParametersMRDC/supportedBandCombinationList/mrdc-Parameters) respectively. So maybe the LSout can be purely on whether the intra-band CA for the MR-DC part need this change as well or not to clarify the concern from some companies.

For Q1, it seems the key issue is whether the R4-required change mandate the UE reporting dualPA to report 2 DC-location. From OPPO perspective, my understanding is No, so this does not go against 117 agreement. Yet due to the preference of LSout in Q2. Maybe no harm to have something in LS, e.g., by copy-paste R2#117 agreement, and ask R4 to confirm whether it is compatible with R4-required change or not.

- Send LS to R4 to ask 1) whether the R4-required change is compatible with R2#117 agreement on DC location reporting, and 2) whether the R4-required change is applicable to dualPA-Architecture in MRDC-Parameters.

# Conclusion

We have the following proposals:

Proposal 1	Send LS to R4 to ask 1) whether the R4-required change is compatible with R2#117 agreement on DC location reporting, and 2) whether the R4-required change is applicable to dualPA-Architecture in MRDC-Parameters.

# Reference

- R2-2204501	LS on clarification of dualPA-Architecture capability (R4-2206503; contact: OPPO)	RAN4	LS in	Rel-17	NR_RF_FR1_enh	To:RAN2

- R2-2204629	Discussion on R4 LS on dual-PA architecture clarification	OPPO	discussion	Rel-17	NR_RF_FR1_enh

- R2-2204630	Extension of dual-PA architecture capability	OPPO	CR	Rel-17	38.306	17.0.0	0700	-	A	NR_RF_FR1_enh

- R2-2204631	Extension of dual-PA architecture capability	OPPO	CR	Rel-16	38.306	16.8.0	0701	-	F	NR_RF_FR1_enh

- R2-2205380	Clarification to dualPA-Architecture	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_RF_FR1_enh-Core

- R2-2205381	Clarification to dualPA-Architecture	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.306	16.8.0	0712	-	F	NR_RF_FR1_enh-Core

- R2-2205382	Clarification to dualPA-Architecture	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.306	17.0.0	0713	-	A	NR_RF_FR1_enh-Core

- R2-2205383	Clarification to dualPA-Architecture	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.8.0	3074	-	F	NR_RF_FR1_enh-Core

- R2-2205384	Clarification to dualPA-Architecture	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.331	17.0.0	3075	-	A	NR_RF_FR1_enh-Core

- R2-2205516	Discussion on dualPA-Architecture capability	Huawei, HiSilicon	discussion	Rel-16	NR_RF_FR1-Core

- R2-2205514	Clarification on DC location reporting for dual PA UE	Huawei, HiSilicon	CR	Rel-16	38.331	16.8.0	3095	-	F	NR_RF_FR1-Core

- R2-2205515	Clarification on DC location reporting for dual PA UE	Huawei, HiSilicon	CR	Rel-17	38.331	17.0.0	3096	-	A	NR_RF_FR1-Core