3GPP TSG-RAN2 Meeting #118-e	R2-220XXXX

Online, 9 – 20 May 2022

Agenda Item:	7.2.1.1

Source:		Huawei, HiSilicon

Title:		RRC CR review

Document for:	Discussion and decision

# Introduction

This document is the report from the following offline discussion:

- [AT118-e][056][IOT NTN] RRC CR 36331 (Huawei)

Scope: For usage by the IoT NTN RRC CR Rapporteur.

Intended outcome: Set by Rapporteur (agreed CR in the end)

Deadline: Set by Rapporteur

It is expected that this email discussion will span over the full meeting and carry on afterwards to capture the agreements in other email discussions and finalise the CR.

It is proposed to process via different rounds as follows:

Round 1: Deadline for comments: W1 Friday May 12th 1200 UTC

Scope:

- to check the changes included in the rapporteur CR in [5]

- to progress the RILs in [4] not concluded and not discussed in other offline discussions.

Round 2: Deadline for settling the corresponding CR details: W2 Wednesday May 12th 1200 UTC

Scope:

- to check the CR update triggered by the outcome of Round 1.

Round 3: Details to be decided later

# Contact

Delegates are encouraged to provide their contact information in the following table:

| Company | Name | Email |
| --- | --- | --- |
| Huawei, HiSilicon | Odile Rollinger | odile.rollinger@huawei.com |

# Discussion

## Miscellaneous RILs

### RILs H001, H002, H003, H004, H005

In RILs H001, H002, H003, H004 and H005, it is proposed to clarify that the requirements of acquiring SIB31 only apply to NTN capable UEs:

Section 5.2.2.3:

The UE shall:

1>	ensure having a valid version, as defined below, of (at least) the following system information, also referred to as the 'required' system information:

2>	if in RRC_IDLE:

3>	if the UE is a NB-IoT UE:

4>	the MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB and SystemInformationBlockType1-NB as well as SystemInformationBlockType2-NB through SystemInformationBlockType5-NB, SystemInformationBlockType22-NB;

3>	else:

4>	the MasterInformationBlock and SystemInformationBlockType1 (or SystemInformationBlockType1-BR depending on whether the UE is a BL UE or the UE in CE) as well as SystemInformationBlockType2 through SystemInformationBlockType8 and SystemInformationBlockType24 (depending on support of the concerned RATs), SystemInformationBlockType17 (depending on support of RAN-assisted WLAN interworking when the UE is connected to EPC), SystemInformationBlockType25 (depending on support of E-UTRA/5GC), SystemInformationBlockType29 (only for BL UE or the UE in CE depending on support of resource reservation), SystemInformationBlockType21, SystemInformationBlockType26 (if UE is capable of V2X sidelink communication and is configured by upper layers to receive or transmit V2X sidelink communication), and SystemInformationBlockType28 (if UE is capable of NR sidelink communication and is configured by upper layers to receive or transmit NR sidelink communication);

3>	if initiating a RRC connection establishment/resume procedure; and

3>	the UE is a BL UE or a UE in CE or a NB-IoT UE:

4>	SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT), if scheduled;

…

2>	if in RRC_CONNECTED and T311 is running; and

2>	the UE is a BL UE or the UE is in CE or the UE is a NB-IoT UE;

3>	the MasterInformationBlock (or MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT), SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT) and SystemInformationBlockType2 (or SystemInformationBlockType2-NB in NB-IoT), SystemInformationBlockType25 (only for BL UE or the UE in CE depending on support of E-UTRA/5GC), SystemInformationBlockType29 (only for BL UE or the UE in CE depending on support of resource reservation), SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) if scheduled, and for NB-IoT SystemInformationBlockType22-NB;

Section 5.2.2.4:

1>	if the UE is a BL UE or a UE in CE or a NB-IoT UE:

2>	if schedulingInfoList indicates that SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) is present:

3>	immediately before establishing, resuming or re-establishing an RRC connection; or

3>	if in RRC_CONNECTED and T317 is not running:

4>	acquire SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT);

Section 5.2.2.5:

2>	else:

3>	if the UE is unable to acquire the SystemInformationBlockType2 (or SystemInformationBlockType2-NB in NB-IoT) and for NB-IoT, SystemInformationBlockType22-NB if scheduled; or

3>	if SystemInformationBlockType25 is broadcast and if the UE is connected to 5GC and is unable to acquire the SystemInformationBlockType25; or

3>	if SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) is broadcast and if the UE is unable to acquire the SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT):

4>	treat the cell as barred in accordance with TS 36.304 [4];

During email discussion [Pre1198-e][012], there were both support and statement that it was not needed and a number of general questions were raised:

- whether SIB31 was only scheduled in a NTN cell and whether to clarify

- whether to clarify that the requirement to acquire SIB31 only applies in a NTN cell.

- whether NTN capable UE was only BL UE or a UE in CE or a NB-IoT UE and if it would be better to refer to NTN capable UE instead of ‘BL UE or a UE in CE or a NB-IoT UE’ everywhere

Discussion Point 1.1: Do you agree that SIB31 is only scheduled in a NTN cell. If yes, should that be specified and where

| Company | Agree (Y/N) | Comments |
| --- | --- | --- |

In the RILs, the proponent argues that a TN only UE (connected to TN cell) will not understand the requirement in the specification as it does not understand SIB31 thus a clarification about the applicability is needed. Possible clarification being either that the requirement applies to ‘NTN capable UEs’ or to ‘UE connected to NTN ’.

Discussion Point 1.2: Do you agree that a clarification of the applicability is needed. If yes, how best to clarify?

| Company | Y/N | Comments |
| --- | --- | --- |

Discussion Point 1.3: Do you agree to replace ‘BL UE or a UE in CE or a NB-IoT UE’ in the NTN specific statements with ‘NTN capable UE’ or ‘’UEs connected to NTN’.

| Company | Y/N | Comments |
| --- | --- | --- |

### RIL Q601

RIL Q601 indicates that t-Service/t-ServiceStart in SIB3 and SIB32 are defined as TimeUTC-r17 (INTEGER (0..549755813887)) which is different from timeInfo-r11 broadcast is SIB16. Does this mean other fields such as leapSeconds-r11 are irrelevant?

-- ASN1START

SystemInformationBlockType16-r11 ::=		SEQUENCE {

timeInfo-r11							SEQUENCE {

timeInfoUTC-r11						INTEGER (0..549755813887),

dayLightSavingTime-r11				BIT STRING (SIZE (2))		OPTIONAL,	-- Need OR

leapSeconds-r11						INTEGER (-127..128)			OPTIONAL,	-- Need OR

localTimeOffset-r11					INTEGER (-63..64)			OPTIONAL	-- Need OR

}																	OPTIONAL,	-- Need OR

lateNonCriticalExtension			OCTET STRING				OPTIONAL,

...,

[[	timeReferenceInfo-r15				TimeReferenceInfo-r15	OPTIONAL	-- Need OR

]]

}

-- ASN1STOP

Discussion Point 2: Do you think other fields. e.g leapSeconds-r11, should be provided in the signalling of t-service in SIB3 and SIB32 ? which ones

| Company | Y/N | Comments |
| --- | --- | --- |

### RIL C501

RIL501 proposes not to support implicit signalling of epochTime in SIB31 in broadcast signalling.

Discussion Point 3: Do you agree with removing the option of implicit signalling of epochTime in SIB31 in broadcast signalling ?

| Company | Y/N | Comments |
| --- | --- | --- |

### RIL X601 (kept open during offline session)

RIL X601 proposes to move the contents of section 3.3.2.2 T317 expiry under 5.2.2 system information

Discussion Point 4: Do you agree to move T317 expiry under 5.2.2 system information. If yes, where exactly ?

| Company | Agree (Y/N) | Comments |
| --- | --- | --- |

## CR review

Companies are invited to provide comments either directly in the CR, using balloons, or for more complex comments or comments in sections not present in the CR in the tables below

### Sections 3.x Definitions and Abbreviations

Discussion Point 5: Companies to indicate any comments on sections 3.1 and 3.2.

| Company | section | Comments |
| --- | --- | --- |

### Sections 5.2.x System information

Discussion Point 6: Companies to indicate any comments on sections 5.2.x.

| Company | section | Comments |
| --- | --- | --- |

    - Sections 5.3.3.x RRC Connection establishment

Discussion Point 7: Companies to indicate any comments on sections 5.3.3.x

| Company | section | Comments |
| --- | --- | --- |

    - Sections 6.2, 6.3 and 6.4 LTE ASN.1

Discussion Point 8: Companies to indicate any comments on sections 6.2, 6.3 and 6.4.

| Company | section | Comments |
| --- | --- | --- |

    - Sections 6.7.x NB-IOT ASN.1

Discussion Point 9: Companies to indicate any comments on sections 6.7.x

| Company | section | Comments |
| --- | --- | --- |

    - Other

Discussion Point 10: Companies to indicate any comments on sections not included in the CR

| Company | section | Comments |
| --- | --- | --- |

# Conclusion

To be completed later

# Reference

- 3GPP TS 36.331 E-UTRAN Radio Resource Control (RRC) (Release 17)

- R1-2202540 Consolidated higher layers parameter list for Rel-17 LTE, RAN1

- R2-2205326 Report of  [Pre118-e][012][IoT-NTN] 36331 CR and rapporteur resolutions (Huawei), Huawei, HiSilicon, RAN2#118-e, May 2022

- R2-2206089 IOT NTN ASN1 RIL List, Huawei, RAN2#118-e, April 2022

- R2-2205325 Corrections to support of Non-Terrestrial Network in NB-IoT and eMTC, Huawei, HiSilicon, RAN2#118-e, May 2022