3GPP TSG-RAN WG2 #118-e	R2-22xxxxx

E-meeting, 09 – 20 May 2022

Agenda item:	6.4.3

Source: 	Samsung

Title: 	Summary of discussion [AT118-e][065][eIAB] MAC (Samsung)

Document for:	Discussion and decision

# Introduction

This contribution summarizes the following discussion:

- [AT118-e][065][eIAB] MAC (Samsung)

Scope: 1. Address the remaining TS issues from tdocs submitted under AI 6.4 (and below), except those issues addressed in specific discussion. Review collect comments identify agreement points, points for online CB etc. 2. Progress the CR, merge all TS impacts into a single CR.

Intended outcome: Report, CR

Deadline: 1 for CB W2 Wed, 2 CR agreement is expected in Post meeting discussion

Please note that the scope of this discussion does not include the issue of introduction of outstanding MAC CEs and the related RRC/MAC CE split discussion. The present discussion looks at submissions to the meeting proposing changes to the existing spec/existing MAC CEs.

In Section 2, companies’ views are collected on the various issues raised in submissions to the meeting. Section 3 contains rapporteur’s proposals based on input in Section 2, and companies’ comments on these proposals. Section 4 then lists final version of proposals, to be reviewed by RAN2 and used in any modification of draft MAC CR (which is made available in parallel with this discussion document).

# Key issues

The following Table 1 contains a list of the documents that the rapporteur believes are relevant to the MAC spec, the summary of proposed changes therein, and rapporteur’s reply/proposed action. Companies are invited to comment by providing their input into Table 1.

| Tdoc no. | Summary of change(s) | Rapporteur’s proposed way forward | Companies’ view [Company name]: comments |
| --- | --- | --- | --- |
| R2-2205268 (rapporteur submission) | 1. change logicalChannelGroup-IABExt to logicalChannelGroup-IAB-Ext (clause 5.4.5), to align with RRC spec 2. change (3072, -3071, …, 1023) to (-3072, -3071, …, 1023), to correct the error made when previous CR was implemented (clause 6.1.3.38) NB – other changes from this submission are to do with introduction of new MAC CEs and are out of scope of present discussion | Implement both changes | [LGE] agree with rapporteur’s change. [Ericsson]: Ok to changes 1/2 indicated by the rapporteur [Apple] Fine with both changes. [ZTE] agree with the two changes. |
| R2-2204901 | 1) If the Extended Long Truncated BSR MAC CE cannot be used for padding BSR with multiple LCGs having data for transmission, proposal is to allow one MAC PDU to carry multiple Extended Short Truncated BSR MAC CEs 2) Due to perceived clash between 5.4.5 and 6.1.3.1, consider whether to remove the description to include the BS fields for Long/Extended Truncated BSR in ascending order of the LCG IDs in Clause 6.1.3.1. | Rapporteur agrees that 1) does identify a potential wastage of padding bits. However this was already discussed at length in the previous meeting and agreement was to live with this wastage. Also, a MAC PDU can only carry one BSR MAC CE. Therefore rapporteur proposes not to pursue 1). Regarding 2), rapporteur feels there is no inconsistency. In 5.4.5, it is stated that when reporting Extended Long Truncated BSR, LCGs should following a decreasing order of the highest priority logical channel in each of these LCG(s). In rapporteur’s understanding, this means that the highest priority LCG is selected for reporting first, followed by second highest priority LCG, and so on until we run out of padding. In 6.1.3.1, it is stated that for the Extended Long Truncated format, the Buffer Size fields are included in ascending order based on the LCGi. Rapporteur’s understanding is as follows: - Based on 5.4.5, we decide which LCGs to include - Based on 6.1.3.1, we decide how to put them together in a BSR MAC CE Therefore there is no apparent “contradiction” in rapporteur’s understanding, and rapporteur proposes not to pursue 2). | [LGE] agree with rapporteur and no changed is needed. In addition, for issue 1), considering the description in LCP section, i.e., “- the UE should maximise the transmission of data;”, if there is a remaining data in a logical channel, the normal UE should include a data into a MAC PDU as much as possible and should not generate that much padding bits for padding BSR as addressed in the R2-2204901. So, we think that wastage would be small and negligible. [Huawei]: Agree with rapporteur’s view on that 2) is not needed (This is legacy issue. And there is no issue in legacy). [Ericsson]: For 1) we agree with the issue, but RAN2 decided to do not address this problem. Further, P1 in R2-2204901 is not a minor change. For 2) we agree with Rapporteur. [Apple] We have some sympathy with 1) but this is an optimization, and it is going to complicate the procedure. We prefer not to adopt this proposal even though the current procedure is not ideal either. For 2) we agree with the rapporteur. [ZTE] Agree with rapporteur’s analysis. |
| R2-2205255 | 1) It is observed that in 5.4.7, for pre-emptive BSR, only generation of Pre-emptive BSR is captured and not the generation of the Extended version. 2) Comment is made that the function of Timing Delta MAC CE for Case-6 timing is not only for the IAB-DU Tx, but also for the IAB-MT Tx, and appropriate correction is proposed. 3) The description of the MAC CE in 6.1.3.39 is missing. | Regarding 1), rapporteur agrees this is a potential issue. However, for legacy BSR we also do not list all types of BSR formats that should be generated – we simply say generate the BSR MAC CE(s) as defined in clause 6.1.3.1. Rapporteur’s proposal is therefore to fix this by removing reference to Pre-emptive BSR MAC CE altogether, as it is clear that the procedure we refer to is the ‘Pre-emptive Bufer status reporting procedure’. Please see accompanying CR draft for rapporteur’s proposed change to resolve this issue. Regarding 2) and 3), rapporteur proposes to agree the original change. | [LGE] agree with Rapporteur’s analysis and change. [Huawei] On 1), prefer the original proposal. But fine with rapp update. [Ericsson]: On 1), agree with Rapporteur´s way forward. On 2) ok. On 3) there is no need to specify that the IAB-MT is a child IAB node: “An child IAB-MT can inform a parent node whether Case 6 timing is required for simultaneous operation by this MAC CE.” [Apple] On 1), we slightly prefer the proposal made by the rapporteur. For change 2) and 3) we are ok to agree it with the update proposed by Ericsson. [ZTE] For 1st change, prefer the rapporteur suggested change. For the 3rd change, agree the update from Ericsson. |
| R2-2205287 | 1) For the IAB node that is configured with logicalChannelGroup-IAB-Ext-r17, the IAB node should report the extended Short BSR, when the maximum configured LCG ID is less than 8. | Rapporteur agrees this is a potential issue which should be fixed. It is theoretically possible (although change to RRC spec would be needed) to use logicalChannelGroup-IAB-Ext-r17 (i.e. the extended formats, especially the Extended Short format) for when we have fewer than 8 LCGs as well, and this would give us better granularity for the Short BSR by using Extended Short BSR, which offers 8 bits instead of 5 for the buffer size. This can be especially useful given the volume of data on the backhaul. Rapporteur therefore proposes to agree the change, but also notes that this needs to be aligned with RRC discussions. | [LGE] agree with Rapporteur’s analysis and change. Considering that 5bits buffer size field in the Legacy Short BSR is designed only for one UE, not for backhaul traffic from many UEs, the IAB node should use an extended Short BSR if the IAB node support extended BSR format. We also think that RRC change would be simple, i.e., just change logicalChannelGroup-IAB-Ext-r17 to start from 0 and that’s it. [Huawei]: No strong view on this issue itself. It is discussed in RRC RIL. We suggest to wait, at least to align the conclusion from RRC and MAC. [Ericsson]: We do not see the need for this change. If more granularity is required for the short BSR, the network can configure the IAB node with the extended logical channel groups. If the network just configures the legacy LCGs 0-7, it means that this extra granularity is not required and the IAB node can just use the legacy BSR, without any need to enhance the way of reporting the BSR for the LCGs 0-7. Using the extended LCG configuration to enhance the legacy reporting of the LCGs 0-7 is strange from a technical perspective. [Apple] We agree with observation 3 and 4 in R2-2205287. However, the extended formats were introduced to facilitate a higher number of LCGs. If an IAB node operates with the same number of LCGs as a UE in legacy, then there might be no need to use the extended formats. Besides RAN2 agreed not to introduce new triggering conditions for BSR/Extended BSR. The logicalChannelGroup-IAB-Ext is used at multiple other places. If we adopt the change, then these places will be affected. Maybe another parameter would be needed then. More importantly, the proposed solution brings back the issue that Extended Long Truncated BSR could be used when there are less than 8 LCGs with data available (even though more are configured), which seems like a waste. We’d tend to not agree this change - but open to discuss more or consider alternative ways. [ZTE] We think this change is not necessary. If the data volume associated with one LCG is really large, donor CU may setup more BH RLC channels and LCGs and thus the available LCGs may be larger than 8. The issue can be solved up to donor CU implementation. |

In Table 2, companies are invited to provide any additional comments:

| Company | Comment |
| --- | --- |

# Proposals

…

# Conclusions

…