- 3GPP TSG RAN WG2 Meeting #118e      	                         R2-2206204
E-Conference, 9th – 20th May 2022

Agenda item:	6.12.2.1

Source:	ZTE Corporation

Title:	[AT118-e][105][RedCap] NCD-SSB aspects—Phase 2

WID/SID:	NR_redcap-Core – Release 17

Document for:	Discussion and Decision

# Introduction

This document summarizes the following offline discussion.

[Pre118-e][105][RedCap] NCD-SSB aspects (ZTE)

Initial scope: Continue the discussion on NCD-SSB aspects, based on R2-2206143, including a possible reply LS to R2-2204486 (RAN4 LS on NCD-SSB issues)

Initial intended outcome: Summary of the offline discussion with e.g.:

         Text/proposals for a possible reply LS to R2-2204486

         List of proposals for agreement (if any)

         List of proposals that require online discussions

         List of proposals that should not be pursued (if any)

Deadline (for companies' feedback): Tuesday 2022-05-10 0800 UTC

Deadline (for rapporteur's summary in R2-2206195): Tuesday 2022-05-10 1000 UTC

# Contact information

| Company | Name and email address |
| --- | --- |
| ZTE | LiuJing (liu.jing30@zte.com.cn) |
| MediaTek | Pradeep Jose (pradeep dot jose at mediatek dot com) |
| Qualcomm | Linhai He (linhaihe@qti.qualcomm.com) |
| Apple | Naveen Palle (naveen.palle@apple.com) |
| NEC | Hisashi Futaki (hisashi.futaki @ nec.com) |
| Samsung | Jaehyuk Jang (jack.jang@samsung.com) |
| InterDigital | Keiichi Kubota (keiichi.kubota@interdigital.com) |
| Huawei, HiSilicon | Yulong (shiyulong5@huawei.com) |
| vivo | Chenli5g@vivo.com |
| DENSO | Hideaki Takahashi (hideaki.takahashi.j6e@jp.denso.com) |
| Intel | Yi. Guo (yi.guo@intel.com) |
| OPPO | Haitao Li (lihaitao@oppo.com) |
| CATT | Xiangdong Zhang (zhangxiangdong@catt.cn) |
| Spreadtrum | Min Xu (Ellen.Xu@unisoc.com) |
| Sequans | Noam Cayron (noam.cayron@sequans.com) |
| Sharp | LIU Lei (lei.liu@cn.sharp-world.com) |
| Ericsson | Emre A. Yavuz (emre.yavuz@ericsson.com) |
| Vodafone | Alexey.kulakov1@vodafone.com |
| BT | Salva Diaz (salva.diazsendra@bt.com) |
| Xiaomi | Liyanhua1@xiaomi.com |

# Phase 1 Discussion

## Serving Cell measurements

For serving cell measurements, as indicated in AI summary [1], the discussion focus on following two questions:

- Q1: Whether the UE can be configured to perform serving cell measurements on CD-SSB when its active BWP is associated with NCD-SSB?

- Q2: How to configure the UE to perform serving cell measurements (i.e. signalling design)?

And following proposals are provided in AI summary.

Proposal 1: To discuss whether UE can be configured to perform serving cell measurements on CD-SSB when the UE’s active BWP is associated with NCD-SSB. Proposal 2.1: If answers “Yes” to P1, to select one of the following options: - Solution A-1: Reuse existing servingCellMO IE. - Solution A-2: Optionally configures a BWP-specific servingCellMO under BWP-DownlinkDedicated IE if BWP-DownlinkDedicated contains nonCellDefiningSSB-r17. If the field is present, the UE shall use this servingCellMO, otherwise, the UE uses legacy servingCellMO. Proposal 2.2: If answers “No” to P1, to select one of the following options: - Solution B-1: Add servingCellMO-List-NCD-r17 in ServingCellConfig, each entry associated to the one NCD-SSB MO, the UE selects corresponding servingCellMO upon BWP switching. - Solution B-2: Configures a BWP-specific servingCellMO under BWP-DownlinkDedicated IE, the field is configured in the BWP if BWP-DownlinkDedicated contains nonCellDefiningSSB-r17.

In this section, companies are invited to express your views to the two questions and above proposals:

Q1: In case a RedCap UE’s activate BWP contains only NCD-SSB, whether the UE can be configured to perform serving cell measurements on CD-SSB?

(E.g. when all neighbour cell measurements are performed on CD-SSB and gap is already configured)

| Company | Yes or No | Comments |
| --- | --- | --- |
| Vodafone | Yes | In case UE’s active BWP only contains NCD-SSB, serving cell measurement can be performed on either NCD-SSB or CD-SSB |
| ZTE | Yes | If not all neighbour cells have deployed NCD-SSB, the network will configure the UE to measure CD-SSB frequency for triggering handover, and gap will be provided because CD-SSB is outside UE’s activate BWP (if the UE does not report ”no-gap” for intra-frequency in NeedForGap). In this case, the network may expect the UE to also perform serving cell measurements on CD-SSB, similar to Rel-15 UEs. So the spec should allow such flexibility. |
| MediaTek | No | The reason for introducing the NCD-SSB concept was so that the UE can use the NCD-SSB for those purposes that the CD-SSB would normally be used for, when operating in a BWP that only includes the NCD-SSB. In this case, if we require the UE to monitor the CD-SSB instead of the NCD-SSB, then we defeat the purpose of the introduction of the NCD-SSB altogether! [Rapp-ZTE]: We would like to clarify the discussion here only relates to RRM measurements, it does not impact other physical operations, so for RLM/BFD, QCL...etc, the UE should still use NCD-SSB. This is similar to Rel-15 operation, e.g. when UE’s active BWP does not contain SSB, the UE uses other RS (i.e. CSI-RS) for RLM/BFD and QCL, but the UE can still perform RRM measurements on SSB with gap assisted. [MTK2] The point of introducing NCD-SSB was so that it can be used as an alternative to the CD-SSB. Now you’re suggesting that it’s a partial alternative, which can be used in some cases (e.g. RLM) and not in others (e.g. RRM). Why? What is there to gain with such a hybrid approach? As far as we can see, such a hybrid approach only adds specification and implementation confusion, with no clear benefit. |
| Qualcomm | See comment | In typical deployment scenarios, if Redcap UE’s dedicated BWP does contain a SSB (does not matter whether it is CD-SSB or NCD-SSB), then its serving cell measurements should be performed based on this particular SSB, to avoid measurement gaps. In some deployment scenarios, e.g. UE’s dedicated BWP contains NCD-SSB but no intra-frequency neighbor cells are configured with SSB on the same frequency as NCD-SSB in UE’s serving cell, it is understandable that network may want to configure serving cell measurements based on CD-SSB instead of NCD-SSB via measurement gaps. But we are not sure how likely this scenario is in actual deployment… We are fine to leave this issue to network configuration, as long as serving cell measurements in the both types of scenarios above can be efficiently performed. That is why we think the configuration of servingCellMO should be BWP specific. |
| Apple | Neutral | While we agree with MediaTek’s view on NCD-SSB standing in for CD-SSB, we also see that this is NW deployment option. We are ok to go with majority. |
| NEC | Yes | We expect there could be the case where some of neighbour cells do not support NCD-SSB, although this would not be the typical deployment. We also assume, as exemplified by ZTE, that from RRM measurement point of view, the scenario is similar to serving cell measurement on SSB, while the active BWP does not contain the SSB in Rel-15. |
| Samsung | Yes | Assuming a gap is configured for the measurement. |
| Interdigital | Yes | We share Qualcomm view that RedCap UE performs serving cell measurement based on the configured SSB and CD-SSB measurement makes sense for the case that the neighbouring cells aren’t configured with any SSB on the same frequency as the NCD-SSB of the current serving cell. Thus we should allow NW to configure CD-SSB to measure for the serving cell measurement. |
| Huawei, HiSilicon | Yes | Agree with Qualcomm that we should give the NW configuration flexibility. |
| Vivo | See comment | In our understanding, NCD-SSB could be used the same as CD-SSB during measurement. In this way, serving cell measurement should be performed always based on CD-SSB. From UE point of view, it is not reasonable to always switch to BWP with CD-SSB for serving cell measurement. Considering this question is asked from network configuration point of view, we assume it is possible that UE could be configured to perform serving cell measurements on CD-SSB in case a RedCap UE’s active BWP contains only NCD-SSB. But we would like to clarify more about the motivation or scenario. As rapporteur mentioned, it is possible when not all neighbour cells have deployed NCD-SSB. The question is whether this scenario is typical. We think it is possible at the early stage of deployment, but it would be a corner case in mature deployment. |
| DENSO | Yes | Agree with Qualcomm to allow the flexible NW configuration. |
| Intel | Yes | We do not see the problem to do so. |
| OPPO | Yes | Agree with Qualcomm. |
| CATT | Yes | We have the same view with ZTE, and suggest leaving to network configuration. |
| Spreadtrum | Yes | Agree with Qualcomm. It is up to network’s configuration based on the deployment scenario. |
| Sequans | Yes | Agree with QC, it is fine to leave this flexibility to the NW |
| Sharp | Yes |  |
| Ericsson | No | We agree with ZTE that in some cases (e.g., neighbor cell measurements on CD-SSB ARFCN including gaps configured) a UE could perform serving cell measurements on the CD-SSB. However, we don’t think that the gNB needs to tell the UE explicitly whether to use the cell’s CD-SSB or the NCD-SSB for performing serving cell measurements. Since the CD- and NCD-SSBs are meant to have the same properties and since they are in the same carrier of the same band, we suggest leaving it up to UE implementation whether it performs serving cell measurements on the cell’s CD-SSB or NCD-SSB. |

Summary:

18 companies provide feedback, 14 companies support the flexibility, 3 companies disagree, one of them thinks it can be up to UE implementation, 1 company is neutral but fine with majority.

Technically, rapporteur thinks we cannot leave it to UE implementation, because besides the frequency/Band info, the UE also need other information (e.g. SMTC, cell quality derivation parameters) in order to perform RRM measurements, so the UE needs to know which MO should be used.

Based on the majority, rapporteur suggests:

Proposal 1: (15/18) RAN2 confirms when RedCap UE’s activate BWP contains NCD-SSB, the UE can be configured to perform serving cell measurements on CD-SSB.

If answers “Yes” to Q1, companies are invited to provide your views to below solutions:

    - Solution A-1: Reuse existing servingCellMO IE (based on the assumption that RRCReconfiguration is always needed (e.g. to reconfigure UE CBW) when switching from a BWP associated with CD-SSB and a BWP associated with NCD-SSB).

    - Solution A-2: Optionally configures a BWP-specific servingCellMO under BWP-DownlinkDedicated IE when the BWP-DownlinkDedicated contains nonCellDefiningSSB-r17. If the field is present, the UE shall use this servingCellMO, otherwise, the UE uses legacy servingCellMO.

    - Solution A-3: Optionally configures servingCellMO-List-NCD-r17 in ServingCellConfig, each entry associated to the one NCD-SSB MO, the UE selects corresponding servingCellMO upon BWP switching. For a NCD-SSB contained in UE’s activate BWP, if corresponding MO is NOT included in servingCellMO-List-NCD-r17, the UE shall use legacy servingCellMO.

Different from the original proposal 2.1 in AI summary, rapporteur has added Solution A-3 based on received offline comments. The solution A-3 is similar to Solution B-1, but it supports optional configuration.

Q2: If answers “Yes” to Q1, which solution do you prefer?

| Company | Solution A-1 or Solution A-2 or Solution A-3 | Comments |
| --- | --- | --- |
| Vodafone | Solution A-1 |  |
| ZTE | Prefer Solution A-1, can accept Solution A-2 if majority supports | As we mentioned in R2-2204547, we think in real deployment, the network will not deploy two SSBs within 20MHz, so for RedCap UE, switching from a BWP containing CD-SSB to another BWP containing NCD-SSB always requires RRCReconfiguration, because the UE specific channel bandwidth needs to be reconfigured. So the network is able to reconfigure existing servingCellMO field in the same RRC message. Thus we think Solution A-1 is sufficient for RedCap UEs in Rel-17. However, if most companies want to consider the case of deploying two SSBs within 20MHz, we can accept Solution A-2. We don’t prefer Solution A-3, because the UE is required to compare the NCD-SSB frequency (indicated in BWP-DownlinkDedicated) with the multiple Mos provided by the servingCellMO-List to determine the applicable serving cell MO, so it is more complex than Solution A-2. |
| MediaTek | None of the above | The NCD-SSB must always be used for serving cell measurements |
| Qualcomm | A-2 | We are the proponent of the proposal. The main motivation is to enable more flexibility for network in configuring serving cell measurements when UE’s dedicated BWP contains NCD-SSB. As we have clarified in our reply to Q1, it may not be an efficient approach if we mandate serving cell measurements only on a specific type of SSB. Hence it is better to allow servingCellMO to be BWP specific. With Solution A-1, network may have to RRC reconfigure servingCellMO whenever UE switches its BWP. Although RRC Reconfig and BWP switch may not need to have the same timeline, that is still a significant overhead, which should be avoided. |
| NEC | Solution A-1 | With the same reason as ZTE that the network will not normally configure CD-SSB and NCD-SSB within 20 MHz, A-1 would be flexible enough. |
| Samsung | A-1 | - |
| Interdigital | A-2 | We can avoid RRCReconfiguration per BWP switch by A-2 and so we can take advantage of the existing BWP switch mechanism via lower layer signalling + it can cover the case the fallback to default BWP upon the inactive timer expiry. |
| Huawei, HiSilicon | Support A-2/3, but not fine with A-1 | Minor wording update to A-2: “when UE is on this BWP, If the field is present, the UE shall use this servingCellMO, otherwise, the UE uses legacy servingCellMO.” On the comments from ZTE “because the UE specific channel bandwidth needs to be reconfigure”, the downlinkChannelBW-PerSCS-List can be configured larger than 20M. This is the carrier BW of NW, just configuring the UE specific value, which has nothing to do with UE capaiblity. NCD-SSB will lose its benefits if it requires RRC reconfiguration upon DCI based BWP switching, as commented by Qualcomm. |
| vivo | None | We think it’s straightforward that UE is configured to perform serving cell measurements on NCD-SSB in case a RedCap UE’s active BWP contains only NCD-SSB. In this case, there is no need to configure an optional indication in BWP-DownlinkDedicated. On the other hand, if the UE is configured to perform serving cell measurements on CD-SSB, existing servingCellMO could be used to configure the UE performs serving cell measurement based on the CD-SSB. With this, there is no need to configure separate servingCellMO. |
| DENSO | A-1 | Agree with ZTE if the carrier bandwidth is 20 MHz. On the other hand, if the carrier bandwidth is 100 MHz and a 20 MHz portion of the carrier bandwidth is used for RedCap UE, there might be the scenario that NW may transmit multiple NCD-SSB, as well as CD-SSB. Even with that, we tend to agree that reconfiguration is sufficient for the initial release of RedCap. |
| Intel | Solution A-1 or Solution A-3 | To our understanding, solution A-1 is the default solution (i.e. aligned with current specification). A-3 is kind of optimization, but we see benefits on this. So we would also be fine with it. |
| OPPO | Solution A-1 |  |
| CATT | Solution A-1 |  |
| Spreadtrum | A-1 |  |
| Sequans | A-2 | Agree with QC, HW |
| Sharp | A-1 |  |
| Ericsson | None of the above | As said above, the UE may use CD-SSB (known also from legacy servingCellMO) or the NCD-SSB for performing serving cell measurements. However, if RAN2 decides to configure the serving cell MO explicitly we support A-1 since (as ZTE said) DCI-based BWP switching is anyway not possible when the target BWP has a different position in frequency domain (need to reconfigure the channel bandwidth). If the target BWP has the same frequency position (e.g., only other parameters different among BWPs) we see no need to use another MO either. |

Summary:

17 companies provide feedback, companies views are summarized below:

- Solution A-1: 11 (Vodafone, ZTE, NEC, Samsung, DENSO, Intel, OPPO, CATT, Spreadtrum, Sharp, Ericsson?)

- Solution A-2: 5 (ZTE, QC, Interdigital, HW, Sequans)

- Solution A-3: 2 (HW, Intel)

- Non of them: 3 (MTK, Vivo, Ericsson)

Based on the feedback, majority companies prefer Solution A-1, which means there is no need to configure additional ServingCellMO in Rel-17, because DCI-based BWP switching is not possible and RRC reconfiguration is required to reconfigure UE’s channel bandwidth.

The rapporteur suggests:

Proposal 2: (11 vs 5 vs 2) For how to indicate serving cell MO for RedCap UE, to adopt solution A-1.

    - Solution A-1: Reuse existing servingCellMO IE (based on the assumption that RRCReconfiguration is always needed (e.g. to reconfigure UE CBW) when switching from a BWP associated with CD-SSB and a BWP associated with NCD-SSB).

If answers “No” to Q1, companies are invited to provide your views to below solutions:

    - Solution B-1: Add servingCellMO-List-NCD-r17 in ServingCellConfig, each entry associated to the one NCD-SSB MO, the UE selects corresponding servingCellMO upon BWP switching.

      - Note: for all configured NCD-SSB, the corresponding MO must be included in servingCellMO-List-NCD-r17.

    - Solution B-2: Configures a BWP-specific servingCellMO under BWP-DownlinkDedicated IE, the field is configured in the BWP if BWP-DownlinkDedicated contains nonCellDefiningSSB-r17.

      - Note: for each BWP configured with NCD-SSB, its BWP-sepecific servingCellMO field must be included and set to the corresponding NCD-SSB MO.

    - Solution B-3: No new servingCellMO. UE uses the MO on the SSB ARFCN of the NCD-SSB used in the BWP.

Please note, the difference between Solution B-1 and Solution A-3 (or Solution B-2 and Solution A-2) is that, the NCD-SSB MO must be configured as serving cell MO, to ensure the UE always perform serving cell measurements on NCD-SSB when its active BWP contains NCD-SSB.

Q3: If answers “No” to Q1, which solution do you prefer?

| Company | Solution B-1 or Solution B-2 | Comments |
| --- | --- | --- |
| MediaTek | B-1 (open to B-2 as well) | We prefer B-1 as it’s a natural extension of the current serving cell MO signaling. |
| vivo | B-1 | B-1 is most straightforward solution. |
| Ericsson | Solution B-3 | The MOs tell the UE primarily where and how to perform neighbor cell measurements. The UE could perform SSB-based serving cell measurements without the information in the MO. Furthermore, the UE can determine the “servingCellMO” by comparing the SSB ARFCNs in the MOs to those in the carrier/BWP configurations. Hence, we see no need to replicate the servingCellMO per BWP. If RAN2 decides anyway that the gNB must provide this information, we prefer solution B-2 since it avoids having to maintain lists with associated entries in two places. |

Summary:

Based on the outcome of Q1, no proposal is provided for this question.

## Neighbour cell measurements

For neighbour cell measurements, the following proposal is provided in AI summary:

Proposal 4: To discuss if there is any problem in applying existing neighbour cell measurements framework to RedCap UEs.

To facilitate the discussion on neighbour cell measurements, the corresponding proposals and rapporteur analysis are kept as below:

| Source | Related proposals |
| --- | --- |
| HW [R2-2205038] | Proposal 3a: When RedCap UE is on the DL BWP with CD-SSB: - UE only performs the measurement on the measurement object associated with CD-SSB (for both serving and neighbor cell measurement); - UE does not perform the measurement on the measurement object associated with the NCD-SSBs configured to its serving cell. Proposal 3b: When RedCap UE is on the DL BWP with NCD-SSB: - UE at least performs the measurement on the measurement object associated with NCD-SSB in its active DL BWP, if configured (for both serving and potential neighbor cell measurement); - UE does not perform the measurement on the measurement object associated with other NCD-SSBs, which is configured to its serving cell but is not on its active DL BWP. - UE may in addition perform the measurement on the measurement object associated with CD-SSB, based on the NW indication, which is a 1bit indication per DL BWP. Proposal 3c: When UE is on the DL BWP without any SSBs: - UE only performs the measurement on the measurement object associated with CD-SSB (for both serving and neighbor cell measurement); - UE does not perform the measurement on the measurement object associated with the NCD-SSBs configured its serving cell. |
| Qualcomm [R2-2204812] | Proposal 4. If UE’s active DL BWP is configured with a BWP-specific servingCellMO, network can include in the BWP-DownlinkDedicated IE the measurement object(s) whose frequency(s) should be changed to the frequency of the servingCellMO in this BWP. |

In general, above proposals are discussing how to perform neighbour cell measurements when the SSB used for serving cell measurements is changed. If legacy principle is followed, then the UE is required to measure the Mos that are associated with measID, and for Mos that need gap assistance but gap is not configured, the UE will not measure those Mos. It is up to the network to decide which target frequencies should be measured and the network can configure Mos accordingly.

Rapporteur understands above proposals want to introduce some enhancements so that MO reconfiguration can be avoid. However, rapporteur also see some problem (or limitation) of above proposals. For instance, even if RedCap UE is on the DL BWP with CD-SSB, the network may want the UE to also measure NCD-SSB frequency (with gap assistance), so that the network knows whether it can trigger BWP addition/release, the spec should not disallow the network to configure inter-frequency measurements on NCD-SSB.

During this offline, companies are invited to show your view on neighbour cell measurements.

Q6: Do companies think anything needs to be fixed or updated for neighbour cell measurements for RedCap Ues?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | So far, no | We think the existing RRM principle is quite flexible for neighbour cell measurements, for instance: 1. The UE is required to measure neighbour cells when the MO is associated with measID. (If network wants the UE to only perform serving cell on a frequency, the network can configure a MO without associating to any reportConfig and measID) 2. For gap-assisted measurements, the UE only perform the measurement when gap is configured. So if the network wants the UE to stop measuring the neighbour cells when the SSB is outside UE’s active BWP, the network can configure measurements without gap configuration. So far, we think the network has flexibility in configuring neighbour cell measurement, but we are open to hear other companies’ views. |
| MediaTek | No | This can be left to NW implementation with appropriate neighbour cell MO configuration |
| Qualcomm | Yes | The following example can be used to illustrate the motivation and need for some enhancements to the existing framework, if servingCellMO is BWP-specific: In this example, UE is configured with two Mos, one on each type of SSB. Suppose UE switches from BWP#1 to BWP#2 and hence also switches its serving cell measurement from CD-SSB to NCD-SSB configured in BWP#2. However, UE’s neighbor cell measurements are still based on MO#1. Hence network needs a way to signal UE to change those measurements to NCD-SSB (if network thinks this change is necessary). One may argue that no enhancement is needed in this case as network can RRC Reconfig UE’s neighbor cell measurements. But the issue here is the same as the one we discussed in Q1 for serving cell measurement. It is more efficient if neighbor cell measurements, just like serving cell measurements, are switched when BWP switch results in a new type of SSB to measure. Please note that in this proposal, network still has full control whether neighbor cell or serving cell measurements should switch BWP. For example, if network does not include any MO under BWP-DedidcatedDownlink, then no neighbor cell measurements need to be switched. |
| Apple | So far no | Similart views as ZTE, but we understand that this depends on outcome of prop1/2 (Q1/Q2/Q3) |
| NEC | So far no | We still do not see need of specification change to introduce something more in Rel-17. |
| Samsung | No | Share the view with many others. |
| Interdigital | Yes | Share Qualcomm view. |
| Huawei, HiSlicon | Yes | Agree with Qualcomm that the motivation is to avoid the MO/measurement gap reconfiguration via RRC reconfiguration upon DCI triggered BWP switching. This indeed depends on the agreements from Q1-3. At least, we should discuss/clarify the neighbor cell measurement behaviors if RAN2 agree the BWP specific serving cell MO. As to the rapporteur comments, we believe the inter-frequency MO will not overlap with the NCD-SSB of serving cell’s BWPs typically. |
| vivo | No | The current measurement configuration is enough for neighbour cell measurements, and there is no need to introduce additional enhancements for neighbour cell measurements for RedCap UEs. It should be up to network configuration how to configure the measurement on neighboring cells. |
| DENSO | so far no | Similar to Apple’s view, is it related to Q1 to Q3. |
| Intel | No |  |
| OPPO | No |  |
| CATT | No |  |
| Spreadtrum | So far no | It can be up to network configuration. |
| Sequans | Yes | Agree with QC, HW |
| Sharp | No |  |
| Ericsson | No | We agree with ZTE |

Summary:

17 companies provide feedback, 4 companies think for neighbour cell measurements, current RRM framework may have problem upon BWP switching, while 13 companies think there is no problem or haven’t identified any problem so far.

Regarding the comments from QC, rapporteur thinks one possible solution is to configure MOs on CD-SSB and NCD-SSB, and both are associated with measID, so the UE is expected to perform neighbour cells on both frequencies, however, if the network does not configure any gap, then the UE will only measure the one that does not require gap assistance. This means when UE switches to BWP2 and gap is not configured, the UE will stop neighbour cell measurements on CD-SSB. Similarly, when UE switches to BWP1, since gap is not provided, the UE will stop neighbour cell measurements on NCD-SSB.

So the following proposal is provided based on majority views, but concerns can be raised during online, and we can further discuss in the next round of offline.

Proposal 3: (13/17) For neighbour cell measurements, the existing RRM mechanism is applied, further enhancement is not needed.

## About reply LS to R2-2204486

### Definition of intra-frequency measurements

The discussion relates to the following questions from RAN4 in LS R2-2204486:

Measurement related: - Is it possible to configure CD-SSB and/or multiple NCD-SSB(s) for serving cell measurements? - If yes, is it feasible from RAN2’s signalling design perspective to explicitly indicate which SSB is the reference SSB to define intra-frequency measurement?

And the following proposal is provided in AI summary.

Proposal 3: For measurement related questions in R2-2204486, reply to RAN4 with the following RAN2 understandings: - Either CD-SSB or NCD-SSB can be configured for serving cell measurements, not both. - The SSB used for serving cell measurements may be changed upon BWP switching. - The outcome of Proposal 1. - From RAN2 signalling point of view, it is feasible to explicitly indicate which SSB is the reference SSB to define intra-frequency measurements. But the indicated SSB may be different from the SSB that UE performs serving cell measurements on. So RAN2 expects further confirmation from RAN4 before introducing such signalling.

As indicated in AI summary, there are two different understandings on how to define the intra-frequency measurements:

- Understanding-1: The SSB used by the UE to perform serving cell measurements is reference SSB;

- Understanding-2: The reference SSB is explicitly configured by the network, it can be different from the SSB that UE performs serving cell measurements.

- Understanding 3: The NCD-SSB of the currently active BWP (if configured), or the CD-SSB (otherwise)

Although RAN4 did not ask RAN2 how to define the intra-frequency measurements, to avoid misunderstanding, it is better to clarify RAN2 understandings as much as possible. So the first three bullets in Proposal 3 are provided regarding to RAN4’s first question.

- Is it possible to configure CD-SSB and/or multiple NCD-SSB(s) for serving cell measurements?

Companies are invited to show your views on the content/proposals for a possible reply LS to R2-2204486.

Q4: For the first measurement related question, do companies agree to reply RAN4 with below RAN2 understandings?

- Either CD-SSB or NCD-SSB can be configured for serving cell measurements, not both.

- The SSB used for serving cell measurements may be changed upon BWP switching.

- The outcome of Proposal 1.

| Company | Yes or No | Comments |
| --- | --- | --- |
| Vodafone | yes | Should follow the outcome from proposal 1 from section 3.1 in this document. |
| ZTE | Yes | Strictly speaking, the second bullet also relates to the discussion of Q1. If solution A-1 is selected, then the second bullet is not valid anymore. In addition, we can also mention that a RedCap UE may be configured with multiple NCD-SSBs, but one BWP can contain at least one NCD-SSB. |
| MediaTek | Partially | We have to clarify that ‘not both’ is BWP specific and not cell specific. For example, with solutions A2, A3, B1 and B2 in section 3.1, the RRC configuration can include CD and multiple NCD SSBs for a given cell, but when operating in a BWP only one of the configured SSBs is used for serving cell measurements. So, we propose the following modification: - CD-SSB and/or multiple NCD-SSBs can be configured for serving cell measurements, with the restriction that only one SSB (CD-SSB or NCD-SSB) can be used for serving cell measurements within a configured BWP. - The SSB used for serving cell measurements may be changed upon BWP switching. - The outcome of Proposal 1. |
| Qualcomm | Yes | We support the clarification to the first bullet proposed by MediaTek |
| Apple | Yes, but with the clarification by MediaTek |  |
| NEC | Yes | It would be good to confirm this after concluding the Q1/P1. |
| Samsung | Yes | We also support the clarification from MediaTek. |
| Interdigital | Yes | We support the clarification proposed by MediaTek. |
| Huawei, HiSilicon | Yes, with the update | We support the intention from MediaTek. We suggest to minor update - CD-SSB and/or multiple NCD-SSBs can be configured for serving cell measurements, with the restriction that only one SSB (CD-SSB or NCD-SSB) can be used for serving cell measurements within an active configured BWP. |
| vivo | Yes | - Agree the clarification proposed by MediaTek. 2nd bullet could be also updated based on the output of above questions. |
| DENSO | Yes | Agree on MediaTek’s clarification |
| Intel | Yes | Agree the proposed change from MediaTek. |
| OPPO | Yes | Agree on MediaTek’s clarification |
| CATT | Yes | We also support the clarification from MediaTek. |
| Spreadtrum | Yes | Agree with MediaTek |
| Sequans | Yes, with update | Agree with MediaTek |
| Sharp | Yes | Agree with MediaTek |
| Ericsson | No | As said for Q1, we don’t see a need to configure explicitly which SSB the UE shall used for serving cell measurements. The UE may use CD-SSB or NCD-SSB. And we assume/believe that the results are sufficiently similar to be comparable. RAN4 should ensure that. |

Summary:

17 companies provide feedback, except 1 company, 16 companies agree with the proposal and most of them prefer the proposed change from MTK.

Regarding the wording “within a configured BWP” or “within an active BWP”, rapporteur thinks it may cause misunderstanding that only the SSB within UE’s activate BWP can be used for serving cell measurements, this may conflict with the discussion in Q1. Considering the intention is to clarify that only one SSB can be used at a given time, and the outcome of P1 is already captured as the third bullet, for the first bullet, rapportuer suggests to change it to “at a given time”.

Proposal 4: (16/17) For the first measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings.

- CD-SSB and/or multiple NCD-SSBs can be configured for serving cell measurements, with the restriction that only one SSB (CD-SSB or NCD-SSB) can be used for serving cell measurements at a given time.

- The SSB used for serving cell measurements may be changed upon BWP switching.

- The outcome of Proposal 1.

Regarding the second question from RAN4, RAN4 asks about the feasibility of explicit indication. As pointed out in AI summary, by using RRC signalling, the indicated referenced SSB can be different from the SSB that used for serving cell measurements, because BWP can be changed via DCI. Then both RAN2 and RAN4 may need more discussion on how to interpret UE gap capability.

So in Proposal 3, rapporteur suggests to highlight this aspect, companies are invited to show your views for the possible reply LS to R2-2204486.

Q5: For the second measurement related question, do companies agree to reply RAN4 with below RAN2 understandings?

- From RAN2 signalling point of view, it is feasible to explicitly indicate which SSB is the reference SSB to define intra-frequency measurements. But the indicated SSB may be different from the SSB that UE performs serving cell measurements on. So RAN2 expects further confirmation from RAN4 before introducing such signalling.

| Company | Yes or No | Comments |
| --- | --- | --- |
| Vodafone | yes | I think it is good to have the clarification before final answer. |
| ZTE | Yes | We have some concern on the RRC indicated reference SSB. The definition of intra-frequency measurement will be used to determine UE’s gap requirement, if the SSB used for actual serving cell measurements can be different from the configured reference SSB, we are afraid mismatch will happen between the network and the UE. For instance, the UE reports “no gap” for intra-frequency measurements in NeedForGap reporting. Network has set reference SSB to CD-SSB, but at the moment the UE is performing serving cell measurements on NCD-SSB, then for neighbour cell measurements on CD-SSB frequency, the network will assume gap is not needed. But this may not be true, because from UE RF point of view, measuring cell on CD-SSB frequency may need RF retuning. |
| MediaTek | No | The purpose of defining intra and inter frequencies is to know which set of UE capabilities and RAN4 requirements apply. For example, measurement gaps are not needed for intra frequency measurements but are needed for inter frequency measurements. If we change the definition altogether, such that the ‘intra frequency’ is different from the frequency of the SSB that the UE performs cell measurements on, we end up with a situation where intra frequency measurements would REQUIRE measurement gaps, while inter frequency measurements would NOT REQUIRE measurement gaps! Such an approach will create a mess out of the specifications and we’ll be dealing with the fallout of this mess for the next two years… We therefore propose the following response: From RAN2 signalling point of view, the NW and the UE always know which SSB is the reference SSB to be used for serving cell measurements at a given point in time. This reference SSB can be used to define intra-frequency measurements. This reference SSB may change with BWP switching. [Rapp-ZTE] On behalf of ZTE, we have the same understanding that the SSB used for serving cell measurements should be regarded as “reference SSB”, so we are fine with the proposed response. |
| Qualcomm | No | We can’t agreement with this statement “But the indicated SSB may be different from the SSB that UE performs serving cell measurements on”. In our view, we do not need to change the current understanding that the frequency used in serving cell measurement should be the one used to define intra- vs inter-frequency. We can only need to update the definition that the frequency used in serving cell measurements can be either CD-SSB or NCD-SSB. |
| Apple | No | We have the same view as Qualcomm and Mediatek, and just the “which” SSB needs discussion, and based on this the existing intra/inter definition can be reused. We are ok with the wording suggested by Mediatek. |
| Samsung | No | Proposal from MediaTek looks good to us. |
| Interdigital | No | MediaTek’s proposal is fine for us. |
| Huawei, HiSilicon | See comment | In any case, the 1st sentence is correct, we can always just confirm the feasibility “From RAN2 signalling point of view, it is feasible to explicitly indicate which SSB is the reference SSB to define intra-frequency measurements.” And, the first part from MTK wording is also reasonable “From RAN2 signalling point of view, the NW and the UE always know which SSB is the reference SSB to be used for serving cell measurements at a given point in time. This reference SSB can be used to define intra-frequency measurements.” So, the key point/controversial part is on whether “This reference SSB may change with BWP switching.” To be honest, this is actually R4 discussion. For sure, RAN2 can also conclude on this and send the agreement back to R4 to help R4 discussion. But, if RAN2 has no consensus, we’d better not to touch this and send the LS immediately to ask for R4 to give the feedback/confirmation by the end of meeting. The most important thing is we need to conclude this in May meeting, since there is RAN2 impact anyway. So, we suggestion the proposal as: Pa: reply RAN4 with below RAN2 understandings: - From RAN2 signalling point of view, it is feasible to explicitly indicate which SSB is the reference SSB to define intra-frequency measurements - From RAN2 signalling point of view, the NW and the UE always know which SSB is the reference SSB to be used for serving cell measurements at a given point in time. This reference SSB can be used to define intra-frequency measurements. Pb: RAN2 to discuss online on whether this reference SSB may change with BWP switching.. |
| Vivo | No | As we prefer the Understanding-1 above, we don’t see any motivation that network indicates UE to perform serving cell measurement on one SSB, while indicating a different SSB as reference SSB. If the network thinks the NCD-SSB in the active BWP is not suitable to be set as reference SSB, it could indicate UE to perform serving cell measurement on CD-SSB as legacy, rather than introducing an explicit indication. |
| DENSO | No | Same view as MediaTek, Qualcomm and Apple. There shouldn’t be mismatch between NW and UE as to which SSB is used for the serving cell measurement. We’re fine with MediaTek’s text proposal. |
| Intel | No | Agree the proposed change from MediaTek. |
| OPPO | No | MediaTek’s proposal is fine for us. |
| CATT | No | Support the proposal from Media Tek. |
| Spreadtrum | No | We agree with MediaTek’s proposal. |
| Sequans | No | Agree with MediaTek comments and proposal |
| Sharp | No | Agree with MediaTek |
| Ericsson | No | Similar view as MediaTek. We assumed that a UE operating on a BWP that contains the NCD-SSB could always perform neighbor cell measurements on that ARFCN without gaps. Hence, we assumed that this NCD-SSB would be the “reference SSB”. If the UE operates on a BWP in which no NCD-SSB is configured, the CD-SSB would be the “reference SSB”. Being able to configure a “reference SSB” to any other/additional ARFCN seems to cause additional problems. |
| Vodafone |  | Even, we probably agree with Mediatek intention, I think the question was if there is a possibility to signal reference SSB and therefore, I think it would be better to say that there is no need for the signaling and explain why. |

Summary:

Almost all companies agree with the modification from MTK. One company suggests to add the following:

“-From RAN2 signalling point of view, it is feasible to explicitly indicate which SSB is the reference SSB to define intra-frequency measurements”

From rapporteur point of view, if RAN2 confirms the reference SSB should be the one used for serving cell measurements, it implies explicit indication should not be considered. So adding above bullet may cause problem.

Based on the majority, rapporteur suggests:

Proposal 5: (16/17) For the second measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings.

- From RAN2 signalling point of view, the NW and the UE always know which SSB is the reference SSB to be used for serving cell measurements at a given point in time. This reference SSB can be used to define intra-frequency measurements. This reference SSB may change with BWP switching.

### Handover to BWP without CD-SSB

The discussion relates to the following questions from RAN4 in L;)S R2-2204486:

Handover related: - Are the following Handover scenarios valid from RAN2’s perspective? - Handover to a target cell’s specific Redcap BWP associated with NCD-SSB directly other than to the initial BWP associated with CD-SSB - Handover to a target cell’s initial BWP and further switch to the specific Redcap BWP to send the RACH - the specific Redcap BWP associated with NCD-SSB - the specific Redcap BWP without presence of NCD-SSB

And following proposals are provided in AI summary:

Proposal 5: RAN2 confirms that RedCap UE can be configured to handover directly to a target cell’s specific Redcap BWP associated with an NCD-SSB, i.e. the UE directly sync to the NCD-SSB and perform RACH on that BWP. Proposal 5.1: In handover command, if the first active BWP is associated with NCD-SSB, the smtc field included reconfigurationWithSync is configured according to the NCD-SSB of target cell. Proposal 6: To discuss whether handover to a target cell’s initial BWP and further switch to the specific Redcap BWP to send the RACH can be supported.

In this section, companies are invited to show your views on the support of scenario 1 and scenario 2 separately.

Scenario 1: Handover to a target cell’s specific Redcap BWP associated with NCD-SSB directly other than to the initial BWP associated with CD-SSB

Q7: From RAN2 perspective, do companies think Scenario 1 can be supported (i.e. the UE can directly sync to the NCD-SSB and perform RACH on that BWP)?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Vodafone | No; Depending on assumption | Vodafone (we don’t see a significant gain on supporting this scenario, under the assumption that CORESET#0 is not in the NCD-SSB, leading to the reconfiguration to happen anyway to obtain information to perform RACH), but if anyone could confirm that our assumption is correct this would be helpful. |
| ZTE | Yes | NCD-SSB also provides SFN/subframe information, so it is feasible to sync to NCD-SSB to obtain DL timing, SIB1 can be provided via dedicated signalling. The drawback of scenario 1 is that, NCD-SSB may have larger transmission periodicity, so it may cause longer handover delay. However, according to RAN4 requirements, if the target cell is known (e.g. based on RRM in source cell), the value of Tsearch is 0ms. So we think the drawback is negligible in case of measurement based handover. |
| MediaTek | Yes |  |
| Qualcomm | Yes | We think it is technically feasible. And it enables faster and more efficient handover, as compared with alternative options (e.g. handover to default initial BWP and then switch to RedCap-specific BWP to RACH). So we think it should be supported. |
| Apple | Yes | We agree. Also this implies that UE doesn’t have to do anything (per spec) with CD-SSB as part of handover in such a case. |
| NEC | Yes | We think it is feasible to support the scenario 1, while it would be good to raise potential delay as explained by ZTE and confirm with RAN4. |
| Samsung | Yes | - |
| Interdigital | Yes | It’s feasible and so Yes. |
| Huawei, HiSilicon | Yes, but | We are fine to compromise. But, we need to add the clarification that: The UE can also directly sync to the CD-SSB and perform RACH on that BWP, as supported in legacy. We don’t want to mandate UE to directly HO to NCD-SSB. That’s also to allow NW to choose HO to CD-SSB rather than NCD-SSB, if the delay is considered as an issue, as commented by ZTE. |
| Vivo | Yes | This is one of the intentions for NCD-SSB in connected mode. It could be helpful to avoid unnecessary RF retuning from UE side. Regarding the potential delay mentioned by ZTE, we assume it depends on which scenarios. There is no need to inform them in LS, as it is RAN4 work scope. |
| DENSO | Yes | Agree that it is feasible and more efficient than the other alternative, even with the delay due to longer SSB periodicity. |
| Intel | Yes | We do not see the problem to do this. |
| OPPO | Yes |  |
| CATT | Yes |  |
| Spreadtrum | Yes |  |
| Sequans | Yes | Agree with ZTE |
| Sharp | Yes |  |
| Ericsson | Yes | Same view as ZTE. We would like to emphasize that the main aspect is that the UE performs the RA directly on the firstActive BWP (which is the BWP with NCD-SSB in this scenario). We don’t see a need for the UE to first acquire the CD-SSB since all necessary information is contained also in the MIB of the NCD-SSB. And UE’s should not be required to acquire SIB1 or other SIBs during HO. But even if the UE would (for some reason) decide to sync/acquire CD-SSB before switching to the firstActive BWP with NCD-SSB and perform RA there, the network would not notice that first step anyway. |

Summary:

All companies except one agree that scenario 1 can be supported.

Based on the majority, rapporteur suggests:

Proposal 6: (16/17) From RAN2 perspective, handover scenario 1 is supported.

- Scenario 1: Handover to a target cell’s specific Redcap BWP associated with NCD-SSB directly other than to the initial BWP associated with CD-SSB (i.e. UE directly sync to the NCD-SSB and perform RACH on that BWP)

If answers “Yes” to Q7, please also provide your views on how to set the “smtc” field in handover command.

Q8: If answers “Yes” to Q7, do companies agree that in handover command, if the first active BWP is associated with NCD-SSB, the smtc field included reconfigurationWithSync is configured according to the NCD-SSB of target cell.

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes | “smtc” field is introduced to assist the UE to search target cell. To support scenario 1, we think the simplest way is to reuse this field and associate it to NCD-SSB when first active BWP is associated with NCD-SSB, it’s better to make sure the network and the UE have the same understanding, so IoT problem won’t occur. So if this can be agreed, we suggest to make following update to the field description. Smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell change and NR Pcell change. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in spCellConfigCommon. For case of NR Pcell change, the smtc is based on the timing reference of (source) Pcell. For case of NR PSCell change, it is based on the timing reference of source PSCell. If both this field and targetCellSMTC-SCG are absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message. For RedCap UE, if the first active DL BWP indicated in this RRC message is configured with nonCellDefiningSSB-r17, the UE assumes this field is configured according to the NCD-SSB indicated by nonCellDefiningSSB-r17, otherwise, the UE assumes this field is configured according to the CD-SSB of target cell. |
| MediaTek | Yes |  |
| Qualcomm | Yes |  |
| Apple | Yes |  |
| NEC | Yes |  |
| Samsung | Yes | - |
| Interdigital | Yes |  |
| Huawei, HiSilicon | Disagree | smtc is mainly to help UE to find the SSB fast. But, for NCD-SSB, NW already provide the frequency and periodicity in nonCellDefiningSSB. So, UE already have the full information to find the NCD-SSB. Introducing smtc for NCD-SSB is enhancement. Reusing the smtc field included reconfigurationWithSync, which is supposed to be for CD-SSB, is not acceptable. Introducing this will has impact to the case of “HO to CD-SSB”. It means we cannot support the below case in scenario 2, where NW may want to use smtc to configure UE to find the CD-SSB for DL sync, rather than NCD-SSB. “Handover to a target cell’s initial BWP and further switch to the specific Redcap BWP to send the RACH  the specific Redcap BWP associated with NCD-SSB”. |
| vivo | Yes | We have agreed before: For connected mode operation if NCD-SSB is configured in a dedicated DL BWP whose paired UL BWP is configured with RACH-ConfigDedicated, RACH-ConfigCommon or BeamFailureRecovery Config, SSB in that RACH configuration (e.g., in CFRA-SSB-Resource IE or in PRACH-ResourceDedicatedBFR IE) refers to the NCD-SSB configured in that DL BWP. Hence, if the target cell’s specific Redcap BWP is associated with an NCD-SSB, SSB in rach-ConfigDedicated and smtc within reconfigurationWithSync should also refer to the NCD-SSB, then UE could perform RACH on NCD-SSB. |
| DENSO | Yes |  |
| Intel | Yes | Make sense to us. |
| OPPO | Yes |  |
| CATT | Yes |  |
| Spreadtrum | Yes |  |
| Sequans | Maybe | We think the concern from HW is valid if we keep the scenario they mention, depending on the conclusion of Q9 |
| Sharp | Yes |  |
| Ericsson | Not sure | Wouldn’t it be preferable (for the UE) to set this field using the source cell as timing reference? If so, why is that not considered possible here? We do agree that the constraints mentioned in the first paragraph of the field description (“The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in spCellConfigCommon”) should apply to the (target) NCD-SSB properties. |

Summary:

14 companies agree with the proposal, 1 company disagrees and 2 companies are not sure.

This indeed relates to the outcome of Q9, so in the proposal, rapporteur added “for scenario 1”.

Proposal 7: (14/17) For scenario 1, in handover command, if the first active BWP is associated with NCD-SSB, the smtc field included reconfigurationWithSync is configured according to the NCD-SSB of target cell.

For handover scenario 2:

Scenario 2: Handover to a target cell’s initial BWP and further switch to the specific Redcap BWP to send the RACH

    - the specific Redcap BWP associated with NCD-SSB

    - the specific Redcap BWP without presence of NCD-SSB

Q9: From RAN2 perspective, do companies think Scenario 2 can be supported (i.e. the UE first sync to the CD-SSB and then autonomously switch to first active BWP to perform RACH)?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Vodafone | Yes | Please see comments to Q7 |
| ZTE | Prefer No | Scenario 2 is more complex than Scenario 1. If scenario 1 can be supported, there seems no additional benefit to support scenario 2. But we won’t object if majority companies would like to support scenario 2. |
| MediaTek | No | We do not see the point in introducing such a case for handover. This approach results in an unnecessarily complicated handover procedure with no real gain: - For the first listed case under Scenario 2, the RedCap specific BWP already has an NCD-SSB. There’s no need to complicate the handover procedure with an intermediate BWP switch, when the procedure can take place without needing such a BWP switch (by using the NCD-SSB as the sync reference) - For the second listed case, RAN4 will not define connected mode measurement requirements for operation in a BWP without an SSB (see R4-2206950). So we end up in a situation where there are no requirements for the UE operating in the first active BWP following handover! It’s a waste of our time and effort to optimize for this case that will only result in a paper design and will not be adopted in real deployments. |
| Qualcomm | No | We share the same view as MediaTek |
| Apple | No |  |
| NEC | No | We do not see the benefit of supporting this considering the additional complexity. |
| Samsung | No | Share the view with many others. |
| Interdigital | No | We don’t see any point to support the scenario. RedCap specific BWP is introduced for the case that RedCap UE doesn’t support the BW of the initial BWP. If RedCap UE can access the initial BWP, then no need to have the RedCap specific BWP from the beginning. |
| Huawei, HiSilicon | Yes | This question is not about the whether it is beneficial. RAN4 is just asking whether it is valid. RAN2 should not preclude the feasibility for NW configuration. This should be up to NW implementation. We don’t see any technical blocking issue from R2 to not support those case. |
| vivo | No | There is no need to support scenario2, which brings more complexity. |
| DENSO | No | Agree with MediaTek |
| Intel | ? | If the UE can sync/access to RedCap specific BWP, it is not clear to us why/when the scenario would happen |
| OPPO | No | Agree with MediaTek. |
| CATT | No |  |
| Spreadtrum | No |  |
| Sequans | Prefer No, but | We agree with companies above that this does not seem very beneficial. However, we are OK to continue discussing this, in view of the possible additional delay of option 1 and that this limits NW configuration flexibility |
| Sharp | No |  |
| Ericsson | No | We are not even sure what “Handover to a target cell’s initial BWP” really means. Since, the UE would anyway send its RA only to the NCD-SSB’s RA resources, the gNB would not even notice that the UE went to the initial BWP first. |

Summary:

All companies except 2 agree that scenario 2 is more complex that does not need to be supported.

Based on the majority, rapporteur suggests:

Proposal 8: (15/17) From RAN2 perspective, handover scenario 2 is not supported.

- Scenario 2: Handover to a target cell’s initial BWP and further switch to the specific Redcap BWP to send the RACH (i.e. UE first sync to the CD-SSB and then autonomously switch to first active BWP to perform RACH)

## SIBs reception in BWP without CD-SSB

For SIB reception, the following proposals are provided in AI summary:

Proposal 7: During handover, if dedicatedSIB1-Delivery IE is not included in the handover command and the first active BWP in the target cell does not contain CD-SSB, UE acquires SI either from CSS for SIBs or via dedicated signaling only within the first active BWP. Proposal 8: RAN2 confirm that RedCap UEs in RRC Connected only need to support the following three options for acquiring SI update or ETWS/CMAS message in a dedicated DL BWP that does not contain CD-SSB: - From CSS for SIBs configured within this DL BWP; - Via dedicated signaling; - Switched by network (either DCI or RRC) to an initial DL BWP where SIBs are sent.

Companies are invited to show your views to above proposals:

Q10: Do companies agree with above Proposal 7?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes | We understand there are only three ways for the UE to obtain SIB, as indicated in P8. But during handover procedure, it doesn’t make sense to switch the UE back to initial BWP to obtain SIB, if that is the case, the network should switch the UE directly to then initial BWP(or a BWP that contains CD-SSB and CORESET#0), then switch to other BWP for data transmission. |
| MediaTek | Yes, with clarification | With the clarification that ‘only within the first active BWP’ applies to both CSS for SIBs and to dedicated signalling. |
| Qualcomm | Yes | We agree with both the comments from MediaTek and ZTE. |
| Apple | Yes |  |
| NEC | Yes |  |
| Samsung | Yes | It should be the case. |
| Interdigital | Yes |  |
| Huawei, HiSilicon | Yes, but | We support the intention. As to the comment from MediaTek, we believe there is still the case “UE HO directly to CD-SSB and switch to NCD-SSB as its first BWP” (see scenario 2.), where UE may still have the chance to obtain SIB on CD-SSB. So, the originally wording is better. Also, we believe another intention is that NW should ensure UE have the SIB1. So, we suggest minor update: “UE is expected to acquire SI either from CSS for SIBs or via dedicated signaling only within the first active BWP.” |
| vivo | Yes |  |
| DENSO | Yes |  |
| Intel | Yes with clarification (see comment) | When UE acquires SI from CSS for SIBs, we assume it is the CSS for SIBs configured within this DL BWP, we wonder whether this is the correct/same understanding as the proponent? If yes, then we agree P7. |
| OPPO | Yes |  |
| CATT | Yes |  |
| Spreadtrum | Yes |  |
| Sequans | Yes |  |
| Sharp | Yes |  |
| Ericsson | Yes, with clarification | We agree that those (legacy) mechanisms allow the NW to provide a UE in those dedicated BWPs with SIBs. But (as in legacy) the NW has the option not to provide up to date SI to such UEs if the NW does not consider it necessary. In such case the UE is not required to acquire the SIB. |

Summary:

All companies agree with Proposal 7, also adopts the suggestion from HW:

Proposal 9: (17/17)During handover, if dedicatedSIB1-Delivery IE is not included in the handover command and the first active BWP in the target cell does not contain CD-SSB, UE is expected to acquire SI either from CSS for SIBs or via dedicated signaling only within the first active BWP.

Q11: Do companies agree with above Proposal 8?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes | We think the proposal is aligned with R15/R16 operations. |
| MediaTek | Yes | Proposal 8 is the generic case |
| Qualcomm | Yes | We think these three possible methods are the most likely options network can apply. Additional options would not offer more benefits but on the other hand would increase uncertainties for UE implementations. They therefore should be excluded. |
| Apple | Yes | Same as legacy operation. |
| NEC | Yes |  |
| Samsung | Yes | - |
| Interdigital | Yes |  |
| vivo | Yes |  |
| DENSO | Yes |  |
| Intel | Yes |  |
| OPPO | Yes |  |
| CATT | Yes |  |
| Spreadtrum | Yes |  |
| Sequans | Yes |  |
| Sharp | Yes |  |
| Ericsson | Yes | We should clarify in an agreement that the legacy mechanisms exist (including the option not to provide those UEs with updated SI if it is not important for the operation of those UEs). |

Summary:

All companies agree with the proposal:

Proposal 10: (17/17)RAN2 confirm that RedCap UEs in RRC Connected only need to support the following three options for acquiring SI update or ETWS/CMAS message in a dedicated DL BWP that does not contain CD-SSB:

- From CSS for SIBs configured within this DL BWP;

- Via dedicated signaling;

- Switched by network (either DCI or RRC) to an initial DL BWP where SIBs are sent.

## Paging reception in BWP without CD-SSB

For paging reception, the following proposals are provided in AI summary:

Proposal 9: Clarify in the RRC field description that the paging search space is configured in an initial BWP only if that BWP includes the CD-SSB. Proposal 10: RAN2 confirms for separate initial DL BWP does not contain CD-SSB and CORESET#0, the BWP will not be configured for paging no matter the RedCap UE is in RRC_IDLE/INACTIVE or RRC_CONNECTED state Proposal 11: Reply to RAN1 and explain there is no need to support paging connected RedCap UEs in case the separate initial DL BWP does not contain CD-SSB and CORESET#0.

Among the proposals, Proposal 10 and 11 are related to the below RAN1 agreements mentioned in RAN1 LS R2-2204422.

RAN1 agreements extracted from LS R2-2204422. - For BWP#0 configuration option 1, - For FR1, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB and the entire CORESET#0. - For FR2, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB. - Note: For BWP#0 configuration option 2, - For FR1, - For a separate initial DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0), if it is configured for paging, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE supporting FG 6-1a does not expect it to contain SSB/CORESET#0/SIB - For FR2, - For a separate initial DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0), if it is configured for paging, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE supporting FG 6-1a does not expect it to contain SSB/CORESET#0/SIB

In short, proponent suggests to clarify there is no need to define different paging principles for RRC_IDLE/INACTIVE and RRC_CONNECTED RedCap Ues, and the agreement made in RAN1 may cause problem when the RedCap UE in RRC_CONNECTED state wants to obtain SIB after receiving system modification indication or ETWS/CMAS notification in Paging message.

Companies are invited to show your views to above proposals:

Q12: Do companies agree with above Proposal 9?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes, but | P9 and P10 are related, for RRC_CONNECTED Ues, although RAN1 agreed the separate initial BWP that does not include CD-SSB can be configured for paging. We see no benefit to support this scenario, so we are aligned with P9. However, it seems not necessary to update the field description, it can be handled by network implementation, if needed, maybe it is better to capture the principle in stage2 spec. |
| MediaTek | Yes | It is important to capture this as we have already agreed to this but it’s missing in the specification (which therefore results in a specification that is the opposite of what we’ve agreed!) |
| Qualcomm | Yes |  |
| Apple | Yes | We think it’s better to have this captured in spec for clarity. There are explicit RAN1 agreements. |
| NEC | Yes | This clarification is useful. |
| Samsung | Yes | - |
| Interdigital | Yes |  |
| vivo | Yes | We prefer to add this field description for paging search space to make things clearer although this field could only present when RedCap initial BWP includes the CD-SSB. |
| DENSO | Yes |  |
| Intel | No | We do not see the problem to support RAN1 agreements. In contrast to Idle mode, in CONNECTED mode UE, the UE doesn’t need to perform any serving cell measurements before paging monitoring. In fact, the UE would not need CD-SSB for monitoring and reception of Paging PDCCH in connected mode. The behavior is same as that of regular PDCCH monitoring in connected mode, i.e. paging monitoring in CONNECTED mode is just to receive PDCCH and thus NCD-SSB can be used. Therefore, RAN1 agreed that there is no need to restrict configuration of paging monitoring in an active DL BWP that may not contain CD-SSB. |
| OPPO | Yes |  |
| CATT | Yes | We think for idle/inactive UE, only the initial BWP contain CD-SSB can include the paging SS. |
| Spreadtrum | Yes |  |
| Sequans | Yes |  |
| Sharp | Yes |  |
| Ericsson | Yes | We agree with MediaTek that a clarification is necessary if the signaling (ASN.1 + field descriptions) enables configurations which are actually not permitted. We wonder whether it is necessary at all to configure the paging search space inside the “initial DL BWP for RedCap”. Wouldn’t it be sufficient and less ambiguous that all UEs (legacy and RedCap) use the downlinkConfigCommon(SIB)=> initialDownlinkBWP => pdcch-ConfigCommon => pagingSearchSpace configuration? That would fit to the previous agreement that also IDLE/INACTIVE RedCap UEs shall perform cell (re-)selection on the CD-SSB and read the SIBs on CORESET#0 (like legacy UEs). |

Summary:

All companies except one agree with the proposal.

Regarding the comment from Intel, rapporteur thinks the key point is not about the feasibility, but about the necessity. As also agreed by RAN1, the connected UEs are not expected to monitor SIB in that BWP, so even if those UE can obtain Paging, the UE is unable to obtain SIB1 or ETWS/CMAS messages. Thus the benefit of monitoring paging is gone.

Proposal 11: (15/16) Clarify in the RRC field description that the paging search space is configured in an initial BWP only if that BWP includes the CD-SSB.

Q13: Do companies agree with above Proposal 10?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes | As we discussed in R2-2205771, even for BWP operation 2, the paging search space is configured in BWP common configuration, which is applicable to RRC_IDLE/INACTIVE and RRC_CONNECTED UEs. So to only ask RRC_CONNECTED Ues to monitor paging requires RRC spec change (e.g. introduce additional paging search space configuration for connected UE). In addition, RRC_CONNECTED UE monitors paging to obtain SI change indication and ETWS/CMAS notification, but RAN1 indicates there is no SIB transmitted in that BWP. So only reading Paging is useless. So from RAN2 perspective, we do not see the benefit of supporting different paging configurations for Ues in different RRC states. |
| MediaTek | Yes |  |
| Qualcomm | Yes | From UE’s perspective, we support Proposal 10 for both RRC Idle/Inactive and RRC Connected. It simplifies UE implementation. |
| Apple | Yes |  |
| NEC | Yes |  |
| Samsung | Yes |  |
| Interdigital | Yes |  |
| vivo | Yes | From RAN2 current signaling configuration perspective, there is no other way to configure CSS for paging for the separate initial DL BWP, hence it can’t be configured for paging. |
| DENSO | Yes |  |
| OPPO | Yes |  |
| CATT | yes |  |
| Spreadtrum | Yes |  |
| Sequans | Yes |  |
| Sharp | Yes |  |
| Ericsson | Yes |  |

Summary:

All companies agree with the proposal.

Proposal 12: (15/15) RAN2 confirms for separate initial DL BWP does not contain CD-SSB and CORESET#0, the BWP will not be configured for paging no matter the RedCap UE is in RRC_IDLE/INACTIVE or RRC_CONNECTED state

Q14: Do companies agree with above Proposal 11?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes | Same our comments to Q13. As we known, RAN1 didn’t discuss the role of paging for connected UEs, so if P10 can be agreed, we should inform RAN1 and ask them to revisit their agreement. |
| MediaTek | Yes |  |
| Qualcomm | Yes |  |
| Apple | Yes | Maybe we can add that paging in this case can be done via dedicated signaling (if there are questions in RAN1) |
| NEC | Yes |  |
| Samsung | Yes | - |
| Interdigital | Yes |  |
| Vivo | Yes |  |
| DENSO | Yes |  |
| OPPO | Yes |  |
| CATT | Yes |  |
| Spreadtrum | Yes |  |
| Sequans | Yes |  |
| Sharp | Yes |  |
| Ericsson | Yes |  |

Summary:

All companies agree with the proposal.

Proposal 13: (15/15) Reply to RAN1 and explain there is no need to support paging connected RedCap UEs in case the separate initial DL BWP does not contain CD-SSB and CORESET#0.

## Number of SSBs

Regarding the number of SSBs, the following proposal is provided in AI summary:

Proposal 12: In Rel-17, one BWP can only contain up to one SSB (CD-SSB or NCD-SSB).

The intention is to align with RAN4 because RAN4 will only define RRM requirements for scenarios where the active BWP includes either a CD-SSB or an NCD-SSB, not both.

In RAN2#117e, RAN2 discussed the number of NCD-SSB and made the following agreement.

Agreements online:

- A RedCap UE may be configured with multiple NCD-SSBs provided that each BWP is configured with at most one SSB

However, rapporteur thinks some company may interpret “at most one SSB” as “at most one NCD-SSB”, that is why contribution is submitted to clarify one BWP can only contain one SSB (either CD-SSB or NCD-SSB). So companies are invited to show your opinion about P12.

Q15: Do companies agree with above Proposal 12?

| Company | Yes or No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| MediaTek | Yes |  |
| Qualcomm | Yes |  |
| Apple | Pls see comments | We think it’s simpler to limit to 1 NCD-SSB for the UE, and the same can be present in multiple BWPs (overlapped), while the current question does not bring this up, we think for R17 it would be simpler to go with this. This also helps with ref SSB switching topic (at the time of BWP switching), and we understand that this restricts the NW, but our view is practically it might not. |
| NEC | Yes |  |
| Samsung | Yes | - |
| Interdigital | Yes |  |
| Huawei, HiSilicon | Yes |  |
| vivo | Yes | If the BWP contains CD-SSB, there is no strong motivation to configure NCD-SSB for this BWP. |
| DENSO | Yes and agree with Apple | NW may transmit multiple NCD-SSB within a carrier bandwidth. On the other hand, from UE point of view, it would be enough to use one of NCD-SSB. One may consider that NW wishes to configure different NCD-SSB per BWP for a given RedCap UE, e.g. for dynamic load balancing by BWP switching. Nevertheless, it is not clear if such a dynamic mechanism is needed for the first release of RedCap. |
| Intel | Yes |  |
| CATT | Yes |  |
| Spreadtrum | Yes |  |
| Sequans | Yes |  |
| Sharp | Yes |  |
| Ericsson | Yes | The formulation is misleading. We assume that “BWP” in the proposal refers to the BWP configuration (BWP-DownlinkDedicated). And this “configuration” will never “contain” the CD-SSB. And if “BWP” refers to the “bandwidth of the BWP”, there is no need for such restriction since there could be SSBs that this UE is either unaware of (and hence maybe rate-matched around) or which it is meant to use for neighbor cell measurements. Isn’t it already clear from the ASN.1 that the NW can configure at most one NCD-SSB in a BWP… and that the UE uses the CD-SSB if the (one) NCD-SSB is absent? |

Summary:

All companies agree with the proposal. Regarding the comment from Ericsson, I have reformulated the proposal as below, i.e. adding “from UE perspective” and “configured BWP”.

Proposal 14: (17/17) In Rel-17, from UE perspective, one configured BWP can only contain up to one SSB (CD-SSB or NCD-SSB).

In addition, two companies suggest to revisit the conclusion on multiple NCD-SSB, so rapporteur adds the following proposal for online discussion:

Proposal 15: To discuss whether only one NCD-SSB can be configured (per-UE) in Rel-17 (revisit previous RAN2 agreement).

Note: The following proposal from AI summary will be discussed under Offline-109:

Proposal 13: When the type of measured SSB (e.g. CD-SSB, NCD-SSB) is changed, the UE shall set the value of SrxlevRefStationaryConnected to the current Srxlev value of the serving cell.

## Other

Besides the questions listed in previous section, if companies think there are other NCD-SSB related issues that need further discussion, please elaborate it in below table.

Q16: Any other NCD-SSB related issues?

| Company | Comments |
| --- | --- |

# Phase 1 Conclusion

To facilitate online discussion, rapporteur reorders the proposal as below:

Proposals for easy agreements:

Proposal 4: (16/17) For the first measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings.

- CD-SSB and/or multiple NCD-SSBs can be configured for serving cell measurements, with the restriction that only one SSB (CD-SSB or NCD-SSB) can be used for serving cell measurements at a given time.

- The SSB used for serving cell measurements may be changed upon BWP switching.

- The outcome of Proposal 1.

Proposal 5: (16/17) For the second measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings.

- From RAN2 signalling point of view, the NW and the UE always know which SSB is the reference SSB to be used for serving cell measurements at a given point in time. This reference SSB can be used to define intra-frequency measurements. This reference SSB may change with BWP switching.

Proposal 6: (16/17) From RAN2 perspective, handover scenario 1 is supported.

- Scenario 1: Handover to a target cell’s specific Redcap BWP associated with NCD-SSB directly other than to the initial BWP associated with CD-SSB (i.e. UE directly sync to the NCD-SSB and perform RACH on that BWP)

Proposal 8: (15/17) From RAN2 perspective, handover scenario 2 is not supported.

- Scenario 2: Handover to a target cell’s initial BWP and further switch to the specific Redcap BWP to send the RACH (i.e. UE first sync to the CD-SSB and then autonomously switch to first active BWP to perform RACH)

Proposal 9: (17/17)During handover, if dedicatedSIB1-Delivery IE is not included in the handover command and the first active BWP in the target cell does not contain CD-SSB, UE is expected to acquire SI either from CSS for SIBs or via dedicated signaling only within the first active BWP.

Proposal 10: (17/17)RAN2 confirm that RedCap UEs in RRC Connected only need to support the following three options for acquiring SI update or ETWS/CMAS message in a dedicated DL BWP that does not contain CD-SSB:

- From CSS for SIBs configured within this DL BWP;

- Via dedicated signaling;

- Switched by network (either DCI or RRC) to an initial DL BWP where SIBs are sent.

Proposal 11: (15/16) Clarify in the RRC field description that the paging search space is configured in an initial BWP only if that BWP includes the CD-SSB.

Proposal 12: (15/15) RAN2 confirms for separate initial DL BWP does not contain CD-SSB and CORESET#0, the BWP will not be configured for paging no matter the RedCap UE is in RRC_IDLE/INACTIVE or RRC_CONNECTED state

Proposal 13: (15/15) Reply to RAN1 and explain there is no need to support paging connected RedCap UEs in case the separate initial DL BWP does not contain CD-SSB and CORESET#0.

Proposal 14: (17/17) In Rel-17, from UE perspective, one configured BWP can only contain up to one SSB (CD-SSB or NCD-SSB).

Proposals for online discussion:

Proposal 1: (15/18) RAN2 confirms when RedCap UE’s activate BWP contains NCD-SSB, the UE can be configured to perform serving cell measurements on CD-SSB.

Proposal 2: (11 vs 5 vs 2) For how to indicate serving cell MO for RedCap UE, to adopt solution A-1.

    - Solution A-1: Reuse existing servingCellMO IE (based on the assumption that RRCReconfiguration is always needed (e.g. to reconfigure UE CBW) when switching from a BWP associated with CD-SSB and a BWP associated with NCD-SSB).

Proposal 3: (13/17) For neighbour cell measurements, the existing RRM mechanism is applied, further enhancement is not needed.

Proposal 7: (14/17) For scenario 1, in handover command, if the first active BWP is associated with NCD-SSB, the smtc field included reconfigurationWithSync is configured according to the NCD-SSB of target cell.

Proposal 15: To discuss whether only one NCD-SSB can be configured (per-UE) in Rel-17 (revisit previous RAN2 agreement).

# Phase 2 Discussion

[AT118-e][105][RedCap] NCD-SSB aspects (ZTE)

Scope: Continue the discussion on NCD-SSB aspects, including a possible reply LS to R2-2204486

Intended outcome: Summary of the offline discussion with e.g.:

         Text/proposals for a possible reply LS to R2-2204486

         List of proposals for agreement (if any)

         List of proposals that require online discussions

         List of proposals that should not be pursued (if any)

Deadline (for companies' feedback):  Friday 2022-05-13 02:00 UTC

Deadline (for rapporteur's summary in R2-2206204):  Friday 2022-05-13 04:00 UTC

Proposals marked "for agreement" in R2-2206204 not challenged until Friday 2022-05-13 16:00 UTC will be declared as agreed via email by the session chair (for the rest the discussion might continue offline).

## Guidance (Read Me)

Based on online discussion on Tuesday, some companies have concern on the wording of proposals, some companies think the proposals may need update when other proposal is agreed or not agreed (e.g. handover scenario 2, 1 NCD-SSB).

To make progress and to avoid companies to repeat their comments in phase 2, rapporteur suggests focus on the wording of proposals in phase 2. Of course, technical comments are allowed if you have changed your position or if you did not join phase 1 discussion.

Therefore, the discussion splits into two parts:

- Technical discussion:

  - Please provide your feedback only if you have changed your views expressed in phase1 (e.g. support -> not support, or not support-> support). Or if you did not join phase 1 discussion.

- Wording refinement:

  - Please comment if you have concern on the wording and please provide your preferred wording.

  - Rapporteur will update the wording based on received comments, so please pay attention to the update and provide comments if any.

## Technical discussion

The remaining phase 1 proposals listed below (P10 is already agreed):

For serving cell measurements Proposal 1: (15/18) RAN2 confirms when RedCap UE’s activate BWP contains NCD-SSB, the UE can be configured to perform serving cell measurements on CD-SSB. Proposal 2: (11 vs 5 vs 2) For how to indicate serving cell MO for RedCap UE, to adopt solution A-1. - Solution A-1: Reuse existing servingCellMO IE (based on the assumption that RRCReconfiguration is always needed (e.g. to reconfigure UE CBW) when switching from a BWP associated with CD-SSB and a BWP associated with NCD-SSB). For neighbour cell measurements Proposal 3: (13/17) For neighbour cell measurements, the existing RRM mechanism is applied, further enhancement is not needed. About reply LS to R2-2204486 Proposal 4: (16/17) For the first measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings. - CD-SSB and/or multiple NCD-SSBs can be configured for serving cell measurements, with the restriction that only one SSB (CD-SSB or NCD-SSB) can be used for serving cell measurements at a given time. - The SSB used for serving cell measurements may be changed upon BWP switching. - The outcome of Proposal 1. Proposal 5: (16/17) For the second measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings. - From RAN2 signalling point of view, the NW and the UE always know which SSB is the reference SSB to be used for serving cell measurements at a given point in time. This reference SSB can be used to define intra-frequency measurements. This reference SSB may change with BWP switching. Proposal 6: (16/17) From RAN2 perspective, handover scenario 1 is supported. - Scenario 1: Handover to a target cell’s specific Redcap BWP associated with NCD-SSB directly other than to the initial BWP associated with CD-SSB (i.e. UE directly sync to the NCD-SSB and perform RACH on that BWP) Proposal 7: (14/17) For scenario 1, in handover command, if the first active BWP is associated with NCD-SSB, the smtc field included reconfigurationWithSync is configured according to the NCD-SSB of target cell. Proposal 8: (15/17) From RAN2 perspective, handover scenario 2 is not supported. - Scenario 2: Handover to a target cell’s initial BWP and further switch to the specific Redcap BWP to send the RACH (i.e. UE first sync to the CD-SSB and then autonomously switch to first active BWP to perform RACH) For SIB reception in BWP without CD-SSB Proposal 9: (17/17)During handover, if dedicatedSIB1-Delivery IE is not included in the handover command and the first active BWP in the target cell does not contain CD-SSB, UE is expected to acquire SI either from CSS for SIBs or via dedicated signaling only within the first active BWP. For Paging reception in BWP without CD-SSB Proposal 11: (15/16) Clarify in the RRC field description that the paging search space is configured in an initial BWP only if that BWP includes the CD-SSB. Proposal 12: (15/15) RAN2 confirms for separate initial DL BWP does not contain CD-SSB and CORESET#0, the BWP will not be configured for paging no matter the RedCap UE is in RRC_IDLE/INACTIVE or RRC_CONNECTED state Proposal 13: (15/15) Reply to RAN1 and explain there is no need to support paging connected RedCap UEs in case the separate initial DL BWP does not contain CD-SSB and CORESET#0. For number of NCD-SSBs Proposal 14: (17/17) In Rel-17, from UE perspective, one configured BWP can only contain up to one SSB (CD-SSB or NCD-SSB). Proposal 15: To discuss whether only one NCD-SSB can be configured (per-UE) in Rel-17 (revisit previous RAN2 agreement).

Q2-1: Any technical comments on above proposals?

- Please provide your feedback if you have changed your view (e.g. support -> not support, or not support-> support), or if you did not join phase 1 discussion.

- For wording improvement, please provide your comments to section 5.3.

| Company | Which proposal(s)? | Comments |
| --- | --- | --- |
| BT | Proposal 4: not support with current wording | Bullet 2 says, “the SSB used for serving cell measurements may be changed upon BWP switching”, but may be change by who? It does not clarify who is responsible to perform the change. Since FR1 goes up to 100 MHz and FR2 up to 400 MHz, it seems reasonable that network can indicate the UE the SSBs that’s needs to be measured, e.g. network can configure the UE to the (NCD- or CD-) SSB of the active BWP. If it is not indicated, the UE have the freedom to do the change or not upon BWP switching. [Rapp-ZTE] Thanks for the comment, I agree that “may be changed” is a bit misleading and with implications. I have updated P4 based on the proposal from QC in Q2-4. |
| Xiaomi | P7/P8 | We agree with P7. But for P8, we still have a question: For P7: •Scenario 1: Handover to a target cell’s specific Redcap BWP associated with NCD-SSB directly other than to the initial BWP associated with CD-SSB (i.e. UE directly sync to the NCD-SSB and perform RACH on that BWP) Here, I think "target cell’s specific Redcap BWP" is the firstActive BWP (which is the BWP with NCD-SSB in this scenario). [Rapp-ZTE] Yes, your understanding is correct. If Scenario 2 is not supported, does that mean the NW is mandatary to provide a firstActive BWP and then mandatary to provide the NCD-SSB in the firstActive BWP? [Rapp-ZTE] No, firstly, firstly, in existing spec, firstActiveBWP ID is mandatory provided upon reconfigurationWithSync, same for RedCap UE. It is possible the firstActiveBWP still contains CD-SSB, not NCD-SSB. If so, how about UE that do not need NCD-SSB as RAN1'Ls says: - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities [FG 6-1a] with supporting CSI-RS, or [FG 6-1a] without supporting CSI-RS. And for Scenario 2 - Scenario 2: Handover to a target cell’s initial BWP and further switch to the specific Redcap BWP to send the RACH (i.e. UE first sync to the CD-SSB and then autonomously switch to first active BWP to perform RACH) And we are not sure what “Handover to a target cell’s initial BWP” really means. Is the Redcap initial DL BWP? How about the case that Redcap initial DL BWP not containing CD-SSB and CORESET#0? Do UE still need to use the general initial DL BWP for sync/measurement and then performs the RA on the Redcap initial DL BWP? [Rapp-ZTE] I would like to say, the first sentence is copied from RAN4 LS in R2-2204486. The term “target cell’s initial BWP” means legacy initial BWP. |

Summary:

Two companies provide feedbacks, mainly on the wording or clarification on the scenario.

Rapporteur already provides feedback, so no proposal is provided.

## Wording refinement

### P1~P2

For serving cell measurements Proposal 1: (15/18) RAN2 confirms when RedCap UE’s activate BWP contains NCD-SSB, the UE can be configured to perform serving cell measurements on CD-SSB. Proposal 2: (11 vs 5 vs 2) For how to indicate serving cell MO for RedCap UE, to adopt solution A-1. - Solution A-1: Reuse existing servingCellMO IE (based on the assumption that RRCReconfiguration is always needed (e.g. to reconfigure UE CBW) when switching from a BWP associated with CD-SSB and a BWP associated with NCD-SSB).

Q2-2: Any comments on wording of proposal 1~2?

| Company | Which proposal(s)? | Comments and your preferred wording. |
| --- | --- | --- |
| Qualcomm | Proposal 1 | RAN2 confirms that when RedCap UE’s active BWP contains NCD-SSB, it is up to network configuration whether the UE performs serving cell measurements on NCD-SSB or CD-SSB. [Rapp-ZTE]: OK. |
| Qualcomm | Proposal 2 | Solution A-1: existing servingCellMO IE under ServingCellConfig is used by RedCap UE in any active BWP. No additional servingCellMO is supported in R17. [Rapp-ZTE]: OK. |
| Rapp-ZTE | Updated P1 and P2 | The proposal from QC is more concise, P1 and P2 are updated as below: Proposal 1: (15/18) RAN2 confirms that when RedCap UE’s active BWP contains NCD-SSB, it is up to network configuration whether the UE performs serving cell measurements on NCD-SSB or CD-SSB. Proposal 2: (11 vs 5 vs 2) For how to indicate serving cell MO for RedCap UE, to adopt solution A-1. - Solution A-1: Existing servingCellMO IE under ServingCellConfig is used by RedCap UE in any active BWP. No additional servingCellMO is supported in R17. Please comment based on this version. |
| vivo | Proposal 1 | Fine with Qualcomm’s re-wording. But similar to the first round, considering this question is asked from network configuration point of view, we assume it is possible. We think it is possible at the early stage of deployment, but it would be a corner case in mature deployment, where NCD-SSB could be used the same as CD-SSB during measurement. [Rapp-ZTE] Thanks for the comment, honestly, I am not sure this is case for RedCap, because RedCap may deploy for specific devices (e.g. machines in factory). Of course, if all neighbour cells have deployed NCD-SSB, the network will configure UE to use NCD-SSB for both serving and neighbour cell measurements. ; ) |
| Huawei, HiSilicon | Proposal 2 | Proposal 2: (11 vs 5 vs 2) For how to indicate serving cell MO for RedCap UE, to adopt solution A-1, assuming only the RRC based BWP switching is supported as the baseline. - Solution A-1: Existing servingCellMO IE under ServingCellConfig is used by RedCap UE in any active BWP (can be CD-SSB or NCD-SSB). No additional servingCellMO is supported in R17. [Rapp-ZTE] Thanks for willing to compromise. But the added first sentence is not 100% correct, because DCI-based BWP switch is support when the target BWP is within 20MHz bandwidth. Based on the comments received so far, I am afraid it is hard to converge the wording if BWP switch is mentioned explicitly, So I decide to put all A series solutions to Proposal 2, let us discuss further online. In addition, based on the discussion on RAN2 reflector, I will provide a separate proposal (P16) for UE channel BW and BWP switch. |
| Rapp-ZTE | Updated P2 | Proposal 2: (11 vs 5 vs 2) For how to indicate serving cell MO for RedCap UE, to choose one of following options: - Solution A-1: Existing servingCellMO IE under ServingCellConfig (can be CD-SSB or NCD-SSB) is used by RedCap UE in any active BWP. No additional servingCellMO is supported in R17. - Solution A-2: Optionally configures a BWP-specific servingCellMO under BWP-DownlinkDedicated IE when the BWP-DownlinkDedicated contains nonCellDefiningSSB-r17. If the field is present, the UE uses this servingCellMO, otherwise, the UE uses legacy servingCellMO IE under ServingCellConfig. - Solution A-3: Optionally configures servingCellMO-List-NCD-r17 in ServingCellConfig, each entry links to one NCD-SSB MO, the UE selects corresponding servingCellMO upon BWP switching. If UE’s active BWP is configured with NCD-SSB but the NCD-SSB MO is NOT included in servingCellMO-List-NCD-r17, the UE uses legacy servingCellMO IE under ServingCellConfig. This proposal will be marked as “for online discussion”. Please comment based on this version. |
| Rapp-ZTE | New Proposal 16 on UE channel BW configuration; New proposal 17 on feasibility of BWP switch cases | Proposal 16: To discuss whether the network can configure UE specific channel bandwidth larger than UE maximum supported channel bandwidth (send LS to RAN1&4 if no consensus in RAN2). Proposal 17: To discuss whether DCI-based BWP switch, timer-based BWP switch, RA-based BWP switch can be supported if target BWP is outside UE’s operating bandwidth which used on source BWP (send LS to RAN1&4 if no consensus in RAN2). Please comment based on this version. |
| Ericsson | Proposal 1 | Agree with QC’s proposal in principle. Here’s an editorial suggestion to improve readability (also considering the suggestion from HW): “RAN2 confirms that when active BWP of a RedCap UE is associated with NCD-SSB, it is up to network to configure the UE to perform serving cell measurements on NCD-SSB or CD-SSB.” |
| Ericsson | Proposal 2 | Agree with QC’s proposal in principle. Here’s an editorial suggestion to improve readability: “A RedCap UE uses the existing servingCellMO IE in ServingCellConfig to perform serving cell measurements in an active BWP regardless of whether there is any SSB, or it is CD-SSB or NCD SSB.” |

Summary:

Based on the received comments and offline discussion with companies, rapporteur suggests to adopt “Proposal 1 + Solution A-2” as a way forward, and this can also decouple the discussion on UE CBW configuration and BWP switch mechanism.

Regarding UE CBW configuration and BWP switch mechanism, due to the limit time in this meeting, rapporteur suggest to postpone the discussion, if companies have concern, please raise it in future meeting. Therefore, Proposal 16 and 17 are withdrawed.

Proposal 1: (15/18) RAN2 confirms that when RedCap UE’s active BWP contains NCD-SSB, it is up to network configuration whether the UE performs serving cell measurements on NCD-SSB or CD-SSB.

Proposal 2: For how to indicate serving cell MO for RedCap UE, to adopt solution A-2.

    - Solution A-2: Optionally configures a BWP-specific servingCellMO under BWP-DownlinkDedicated IE when the BWP-DownlinkDedicated contains nonCellDefiningSSB-r17. If the field is present, the UE uses this servingCellMO for serving cell measurements, otherwise, the UE uses legacy servingCellMO IE under ServingCellConfig.

### P3

For neighbour cell measurements Proposal 3: (13/17) For neighbour cell measurements, the existing RRM mechanism is applied, further enhancement is not needed.

Q2-3: Any comments on wording of proposal 3?

| Company | Comments and your preferred wording. |
| --- | --- |
| Huawei, HiSilicon | This is conditional proposal. Only if P2 is agreed, this can be easy proposal. [Rapp-ZTE] This will be marked as “for discussion”. |

Summary:

Based on the feedback, same as in round 1, the proposal remains the same, but it will be marked as to discuss.

Proposal 3: (13/17) For neighbour cell measurements, the existing RRM mechanism is applied, further enhancement is not needed.

### P4~P8

About reply LS to R2-2204486 Proposal 4: (16/17) For the first measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings. - CD-SSB and/or multiple NCD-SSBs can be configured for serving cell measurements, with the restriction that only one SSB (CD-SSB or NCD-SSB) can be used for serving cell measurements at a given time. - The SSB used for serving cell measurements may be changed upon BWP switching. - The outcome of Proposal 1. Proposal 5: (16/17) For the second measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings. - From RAN2 signalling point of view, the NW and the UE always know which SSB is the reference SSB to be used for serving cell measurements at a given point in time. This reference SSB can be used to define intra-frequency measurements. This reference SSB may change with BWP switching. Proposal 6: (16/17) From RAN2 perspective, handover scenario 1 is supported. - Scenario 1: Handover to a target cell’s specific Redcap BWP associated with NCD-SSB directly other than to the initial BWP associated with CD-SSB (i.e. UE directly sync to the NCD-SSB and perform RACH on that BWP) Proposal 7: (14/17) For scenario 1, in handover command, if the first active BWP is associated with NCD-SSB, the smtc field included reconfigurationWithSync is configured according to the NCD-SSB of target cell. Proposal 8: (15/17) From RAN2 perspective, handover scenario 2 is not supported. - Scenario 2: Handover to a target cell’s initial BWP and further switch to the specific Redcap BWP to send the RACH (i.e. UE first sync to the CD-SSB and then autonomously switch to first active BWP to perform RACH)

Q2-4: Any comments on wording of proposal 4~8?

| Company | Which proposal(s)? | Comments and your preferred wording. |
| --- | --- | --- |
| Qualcomm | Proposal 4 | Proposal 4: (16/17) For the first measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings. • Either CD-SSB or NCD-SSBs can be configured for serving cell measurements, with the restriction that only one SSB (CD-SSB or NCD-SSB) can be used for serving cell measurements at a given time. • The SSB used for serving cell measurements may be RRC reconfigured upon BWP switching. • The outcome of Proposal 1. [Rapp-ZTE] Ok, thanks. |
| Qualcomm | Proposal 5 | Proposal 5: (16/17) For the second measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings. • From RAN2 signalling point of view, the SSB defined in servingCellMO under ServingCellConfig is always the reference SSB to be used for serving cell measurements at a given point in time. This reference SSB is used to define intra-frequency measurements. This reference SSB may change after the UE’s active BWP is switched or reconfigured. [Rapp-ZTE] Ok, for the last sentence, it duplicated with the 2nd bullet in P4, so I remove it, please check. |
| BT | Proposal 4 | The SSB used for serving cell measurements may be implicitly changed by NW upon BWP switching or reconfiguration. We are also fine with QC proposal [Rapp-ZTE] Thanks, P4 is updated based on QC’s proposal. |
| BT | Proposal 6 | Scenario 1: Handover to a target cell’s specific Redcap BWP associated with NCD-SSB directly other than besides to the initial BWP associated with CD-SSB (i.e. UE directly sync to the NCD-SSB and perform RACH on that BWP). [Rapp-ZTE] Ok. |
| Rapp-ZTE | Updated P4, 5, 6 | Proposal 4: (16/17) For the first measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings. - Either CD-SSB or NCD-SSB can be configured for serving cell measurements, with the restriction that only one SSB (CD-SSB or NCD-SSB) can be used for serving cell measurements at a given time. - The SSB used for serving cell measurements may be RRC reconfigured upon BWP switching. - The outcome of Proposal 1. Proposal 5: (16/17) For the second measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings. - From RAN2 signalling point of view, SSB defined in servingCellMO under ServingCellConfig is always the reference SSB to be used for serving cell measurements at a given point in time. This reference SSB is used to define intra-frequency measurements. Proposal 6: (16/17) From RAN2 perspective, handover scenario 1 is supported. - Scenario 1: Handover to a target cell’s specific Redcap BWP associated with NCD-SSB besides to the initial BWP associated with CD-SSB (i.e. UE directly sync to the NCD-SSB and perform RACH on that BWP) For P4, P5, P6, please comment based on this version. |
| OPPO | P4 | For the updated P4, regarding the bullet: - The SSB used for serving cell measurements may be RRC reconfigured upon BWP switching. probably the following consequence needs to be captured. Upon BWP switching, the SSB used for serving cell measurements remains unchanged until reconfigured by the network, which means that UE autonomous change for the SSB used for serving cell measurements is not supported. [Rapp-ZTE] The updated P2 includes other solutions, so I will put a bracket to 2nd bullet, see below update. |
| vivo | Proposal 4 | 2nd bullet in P4: The SSB used for serving cell measurements may be changed by NW upon BWP switching or reconfiguration. [Rapp-ZTE] Same comment as to OPPO. |
| Huawei, HiSilicon | P4/5 | Based on the above comment from OPPO/vivo, it is important to include the P2 in the LS to explain the reason why “From RAN2 signalling point of view, SSB defined in servingCellMO under ServingCellConfig is always the reference SSB to be used for serving cell measurements at a given point in time.” Proposal 4: (16/17) For the first measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings. - Either CD-SSB or NCD-SSB can be configured for serving cell measurements, with the restriction that only one SSB (CD-SSB or NCD-SSB) can be used for serving cell measurements at a given time. - The SSB used for serving cell measurements may be RRC reconfigured upon BWP switching. - The outcome of Proposal 1. - The outcome of Proposal 2. Proposal 5: (16/17) For the second measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings, assuming only the RRC based BWP switching is supported as the baseline. - From RAN2 signalling point of view, SSB defined in servingCellMO under ServingCellConfig is always the reference SSB to be used for serving cell measurements at a given point in time. This reference SSB is used to define intra-frequency measurements. [Rapp-ZTE] For P4, see below update, For P5, same comment as to P2. |
| Huawei, HiSilicon | P8 | It is important to clarify the legacy manner (directly sync to CD-SSB and RACH on that BWP) is not excluded. Proposal 8: (15/17) From RAN2 perspective, handover scenario 2 is not supported. Scenario 2: Handover to a target cell’s initial BWP and further switch to the specific Redcap BWP to send the RACH (i.e. UE first sync to the CD-SSB and then autonomously switch to first active BWP (the specific Redcap BWP) to perform RACH) [Rapp-ZTE] The Scenario 2 from RAN4 LS does not include legacy manner. So change is not necessary, let’s focus on the scenario asked by RAN4. |
| Rapp-ZTE | Updated P4, 5 | Proposal 4: (16/17) For the first measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings. - Either CD-SSB or NCD-SSB can be configured for serving cell measurements, with the restriction that only one SSB (CD-SSB or NCD-SSB) can be used for serving cell measurements at a given time. - The SSB used for serving cell measurements may be RRC reconfigured upon BWP switching.(can update based on the outcome of Proposal 2) - The outcome of Proposal 1. Proposal 5: (16/17) For the second measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings. - From RAN2 signalling point of view, SSB defined in servingCellMO under ServingCellConfig is always the reference SSB to be used for serving cell measurements at a given point in time. This reference SSB is used to define intra-frequency measurements. (can update based on the outcome of Proposal 2) Please comment based on this version. |
| Ericsson | Proposal 4 | Agree with rapporteur’s formulation of the first bullet. Here’s a suggestion to improve the readability: - Either CD-SSB or NCD-SSB can be configured for performing serving cell measurements, with the restriction that only one SSB (CD-SSB or NCD-SSB) can be used for serving cell measurements at a given time. It is not clear to us what message RAN2 would try to convey with he way bullet 2 is currently formulated. There is an ongoing discussion regarding whether e.g., DCI-based or timer-based BWP switching is (or should be) supported, and we assume that the intention with the second bullet is to point at the alternative mechanism, i.e., BWP switching via RRC reconfiguration, which can justify the outcome of the discussion on Proposal 2. The way bullet 2 is currently formulated can be confusing for RAN4 since its association to currently possible signaling captured in ASN.1 may not be clear. We think that it would be better to wait until the discussion on Proposal 2 is concluded before formulating this bullet. |

Summary:

Proposal 4 and Proposal 5 are updated based on P1 and P2.

Proposal 4: (16/17) For the first measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings.

- Either CD-SSB or NCD-SSB can be configured for serving cell measurements, with the restriction that only one SSB (CD-SSB or NCD-SSB) can be used for serving cell measurements at a given time.

- The SSB used for serving cell measurements is configured by the network, the SSB can be different when the UE in different active BWPs.

- When RedCap UE’s active BWP contains NCD-SSB, it is up to network configuration whether the UE performs serving cell measurements on NCD-SSB or CD-SSB.

Proposal 5: (16/17) For the second measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings.

- From RAN2 signalling point of view, a BWP-specific servingCellMO is defined under BWP-DownlinkDedicated, the SSB defined in this servingCellMO is the reference SSB to be used for serving cell measurements when the UE is in this active BWP; if the field is absent, SSB defined in servingCellMO under ServingCellConfig is the reference SSB to be used for serving cell measurements. This reference SSB is used to define intra-frequency measurements.

The wording of P6 is update based on QC’s comments, P7/P8 remain the same.

Proposal 6: (16/17) From RAN2 perspective, handover scenario 1 is supported.

- Scenario 1: Handover to a target cell’s specific Redcap BWP associated with NCD-SSB besides to the initial BWP associated with CD-SSB (i.e. UE directly sync to the NCD-SSB and perform RACH on that BWP)

Proposal 7: (14/17) For scenario 1, in handover command, if the first active BWP is associated with NCD-SSB, the smtc field included reconfigurationWithSync is configured according to the NCD-SSB of target cell.

Proposal 8: (15/17) From RAN2 perspective, handover scenario 2 is not supported.

- Scenario 2: Handover to a target cell’s initial BWP and further switch to the specific Redcap BWP to send the RACH (i.e. UE first sync to the CD-SSB and then autonomously switch to first active BWP to perform RACH)

### P9

For SIB reception in BWP without CD-SSB Proposal 9: (17/17)During handover, if dedicatedSIB1-Delivery IE is not included in the handover command and the first active BWP in the target cell does not contain CD-SSB, UE is expected to acquire SI either from CSS for SIBs configured in this DL BWP or via dedicated signaling only within the first active BWP.

Based on online comments, rapporteur updated the wording, add “configured in this DL BWP” to align with agreed P10.

Q2-5: Any comments on wording of proposal 9?

| Company | Comments and your preferred wording. |
| --- | --- |
| Qualcomm | We believe the following was what MediaTek suggested, which we think is indeed more precise than the original wording: During handover, if dedicatedSIB1-Delivery IE is not included in the handover command and the first active BWP in the target cell does not contain CD-SSB, UE is expected to acquire SI only within this first active BWP either from CSS for SIBs or via dedicated signaling. [Rapp-ZTE] Actually, I was referring to the online comment from Intel, not MTK, but OK with the change. ; ) |
| [Rapp-ZTE] | Proposal 9: (17/17)During handover, if dedicatedSIB1-Delivery IE is not included in the handover command and the first active BWP in the target cell does not contain CD-SSB, UE is expected to acquire SI only within the first active BWP either from CSS for SIBs or via dedicated signaling. Please comment based on this version. |
| vivo | We would like to clarify it is legacy behavior, so the suggested wording: During handover, if dedicatedSIB1-Delivery IE is not included in the handover command and the first active BWP in the target cell does not contain CD-SSB, UE is expected to acquire SI only within the first active BWP either from CSS for SIBs or via dedicated signaling, as legacy. [Rapp-ZTE] As clarified by QC during online, the two actions are legacy behaivour, but the entire proposal is not as legacy, because it excludes the BWP-switch option, which does not make sense to use during handover procedure. |
| Huawei, HiSilicon | This is conditional proposal. Only if P8 is agreed, this can be easy proposal. [Rapp-ZTE] Both P8 and P9 are supported by large majority, I suggest we focus on important things, otherwise, we don’t have time to discuss other remaining issues. |
| Ericsson | It is also possible that the network sees no need to provide SIB(s), so this proposal should not be interpreted as if it is mandatory for the network to provide such information. We suggest the following: “During handover, if dedicatedSIB1-Delivery IE is not included in the handover command and the first active BWP in the target cell is not associated with does not contain CD-SSB, UE may is expected to acquire SI only within the first active BWP either from CSS for SIBs or via dedicated signaling.” |

Summary:

Proposal 9: (17/17)During handover, if dedicatedSIB1-Delivery IE is not included in the handover command and the first active BWP in the target cell does not contain CD-SSB, UE is expected to acquire SI only within the first active BWP either from CSS for SIBs or via dedicated signaling.

### P11~P13

For Paging reception in BWP without CD-SSB Proposal 11: (15/16) Clarify in the RRC field description that the paging search space is configured in an initial BWP only if that BWP includes the CD-SSB. Proposal 12: (15/15) RAN2 confirms for separate initial DL BWP does not contain CD-SSB and CORESET#0, the BWP will not be configured for paging no matter the RedCap UE is in RRC_IDLE/INACTIVE or RRC_CONNECTED state Proposal 13: (15/15) Reply to RAN1 and explain there is no need to support paging connected RedCap UEs in case the separate initial DL BWP does not contain CD-SSB and CORESET#0.

Q2-6: Any comments on wording of proposal 11~13?

| Company | Which proposal(s)? | Comments and your preferred wording. |
| --- | --- | --- |
| Qualcomm | Proposal 12 | RAN2 confirms that if aRedCap-specific initial DL BWP does not contain CD-SSB and CORESET#0, then this BWP will not be configured with a paging search space in any RRC state. [Rapp-ZTE] Ok. |
| Intel | Proposal 12 | Based on companies’ explanation, for RRC_CONNECTED UE, if active BWP does not contain the CD-SSB and CORESET#0, then the RedCap UE is not required to monitor paging. It should be captured in proposals, otherwise companies may interpret this as same as IDLE/INACTIVE, the RedCap UE needs to monitor paging via BWP with CD-SSB and CORESET#0. Therefore we propose to update the P12 as Proposal 12: (15/15) RAN2 confirms for separate initial DL BWP does not contain CD-SSB and CORESET#0, the BWP will not be configured for paging no matter the RedCap UE is in RRC_IDLE/INACTIVE or RRC_CONNECTED state. The RedCap UE in RRC_CONNECTED state is not required to read paging if paging is not configured in the active BWP. [Rapp-ZTE] I understand your concern, I have revised the sentence a bit, see below. |
| BT | Proposal 12 | We are fine with QC’s wording |
| Rapp-ZTE | Updated P12 | Proposal 12: (15/15) RAN2 confirms that if RedCap-specific initial DL BWP does not contain CD-SSB and CORESET#0, then this BWP will not be configured with a paging search space in any RRC state. In this case, the RedCap UE in RRC_CONNECTED state is not required to read paging. Please comment based on this version. |
| vivo | Proposal 12 | Prefer wording suggested by Intel, especially for “if paging is not configured in the active BWP”. [Rapp-ZTE] The first sentence already mentions “this BWP will not be configured for paging”, so I use “in this case” to avoid duplication. I think there is no room for misunderstanding, right? |
| Huawei, HiSilicon | P12 | Not wording comment, but just one general comment. Does P12 have additional spec impact than P11? If not, we do not even need to agree this, since we are already in the CR phase. If yes, it is better to discuss this based on TP. [Rapp-ZTE] P12 has no additional spec impact, but it is important to confirm the understanding in RAN2, because we need to send it back to RAN1 in the reply LS. |
| Ericsson | Proposal 11 | Editorial suggestion: “Clarify in the RRC field description that the paging search space is configured in an initial BWP only if that BWP is associated with includes the CD-SSB.” |
| Ericsson | Proposal 12 | Please see below for comments: RAN2 confirms that if RedCap-specific initial DL BWP does not contain CD-SSB and CORESET#0, then this BWP will not be configured with a paging search space in any RRC state. In this case, the RedCap UE in RRC_CONNECTED state is not required to read paging. The proposed text in red does not seem to capture the behavior in general since the text only refers to the case when RedCap-specific initial DL BWP does not contain CD-SSB and CORESET#0. How about the case when UE is configured with a gap to perform measurements based on BWP associated with CD-SSB and contains CORESET#0? We should either remove the text in red, and rely on implicit interpretation or improve the text to capture the behavior considering other cases. |

Summary:

Proposal 11: (15/16) Clarify in the RRC field description that the paging search space is configured in an initial BWP only if that BWP includes the CD-SSB.

Proposal 12: (15/15) RAN2 confirms that if RedCap-specific initial DL BWP does not contain CD-SSB and CORESET#0, then this BWP will not be configured with a paging search space in any RRC state. In this case, the RedCap UE in RRC_CONNECTED state is not required to read paging.

Proposal 13: (15/15) Reply to RAN1 and explain there is no need to support paging connected RedCap UEs in a RedCap-specific initial DL BWP which does not contain CD-SSB and CORESET#0.

### P14~P15

For number of NCD-SSBs Proposal 14: (17/17) In Rel-17, from UE perspective, one configured BWP can only contain up to one SSB (CD-SSB or NCD-SSB). Proposal 15: To discuss whether only one NCD-SSB can be configured (per-UE) in Rel-17 (revisit previous RAN2 agreement).

Q2-7: Any comments on wording of proposal 14~15?

| Company | Which proposal(s)? | Comments and your preferred wording. |
| --- | --- | --- |

Summary:

Proposal 14: (17/17) In Rel-17, from UE perspective, one configured BWP can only contain up to one SSB (CD-SSB or NCD-SSB).

Proposal 15: To discuss whether only one NCD-SSB can be configured (per-UE) in Rel-17 (revisit previous RAN2 agreement).

# Phase 2 Conclusion

Proposals for easy agreements:

Proposal 1: RAN2 confirms that when RedCap UE’s active BWP contains NCD-SSB, it is up to network configuration whether the UE performs serving cell measurements on NCD-SSB or CD-SSB.

Proposal 2: For how to indicate serving cell MO for RedCap UE, to adopt solution A-2.

    - Solution A-2: Optionally configures a BWP-specific servingCellMO under BWP-DownlinkDedicated IE when the BWP-DownlinkDedicated contains nonCellDefiningSSB-r17. If the field is present, the UE uses this servingCellMO for serving cell measurements, otherwise, the UE uses legacy servingCellMO IE under ServingCellConfig.

Proposal 4: (16/17) For the first measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings.

- Either CD-SSB or NCD-SSB can be configured for serving cell measurements, with the restriction that only one SSB (CD-SSB or NCD-SSB) can be used for serving cell measurements at a given time.

- The SSB used for serving cell measurements is configured by the network, the SSB can be different when the UE in different active BWPs.

- When RedCap UE’s active BWP contains NCD-SSB, it is up to network configuration whether the UE performs serving cell measurements on NCD-SSB or CD-SSB.

Proposal 5: (16/17) For the second measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings.

- From RAN2 signalling point of view, a BWP-specific servingCellMO is defined under BWP-DownlinkDedicated, the SSB defined in this servingCellMO is the reference SSB to be used for serving cell measurements when the UE is in this active BWP; if the field is absent, SSB defined in servingCellMO under ServingCellConfig is the reference SSB to be used for serving cell measurements. This reference SSB is used to define intra-frequency measurements.

Proposal 6: (16/17) From RAN2 perspective, handover scenario 1 is supported.

- Scenario 1: Handover to a target cell’s specific Redcap BWP associated with NCD-SSB besides to the initial BWP associated with CD-SSB (i.e. UE directly sync to the NCD-SSB and perform RACH on that BWP)

Proposal 8: (15/17) From RAN2 perspective, handover scenario 2 is not supported.

- Scenario 2: Handover to a target cell’s initial BWP and further switch to the specific Redcap BWP to send the RACH (i.e. UE first sync to the CD-SSB and then autonomously switch to first active BWP to perform RACH)

Proposal 9: (17/17)During handover, if dedicatedSIB1-Delivery IE is not included in the handover command and the first active BWP in the target cell does not contain CD-SSB, UE is expected to acquire SI only within the first active BWP either from CSS for SIBs or via dedicated signaling.

Proposal 11: (15/16) Clarify in the RRC field description that the paging search space is configured in an initial BWP only if that BWP includes the CD-SSB.

Proposal 12: (15/15) RAN2 confirms that if RedCap-specific initial DL BWP does not contain CD-SSB and CORESET#0, then this BWP will not be configured with a paging search space in any RRC state. In this case, the RedCap UE in RRC_CONNECTED state is not required to read paging.

Proposal 13: (15/15) Reply to RAN1 and explain there is no need to support paging connected RedCap UEs in a RedCap-specific initial DL BWP which does not contain CD-SSB and CORESET#0.

Proposal 14: (17/17) In Rel-17, from UE perspective, one configured BWP can only contain up to one SSB (CD-SSB or NCD-SSB).

Proposals for online discussion:

Proposal 3: (13/17) For neighbour cell measurements, the existing RRM mechanism is applied, further enhancement is not needed.

Proposal 7: (14/17) For scenario 1, in handover command, if the first active BWP is associated with NCD-SSB, the smtc field included reconfigurationWithSync is configured according to the NCD-SSB of target cell.

Proposal 15: To discuss whether only one NCD-SSB can be configured (per-UE) in Rel-17 (revisit previous RAN2 agreement).

# Final round Discussion

The followings are agreed via email:

Agreements via email – from offline 105 – second round:

- RAN2 confirms that when RedCap UE’s active BWP contains NCD-SSB, it is up to network configuration whether the UE performs serving cell measurements on NCD-SSB or CD-SSB.

- For how to indicate serving cell MO for RedCap UE, adopt solution A-2.

Solution A-2: Optionally configures a BWP-specific servingCellMO under BWP-DownlinkDedicated IE when the BWP-DownlinkDedicated contains nonCellDefiningSSB-r17. If the field is present, the UE uses this servingCellMO for serving cell measurements, otherwise, the UE uses legacy servingCellMO IE under ServingCellConfig.

- From RAN2 perspective, handover scenario 1 is supported.

Scenario 1: Handover to a target cell’s specific Redcap BWP associated with NCD-SSB besides to the initial BWP associated with CD-SSB (i.e. UE directly sync to the NCD-SSB and perform RACH on that BWP)

- From RAN2 perspective, handover scenario 2 is not supported.

Scenario 2: Handover to a target cell’s initial BWP and further switch to the specific Redcap BWP to send the RACH (i.e. UE first sync to the CD-SSB and then autonomously switch to first active BWP to perform RACH)

- Clarify in the RRC field description that the paging search space is configured in an initial BWP only if that BWP includes the CD-SSB.

- RAN2 confirms that if RedCap-specific initial DL BWP does not contain CD-SSB and CORESET#0, then this BWP will not be configured with a paging search space in any RRC state. In this case, the RedCap UE in RRC_CONNECTED state is not required to read paging.

- Reply to RAN1 and explain there is no need to support paging connected RedCap UEs in a RedCap-specific initial DL BWP which does not contain CD-SSB and CORESET#0.

- In Rel-17, from UE perspective, one configured BWP can only contain up to one SSB (CD-SSB or NCD-SSB).

In this final round, we mainly discuss the following proposals, other remaining proposals (P3, P7, P9, P15) will be discussed online.

Proposal 4	For the first measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings.

- Either CD-SSB or NCD-SSB can be configured for serving cell measurements, with the restriction that only one SSB (CD-SSB or NCD-SSB) can be used for serving cell measurements at a given time.

- The SSB used for serving cell measurements is configured by the network, the SSB can be different when the UE in different active BWPs.

- When RedCap UE’s active BWP contains NCD-SSB, it is up to network configuration whether the UE performs serving cell measurements on NCD-SSB or CD-SSB.

Proposals from Samsung (Jaehyuk), mainly to facilitate the discussion in power saving session.

Rapporteur has split them into two proposals, for proposal 17, rapporteur has removed RAR, because based on previous RAN1 agreement, RAR search space can be configured different from the one in RedCap-specific initial DL BWP.

Proposal 18: RedCap UE in idle/inactive mode monitors paging in the RedCap-specific initial DL BWP if RedCap-specific initial DL BWP contains CD-SSB and the RedCap-specific initial DL BWP is configured with search space for paging (i.e. pagingSearchSpace).

Proposal 19: If paging and OSI search space are configured in the RedCap-specific initial DL BWP which contains CD-SSB, the associated physical time/frequency domain resources are the same as the ones in the legacy initial DL BWP.

### P4

Based on the discussion on RAN2 reflector, some company showed concern to the 2nd bullet of Proposal 4, the intention of the 2nd bullet is to clarify when the UE is in different active BWP, the SSB used for serving cell measurements can be different. However, considering we already have proposal 5 which describes the signalling design for BWP-specific servingCellMO, it can also capture the above intention.

So to address the concern from company, rapporteur would like to ask if companies are fine to remove the 2nd bullet from Proposal 4, like below:

Proposal 4	For the first measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings.

- Either CD-SSB or NCD-SSB can be configured for serving cell measurements, with the restriction that only one SSB (CD-SSB or NCD-SSB) can be used for serving cell measurements at a given time.

- The SSB used for serving cell measurements is configured by the network, the SSB can be different when the UE in different active BWPs.

- When RedCap UE’s active BWP contains NCD-SSB, it is up to network configuration whether the UE performs serving cell measurements on NCD-SSB or CD-SSB.

Q3-1: Do you agree to remove the 2nd bullet from Proposal 4?

| Company | Yes or No | Comments and your preferred wording. |
| --- | --- | --- |
| Qualcomm | See comment | It is OK to remove the 2nd bullet if the same sentence is kept in Proposal 5. Or keep it in Proposal 4 but remove the same sentence in Proposal 5. Either approach is fine with us, as long as it is included in the reply LS to RAN4. |
| Vodafone | yes | I think P5 already covers it and it is agreed |
| MediaTek | No, but | This text provides clarity on the SSB to be used for measurements and must be included in the reply LS to avoid confusion in RAN4 discussions. We don’t have a strong view whether this information is provided as part of this specific text or as Proposal 5 (that is agreed). |
| Apple | As long as the intent is present in the LS we are ok | Same view as MediaTek in that RAN4 should be informed with clarity. |
| NEC | No, but | Agree with MediaTek |
| Interdigital | Either way is fine | Indeed P5 has already covered the case but even if we keep it, that’ s harmless. |
| Ericsson | Yes |  |
| Huawei, HiSilicon | No | We understand the 2nd bullet clarify that the serving cell measurement SSB changes when UE switches among BWPs. [P5 is more about reference SSB, not exactly on SSB to measure] So, it is useful to keep the 2nd bullet. Otherwise, we need to add “of active BWP” in the agreed P2. 2nd bullet in P4 •The SSB used for serving cell measurements is configured by the network, the SSB can be different when the UE in different active BWPs. Agreed P1.RAN2 confirms that when RedCap UE’s active BWP contains NCD-SSB, it is up to network configuration whether the UE performs serving cell measurements on NCD-SSB or CD-SSB. Agreed P2.For how to indicate serving cell MO for RedCap UE, adopt solution A-2. Solution A-2: Optionally configures a BWP-specific servingCellMO under BWP-DownlinkDedicated IE when the BWP-DownlinkDedicated contains nonCellDefiningSSB-r17. If the field is present, the UE uses this servingCellMO of active BWP for serving cell measurements, otherwise, the UE uses legacy servingCellMO IE under ServingCellConfig. Agreed P5.For the second measurement related question in R2-2204486, reply to RAN4 with the following RAN2 understandings: From RAN2 signalling point of view, a BWP-specific servingCellMO is defined under BWP-DownlinkDedicated, the SSB defined in this servingCellMO is the reference SSB to be used for serving cell measurements when the UE is in this active BWP; if the field is absent, SSB defined in servingCellMO under ServingCellConfig is the reference SSB to be used for serving cell measurements. This reference SSB is used to define intra-frequency measurements. |
| Xiaomi | Yes | P5 clearly covers that SSB used for serving cell measurement can be changed upon BWP switching. |
| OPPO | Yes | Already covered by P5. |
| vivo | Yes | We think RAN4 could get this understanding form P5, assuming P5 would also be included in the reply LS. |

### Paging/OSI configuration in RedCap-specific BWP

Based on the comments from Samsung, power saving session is discussing the paging configuration in RedCap-specific BWP, and it is expected to confirm the understanding from RedCap perspective.

Rapporteur has split them into two proposals, for proposal 17, rapporteur has removed RAR, because based on previous RAN1 agreement, RAR search space can be configured different from the one in RedCap-specific initial DL BWP.

Proposal 18: RedCap UE in idle/inactive mode monitors paging in the RedCap-specific initial DL BWP if RedCap-specific initial DL BWP contains CD-SSB and the RedCap-specific initial DL BWP is configured with search space for paging (i.e. pagingSearchSpace).

Proposal 19: If paging and OSI search space are configured in the RedCap-specific initial DL BWP which contains CD-SSB, the associated physical time/frequency domain resources are the same as the ones in the legacy initial DL BWP.

Q3-2: Do you agree with above Proposal 18?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Samsung | Yes | This makes the behavior of the RedCap UE in idle/inactive mode clear. Without this it is unclear whether RedCap UE in idle/inactive mode monitors paging in RedCap-specific initial DL BWP or non RedCap-specific initial DL BWP when paging search space is configured in RedCap-specific initial DL BWP. We understand that “pagingSearchSpace and firstPDCCH-MonitoringOccasionOfPO” configured in RedCap-specific initial DL BWP is used for monitoring paging in RedCap-specific initial DL BWP. |
| Qualcomm | Yes | We are fine with this clarification, which we believe has been a common understanding. |
| Vodafone | Yes |  |
| MediaTek | Yes | Assume that this has been the common understanding for a while now. |
| Apple | Yes |  |
| NEC | Yes | This is good clarification. |
| Interdigital | Yes | Looks clearer. |
| Ericsson | Yes | We think this was already the common understanding |
| Huawei, HiSilicon | Yes |  |
| Xiaomi | Yes |  |
| OPPO | Yes |  |
| Vivo | Yes |  |

Q3-3: Do you agree with above Proposal 19?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Samsung | No, see comments | In case network configures a separate initial DL BWP for redcap UEs, it is preferable to follow the current BWP design principle as per which search space configurations are independently configured for each BWP. It’s up to network configuration whether the associated physical time/frequency domain resources for PDCCH monitoring are same or different in different BWPs (i.e. RedCap-specific initial DL BWP and legacy initial DL BWP in case of proposal 19). Note that even in legacy paging/OSI can be supported in multiple DL BWPs and there is no restriction that the associated physical time/frequency domain resources for PDCCH monitoring in these DL BWPs are the same. |
| Qualcomm | No, see comment | We have similar view as Samsung. Our current understanding is that it is not necessarily true. For example, paging search space can be configured without coreset#0. Hence it can be configured in a BWP which contains CD-SSB but not coreset#0, which means the physical time/freq resources for the paging search space in the RedCap-specific BWP may not the same as those in the default initial BWP. Since the original intention of Proposal 18 and 19 is to clarify paging search space configuration for RedCap Ues (e.g. to help the discussion in ePowSav session), we think Proposal 18 alone probably is sufficient for the purpose. |
| Vodafone | See comment | I think, it should be highlighted that the configurations might be the same |
| MediaTek | Yes, see comment | Given that the legacy initial DL BWP and RedCap-specific initial DL BWP have to overlap in case paging is configured, it doesn’t really make sense that different physical resources are used. We agree that paging search space configuration has to be BWP specific. Even if the physical resources are the same, the coreset that the search space is mapped to is relative to the start of that particular BWP, so the paging search space has to be configured per BWP. However, if the same physical resources are used, the ‘firstPDCCH-MonitoringOccasionOfPO’ which defines where the UE starts monitoring paging in a PO would be the same as the legacy initial BWP (which is provided in PCCH-Config of the serving cell for the initial BWP). If the same physical resources are not used, then the RRC spec needs to be updated to reflect this. |
| Apple | No, but we tend to agree with MediaTek on the spec change. |  |
| NEC | Yes, as slight preference | As both paging and OSI SS are configured and CD-SSB is contained, and also considering the point raised by MediaTek, we slightly prefer to keep it simple (i.e. Yes) rather than the flexibility whose necessity is not sure in a real deployment. If majority wants to leave some flexibilities, then P19 can be rephrased a bit to e.g. “.. the associated physical time/frequency domain resources can be the same as the ones in the legacy initial DL BWP”, instead of removing. |
| Interdigital | No | Same view as Samsung. Let’s leave it up to NW implementation. |
| Ericsson | No | It would be better for the network to have such flexibility for configuration. This issue is already under discussion in RAN1. |
| Huawei, HiSilicon | No | Agree it is similar to legacy, with no restriction. |
| Xiaomi | Yes | We do not see the difference of paging Redcap UE and eMBB UE since for Redcap UE we did not introduce any DL enhancement. So paging SS do not need to be separated. Even paging search space configuration has to be BWP specific, it does not mean NW sending paging twice. |
| OPPO | No | Up to NW implementation. |
| vivo | No | We agree the paging and OSI search space may be same considering they both are configured in CORESET#0 no matter in legacy initial DL BWP or RedCap-specific initial DL BWP. However, it is up to network configuration, and the restriction is not needed. |

# Final round Conclusion

TBD

# References

AI summary of AI 6.12.2

[1] R2-2206143 [Pre118-e][105][RedCap] Summary of AI 6.12.2.1 on NCD-SSB aspects	ZTE Corporation

Relevant incoming LS:

[2] R2-2204486 LS on NCD-SSB issues for RedCap UE (R4-2207104; contact: Ericsson)	RAN4 LS in	Rel-17	To:RAN2

[3] R2-2204422 LS on operation with and without SSB for RedCap UE (R1-2202886; contact: Ericsson)	RAN1	LS in	Rel-17	To:RAN2, RAN4

NCD-SSB related company contributions submitted to RAN2#118e:

[4] R2-2204544 Handover to BWP without CD-SSB   ZTE Corporation, Sanechips

[5] R2-2204547 Discussion on serving cell measurements on NCD-SSB   ZTE Corporation, Sanechips

[6] R2-2204812 Discussion on NCD-SSB for RedCap UEs	vivo, Guangdong Genius

[7] R2-2205038 Discussion on NCD-SSB aspects for RedCap UE   Huawei, HiSilicon

[8] R2-2205285 [J002] Clarification on reference value in connected RRM relaxation critrion	Sharp

[9] R2-2205522 Aspects related to the use of NCD-SSB   MediaTek Inc.

[10] R2-2205636 Discussion on NCD-SSB handling at handover	Apple discussion

[11] R2-2205771 About paging monitoring in BWP#0 without CD-SSB   ZTE Corporation, Sanechips Late

[12] R2-2206019 [DRAFT] Reply LS on NCD-SSB issues for RedCap UE   Ericsson   LS out   To: RAN4

[13] R2-2206032 Further discussion on SI acquisition in RedCap-specific BWP    Qualcomm Incorporated

[14] R2-2206033 Measurement object configuration with NCD-SSB   Qualcomm Incorporated	discussion