3GPP TSG RAN WG2 #118-e 	R2-2206416

- Online, 9 - 20 May, 2022

Title:	[draft] Reply LS on system information

Response to:	R2-2204468/R1-2202843

Release:	Release 17

Work Item:	NR_NTN_solutions-Core

Source:	Huawei [to be RAN2]

To:	RAN1

Cc:

Contact Person:

#### Name:	Lili Zheng

E-mail Address:	zhenglili4@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 thanks RAN1 for the reply.  RAN2 has agreed that ephemeris, common TA parameters, epoch time and validity duration are valid during validity duration, but the NW is not prevented from triggering the SI modification if there are major changes. RAN2 understands that the NW should not change validity duration alone.

RAN2 also discussed the potential ambiguity of the cell-specific K_offset. From RAN2 perspective, the cell-specific K_offset will not change frequently and the issue can be handled by gNB implementation, therefore RAN2 will not further specify the application time of the updated cell-specific K_offset.

2. Actions:

To RAN1

ACTION: 	RAN1 is kindly requested to take the above information into account.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#119-e 	15 – 26 Aug 2022	e-meeting

RAN2#119-bis-e 	10 – 19 Oct 2022	e-meeting