3GPP TSG RAN WG2 #118-e 	R2-2206416

- Online, 9 - 20 May, 2022

Title:	[draft] Reply LS on system information

Response to:	R2-2204468/R1-2202843

Release:	Release 17

Work Item:	NR_NTN_solutions-Core

Source:	Huawei [to be RAN2]

To:	RAN1

Cc:

Contact Person:

#### Name:	Lili Zheng

E-mail Address:	zhenglili4@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 thanks RAN1 for the reply.

Regarding DL and UL polarization, the parameters of serving cell have been included in SIB19 as ntn-PolarizationDL-r17 and ntn-PolarizationUL-r17. With respect to neighbour cells, RAN2 has agreed that

1) For UEs in RRC_IDLE/RRC_INACTIVE, DL and UL polarization is broadcast per neighbour cell;

2) 	For UEs in RRC_CONNECTED, polarization information for RRM measurement purposes is provided in measurement configuration; target cell’s polarization information is also provided during handover.

On validity duration, RAN2 will follow RAN1 agreement that it applies only to ephemeris and common TA parameters including the epoch time. RAN2 understanding is that ephemeris, common TA parameters, epoch time and validity duration can be updated without invoking the SI modification procedure, and other parameters in SIB19 follow the normal SI modification..

RAN2 also discussed the potential ambiguity of the cell-specific K_offset. From RAN2 perspective, the cell-specific K_offset will not change frequently and the issue can be handled by gNB implementation, therefore RAN2 will not further specify the application time of the updated cell-specific K_offset.

2. Actions:

To RAN1

ACTION: 	RAN1 is kindly requested to take the above information into account.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#119-e 	15 – 26 Aug 2022	e-meeting

RAN2#119-bis-e 	10 – 19 Oct 2022	e-meeting