3GPP TSG RAN WG2 #118-e 	R2-22xxxxx

- Online, 9 - 20 May, 2022

Title:	[draft] Reply LS on system information

Response to:	R2-2204468/R1-2202843

Release:	Release 17

Work Item:	NR_NTN_solutions-Core

Source:	Huawei [to be RAN2]

To:	RAN1

Cc:

Contact Person:

#### Name:	Lili Zheng

E-mail Address:	zhenglili4@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 thanks RAN1 for the reply. On validity duration, RAN2 will follow RAN1 agreement that the validity duration applies only to ephemeris and common TA parameters including the epoch time, and other parameters in SIB19 follow the normal SI modification procedure.

RAN2 also discussed the potential ambiguity of cell-specific K_offset. RAN2 understanding is that the cell-specific K_offset will not change frequently and the issue can be handled by gNB implementation, and RAN2 will not further specify the application time of the updated cell-specific K_offset.

2. Actions:

To RAN1

ACTION: 	RAN1 is kindly requested to take the above information into account.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#119-e 	15 – 26 Aug 2022	e-meeting

RAN2#119-bis-e 	10 – 19 Oct 2022	e-meeting