3GPP TSG RAN WG2 Meeting #118-e	                                        R2-2206161

- Electronic, 9th – 20th May 2022

Agenda item:		4.5, 7.4

Source:			Samsung

Title:		Report [AT118-e][201][LTE] LTE legacy CRs (Samsung)

Document for:	 	Discussion and decision

# Introduction

This report summarizes the email discussion below that took place during RAN2#118-e meeting:

- [AT118-e][201][LTE] LTE legacy CRs (Samsung)

Scope: 1st phase: Discuss LTE CRs marked for this discussion (under AI 4.5 and 7.4). 2nd phase: Provided updated CRs based on online agreements.

Intended outcome: 1st phase: Discussion report in R2-2206161 (for 1st week online discussion). 2nd phase: Agreeable CRs (by proponents) based on online agreements.

Deadline: Deadline 1 (1st week online) / Deadline 5 (CR finalization)

# Reference

The following documents are treated in this email discussion:

- R-2205199	Minor changes collected by Rapporteur	Samsung	CR	Rel-16	36.331	16.8.0	4790	-	F	LTE_euCA-Core

- R2-2205200	Clarifications on CQI-ReportPeriodicScell	Samsung	discussion	LTE_euCA-Core

- R2-2205201	Correction on the CQI-ReportPeriodicScell	Samsung	CR	Rel-15	36.331	15.17.0	4791	-	F	LTE_euCA-Core

- R2-2205202	Correction on the CQI-ReportPeriodicScell	Samsung	CR	Rel-16	36.331	16.8.0	4792	-	A	LTE_euCA-Core

- R2-2205203	Correction on the CQI-ReportPeriodicScell	Samsung	CR	Rel-17	36.331	17.0.0	4793	-	A	LTE_euCA-Core

- R2-2205427	Correction on evaluation of conditional reconfiguration	CATT	CR	Rel-16	36.331	16.8.0	4800	-	F	LTE_feMob-Core

# Contact information

| Company | Contact Name, Email |
| --- | --- |
| Lenovo | Hyung-Nam Choi, hchoi5@lenovo.com |
| Nokia | Jarkko Koskela, jarkko.t.koskela@nokia.com |

# Discussion

## Minor changes collected by Rapporteur

In the CR [1] the following editorial changes to TS 36.331 R16 are proposed:

- Change the field name of meas-Parameters-r16 and IE name of Meas-Parameters-NB-r16 to measParameters-r16 and MeasParameters-NB-r16, respectively.

Question 1: Do companies agree on the proposed changes to TS 36.331 R16, please provide further comments can be included in the Rapporteur CR?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Lenovo | Yes but | Cover page: Wrong meeting dates; wrong meeting#. What about the R17 shadow CR? Other issues which can be fixed in R16 and R17: - Add space between "NR" and "SCG" in the title of Figure 5.6.13a.1-1. Figure 5.6.13a.1-1: NRSCG failure information - In RRCConnectionResume-v1610-Ies: Suffix “-r16” is missing for the fields restoreMCG-SCells, restoreSCG and nr-SecondaryCellGroupConfig. |
| Nokia | Yes | These seems to be good changes – agree with Lenovo comments |

## Clarifications on CQI-ReportPeriodicScell

In the contribution [2] and CRs [3], [4], [5] the following changes to TS 36.331 for R15, R16 and R17 are proposed:

- Background of the issue:

- In RAN1#91 meeting, RAN1 made following agreements regarding the CQI periodic reporting for the new state (i.e. dormant state)

Agreement: UE can operate with single CSI subframe set or multiple CSI subframe sets, which are separately configured from the CSI subframe sets in active state.

- RAN1 specification (TS 36.213, subcluase 7.2.2) specifies the separate configuration of single CSI subframe set and multiple CSI subframe sets regarding cqi-pmi-ConfigIndexDormant/ cqi-pmi-ConfigIndex2Dormant and ri-ConfigIndexDormant /ri-ConfigIndex2Dormant.

- However, RRC only allows to configure cqi-pmi-ConfigIndexDormant and ri-ConfigIndexDormant i.e. there are no RRC signaling to configure cqi-pmi-ConfigIndex2Dormant and ri-ConfigIndex2Dormant.

- Proposed changes in CRs [3], [4], [5]:

- Introduce the new IE CQI-ReportPeriodicSCell-v15xy including cqi-pmi-ConfigIndex2Dormant and ri-ConfigIndex2Dormant field

Question 2: Do companies agree on the proposed changes to TS 36.331 for R15, R16 and R17?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Nokia | Neutral | At the time this was done they were done based on Nokia and QC views. We only introduced simple parameters to keep it easier. We can see the discrepancy between RAN1 and RAN2. We could correct the ASN.1 as proposed by Samsung but we need to consider if one would need capability for this? We wonder if we don’t do the change then would we need CR to RAN1 instead to remove possibility for different configuration for multiple CSI subframe sets – then same configuration be applied for both cases? |

## Correction on evaluation of conditional reconfiguration

In the CR [6] the following changes to TS 36.331 for R16 are proposed:

- 5.3.5.9.4 Conditional reconfiguration evaluation

- For CHO case, the target cell is included within MobilityControlInfo. Thus, change the ServingCellConfigCommon to MobilityControlInfo

Question 2: Do companies agree on the proposed changes to TS 36.331 for R16?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Lenovo | Yes but | Change for R16/R17 can be merged into the RRC rapporteur CRs. |
| Nokia | Yes | This seems to be error when copying NR text to LTE. We should agree the CR. And we assume shadow CR to R17 is needed as well. |

# Conclusion

Based on company’s feedback the following proposals are made:

<To be updated>