3GPP TSG RAN WG2 Meeting #118-e	                                        R2-2206161

- Electronic, 9th – 20th May 2022

Agenda item:		4.5, 7.4

Source:			Samsung

Title:		Report [AT118-e][201][LTE] LTE legacy CRs (Samsung)

Document for:	 	Discussion and decision

# Introduction

This report summarizes the email discussion below that took place during RAN2#118-e meeting:

- [AT118-e][201][LTE] LTE legacy CRs (Samsung)

Scope: 1st phase: Discuss LTE CRs marked for this discussion (under AI 4.5 and 7.4). 2nd phase: Provided updated CRs based on online agreements.

Intended outcome: 1st phase: Discussion report in R2-2206161 (for 1st week online discussion). 2nd phase: Agreeable CRs (by proponents) based on online agreements.

Deadline: Deadline 1 (1st week online) / Deadline 5 (CR finalization)

# Reference

The following documents are treated in this email discussion:

- R2-2205199	Minor changes collected by Rapporteur	Samsung	CR	Rel-16	36.331	16.8.0	4790	-	F	LTE_euCA-Core

- R2-2205200	Clarifications on CQI-ReportPeriodicScell	Samsung	discussion	LTE_euCA-Core

- R2-2205201	Correction on the CQI-ReportPeriodicScell	Samsung	CR	Rel-15	36.331	15.17.0	4791	-	F	LTE_euCA-Core

- R2-2205202	Correction on the CQI-ReportPeriodicScell	Samsung	CR	Rel-16	36.331	16.8.0	4792	-	A	LTE_euCA-Core

- R2-2205203	Correction on the CQI-ReportPeriodicScell	Samsung	CR	Rel-17	36.331	17.0.0	4793	-	A	LTE_euCA-Core

- R2-2205427	Correction on evaluation of conditional reconfiguration	CATT	CR	Rel-16	36.331	16.8.0	4800	-	F	LTE_feMob-Core

# Contact information

| Company | Contact Name, Email |
| --- | --- |
| Lenovo | Hyung-Nam Choi, hchoi5@lenovo.com |
| Nokia | Jarkko Koskela, jarkko.t.koskela@nokia.com |
| Qualcomm | Umesh Phuyal, uphuyal@qti.qualcomm.com |
| Huawei, HiSilicon | Jun Chen, jun.chen@huawei.com |
| Samsung | seungri.jin@samsung.com |
| Ericsson | robert.s.karlsson AT ericsson.com |

# Discussion

## Minor changes collected by Rapporteur

In the CR [1] the following editorial changes to TS 36.331 R16 are proposed:

- Change the field name of meas-Parameters-r16 and IE name of Meas-Parameters-NB-r16 to measParameters-r16 and MeasParameters-NB-r16, respectively.

Question 1: Do companies agree on the proposed changes to TS 36.331 R16, please provide further comments can be included in the Rapporteur CR?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Lenovo | Yes but | Cover page: Wrong meeting dates; wrong meeting#. What about the R17 shadow CR? Other issues which can be fixed in R16 and R17: - Add space between "NR" and "SCG" in the title of Figure 5.6.13a.1-1. Figure 5.6.13a.1-1: NRSCG failure information - In RRCConnectionResume-v1610-Ies: Suffix “-r16” is missing for the fields restoreMCG-SCells, restoreSCG and nr-SecondaryCellGroupConfig. |
| Nokia | Yes | These seems to be good changes – agree with Lenovo comments |
| Qualcomm | Yes | In addition to Lenovo’s suggestions above, since this is a CR to capture all editorial/minor suggestions, , we have the following: 5.5.2.13: else;  (‘;’ to ‘:’) 5.6.2.3: Can update the following to be consistent: 2> if the UE is a NB-IoT UE: 3> set the dedicatedInfoNAS to include the information received from upper layers; 2> else: 3> , set the dedicatedInfoType to include the dedicatedInfoNAS; 5.6.3.3: there are a few “else” without “:”, can add “:” to them. <skip> 3> else 4> if the UECapabilityEnquiry message includes requestedFrequencyBands and UE supports requestedFrequencyBands: 5> include in supportedBandCombination as many as possible of the band combinations included in the list of candidates, including the non-CA combinations and up to 5DL+5UL CA band combinations, determined according to the rules and priority order defined above; 5> include in supportedBandCombinationAdd as many as possible of the remaining band combinations included in the list of candidates, (i.e. the candidates not included in supportedBandCombination), up to 5DL+5UL CA band combinations, determined according to the rules and priority order defined above; 4> else 5> include in supportedBandCombination as many as possible of the band combinations included in the list of candidates, including the non-CA combinations and up to 5DL+5UL CA band combinations, determined according to the rules defined above; 5.6.8.2: Following “if” should be removed. <<skip>> 2> else if: 3> if the UE is in any cell selection state (as specified in TS 36.304 [4]): <<skip>> 5> set the measResultServCell to include the quantities of the last logged cell the UE was camping on; 4> else:  This should have styple B4, instead of B3. 5> set the servCellIdentity to indicate global cell identity of the cell the UE is camping on; 5.10.7.3: missing “:” after else. 3> else 5.10.8.2: several usages of “If”, can change to “if” (lowercase I) 5.10.8a: several usages of “If”, can change to “if” (lowercase I) ASN.1 comments: In the following, -rxx are missing. But they are rel15, so leave it up-to you. RRCConnectionRequest-5GC-r15-IEs ::= SEQUENCE { ue-Identity-r15 InitialUE-Identity-5GC-r15, establishmentCause-r15 EstablishmentCause-5GC-r15, spare BIT STRING (SIZE (1)) } InitialUE-Identity-5GC-r15 ::= CHOICE { ng-5G-S-TMSI-Part1 BIT STRING (SIZE (40)), randomValue BIT STRING (SIZE (40)) } EstablishmentCause-5GC-r15 ::= ENUMERATED { emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, mo-VoiceCall, spare2, spare1} In several cases, since the choice values were added in the same release when the field containing them were added (and not extended later), r1x is not needed in the choice values below according to general convention. But since some of them go back to rel15, I leave it for your discretion. Also, this is not exhaustive list. DLInformationTransfer-r15-IEs ::= SEQUENCE { dedicatedInfoType-r15 CHOICE { dedicatedInfoNAS-r15 DedicatedInfoNAS, dedicatedInfoCDMA2000-1XRTT-r15 DedicatedInfoCDMA2000, dedicatedInfoCDMA2000-HRPD-r15 DedicatedInfoCDMA2000 } OPTIONAL, -- Need ON timeReferenceInfo-r15 TimeReferenceInfo-r15 OPTIONAL, -- Need ON nonCriticalExtension DLInformationTransfer-v8a0-IEs OPTIONAL } FailureInformation-r16 ::= SEQUENCE { criticalExtensions CHOICE { failureInformation-r16 FailureInformation-r16-IEs, criticalExtensionsFuture SEQUENCE {} } } SecurityConfigHO-v1530 ::= SEQUENCE { handoverType-v1530 CHOICE { intra5GC-r15 SEQUENCE { securityAlgorithmConfig-r15 SecurityAlgorithmConfig OPTIONAL, -- Cond HO-toEUTRA keyChangeIndicator-r15 BOOLEAN, nextHopChainingCount-r15 NextHopChainingCount, nas-Container-r15 OCTET STRING OPTIONAL -- Need ON }, fivegc-ToEPC-r15 SEQUENCE { securityAlgorithmConfig-r15 SecurityAlgorithmConfig, nextHopChainingCount-r15 NextHopChainingCount }, epc-To5GC-r15 SEQUENCE { securityAlgorithmConfig-r15 SecurityAlgorithmConfig, nas-Container-r15 OCTET STRING } }, ... } RAN-NotificationAreaInfo-r15 ::= CHOICE { cellList-r15 PLMN-RAN-AreaCellList-r15, ran-AreaConfigList-r15 PLMN-RAN-AreaConfigList-r15 } RedirectedCarrierInfo-r15-IEs ::= CHOICE { eutra-r15 ARFCN-ValueEUTRA-r9, geran-r15 CarrierFreqsGERAN, utra-FDD-r15 ARFCN-ValueUTRA, cdma2000-HRPD-r15 CarrierFreqCDMA2000, cdma2000-1xRTT-r15 CarrierFreqCDMA2000, utra-TDD-r15 CarrierFreqListUTRA-TDD-r10 } RLF-Report-r9 ::= SEQUENCE { <<skip>> [[ measResultListNR-r16 MeasResultCellListNR-r15 OPTIONAL, previousNR-PCellId-r16 CellGlobalIdNR-r16 OPTIONAL, failedNR-PCellId-r16 CHOICE { cellGlobalId-r16 CellGlobalIdNR-r16, pci-arfcn-r16 SEQUENCE { physCellId-r16 PhysCellIdNR-r15, carrierFreq-r16 ARFCN-ValueNR-r15 } } OPTIONAL, reconnectCellId-r16 CHOICE { nrReconnectCellId-r16 CellGlobalIdNR-r16, eutraReconnectCellId-r16 SEQUENCE { cellGlobalId-r16 CellGlobalIdEUTRA, trackingAreaCode-EPC-r16 TrackingAreaCode OPTIONAL, trackingAreaCode-5GC-r16 TrackingAreaCode-5GC-r15 OPTIONAL } } OPTIONAL, timeUntilReconnection-r16 TimeUntilReconnection-r16 OPTIONAL ]], [[ measResultListNR-v1640 SEQUENCE { carrierFreqNR-r16 ARFCN-ValueNR-r15 } OPTIONAL, measResultListExtNR-r16 MeasResultFreqListNR-r16 OPTIONAL ]] } Etc. |
| Huawei, HiSilicon | Yes |  |
| Samsung | Yes | Agree with above comments. For the mirror R17 CR, actually this change is included in “R2-2205208 Corrections on the general ASN.1 issues”. Need to determine the separate R17 Rap CR is needed or all minor changes will be added in general ASN.1 CR. |
| Ericsson | Yes |  |

All companies agree for the Rapporteur CR and some companies provided further changes to be added in this CR. In addition, Rel-17 mirror CR seems required based on the comments.

Proposal 1: Revise the Rel-16 LTE RRC Rapporteur CR (R2-2205199) based on the feedback in this offline discussion.

Proposal 2: Prepare Rel-17 LTE RRC Rapporteur CR (i.e. shadow CR).

## Clarifications on CQI-ReportPeriodicScell

In the contribution [2] and CRs [3], [4], [5] the following changes to TS 36.331 for R15, R16 and R17 are proposed:

- Background of the issue:

- In RAN1#91 meeting, RAN1 made following agreements regarding the CQI periodic reporting for the new state (i.e. dormant state)

Agreement: UE can operate with single CSI subframe set or multiple CSI subframe sets, which are separately configured from the CSI subframe sets in active state.

- RAN1 specification (TS 36.213, subcluase 7.2.2) specifies the separate configuration of single CSI subframe set and multiple CSI subframe sets regarding cqi-pmi-ConfigIndexDormant/ cqi-pmi-ConfigIndex2Dormant and ri-ConfigIndexDormant /ri-ConfigIndex2Dormant.

- However, RRC only allows to configure cqi-pmi-ConfigIndexDormant and ri-ConfigIndexDormant i.e. there are no RRC signaling to configure cqi-pmi-ConfigIndex2Dormant and ri-ConfigIndex2Dormant.

- Proposed changes in CRs [3], [4], [5]:

- Introduce the new IE CQI-ReportPeriodicSCell-v15xy including cqi-pmi-ConfigIndex2Dormant and ri-ConfigIndex2Dormant field

Question 2: Do companies agree on the proposed changes to TS 36.331 for R15, R16 and R17?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Nokia | Neutral | At the time this was done they were done based on Nokia and QC views. We only introduced simple parameters to keep it easier. We can see the discrepancy between RAN1 and RAN2. We could correct the ASN.1 as proposed by Samsung but we need to consider if one would need capability for this? We wonder if we don’t do the change then would we need CR to RAN1 instead to remove possibility for different configuration for multiple CSI subframe sets – then same configuration be applied for both cases? |
| Qualcomm | Neutral | Similar to Nokia comment: No strong view. If companies prefer to have the CR, we have following comments on the CRs: - Interoperability statement missing - v15xy can be hardcoded as v15i0 (?) as the ASN.1 is already frozen - comma to be removed from ri-ConfigIndex2Dormant-v15xy INTEGER (0..1023) OPTIONAL, -- Need OR - Maybe better to add SetupRelease somewhere. (Either at the top level) physicalConfigDedicatedSCell-v15xy PhysicalConfigDedicatedSCell-v15xy OPTIONAL -- Need ON Or somewhere down the chain. |
| Huawei, HiSilicon | Neutral | Share the same view as Nokia that we may need to consider new capabilities as the changes put extra impacts to UE side. |
| Samsung | Proponent | At least, we need to resolve the discrepancy between RAN1 and RAN2. As companies provided comments, new capabilities and further CR work can be discussed during the phase 2 discussion. The submitted CRs would be the good start point from our understanding. |
| Ericsson | Neutral | CR needs to be made backwards compatible Interoperability is missing |

Most of companies provide the view as “Neutral” for these CRs but I think all companies agree that there are discrepancy between RAN1 and RAN2 specifications. Rapporteur understood that the revision of CRs could be acceptable for the companies i.e. without this different configuration for multiple CSI subframe sets  will not be supported.

Proposal 3: Introduce the new capability to support different configuration for multiple CSI subframe sets.

Proposal 4: Revise the CRs (R2-2205201, R2-2205202, R2-2205203) based on the feedback in this offline discussion, and further check during the phase 2 discussion.

## Correction on evaluation of conditional reconfiguration

In the CR [6] the following changes to TS 36.331 for R16 are proposed:

- 5.3.5.9.4 Conditional reconfiguration evaluation

- For CHO case, the target cell is included within MobilityControlInfo. Thus, change the ServingCellConfigCommon to MobilityControlInfo

Question 2: Do companies agree on the proposed changes to TS 36.331 for R16?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Lenovo | Yes but | Change for R16/R17 can be merged into the RRC rapporteur CRs. |
| Nokia | Yes | This seems to be error when copying NR text to LTE. We should agree the CR. And we assume shadow CR to R17 is needed as well. |
| Qualcomm | Yes but | Agree with Lenovo’s comments. (Otherwise Rel17 mirror is missing also.) |
| Huawei, HiSilicon | Yes |  |
| Samsung | Yes | Can be merged into Rap CRs. |
| Ericsson | Yes | Agree with the change. |

All companies agree to the proposed changes in this CR (R2-2205427) but this change can be merged into the Rapporteur CR for Rel-16/17.

Proposal 5: Changes in this CR (R2-2205427) is merged into the LTE RRC Rapporteur CRs for Rel-16/17.

# Conclusion

Based on company’s feedback the following proposals are made:

Proposal 1: Revise the Rel-16 LTE RRC Rapporteur CR (R2-2205199) based on the feedback in this offline discussion.

Proposal 2: Prepare Rel-17 LTE RRC Rapporteur CR (i.e. shadow CR).

Proposal 3: Introduce the new capability to support different configuration for multiple CSI subframe sets.

Proposal 4: Revise the CRs (R2-2205201, R2-2205202, R2-2205203) based on the feedback in this offline discussion, and further check during the phase 2 discussion.

Proposal 5: Changes in this CR (R2-2205427) is merged into the LTE RRC Rapporteur CRs for Rel-16/17.