3GPP TSG-RAN WG2 Meeting #118-e	R2- 220xxxx

Online, May, 2022

Agenda:	6.7.1

Source:	ZTE

Title:	Summary of [AT118-e][619][Relay] LS on SDU type in PDCP (ZTE)

Document for:	Discussion, Decision

# 1	Introduction

This document captures the following email discussion:

 [AT118-e][619][Relay] LS on SDU type in PDCP (ZTE)

Scope: Discuss the LS in R2-2204447 and related contributions (R2-2204633, R2-2204771, R2-2204772, R2-2204798, R2-2204799).  Phase 1 should determine a way forward and a recommendation to be taken into account in the PDCP rapporteur CR; Phase 2 is to draft and approve the LS.

Intended outcome: Approved LS (without CB)

Deadline:  Phase 1 Friday 2022-05-13 1800 UTC,

Phase 2 Wednesday 2022-05-18 0400 UTC

# Discussion

CT1 sent an LS (R2-220447) to RAN2 on the SDU type used over user plane for NR PC5 reference point, which checks with RAN2 whether “Ethernet PDCP SDU type” and “Unstructured PDCP SDUtype” are supported by AS layer.

In clause 6.1.2.2 of TS 23.304 v17.1.0, it specifies that "IP, Ethernet and Unstructured PDCP SDU types are supported. For IP PDCP SDU type, both IPv4 and IPv6 are supported." Hence, the "Ethernet PDCP SDU type" and "Unstructured PDCP SDU type" are required over the user plane for NR PC5 reference point. In order to implement the requirement above, CT1 would like to check with RAN2 whether "Ethernet PDCP SDU type" and "Unstructured PDCP SDU type" are supported by AS layer. To RAN WG2 group. ACTION: CT1 kindly asks RAN2 to provide the answer to the question above.

According to the latest TS 23.304, IP, Ethernet and Unstructured PDCP SDU types are supported. For IP PDCP SDU type, both IPv4 and IPv6 are supported. As we can see from the user plane protocol stack in Figure 1, the packets from ProSe application layer may be IP, Ethernet and Unstructured packets.

Figure 1 User plane protocol stack for NR PC5 reference point (TS 23.304)

To be specific, the Ethernet, unstructured and IP traffic are supported for both 5G ProSe Direc Communication and U2N relay according to TS 23.304. The detailes are listed as follows:

- 5G ProSe Direc Communication

- For broadcast and groupcast mode: the data unit types of IPv4, IPv6, Ethernet, Unstructured, and Address Resolution Protocol should be supported.

- For unicast mode: the data unit types of IPv4, IPv6, Ethernet, and Unstructured should be supported.

- U2N relay

- For both L2 and L3 U2N relay: unicast traffic (uplink and downlink) between the 5G ProSe Remote UE and the network supports IP, Ethernet or Unstructured traffic type.

In addition, during RAN2#115 meeting, RAN2 sent the reply LS (R2-2109124) to SA2, which confirmed that the new data unit type of ARP for broadcast and groupcast mode ProSe Direct Communication can be supported by AS layer with some update in TS 38.323 specification. However, it is not yet captured in TS 38.323 yet.

According to [4] (CATT, R2-2204771) and [6] (ZTE, R2-2204798), it is suggested to introduce new code point for PDCP SDU types of “Ethernet”, “Unstructured” and “ARP”. On the other hand, it is mentioned in [3](OPPO, R2-2204633) that TS 38.323 include a Non-IP SDU type, which may be used to carry the "Ethernet" and "Unstructured" SDU type without differentiation of the two using separate code-points. And [3] suggest to further clarify this with CT1 and SA2 on whether there is a need to introduce separate code-points for “Ethernet”, “Unstructured” and “ARP” PDCP SDU Type.

Table 6.3.12-1: SDU Type (TS 38.323)

| Bit | Description |
| --- | --- |
| 000 | IP |
| 001 | Non-IP |
| 010-111 | Reserved |

Furthermore, [8] (Samsung, R2-2205611) presents that CT1 specifies Ethernet, ARP, Unstructured data as non-IP PDU type in TS 24.554[9]. The “Non-IP” code point in current PDCP SDU type can be used to support the Ethernet, Unstructured and ARP packet. RAN2 can add a text to specify that non-IP in PDCP SDU type includes Ethernet, Unstructured data or ARP in PDCP specification.

## 11.5 Non-IP PDU format The non-IP PDU is coded according to figure 11.5.1 and table 11.5.1. Bits. 8; 7; 6; 5; 4; 3; 2; 1; Octets. Non-IP type; 1. Non-IP payload; 2. n. Figure 11.5.1: Non-IP PDU format Table 11.5.1: Non-IP PDU values Non-IP type (octet 1) Bits. 8; 7; 6; 5; 4; 3; 2; 1. 0; 0; 0; 1; 0; 0; 0; 0; Ethernet. 0; 0; 0; 1; 0; 0; 0; 1; Address Resolution Protocol (see RFC 826 [32]). 0; 0; 0; 1; 0; 0; 1; 0; Unstructured. All other values are reserved.. Octets 2 to n contain the non-IP payload field containing the data provided by the upper layer as indicated in Non-IP type..

Q1) Which option do you prefer for the support of “Ethernet”, “Unstructured” and “ARP” packet? Provide your comments.

Option 1: introduce three new code points of PDCP SDU type to support “Ethernet”, “Unstructured” and “ARP” packet;

Option 2: reuse the legacy “Non-IP” code point to support “Ethernet”, “Unstructured” and “ARP” packet;

| Company | Option | Comments |
| --- | --- | --- |
| Qualcomm | Option 1 | To not overload the legacy “Non-IP” code point and not incur additional non-IP header overhead for each packet |

Q2) Is it necessary to further clarify with CT1 and SA2 on whether to reuse legacy “Non-IP” code point or to introduce three new code points of PDCP SDU type to support “Ethernet”, “Unstructured” and “ARP” packet?

| Company | Response (Y/N) | Comments |
| --- | --- | --- |
| Qualcomm | No | In SA2 TS 23.304, it is already assumed the PDCP SDU types are defined. So, we do not think it is necessary to clarify with SA2 further. As CT1 sent the LS, we think a reply with RAN2 decision is suitable approach. |

# 4	Conclusions

In this contribution, we have the following proposals based on the email discussion.

# 5	References

- R2-2204447	LS on the SDU type used over user plane for NR PC5 reference point (C1-221835; contact: ZTE).

- TS 23.304 3GPP TSG service and system aspects, ProSe in 5GS, R17.

- R2-2204633	Discussion on CT1 LS on SDU type (C1-221835)	OPPO.

- R2-2204771	Issues on the SDU Type Used over User Plane for NR PC5 Reference Point	CATT.

- R2-2204772	Correciton on PDCP for SL relay CATT.

- R2-2204798	Discussion on the SDU type used over user plane for NR PC5 reference point ZTE.

- R2-2204799	Draft reply LS on SDU type used over user plane for NR PC5 reference point	 ZTE.

- R2-2205611 Support of non-IP PDU type in PDCP protocol Samsung.

- TS 24.554 Proximity-services (ProSe) in 5G System (5GS) protocol aspects; Stage 3.