3GPP TSG-RAN WG2 Meeting #118 electronic	R2-220

Online, May 9th –20th, 2022

Agenda Item:	6.6.2

Source: 	Huawei, HiSilicon

Title:  	Email discussion for MAC CR for positioning

Document for:	Discussion and Decision

# Introduction

The following email discussion has been planned for the MAC CR for positioning

- [AT118-e][625][POS] 38321 positioning CR (Huawei)

Scope: Develop a rapporteur CR, taking into account decisions of this meeting.  Discussion should coordinate with the handling of agenda item summaries.

Intended outcome: Agreeable CR in R2-2206248

Deadline:  Tuesday 2022-05-17 1800 UTC

# 2. PPW activation/deactivation

During the last R1 meeting, the following agreements have been made on PPW activation/deactivation MAC CE:

Agreement The maximum number of PRS processing windows that can be activated/deactivated by a DL MAC CE is 1.

Based on the agreement from R1 above, the rapporteur would like to provide the following texr proposal

- 6.1.3.42 PPW Activation/Deactivation Command MAC CE The PPW Activation/Deactivation Command MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b. It consists of a single octet defined as follows (Figure 6.1.3.42-1): - Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits; - PPW ID: This field indicates the PPW configured on active DL BWP of the Serving Cell identified by the above Serving Cell ID. The length of the field is 2 bits; - A/D: This field indicates the activation or deactivation of the PPW. The field is set to 1 to indicate activation, otherwise it indicates deactivation. The length of the field is 1 bit. Figure 6.1.3.42-1: PPW Activation/Deactivation Command MAC CE

###### Question1: Do companies agree with the text proposal above for PPW activation/deactivation command?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSIlicon | Yes |  |
| Intel | Yes |  |
| Apple | Yes |  |
| Qualcomm | No | The PPW ID requires 4-bits to cover the range 0…15: The maximum number of preconfigured PRS processing window per DL BWP is 4. Unclear why the Serving Cell ID is needed. Would a PPW pre-configuration survive a cell change? The MAC-CE can be analogous to the MGs below. |
| CATT | No | According to the following agreement: ------------------------------------------------------------------------------------------- For Issue#5, RAN1 have reached the following agreement in RAN1#108e: Agreement RAN1 Answer: RAN1 agreed that PRS processing window configuration is provided per DL BWP. UE can be configured with multiple PRS processing windows in one DL BWP. The maximum number of PPW configuration is 4 per DL BWP, but the number of activated PRS processing window per DL BWP is 1. In addition, RAN1 would like to note the maximum number of activated PRS processing windows across all active DL BWPs is 4, and those activated PRS processing windows are not overlapping in time. It is RAN1 understanding that UE should monitor PDCCH during RAR window/msgB window or contention resolution timer for the affected symbols by the PRS processing window. ---------------------------------------------------------------------------------------------- On BWP ID, we think maybe size 2 is enough, since only the PPW within the active BWP can be activated according to per RAN1 LS. Thus, the number space of PPW can be per BWP unique, but not per cell unique, which can save the signalling of MAC CE, i.e., 2 bit is enough. In addition, it seems that up to 4 PPW can be activated simultaneously, thus we think the current PPW MAC CE still make sense, which can active up to 4 PPW flexibly based on the field of “numentry”. And the only thing need to be changes is to correct the value of numentry as proposed by R2-2204742 |
| OPPO | No. | Agree with Qualcomm that the intention of including the serving cell ID in the MAC CE is not clear. The serving cell should be the one from which the UE receives the MAC CE. |
| ZTE | No | Agree with CATT that the previous structure is good |
| vivo | Depends on the further conclusion of RAN1 | RAN1 is discussing whether to revert the agreement and RAN2 shall just follow RAN1’s new conclusion. For the Cell ID, we think it’s the CellIndex associated with the active DL BWP. As each BWP can only configured with 4 PPW, 2 bits are enough. |
| Ericsson | Further discussion needed? | At least in the RAN1 parameter list; the cellID is removed. However, my interpretation was that it is removed from RRC as it has been included in MAC. But is it then it is also not needed in MAC. What happens when UE is configured with dual connectivity, multicarrier with PCell, Scell etc; is the cellID needed then or BWP can uniquely resolve? |

.

Moreover, the following two proposals from the summary R2-2206340 have been decided to covered by this email discussion

================================BEGIN=============================================

Proposal 1: RAN2 to agree the change in 38.321 draft CR R2-2204700 and R2-2205656 for pre-configured MG MAC CE activation/deactivation request/command design.

Proposal 8: RAN2 to agree the change in draft CR R2-2205309 for capturing the cancellation procedure of UL MAC CE for pre-configured MG in 38.321.

================================END==============================================

On the MAC CE for MG activation/deactivation command/request, the following text proposal has been given in R2-2204996 and R2-2204700:

- 6.1.3.40 Positioning Measurement Gap Activation/Deactivation Request MAC CE The Positioning Measurement Gap Activation/deactivation request MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-2b. It has a fixed 8-bit size and consists of a single octet defined as follows (Figure 6.1.3.40-1): - Positioning MG ID: This field indicates the identifier for the preconfigured positioning measurement gap. The length of the field is 4 bits. - A/D: This field indicates the activation or deactivation of the Positioning Measurement Gap. The field is set to 1 to indicate activation, otherwise it indicates deactivation. The length of the field is 1 bit. Figure 6.1.3.40-1: Positioning Measurement Gap Activation/Deactivation Request MAC CE - 6.1.3.41 Positioning Measurement Gap Activation/Deactivation Command MAC CE The Positioning Measurement Gap Activation/Deactivation Command MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a fixed 8-bit size and consists of a single octet defined as follows (Figure 6.1.3.41-1): - Positioning MG ID: This field indicates the identifier for the preconfigured positioning measurement gap. The length of the field is 4 bits. - A/D: This field indicates the activation or deactivation of the Positioning Measurement Gap. The field is set to 1 to indicate activation, otherwise it indicates deactivation. The length of the field is 1 bit. Figure 6.1.3.41-1: Positioning Measurement Gap Activation/Deactivation Command MAC CE ’

###### Question: Do companies agree with the text proposal above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSIlicon | Yes |  |
| Intel | Yes |  |
| Apple | Yes | Do we need the “has a fixed 8-bit size and”? That’s what “single octet” means anyway. Seems redundant. |
| Qualcomm | Yes |  |
| CATT | Yes |  |
| OPPO | Yes |  |
| ZTE | Yes |  |
| vivo | Yes |  |
| Ericsson | No | Agree with Apple comments. It would have been good if in the same MAC CE UE can indicate multiple preference and also possibility to signal deactivate for a MG e.g ID 1 and activate MG e.g. ID 2 simultaneously. With the proposed current structure this is not possible. |

In addition, on the cancellation of the MAC CE, the following text proposal has been given in

## 5.25 Positioning Measurement Gap Activation/Deactivation Request If the UE is configured with pre-configured measurement gap, the UE may request the network to activate or deactivate the Positioning measurement gap with UL MAC CE for Positioning Measurement Gap Activation/Deactivation Request in clause 6.1.3.40. The MAC entity shall, when triggered by the upper layer to send Positioning Measurement Gap Activation/Deactivation Request, cancel the triggered Positioning Measurement Gap Activation/Deactivation Request, if any and trigger another Positioning Measurement Gap Activation/Deactivation Request according to the upper layer’s request. The MAC entity shall, - if Positioning Measurement Gap Activation/Deactivation Request MAC CE has been triggered by upper layer: 2>If a request from upper layers to transmit a new request to gNB for a new/modified gap configuration is received, or 2>an indication from upper layers that the gaps are not needed any more is received, or 2>an indication from upper layers that a gap with a new id needs to be activated is received: 3>cancel triggered Positioning Measurement Gap Activation/Deactivation Request MAC CE. 2>else: 3>if UL-SCH resources are available for a new transmission and these UL-SCH resources can accommodate the Positioning Measurement Gap Activation/Deactivation Request MAC CE plus its subheader as a result of logical channel prioritization: 4> instruct the Multiplexing and Assembly procedure to generate the Positioning Measurement Gap Activation/Deactivation Request MAC CE according to the upper layer’s request; 4> cancel triggered Positioning Measurement Gap Activation/Deactivation Request MAC CE. - else: 4> trigger a Scheduling Request for Positioning Measurement Gap Activation/Deactivation Request MAC CE.

###### Question3: Do companies agree with the text proposal above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | The MAC layer just follows the request from RRC layer for activation/deactivation or the possible cancellation of the MAC CE. Not need to capture this in the MAC spec. |
| Intel | No | Agree with Huawei. MAC does not need to be aware why the RRC layer want to cancel the MAC CE. The details should be captured in RRC layer. In MAC layer, MAC only needs to follow cancellation request from upper layer. |
| Apple | No | Agree with Huawei and Intel |
| CATT |  | The current cancel operation for other UL MAC CE is also specified in the MAC spec. Moreover, if the procedure text is not agreed, maybe a note is needed, to make the specification clear. |
| OPPO | No | Agree with Huawei |
| ZTE | Yes | If the previous agreement of cancellation procedure triggered by upper layer does not revise, then it is needed to capture in both RRC and MAC(e.g., briefly in RRC while specifically in MAC). In the offline discussion 634 Q11, some companies think these cancellation procedures are MAC restriction and should be specified in MAC spec. So we should discuss how to capture the cancellation condition in MAC: - a request from upper layers to transmit a new request to gNB for a new/modified gap configuration is received; - an indication from upper layers that the gaps are not needed any more or a gap with a new id needs to be activated is received This 5.25 change can used as a baseline |
| vivo | No | Agree with Huawei and Intel |
| Ericsson | No, but | It is fine for us to not specify explicitly the reasons for cancelling the trigger in MAC. But we are fine to have discussion if proponent can motivate if something is broken if not specified explicitly. Our view is that: It is already specified in RRC that UE can request for new MG and also it is already mentioned in MAC that: The MAC entity shall, when triggered by the upper layer to send Positioning Measurement Gap Activation/Deactivation Request, cancel the triggered Positioning Measurement Gap Activation/Deactivation Request, if any and trigger another Positioning Measurement Gap Activation/Deactivation Request according to the upper layer’s request. Hence, it is clear that MAC would cancel the existing trigger if new one is requested. |

# 4.	Conclusion

Based on the discussion we give the following proposals:

# 4. Reference