3GPP TSG-RAN WG2 #118-e	R2-220xxxx

E-meeting, May 2022

Agenda Item:	x.x.x.x

Source:	OPPO

Title:	Summary of [640] Cast type for discovery (OPPO)

Document for:	Discussion, Decision

# Introduction

This is for the discussion below

- [AT118-e][640][Relay] Cast type for discovery (OPPO)

Scope: Discuss the options on groupcast and unicast for discovery (P6 of R2-2206243) and determine whether spec impact and/or an LS to SA2 is needed.

Intended outcome: Report to Monday week 2 session

Deadline:  Friday 2022-05-13 1800 UTC

# Discussion

Firstly, it is good to clarify the related discovery message, i.e.,

| Discovery messages | Cast type |
| --- | --- |
| Announcement | BC |
| Response | UC or BC? |
| Solicitation | BC |
| UE-to-network relay discovery announcement | BC |
| UE-to-network relay discovery response | UC or BC? |
| UE-to-network relay discovery solicitation | BC |
| Group member discovery announcement | GC or BC? |
| Group member discovery response | UC or BC? |
| Group member discovery solicitation | GC or BC? |
| Relay discovery additional information | BC |

Rapp observed two directions:

- Either we allow UC / GC based discovery

- Or we limit to BC based discovery only.

The following discussion is to check the feasibility of each direction.

# Feasibility of Alt-1: Allow UC/GC based discovery

In the following paper

R2-2205963	Correction on Groupcast transmission mode support for sidelink discovery	Qualcomm Incorporated	draftCR	Rel-17	38.322	17.0.0	C	NR_SL_relay-Core

It says

Sidleink groupcast transmission mode requires HARQ, and since it was agreed that discovery transmission does not have HARQ feedback support, groupcast cannot be used for sidelink discovery.

Q1-1: Do you agree based on the current AS-layer spec “groupcast CANNOT support HARQ transmission w/o feedback”?

| Company | Agree/Disagree | Comment |
| --- | --- | --- |
| OPPO | Disagree | In either SCI-2A (can be used for all cast types) and SCI-2B (can be used for GC only), both w/ and w/o FB can be supported. So not see the reason of this statement. |
| MediaTek | Disagree | We think the HARQ FB can be actually disabled for GC |

In the email discussion, QUALCOMM claimed

However, for supporting unicast or groupcast casttypes, we need to discuss the issues that we raised (unicast link setup need, PDCP format for unicast, without Upper layer indication how can AS layer decide which casttype to use) in the email discussion to conclude they can be supported by RAN2.

If Rapp get the point clearly, it is to say since discovery may happen before the UC link establishment, so UC cast type cannot be used.

Q1-2: Do you agree that based on the current AS-layer spec “since discovery may happen before the UC link establishment, so UC cast type cannot be used”?

| Company | Agree/Disagree | Comment |
| --- | --- | --- |
| OPPO | Disagree | So not see the reason of this statement, AS-layer spec does not prevent the usage of UC based cast-type-indicator for discovery message w/o UC-based communication link establishment, there is no coupling between the two. |
| MediaTek | Agree | As discussed during on session, we fail to see the scenario in practice, where the discovery message needs to be carried by a UC link. Since if there is a UC link between two peer UEs, this means they know each other. We do not see the need for one UE to discover the peer UE. |

In the email discussion, QUALCOMM claimed

However, for supporting unicast or groupcast casttypes, we need to discuss the issues that we raised (unicast link setup need, PDCP format for unicast, without Upper layer indication how can AS layer decide which casttype to use) in the email discussion to conclude they can be supported by RAN2.

If Rapp get the point clearly, it is to say since PDCP for UC is different from the format for BC, so UC cast type cannot be used.

Q1-3: Do you agree that based on the current AS-layer spec “since PDCP for UC is different from the format for BC, so UC cast type cannot be used”?

| Company | Agree/Disagree | Comment |
| --- | --- | --- |
| OPPO | Disagree | So not see the reason of this statement, 323 only define a single format for discovery SL-SRB4, w/o differentiating cast types. |
| MediaTek | Agree | We see the point made by Qualcomm during online session on that PDCP for UC may be different from the format for BC. With this understanding, when we design SL-SRB4, the intention is to deliver it not based on UC mode |

In the email discussion, QUALCOMM claimed

However, for supporting unicast or groupcast casttypes, we need to discuss the issues that we raised (unicast link setup need, PDCP format for unicast, without Upper layer indication how can AS layer decide which casttype to use) in the email discussion to conclude they can be supported by RAN2.

If Rapp get the point clearly, it is to say since upper layer does not provide cast type indication to lower layer, so AS layer cannot make use of UC / GC.

Q1-4: Do you agree that in the current spec “since upper layer does not provide cast type indication to lower layer, so AS layer cannot make use of UC / GC”?

| Company | Agree/Disagree | Comment |
| --- | --- | --- |
| OPPO | Agree | Our S2 colleague also agree it is missing in SA2 spec. |
| MediaTek | Comment | We do not think the upper layer need to indicate the cast type to AS layer for each message. |

Q1-5: If yes to any one of Q1-1/2/3/4, how do you think it should be solved?

| Company | Comment |
| --- | --- |
| OPPO | Either we rely on S2 to handle it by their own since it is merely a S2 oversight, Or we can send a LS to urge them to fix it. |
| MediaTek | We think AS layer can handle the issue if there is no indication from upper layer. E.g. if there is no GC address provided, AS can deliver it via BC. |

# Feasibility of Alt-2: Disallow UC/GC based discovery

Based on the SA2 spec, at transmitter side, the L2 ID setting is as follows, i.e., obviously the L2 ID setting is based on the assumption of UC and GC based transmission.

<23.304 start>

1.	The Discoverer UE sends a Solicitation message. The Solicitation message may include Type of Discovery Message, ProSe Query Code, security protection element.

The Destination Layer-2 ID and Source Layer-2 ID used to send the Solicitation message are specified in clause 5.8.1.2 and clause 5.8.1.3.

How the Discoveree UE determines the Destination Layer-2 ID for signalling reception is specified in clause 5.8.1.2.

2.	The Discoveree UE that matches the solicitation message responds to the Discoverer UE with the Response message. The Response message may include Type of Discovery Message, ProSe Response Code, security protection element, [metadata information]. The Application layer metadata information may be included as metadata in the Response message.

The Source Layer-2 ID used to send the Response message is specified in clause 5.8.1.3. The Destination Layer-2 ID is set to the Source Layer-2 ID of the received Solicitation message.

[…]

For Group member discovery:

-	If an Application Layer Group ID has a configured Layer-2 Group ID, which is provisioned as specified in clause 5.1.2.1, the UE uses this Layer-2 Group ID as the Destination Layer-2 ID,

-	otherwise, the UE converts the Application Layer Group ID into a Destination Layer-2 ID.

NOTE:	The mechanism for converting the application layer provided Application Layer Group ID to the Destination Layer-2 ID is defined in Stage 3.

<23.304 stop>

At receiver side, the L2 ID setting is also configured, so that Rx UE can be aware of it for correct reception.

<23.304 start>

2)	Parameters used for 5G ProSe Direct Discovery:

-	The mapping of ProSe services (i.e. ProSe identifiers) to Destination Layer-2 ID(s) for sending/receiving initial signalling of discovery messages.

[…]

5)	Group Member Discovery parameters:

-	For each discovery group that the UE belongs to include the following parameters that enable the UE to perform Group Member Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	Application Layer Group ID: Identifies an application layer group or a discovery group that the UE belongs to;

-	Layer-2 Group ID: layer-2 ID for Application Layer Group ID;

<23.304 stop>

Then the filtering of L2 ID, which is now located at SCI and MAC-header, relies on the following part in MAC spec

<38.321 start>

1>	if the data for this TB was successfully decoded before:

2>	if this is the first successful decoding of the data for this TB:

3>	if this TB is associated to unicast, the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI, and the SRC field of the decoded MAC PDU subheader is equal to the 16 MSB of any of the Destination Layer-2 ID(s) of the UE for which the 8 LSB are equal to the Source ID in the corresponding SCI; or

3>	if this TB is associated to groupcast or broadcast and the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Destination Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI:

4>	deliver the decoded MAC PDU to the disassembly and demultiplexing entity;

<38.321 stop>

Where the cast type differentiation relies on the SCI: 1) SCI format 2-A: where a 2-bit field is used; or 2) SCI format 2-B: which is limited to GC case only.

<38.212 start>

SCI format 2-A is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes ACK or NACK, when HARQ-ACK information includes only NACK, or when there is no feedback of HARQ-ACK information.

The following information is transmitted by means of the SCI format 2-A:

-	HARQ process number –  bits.

-	New data indicator – 1 bit.

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2.

-	Source ID – 8 bits as defined in clause 8.1 of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause 8.1 of [6, TS 38.214].

-	HARQ feedback enabled/disabled indicator – 1 bit as defined in clause 16.3 of [5, TS 38.213].

-	Cast type indicator – 2 bits as defined in Table 8.4.1.1-1 and in clause 8.1 of [6, TS 38.214].

-	CSI request – 1 bit as defined in clause 8.2.1 of [6, TS 38.214] and in clause 8.1 of [6, TS 38.214].

Table 8.4.1.1-1: Cast type indicator

| Value of Cast type indicator | Cast type |
| --- | --- |
| 00 | Broadcast |
| 01 | Groupcast when HARQ-ACK information includes ACK or NACK |
| 10 | Unicast |
| 11 | Groupcast when HARQ-ACK information includes only NACK |

[…]

SCI format 2-B is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes only NACK, or when there is no feedback of HARQ-ACK information.

The following information is transmitted by means of the SCI format 2-B:

-	HARQ process number –  bits.

-	New data indicator – 1 bit.

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2.

-	Source ID – 8 bits as defined in clause 8.1 of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause 8.1 of [6, TS 38.214].

-	HARQ feedback enabled/disabled indicator – 1 bit as defined in clause 16.3 of [5, TS 38.213].

-	Zone ID – 12 bits as defined in clause 5.8.11 of [9, TS 38.331].

-	Communication range requirement – 4 bits determined by higher layer parameter sl-ZoneConfigMCR-Index.

<38.212 stop>

So originally, the MAC layer filtering is used to do the following check

|  | SRC @ MAC header and source-ID in SCI | DST @ MAC header and destination-ID in SCI |
| --- | --- | --- |
| UC | = destination UC L2 ID? | = source L2 ID? |
| GC |  | = destination GC L2 ID? |
| BC |  | = destination BC L2 ID? |

And if we force the AS-layer to use BC only, i.e., to use BC-based cast-type-indicator for all cases, the checking will be as follows

|  | SRC @ MAC header and source-ID in SCI | DST @ MAC header and destination-ID in SCI |
| --- | --- | --- |
| UC | = destination UC L2 ID? | = source L2 ID? = destination BC L2 ID? |
| GC |  | = destination GC L2 ID? = destination BC L2 ID? |
| BC |  | = destination BC L2 ID? |

Then basically this kind of MAC filtering (if following the current spec) basically is to check the UC/BC-based L2 ID in the way of BC-based L2 ID, and we foresee two possible results

- If the UE is not of interested of a UC/GC L2 ID but of interested in a BC L2 ID, the useless discovery message may by mistake pass the MAC layer filtering, and be delivered by upper layer;

- Or if the is of interested of a UC/GC L2 ID but not of interested in a BC L2 ID, the useless discovery message may be by mistake fail to pass the MAC layer filtering, and be discarded by lower layer;

In case-1, considering upper layer (ProSe layer) cannot further differentiate the discovery message via L2 ID (which is invisible to upper layer), some mis-operation would be cause, e,g., a UE-A who sent solicitation message, may receive a discovery response message from UE-B, even though UE-B meant to response UE-C..

Q2-1: Do you agree that based on the current AS-layer spec, if sending all discovery messages using BC cast-type-indicator, it may cause at Rx-UE side “the unrelated discovery message may by mistake pass the MAC layer filtering, and be delivered by upper layer” and thus lead to upper layer mis-operation?

| Company | Agree/Disagree | Comment |
| --- | --- | --- |
| OPPO | Agree | With the misalignment between cast-type-indicator and L2 ID, as analysed above, this will happen. At ProSe layer, without this info, mis-operation would be caused. |
| MediaTek | Comment | MAC layer filtering may pass the message to Upper layer, However we do not think upper layer will do mis-operation. |

In Case-2, obviously the discarding would cause necessary discovery failure.

Q2-2: Do you agree that based on the current AS-layer spec, if sending all discovery messages using BC manner, it may cause “the useful discovery message may by mistake fail to pass the MAC layer filtering, and be discarded” and thus lead to unnecessary discovery failure?

| Company | Agree/Disagree | Comment |
| --- | --- | --- |
| OPPO | Agree | With the misalignment between cast-type-indicator and L2 ID, as analysed above, this will happen. |
| MediaTek | Comment | We did not see the possibility for BC based discovery message to fail to pass the MAC layer filtering |

Q2-3: If yes to either one of Q2-1/2, how do you think it should be solved?

| Company | Comment |
| --- | --- |
| OPPO | With cast-type-indicator abandoned, Rx-UE cannot know the real cast-type when receiving this at MAC-layer, even if we revise the MAC-layer behavior, e.g., to do the MAC delivery considering all the possibility of UC, GC, BC cases, - It cannot solve the issue in Q2-1, i.e., some irrelated discovery message would be sent to upper layer, which is not able to further differentiate - This would cause NBC change since this behavior starts from R16. So we do not see a feasible solution here. |

# Final Suggestion

Q3: Combine the two sides, what is your suggestion as next-step?

| Company | Comment |
| --- | --- |
| OPPO | We should not allow the possibility to go to Alt-2 since there is no feasible solution technically. We are open to discuss the next-step in the direction of Alt-1, e.g., sending LS to SA2, but that should not be used as a tool to open the door to Alt-2. |
| MediaTek | In general, we think BC only based discovery transmission can work even though the current MAC filtering operation may be not optimal. We did not see a must to send LS to SA2. By the way, there may be some possible optimizations for PHY/MAC filtering operation to support more graceful transmission of discovery message. But we see a difficult to go that far at this late stage of R17. |

- xxx.

# Conclusion

We have the following proposals:

Proposal 1	xxx.

# Reference

[1] R2-2206243		Initial comments on discovery and (re)selection of AI 6.7.2.5	vivo	discussion