3GPP TSG-RAN WG2 #118-e	R2-220xxxx

E-meeting, May 2022

Agenda Item:	x.x.x.x

Source:	OPPO

Title:	Summary of [640] Cast type for discovery (OPPO)

Document for:	Discussion, Decision

# Introduction

This is for the discussion below

- [AT118-e][640][Relay] Cast type for discovery (OPPO)

Scope: Discuss the options on groupcast and unicast for discovery (P6 of R2-2206243) and determine whether spec impact and/or an LS to SA2 is needed.

Intended outcome: Report to Monday week 2 session

Deadline:  Friday 2022-05-13 1800 UTC

# Discussion

Firstly, it is good to clarify the related discovery message, i.e.,

| Discovery messages | Cast type |
| --- | --- |
| Announcement | BC |
| Response | UC or BC? |
| Solicitation | BC |
| UE-to-network relay discovery announcement | BC |
| UE-to-network relay discovery response | UC or BC? |
| UE-to-network relay discovery solicitation | BC |
| Group member discovery announcement | GC or BC? |
| Group member discovery response | UC or BC? |
| Group member discovery solicitation | GC or BC? |
| Relay discovery additional information | BC |

Rapp observed two directions:

- Either we allow UC / GC based discovery

- Or we limit to BC based discovery only.

The following discussion is to check the feasibility of each direction.

# Feasibility of Alt-1: Allow UC/GC based discovery

In the following paper

R2-2205963	Correction on Groupcast transmission mode support for sidelink discovery	Qualcomm Incorporated	draftCR	Rel-17	38.322	17.0.0	C	NR_SL_relay-Core

It says

Sidleink groupcast transmission mode requires HARQ, and since it was agreed that discovery transmission does not have HARQ feedback support, groupcast cannot be used for sidelink discovery.

Q1-1: Do you agree based on the current AS-layer spec “groupcast CANNOT support HARQ transmission w/o feedback”?

| Company | Agree/Disagree | Comment |
| --- | --- | --- |
| OPPO | Disagree | In either SCI-2A (can be used for all cast types) and SCI-2B (can be used for GC only), both w/ and w/o FB can be supported. So not see the reason of this statement. |
| MediaTek | Disagree | We think the HARQ FB can be actually disabled for GC |
| Ericsson | disagree |  |
| Apple | See comment | I think the point is that from AS layer perspective SL GC wo/FB = SL BC. So, if SA2 provide a justification for this ”SL GC w/o FB” case, we can support. If they do not think this is needed, then we do not need support. There is no AS layer reasons to support “SL GC w/o FB”. |
| InterDigital | disagree | Groupcast without HARQ feedback is possible. |
| Samsung | Disagree | Groupcast with/wo HARQ feedback is possible. |
| Sharp | Disagree | Groupcast without HARQ is possible. |
| CATT | Disagree |  |
| ZTE | Disagree | HARQ feedback is not madatory for groupcast. |
| Lenovo | Disagree |  |

In the email discussion, QUALCOMM claimed

However, for supporting unicast or groupcast casttypes, we need to discuss the issues that we raised (unicast link setup need, PDCP format for unicast, without Upper layer indication how can AS layer decide which casttype to use) in the email discussion to conclude they can be supported by RAN2.

If Rapp get the point clearly, it is to say since discovery may happen before the UC link establishment, so UC cast type cannot be used.

Q1-2: Do you agree that based on the current AS-layer spec “since discovery may happen before the UC link establishment, so UC cast type cannot be used”?

| Company | Agree/Disagree | Comment |
| --- | --- | --- |
| OPPO | Disagree | So not see the reason of this statement, AS-layer spec does not prevent the usage of UC based cast-type-indicator for discovery message w/o UC-based communication link establishment, there is no coupling between the two. |
| MediaTek | Agree | As discussed during on session, we fail to see the scenario in practice, where the discovery message needs to be carried by a UC link. [OPPO] Me neither. Since if there is a UC link between two peer UEs, this means they know each other. We do not see the need for one UE to discover the peer UE. [OPPO] Not sure if we are talking about the same issue, I thought the opponent of this option holds the view that UC-based discovery CANNOT be sent without a UC link established, which I do not agree, meaning I think there is no restriction and thus UC-based discovery CAN be sent without a UC link established. Please correct me if any missing point. |
| Ericsson | disagree | RAN2 has already agreed to support all cast types for discovery. There is no strong reason to revert RAN2 agreements. Also, at least discovery response can be transmitted in unicast fashion. In addition, restrict cast type for discovery, would just add unnecessary limit on potential L2 ID space which can be used to transmit discovery. |
| Apple | Agree | It is a common concept in R16 NR SL design that PC5-RRC is needed to enable SL unicast (which is a major feature in NR V2X, compare to LTE V2X). To allow discovery message (prior to PC5 link setup) to be indicated as SL unicast in SCI has created an unnecessary and useless exception. We need avoid this from the system design perspective. [OPPO] Yet what I observed in the response above is it seems like preference yet no tech blocking issue identified? Without AS-layer security, and with specified configuration, I have not identified blocking issue. Please correct me. Again, we need to let SA2 to provide a justification for enable this in discovery. But so far, we do not seen SA2 spec has explicitly required to support this. |
| InterDigital | Agree | I think the point here is that to use the unicast-based mechanisms in the AS layer, a PC5-RRC connection is needed. This is not possible for the transmission of discovery. [OPPO] Same response as for Apple. |
| Samsung | Disagree | We think that discovery can be sent using UC L2 ID even before L2 link establishment. BTW, we wonder DCR is BC type or UC type. Haven’t the DCR message treated as UC type which is used before unicast link establishment? |
| Sharp | Agree | We share the view with Media Tek. We also fail to see the scenario in practice, where the discovery message needs to be carried by a UC link. If there is no scenario for which discovery message is transmitted by a UC link, why we have to support it in a UC mechanism. |
| CATT | Disagree | Same view as OPPO and E. |
| ZTE | Disagree | We think that if the unicast identifier is used for the DST field in the MAC header for SL-SCH, it means the unicast sidelink transmission. In this sense, the discovery response message should be unicast in nature. |
| Lenovo | Disagree | Agree with Oppo. UE based discovery can be sent before UC link establishment. |

In the email discussion, QUALCOMM claimed

However, for supporting unicast or groupcast casttypes, we need to discuss the issues that we raised (unicast link setup need, PDCP format for unicast, without Upper layer indication how can AS layer decide which casttype to use) in the email discussion to conclude they can be supported by RAN2.

If Rapp get the point clearly, it is to say since PDCP for UC is different from the format for BC, so UC cast type cannot be used.

Q1-3: Do you agree that based on the current AS-layer spec “since PDCP for UC is different from the format for BC, so UC cast type cannot be used”?

| Company | Agree/Disagree | Comment |
| --- | --- | --- |
| OPPO | Disagree | So not see the reason of this statement, 323 only define a single format for discovery SL-SRB4, w/o differentiating cast types. |
| MediaTek | Agree | We see the point made by Qualcomm during online session on that PDCP for UC may be different from the format for BC. With this understanding, when we design SL-SRB4, the intention is to deliver it not based on UC mode. [OPPO] After a talk with MTK, I understand the intention of this comment is to say some companies have not thought about the usage of this format to UC-based discovery (?) but not saying the current PDCP spec has defined a different format for UC-based discovery already. If that is the case, I just would like to clarify at least from proponent perspective, indeed when we adopt the format in PDCP spec, it was thought to apply to UC case as well without further differentiation. And so far we have not identify blocking issue to apply this to UC case yet – if there is, please elaborate, thanks! |
| Ericsson | Disagree. | Arguments doesn’t make sense. |
| Apple | Agree | The problem is that we have to mention this exception (SLSRB4 UC is not UC) consistently in all the AS specs, which is a problem shall be avoided. For example, the following text in 38323 is not consistent if we support SL SRB4 UC. For the same SRB, we now have two different ways to set RX_NEXT variable: RX_NEXT This state variable indicates the COUNT value of the next PDCP SDU expected to be received. The initial value is 0, except for sidelink broadcast and groupcast, for SRBs configured with state variables continuation, and for MRBs. For NR sidelink communication for broadcast and groupcast or sidelink SRB4 for broadcast and groupcast based sidelink discovery, the initial value of the SN part of RX_NEXT is (x +1) modulo (2[sl-PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU. For MRBs, the initial value of the SN part of RX_NEXT is (x +1) modulo (2[PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB. NOTE: For NR sidelink communication for broadcast and groupcast or sidelink SRB4 for broadcast and groupcast based sidelink discovery, it is up to UE implementation to select the HFN part for RX_NEXT such that initial value of RX_DELIV should be a positive value. [OPPO] This seems to be just specification clarification, yet not tech blocking issue? [Samsung] The sentence is not to exclude discovery with UC. The sentence is to specify the case for discovery with GC/BC which is handled differently from the case for discovery with UC. For the discovery with UC it follows normal procedure i.e., the initial value is set to 0 in the above example procedure. |
| InterDigital | Agree | Agree with Apple. |
| Samsung | Disagree | The PDCP Data PDU format can be same for any cast type of discovery. The same format as for DCR, here we understand DCR as UC type. |
| Sharp | Agree | Agree with Apple |
| CATT | Disagree | We share the same view as OPPO and Samsung. |
| ZTE | Disagree | We think the PDCP format for SL-SRB4 could be applied to any cast type. |
| Lenovo | Disagree |  |

In the email discussion, QUALCOMM claimed

However, for supporting unicast or groupcast casttypes, we need to discuss the issues that we raised (unicast link setup need, PDCP format for unicast, without Upper layer indication how can AS layer decide which casttype to use) in the email discussion to conclude they can be supported by RAN2.

If Rapp get the point clearly, it is to say since upper layer does not provide cast type indication to lower layer, so AS layer cannot make use of UC / GC.

Q1-4: Do you agree that in the current spec “since upper layer does not provide cast type indication to lower layer, so AS layer cannot make use of UC / GC”?

| Company | Agree/Disagree | Comment |
| --- | --- | --- |
| OPPO | Agree | Our S2 colleague also agree it is missing in SA2 spec. |
| MediaTek | Comment | We do not think the upper layer need to indicate the cast type to AS layer for each message. |
| Ericsson | Agree | SA2 spec is indeed missing cast type mapping rule. So, we can just directly indicate to SA2 to fix this missing part, which doesn’ |
| Apple | Agree | Yes, we need SA2 to indicagte the support of this first. There is no AS layer reasons to support this. |
| InterDigital | Agree |  |
| Samsung | Agree | Even SA2 spec does not say about BC type for discovery. |
| Sharp | Agree |  |
| CATT | Agree | We also get the feedback(the question part is missing in SA2’s spec) from our SA2. |
| ZTE | Agree |  |
| Lenovo | Agree |  |

Q1-5: If yes to any one of Q1-1/2/3/4, how do you think it should be solved?

| Company | Comment |
| --- | --- |
| OPPO | Either we rely on S2 to handle it by their own since it is merely a S2 oversight, Or we can send a LS to urge them to fix it. |
| MediaTek | We think AS layer can handle the issue if there is no indication from upper layer. E.g. if there is no GC address provided, AS can deliver it via BC. |
| Ericsson | We can just send LS to SA2 to let them to add the mapping rule for cast type for discovery. |
| Apple | We can send LS to SA2 to check if the support of UC/GC for SL-SRB4 is needed. |
| InterDigital | If we want to support UC/GC, LS to SA2 would be needed. Then RAN2 would need to address the SL-SRB4 limitations, and also clarify that UC-based discovery transmission is not AS-layer based unicast (i.e. no PC5-RRC connection). |
| Samsung | We can ask SA2 to fix it if it is missing or to confirm the cast type for discovery. |
| Sharp | A LS to SA2 for clarification is necessary. |
| CATT | Suggest sending LS to urge SA2 to fix this issue. |
| ZTE | We can send an LS to SA1 to fix this issue. |
| Lenovo | We can write an LS to them. |

# Feasibility of Alt-2: Disallow UC/GC based discovery

Based on the SA2 spec, at transmitter side, the L2 ID setting is as follows, i.e., obviously the L2 ID setting is based on the assumption of UC and GC based transmission.

<23.304 start>

  - The Discoverer UE sends a Solicitation message. The Solicitation message may include Type of Discovery Message, ProSe Query Code, security protection element.

The Destination Layer-2 ID and Source Layer-2 ID used to send the Solicitation message are specified in clause 5.8.1.2 and clause 5.8.1.3.

How the Discoveree UE determines the Destination Layer-2 ID for signalling reception is specified in clause 5.8.1.2.

  - The Discoveree UE that matches the solicitation message responds to the Discoverer UE with the Response message. The Response message may include Type of Discovery Message, ProSe Response Code, security protection element, [metadata information]. The Application layer metadata information may be included as metadata in the Response message.

The Source Layer-2 ID used to send the Response message is specified in clause 5.8.1.3. The Destination Layer-2 ID is set to the Source Layer-2 ID of the received Solicitation message.

[…]

For Group member discovery:

-	If an Application Layer Group ID has a configured Layer-2 Group ID, which is provisioned as specified in clause 5.1.2.1, the UE uses this Layer-2 Group ID as the Destination Layer-2 ID,

-	otherwise, the UE converts the Application Layer Group ID into a Destination Layer-2 ID.

NOTE:	The mechanism for converting the application layer provided Application Layer Group ID to the Destination Layer-2 ID is defined in Stage 3.

<23.304 stop>

At receiver side, the L2 ID setting is also configured, so that Rx UE can be aware of it for correct reception.

<23.304 start>

2)	Parameters used for 5G ProSe Direct Discovery:

-	The mapping of ProSe services (i.e. ProSe identifiers) to Destination Layer-2 ID(s) for sending/receiving initial signalling of discovery messages.

[…]

5)	Group Member Discovery parameters:

-	For each discovery group that the UE belongs to include the following parameters that enable the UE to perform Group Member Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	Application Layer Group ID: Identifies an application layer group or a discovery group that the UE belongs to;

-	Layer-2 Group ID: layer-2 ID for Application Layer Group ID;

<23.304 stop>

Then the filtering of L2 ID, which is now located at SCI and MAC-header, relies on the following part in MAC spec

<38.321 start>

  - if the data for this TB was successfully decoded before:

2>	if this is the first successful decoding of the data for this TB:

3>	if this TB is associated to unicast, the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI, and the SRC field of the decoded MAC PDU subheader is equal to the 16 MSB of any of the Destination Layer-2 ID(s) of the UE for which the 8 LSB are equal to the Source ID in the corresponding SCI; or

3>	if this TB is associated to groupcast or broadcast and the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Destination Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI:

4>	deliver the decoded MAC PDU to the disassembly and demultiplexing entity;

<38.321 stop>

Where the cast type differentiation relies on the SCI: 1) SCI format 2-A: where a 2-bit field is used; or 2) SCI format 2-B: which is limited to GC case only.

<38.212 start>

SCI format 2-A is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes ACK or NACK, when HARQ-ACK information includes only NACK, or when there is no feedback of HARQ-ACK information.

The following information is transmitted by means of the SCI format 2-A:

-	HARQ process number –  bits.

-	New data indicator – 1 bit.

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2.

-	Source ID – 8 bits as defined in clause 8.1 of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause 8.1 of [6, TS 38.214].

-	HARQ feedback enabled/disabled indicator – 1 bit as defined in clause 16.3 of [5, TS 38.213].

-	Cast type indicator – 2 bits as defined in Table 8.4.1.1-1 and in clause 8.1 of [6, TS 38.214].

-	CSI request – 1 bit as defined in clause 8.2.1 of [6, TS 38.214] and in clause 8.1 of [6, TS 38.214].

Table 8.4.1.1-1: Cast type indicator

| Value of Cast type indicator | Cast type |
| --- | --- |
| 00 | Broadcast |
| 01 | Groupcast when HARQ-ACK information includes ACK or NACK |
| 10 | Unicast |
| 11 | Groupcast when HARQ-ACK information includes only NACK |

[…]

SCI format 2-B is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes only NACK, or when there is no feedback of HARQ-ACK information.

The following information is transmitted by means of the SCI format 2-B:

-	HARQ process number –  bits.

-	New data indicator – 1 bit.

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2.

-	Source ID – 8 bits as defined in clause 8.1 of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause 8.1 of [6, TS 38.214].

-	HARQ feedback enabled/disabled indicator – 1 bit as defined in clause 16.3 of [5, TS 38.213].

-	Zone ID – 12 bits as defined in clause 5.8.11 of [9, TS 38.331].

-	Communication range requirement – 4 bits determined by higher layer parameter sl-ZoneConfigMCR-Index.

<38.212 stop>

So originally, the MAC layer filtering is used to do the following check

|  | SRC @ MAC header and source-ID in SCI | DST @ MAC header and destination-ID in SCI |
| --- | --- | --- |
| UC | = destination UC L2 ID? | = source L2 ID? |
| GC |  | = destination GC L2 ID? |
| BC |  | = destination BC L2 ID? |

And if we force the AS-layer to use BC only, i.e., to use BC-based cast-type-indicator for all cases, the checking will be as follows

|  | SRC @ MAC header and source-ID in SCI | DST @ MAC header and destination-ID in SCI |
| --- | --- | --- |
| UC | = destination UC L2 ID? | = source L2 ID? = destination BC L2 ID? |
| GC |  | = destination GC L2 ID? = destination BC L2 ID? |
| BC |  | = destination BC L2 ID? |

Then basically this kind of MAC filtering (if following the current spec) basically is to check the UC/BC-based L2 ID in the way of BC-based L2 ID, and we foresee two possible results

- If the UE is not of interested of a UC/GC L2 ID but of interested in a BC L2 ID, the useless discovery message may by mistake pass the MAC layer filtering, and be delivered by upper layer;

- Or if the is of interested of a UC/GC L2 ID but not of interested in a BC L2 ID, the useless discovery message may be by mistake fail to pass the MAC layer filtering, and be discarded by lower layer;

In case-1, considering upper layer (ProSe layer) cannot further differentiate the discovery message via L2 ID (which is invisible to upper layer), some mis-operation would be cause, e,g., a UE-A who sent solicitation message, may receive a discovery response message from UE-B, even though UE-B meant to response UE-C..

Q2-1: Do you agree that based on the current AS-layer spec, if sending all discovery messages using BC cast-type-indicator, it may cause at Rx-UE side “the unrelated discovery message may by mistake pass the MAC layer filtering, and be delivered by upper layer” and thus lead to upper layer mis-operation?

| Company | Agree/Disagree | Comment |
| --- | --- | --- |
| OPPO | Agree | With the misalignment between cast-type-indicator and L2 ID, as analysed above, this will happen. At ProSe layer, without this info, mis-operation would be caused. |
| MediaTek | Comment | MAC layer filtering may pass the message to Upper layer, However we do not think upper layer will do mis-operation. [OPPO] After a short check with MTK, seems the view is that maybe we can rely on upper layer (instead of lower layer) to do the filtering. Yet just would like to highlight that: in discovery message, there is no such L2 ID related field, so ProSe layer cannot do the filtering (NOTE that this is different from discovery message defined in LTE, where the L2 ID is included in discovery message as upper layer payload, and at MAC layer we use a transparent mode (without MAC header), so the filter was done at upper layer). |
| Ericsson | comments | Don’t think this question is necessary. Our view is that no change of AS spec, we stick to previous RAN2 agreements, and send LS to SA2 to add missing part, i.e., defining all cast types for discovery. |
| Apple | Disagree | Ther L2 Destination ID is always check in MAC according to 38.321 for each message no matter what cast type is used. We do not think there is a filtering problem. For a receiver, the MAC layer filter is still done based on the L2 ID provided by upper layer. For example, the model B discover UE’s MAC layer will filter the incoming messages matching its own Src L2 ID. [OPPO] how to achieve this Src L2 ID based checking? Which is specified in 321 for unicast only, and if we always set the cast-type-indicator as BC, why would the discoverer UE to use the UC-based changed instead? The model A discovery UE will filter based on provisioned broadcast address for discovery. We do not think the filtering behaviour has disabled by disabling SL UC/GC for discovery. |
| InterDigital | Disagree | The upper layers should configure the AS layers with the appropriate L2 ID for the filtering. [OPPO] See my question to Apple above. |
| Samsung | See comment | We have similar concern with Ericsson. SA2 should fix the cast type indicator for discovery regardless of which cast type support since BC indicator is needed for discovery in NR. |
| Sharp | comments | It could happen but it could be done by UE implementation. |
| CATT | Agree | Indeed, there is a risk of sending all discovery messages using BC and this will destroy the stability of RAN layer deeply. |
| ZTE | Comment | Sending all the discovery message with BC cast type indicator is acutally not aligned with the current SA2 and RAN2 design principle. |
| Lenovo | Disagree | We do not see (or understand) a problem even if the 2 bits cast type in SCI format 2A is forced as BC. The Phy + MAC filtering do check the entire 24 bits which we assume is unique across all cast types. |

In Case-2, obviously the discarding would cause necessary discovery failure.

Q2-2: Do you agree that based on the current AS-layer spec, if sending all discovery messages using BC manner, it may cause “the useful discovery message may by mistake fail to pass the MAC layer filtering, and be discarded” and thus lead to unnecessary discovery failure?

| Company | Agree/Disagree | Comment |
| --- | --- | --- |
| OPPO | Agree | With the misalignment between cast-type-indicator and L2 ID, as analysed above, this will happen. |
| MediaTek | Comment | We did not see the possibility for BC based discovery message to fail to pass the MAC layer filtering. [OPPO] Please note that when MAC layer perform BC-based filtering, based on the current AS layer spec, it would check if the received packet is with “BC-based destination L2 ID that of interested”, so if the discovery-response-message, which use the source L2 ID of solicitation message as the destination L2 ID, yet this destination L2 ID is not in the “BC-based destination L2 ID that of interested”, it will be dropped, and one cannot ensure every “source L2 ID of solicitation message” is within “BC-based destination L2 ID that of interested” based on the current spec. |
| Ericsson | comments | Don’t think this question is necessary. Our view is that no change of AS spec, we stick to previous RAN2 agreements, and send LS to SA2 to add missing part, i.e., defining all cast types for discovery. |
| Apple | Yes with comments | The current text needs to be changed because a discovery RX UE need to match any GC/BC destination ID + Src L2 ID for GC/BC case. However, the current UC-based filtering also fails as it allows the SL-SRB4 UC case, as the current text read as below: if this TB is associated to unicast, the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI, and the SRC field of the decoded MAC PDU subheader is equal to the 16 MSB of any of the Destination Layer-2 ID(s) of the UE for which the 8 LSB are equal to the Source ID in the corresponding SCI; or Ther is no chance for model B discover UE to alreasy have a “L2 Destionon ID” match the discoveree UE’s address in model B message as model-B discover UE’s existing destination address is only a BC address provisioned for discovery purpose. So, we need some fix in 38.321 anyway for the discovery case. [OPPO] Not sure if we are on the same page: if we limit to model-B discoverer UE: For UC case, it needs to do SRC @ MAC header and source-ID in SCI; DST @ MAC header and destination-ID in SCI. = destination UC L2 ID?; = source L2 ID?. For BC case, it needs to do SRC @ MAC header and source-ID in SCI; DST @ MAC header and destination-ID in SCI. = destination UC L2 ID?; = source L2 ID? = destination BC L2 ID?. What Apple mentioned is the difficulty to do checking for “SRC @ MAC header and source-ID in SCI”, that is similar to what we are discussing in 702, i.e., similar change can be applied. And even after that change, the difference on “DST @ MAC header and destination-ID in SCI” based checking remains, i.e., checking whether a “DST @ MAC header and destination-ID in SCI” = source L2 ID or = destination BC L2 ID of interest is still different. |
| InterDigital | Agree |  |
| Samsung | See comment | Same as Q2-1 |
| Sharp | Disagree | According to the response from rapp, the failure is based on the assumption that “one cannot ensure every “source L2 ID of solicitation message” is within “BC-based destination L2 ID that of interested” based on the current spec.” From our understanding, the Discoverer UE should be aware of its L2 ID, i.e. the one carried in the solicitation message which is transmitted by itself. As we commented before, we has not seen the scenario of discover for the UC link. So the response to the solicitation message is also in BC mode. Then the assumption is not existing. [OPPO] As I tried to clarify, the operation of comparing “the one carried in the solicitation message which is transmitted by itself”, i.e., the source L2 ID w.r.t “DST @ MAC header and destination-ID in SCI” the is done for UC based cast-type-indicator only, if following the current spec. |
| CATT | Agree | Please see our answer in the previous question. |
| ZTE | Comment | Sending all the discovery message with BC cast type indicator is acutally not aligned with the current SA2 and RAN2 design principle. |
| Lenovo | Not sure/ disagree |  |

Q2-3: If yes to either one of Q2-1/2, how do you think it should be solved?

| Company | Comment |
| --- | --- |
| OPPO | With cast-type-indicator abandoned, Rx-UE cannot know the real cast-type when receiving this at MAC-layer, even if we revise the MAC-layer 10ehaviour, e.g., to do the MAC delivery considering all the possibility of UC, GC, BC cases, - It cannot solve the issue in Q2-1, i.e., some irrelated discovery message would be sent to upper layer, which is not able to further differentiate - This would cause NBC change since this 11ehaviour starts from R16. So we do not see a feasible solution here. |
| Apple | As explained in Q2-1 and Q2-2, If we fix the 321 spec for MAC-filtering, the problem is solved. Please note the change only affects the discovery message, so there is no NBC issue, We do not mean to “completely abandon the cast-type”. [Qianxi] The NBC issue we are talking about is: We may have shared pool for R16 and R17 UE, and the revised packet filtering may be applied to in-coming R16 packet as well. And we do not think there is a feasible solution for Q2-1. |
| InterDigital | MAC filtering changes can be made if we go with this option. |
| CATT | At least we indeed need more discussion for the AS solution if needed. |

# Final Suggestion

Q3: Combine the two sides, what is your suggestion as next-step?

| Company | Comment |
| --- | --- |
| OPPO | We should not allow the possibility to go to Alt-2 since there is no feasible solution technically. We are open to discuss the next-step in the direction of Alt-1, e.g., sending LS to SA2, but that should not be used as a tool to open the door to Alt-2. |
| MediaTek | In general, we think BC only based discovery transmission can work even though the current MAC filtering operation may be not optimal. We did not see a must to send LS to SA2. [OPPO] no LS is also fine for us. By the way, there may be some possible optimizations for PHY/MAC filtering operation to support more graceful transmission of discovery message. But we see a difficult to go that far at this late stage of R17. [OPPO] Our position is also that we can keep the current AS layer spec without further change. After a further check with MTK, we may say there are generally two directions: 1) one is to do the message filtering @ MAC, 2) the other is to do the filtering @ upper layer. We understand alt-2 requires a) a change @ MAC to ensure all related messages to upper layer without any lost / missing (we understand the current spec cannot achieve that), and b) upper layer can in some way see the L2 ID so that can do the filtering, yet so far L2 ID is not a part of the discovery message payload. So seems 2) is difficult. And 1) does not require AS-layer change, yet just an upper-layer change that is to send the cast-type indicator to lower layer, which is much easier to go. |
| Ericsson | As we commented, we need to stick to previous RAN2 agreements, i.e., support all cast types for discovery. Since we should avoid to add any unnecessary limit on potential L2 ID space which can be used by UE to transmit discovery message. We should send LS to SA2 to add missing parts for discovery. |
| Apple | In general, we also think BC-based discovery can work. There are some small tweak in MAC spec which needs to be fixed, which can be solved with a post-meeting short email discussion. If there is no consensus, we can send a LS to SA2 to check whether GC/UC cast-type for discovery messages are to be passed down to AS layer. |
| InterDigital | It would be preferred to avoid LS at this stage and address this problem in RAN2 only. |
| Samsung | We should send LS to SA2 to fix the missing part. |
| Sharp | We could send a LS to SA2 for clarification whether GC/UC cast-type for discovery messages is required. |
| CATT | It is clear for SA2 to solve this issue is more easy. Hence, we suggest RAN2 to send a LS to SA2 for adding the indication of cast-type for discovery message. |
| ZTE | Send an LS to SA2 to clarify and fix the issue. |
| Lenovo | We request SA2 to align this case with other i.e., pass on the cast type along with L2 IDs together with the payload (Discovery message to AS). |

- xxx.

# Conclusion

We have the following proposals:

Proposal 1	xxx.

# Reference

[1] R2-2206243		Initial comments on discovery and (re)selection of AI 6.7.2.5	vivo	discussion