3GPP TSG-RAN WG2 #118-e	R2-220xxxx

E-meeting, May 2022

Agenda Item:	6.15.2.3

Source:	OPPO

Title:	[Draft]Summary of [AT118-e][712][V2X/SL] User plane discussion (OPPO)

Document for:	Discussion, Decision

# Introduction

This document is a report on the following email discussion:

- [AT118-e][712][V2X/SL] User plane discussion (OPPO)

Scope: Discuss and conclude pre-selected issues for online discussion above.

Intended outcome: Discussion summary in R2-2206309

Deadline: 5/20 10:00am UTC

- According to the scope of this offline discussion, the following issues will be discussed:

Issue 1: Confirm the previous WAs?

-	“If there is no SL grant in the SL DRX active time of the destination that has data to be sent, trigger resource reselection.”.

-	“For mode-1 re-transmission grant, if the re-transmission grant is dropped due to no Rx-UE in active time, Tx-UE report NACK to network via PUCCH.”

Issue 2: Number of configured HARQ RTTs? (e.g. 2 timers in R2-2206138 (same timer value for timer#2 and timer#3), 3 timers in R2-2204579, 1 timer in R2-2205185 (timer#1 only))

-	Timer#1: HARQ enabled w/ PSFCH

-	Timer#2: HARQ disabled w/ PSFCH

-	Timer#3: HARQ disabled w/o PSFCH

Issue 3: Calculation of sl-drx-SlotOffset (e.g. in R2-2205136)?

Issue 4: SL triggering for SL DRX command indication (e.g. in R2-2205136)?

Issue 5: Need of active time extension after the announced periodic resource (e.g. in R2-2205833)?

# Contact Information

| Company | Name | Email Address |
| --- | --- | --- |
| OPPO | Bingxue Leng | lengbingxue@oppo.com |
| Huawei HiSilicon | Li Zhao | zhaoli8@huawei.com |
| LG | Giwon Park | giwon.park@lge.com |
| ASUSTeK | Xinra Kung | Xinra_Kung@asus.com |

# Discussion

## Issue 1:  Confirm the previous WAs

The following 2 WAs related to user plane procedure are made in last RAN2 meeting

28:	Working assumption: if there is no SL grant in the SL DRX active time of the destination that has data to be sent, trigger resource reselection.

32:	Working assumption: For mode-1 re-transmission grant, if the re-transmission grant is dropped due to no Rx-UE in active time, Tx-UE report NACK to network via PUCCH

The following contributions have discussed whether to confirm the above 2 Working Assumptions:

| R2-2204580 | OPPO | Proposal 1 RAN2 confirm the WA “if there is no SL grant in the SL DRX active time of the destination that has data to be sent, trigger resource reselection”. Proposal 2 RAN2 confirm the WA “For mode-1 re-transmission grant, if the re-transmission grant is dropped due to no Rx-UE in active time, Tx-UE report NACK to network via PUCCH”. |
| --- | --- | --- |
| R2-2205105 | ZTE Corporation, Sanechips | Proposal1: RAN2 is suggested to confirm the working assumption: If there is no SL grant in the SL DRX active time of the destination that has data to be sent, trigger resource reselection. Proposal2： For mode-1 re-transmission grant, if the re-transmission grant is dropped due to no Rx-UE in active time, Tx-UE report NACK to network via PUCCH. |
| R2-2205536 | Samsung | [Proposal 1]: RAN2 is asked to confirm the working assumption “if there is no SL grant in the SL DRX active time of the destination that has data to be sent, trigger resource reselection.” as an agreement. [Proposal 2]: RAN2 is asked to confirm the working assumption “For mode-1 re-transmission grant, if the re-transmission grant is dropped due to no Rx-UE in active time, Tx-UE report NACK to network via PUCCH” as an agreement. |

The following Qs are to check companies’ view on the confirmation of these 2 Working Assumptions:

Question 1-1: Do you agree on RAN2 to confirm the WA “if there is no SL grant in the SL DRX active time of the destination that has data to be sent, trigger resource reselection.”?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Huawei HiSilicon | Yes |  |
| LG | Yes |  |
| ASUSTeK | Yes |  |

Question 1-2: Do you agree on RAN2 to confirm the WA “For mode-1 re-transmission grant, if the re-transmission grant is dropped due to no Rx-UE in active time, Tx-UE report NACK to network via PUCCH.”?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Huawei HiSilicon | Yes |  |
| LG | Yes |  |
| ASUSTeK | Yes |  |

## Issue 2: Number of configured HARQ RTT timer length

The following agreements have been made in RAN2 #115 and RAN2 #117 for the values of RTT timer in different cases

HARQ RTT is supported for both HARQ enabled and HARQ disabled cases by allowing HARQ RTT timer to be set to different values.  FFS on the specific values that can be used for HARQ disabled case.

Recommendation 2.3.1-3a [15/17]: For resource pool without PSFCH, if SCI does not indicate re-transmission resource, allow sl-drx-HARQ-RTT-Timer timer length configuration different from the value for resource pool with PSFCH. The value of the RTT timer length (fixed to be zero, or allow non-zero value configuration as well) is FFS.

In summary, there are 3 different HARQ RTT timer values in the following cases:

- RTT-timer value1 for HARQ enabled case;

- RTT-timer value2 for HARQ disabled and resource pool with PSFCH case;

- RTT-timer value3 for HARQ disabled and resource pool without PSFCH case;

And in this meeting, the following contributions are discussing the configured HARQ RTT timer values for the 3 cases

| R2-2204580 | OPPO | Proposal 7 Confirm a first RTT timer length is needed for resource pool with PSFCH, and if feedback is enabled. Proposal 8 Confirm a second RTT timer length is needed for resource pool without PSFCH. Proposal 9 Add a third RTT timer length into the CR for resource pool with PSFCH, and if feedback is disabled. |
| --- | --- | --- |
| R2-2205185 | Ericsson | 1. Update the field description that for resource pool without PSFCH, the RTT timer length is fixed to be zero. 2. Remove one of the timers, and maintain only one timer for resource pool with PSFCH. |
| R2-2205790 | Intel Corporation | Proposal 3: For resource pool without PSFCH, if SCI does not indicate re-transmission resource, the sl-drx-HARQ-RTT-Timer timer length is fixed to be zero. |
| R2-2206138 | Huawei, HiSilicon | Proposal 12: There are two different HARQ RTT timers for SL DRX: - HARQ RTT Timer1: used for HARQ enabled case in RP with PSFCH - HARQ RTT Timer2: used for HARQ disabled case in RP with/without PSFCH |

The following Q is to check companies’ view on the different RTT timer values:

Question 2: What’s your view on the following options for the number of configured RTT timer values?

- Option 1: All the 3 configured timer values are needed;

- Option 2: 2 configured timer values (Timer1 for HARQ FB enabled case and Timer2 for HARQ FB disabled case (including both with and without PSFCH cases)) are needed;

- Option 2b: 2 configured timer values (Timer1 for HARQ enabled W/ PSFCH case and Timer2 for HARQ disabled W/ PSFCH case) are needed (and fix the timer in case no PSFCH as “0”);

- Option 3: only one configured timer value (for RP W/ PSFCH (including both HARQ enable and disables case) )is needed (and fix the timer in case no PSFCH as “0”);

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | Option 1 | From OPPO perspective, we understand we already agree on different RTT timer value for the listed 3 cases, i.e., for HARQ FB enabled/disabled case, the different value has been agreed in 115, and for RP W/ and W/O PSFCH, the different value has been agreed in 117, so to us, the discussion here is somehow revert of the previous conclusion. So we think by following the previous RAN2 agreement, 3 configured timer value is needed. Especially, for the reason to diff between timer2 and timer3, is because the minimum gap requirement (the RAN1 defined “a’ and “b” latency) for the next transmission only holds for resource pool with PSFCH, so cannot use a same RTT timer to handle the two cases. |
| Huawei HiSilicon | Option 2 | We think it is not necessary to distinguish between PSFCH configured and PSFCH not configured cases when HARQ is disabled. Two timers are enough. Also we don’t think the minimum time gap has anything to do with the value range of RTT timer. A proper value can work for both PSFCH configured and PSFCH not configured cases when HARQ is disabled and this can be ensured by NW implementation. |
| LG | Option 1 | Agree with OPPO |
| ASUSTeK | Option 1 or 2b | RTT timer for resource pool without PSFCH can be fixed to 0 or can be configured with a smaller value for receiving subsequent blind retransmissions. |

## Issue 3: Calculation of sl-drx-SlotOffset

As proposed by R2-2205136, the equation to calculate sl-drx-SlotOffset is not fully discussed and the calculation now (sl-drx-SlotOffset = Destination Layer-2 ID modulo sl-drx-onDurationTimer.) is wrong since the UE could derive a slot offset larger than the number of slot in one subframe based on the current formula.

| R2-2205136 | ASUSTek | Proposal 1: sl-drx-SlotOffset calculation should be DST layer-2 ID modulo the number of slots in one millisecond. |
| --- | --- | --- |

The following Q is to check companies’ view on the change proposed in paper R2-2205136.

Question 3: Do you agree on the Proposal 1 of R2-2205136, i.e., sl-drx-SlotOffset calculation should be DST layer-2 ID modulo the number of slots in one millisecond.?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes | We agree with the issue raised by this proposal. I.e., one has to ensure the calculated value of slot-offset is less than the “the number of slots in one subframe”. |
| Huawei HiSilicon | Option 2 | Agree with the intention. |
| LG | No | RAN2 agreed to use cycle length as an argument in the equation of startoffset to inherit the Uu DRX concept. Therefore, slotoffset can use the same onduration timer value as an argument. Also, even if slot offset beyond one subframe is applied, there is no critical problem. If you're worried about slotoffset going out of cycle, I don't think that will happen because the network can set the cycle and onduation timer values ​​well. So we prefer to keep the equation of the current spec. |
| ASUSTeK | Yes | The value of onduration timer for SL can be either in multiples of 1/32 ms or in ms, based on the configuration, which may not be a suitable argument for calculating the slot offset. In Uu, the slot offset is configured by NW and the NW is able to configure the value being in the range in one subframe. We think using number of slots in one millisecond is simple and can ensure the result is in one subframe. |

## Issue 4: SL triggering for SL DRX command indication

We have the following agreement on the SR configuration for SL DRX command MAC CE：

RAN2 does not define a separate SR configuration for SL DRX Command MAC CE.

As proposed by R2-2205136, in current specification, the SL DRX Command MAC CE can trigger a SR, however since we didn’t define the separate SR configuration for SL DRX Command MAC CE, how for the UE to decide the SR configuration for SL DRX Command MAC CE is missing.

| R2-2205136 | ASUSTek | Proposal 2: RAN2 selects one of the following options regarding SR triggering and SL DRX command indication: Option 1: SL DRX command indication does not trigger a Scheduling Request. Option 2: SL DRX command indication can trigger a Scheduling Request. Clarify in the specification that which logical channel triggered a Scheduling Request for SL DRX command indication is up to UE implementation. |
| --- | --- | --- |

The following Q is to check companies’ view on this issue:

Question 4: What’s your view on the following options for the above SR configuration issue?

- Option 1: SL DRX command indication does not trigger a Scheduling Request.

- Option 2a: SL DRX command indication can trigger a Scheduling Request. Clarify in the specification that which logical channel triggered a Scheduling Request for SL DRX command indication is up to UE implementation.

- Option 2b: SL DRX command indication can trigger a Scheduling Request. The SR configuration for CSI report can be reused for the SL DRX command indication.

- Option 3: Others

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | Option 2b | Option 2b is the easiest and feasible solution, since it enables the network to know there is a need for grant due to MAC-CE reasons, either CSI report or DRX command. Option 1 prevent the usage of DRX command MAC-CE in mode-1, which is not preferred; Option 2a rely on the SR for LCH by UE implementation, which leads to the difficulty by network to figure out whether the received SR is for data or for MAC-CE. |
| Huawei HiSilicon | Option 2a or 2b | We think either 2a or 2b works. Fine to follow the majority. |
| LG | Option 3 | Current MAC specification is specifying the LCID for SL-SCH. In the LCID table, LCID for SL DRX command MAC CE is specified as 61. So, Tx UE can send SR with SR configuration matching LCID index (61) of SL DRX Command MAC CE. There is no problem at all with the current UE behavior for sending SR. so additional agreement is not required. [OPPO]: Thanks for the reply, just one question for better understanding: what is the SR configuration matching LCID? Since our understanding is the SR configuration is per-LCH but not per-LCID. (Please correct me if any misunderstanding, thanks) |
| ASUSTeK | Option 1 or 2a | Since DRX command is not as time-critical as CSI-reporting MAC CE, we think option 1 is the simpler solution. In RAN2#116bis-e, it was agreed that RAN2 does not define a separate SR configuration for SL DRX Command MAC CE. Option 2b more or less reverts this previous agreement. In addition, currently there’s no restriction for the SR configuration for SL-CSI reporting to be different from SL LCH’s SR configuration, so it’s up to NW to figure out the SR is for data or for MAC CE already, therefore option 2a should work as well. |

## Issue 5: Need of active time extension after the announced periodic resource

The following agreement has been made regarding the periodic announced active time, and it is already been captured in the specification:

Slots associated with the announced periodic transmissions by the TX UE are considered as SL active time of the RX UE.

In paper R2-2205833, the issue that resource reselection due to pre-emption may cause the data loss at Rx UE since the re-selected resource may be not in the active time of the Rx UE.

| R2-2205833 | Nokia, Nokia Shanghai Bell | Proposal 1: RAN2 to agree that the active time of SL DRX may be extended even after the announced periodic transmission i.e. due to pre-emption. |
| --- | --- | --- |

The following Q is to check companies’ view on the issue raised in R2-2205833.

Question 5: Do you agree on the Proposal 1 of R2-2205833, i.e., RAN2 to agree that the active time of SL DRX may be extended even after the announced periodic transmission i.e. due to pre-emption.?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | No | We fail to see this as a valid issue since even resource re-selection due to pre-emption or any other reason happens, the Tx UE will select new resource in the SL DRX active time of the Rx UE according to the current specification, i.e., no need to further complicate the scheme of periodical-transmission-based active time. |
| Huawei HiSilicon | See comments | We think this issue somehow relates to the issue 1 of offline#707, and we already agree with the intention that for resource re-selection of the pre-emption check in SL DRX, the reselected resource should be within the active time of UE. See proposal 1 of offline#707. Proposal 1: RAN2 is to agree on the intention of the proposal 1 (“For resource re-selection of the pre-emption check in SL DRX, the time gap between the re-selected resource and the reported pre-empted resource is not larger than the duration of SL HARQ Retransmission timer.”) in the R2-2204552, FFS on the detailed shape of the change.. With the above clarification, we think nothing additional is needed. There is no need to distinguish if the pre-empted resource is a periodic transmission or not. |
| LG | No | Tx UE can use other active time. For example, onduration timer/inactivity timer/retransmission timer that is running or will be running in the RX UE. |
| ASUSTeK | No | Agree with OPPO. |

# Conclusion

We have the following proposals:

Proposal 1	xxx.

# Reference

- R2-2204580	Discussion on DRX left issues for user plane aspect	OPPO	discussion	Rel-17	NR_SL_enh-Core

- R2-2205105	Discussion on user plane FFS issues for SL DRX	ZTE Corporation, Sanechips	discussion	Rel-17	NR_SL_enh-Core

- R2-2205536	MAC open issues	Samsung	discussion

- R2-2205185	Correction on RIL issue E047	Ericsson	draftCR	Rel-17	38.331	17.0.0	NR_SL_enh-Core

- R2-2205790	Open issues for SL DRX	Intel Corporation	discussion	Rel-17	NR_SL_enh-Core

- R2-2206138	Rapporteur resolution for various RILs	Huawei, HiSilicon	discussion	Rel-17	NR_SL_enh-Core

- R2-2205136	Discussion on SL MAC aspects	ASUSTeK	discussion	Rel-17	38.321	NR_SL_enh-Core

- R2-2205833	Discussion on active time of SL DRX for the announced periodic transmissions	Nokia, Nokia Shanghai Bell	discussion	NR_SL_enh-Core