List of RRC Editor’s notes and proposed handling

# Introduction

The CRs for IOT NTN were approved at RAN2#117-e and included in Rel-17 specifications.

However, there are a number of Editor’s notes in RRC that need to be closed and it was agreed not to raise RILs for this.

In this document, RRC CR rapporteur suggests how to handle these Editor’s Notes.

# Discussion

Editor’s note 1: SIB31 Validity and notification of changes

Editor’s note: FFS whether changes to parameters other than satellite ephemeris and common TA parameters can only occur at the modification period boundary and notified via system information update notification.

Many RILs have been raised on this issue and more discussions are needed.

Proposal 1: Discuss Editor’s note 1: SIB31 Validity and notification of changes based on contributions in AI 7.2.3.2.

Editor’s note 2: System information acquisition in RRC_CONNECTED

Editor’s Note: FFS Whether the UE may acquire other system information (e.g. MIB, SIB …) in RRC_CONNECTED.

Proposal 2: Discuss Editor’s note 2: System information acquisition in RRC_CONNECTED based on contributions in AI 7.2.3.2.

Editor’s note 3: Reporting of the remaining GNSS validity time

Editor’s Note: Agreement: RAN2 will follow the RAN1 agreement that UE will report the remaining GNSS validity duration to the network. FFS: value range (not clear if the values of RAN1 agreement can be used). FFS which message.

This is an open issue in the exception sheet and should be discussed in AI 7.2.2.

Proposal 3: Discuss Editor’s note 3: Reporting of the remaining GNSS validity time based on contributions in AI 7.2.2.

Editor’s note 4: Release cause when GNSS fix become out-of-date

Editor’s Note: FFS release cause ‘RRC Connection Failure’ or ‘other’.

Discussion point 4: Please provide your views on what should be the release cause

| Company | Comments |
| --- | --- |
| Huawei | we think it should be “RRC connection failure’ which is the cause used when UE moves autonomously to RRC_IDLE while a RRC connection is ongoing |
| OPPO | “RRC Connection Failure” |
| Qualcomm | It should be “other”. We think this does not have to be treated as autonomously moving to IDLE without network knowledge. As UE will send remaining GNSS validity duration, then network has knowledge on when UE is going to IDLE mode, so there is no need for NAS recovery. When network knows UE’s state, NAS recovery is too much power consuming procedure. |
| ZTE | We have same view as Qualcomm. |

Editor’s note 5: How to refer to GNSS in RRC

Editor’s Note: FFS whether GNSS is considered as lower layers, upper layers or something else.

Discussion point 5: Please provide your views on how to refer to GNSS in RRC specification

| Company | Comments |
| --- | --- |
| Huawei | we think it should be “upper layers” as the GNSS device is not controlled by RRC |
| OPPO | GNSS module is out of 3GPP, so we may not need to discuss which layer GNSS is considered in. This EN could be removed. The indication that the GNSS position has become out-of-date while in RRC_CONNECTED is controlled by timer in physical layer. Although GNSS is out of 3GPP, the indication is from lower layers. |
| Qualcomm | Leave the “interaction with GNSS receiver” to UE. |
| ZTE | We have similar view as Qualcomm (maybe also Huawei?). It seems infeasible to trigger a timer in lower layer (physical layer or MAC layer?) when UE (re)acquire GNSS position before accessing network. So we tend to think no need of timer or indication from lower layer. |

Editor’s notes 6-1 & 6-2: Guard Timer T318 for SIB31 acquisition

Editor’s Note: Agreement: Introduce a guard timer TXXXX for SIBXX acquisition in connected mode. At TXXX expiry, UE triggers RLF (if it can be shown in Q2 that UE will loose RLM when UE tunes away, it can be discussed to skip this timer).

Editor’s Note: Editor: FFS whether a new timer T31Y is signalled or the value signalled for T310 is used.

Proposal 6: Discuss Editor’s notes 6-1 & 6-2: Guard Timer T318 for SIB31 acquisition based on contributions in AI 7.2.3.2.

Editor’s note 7: Signalling of Part-EARFCN in MIB for eMTC

Editor’s Note: Agreement: Introduce a presence indicator in addition to the 2 bit LSB EARFCN in the NB-IoT MIB (eMTC - all aspects FFS).

Based on RAN1 spreadsheet [3] and as confirmed by some companies during the RRC CR review after RAN2#117-e, the signalling of part-EARFCN in MIB also applies to eMTC. Thus, rapporteur proposes to reuse the signalling introduced for NB-IoT.

Proposal 7: Editor’s Note 7: Signalling of part-EARFCN in MIB in eMTC: Reuse the signalling introduced in NB-IoT.

MasterInformationBlock ::=			SEQUENCE {

dl-Bandwidth						ENUMERATED {

n6, n15, n25, n50, n75, n100},

phich-Config						PHICH-Config,

systemFrameNumber					BIT STRING (SIZE (8)),

schedulingInfoSIB1-BR-r13			INTEGER (0..31),

systemInfoUnchanged-BR-r15			BOOLEAN,

partEARFCN-17					CHOICE {

spare 							BIT STRING (SIZE (2)),

earfcn-LSB						BIT STRING (SIZE (2)),

spare								BIT STRING (SIZE (14))

}

Discussion point 7: Do you agree with proposal 7

| Company | Agree (yes/No) | Comments |
| --- | --- | --- |
| Huawei | yes |  |
| OPPO | yes |  |
| Qualcomm | Yes |  |
| ZTE | Yes |  |

Editor’s note 8: Signalling of SIB31 in RRCConnectionReconfiguration not for HO

Editor’s Note: FFS whether SIB31 can be provided in other cases than handover to a NTN cell.

RIL O305 and RIL X501 have been raised on this issue.

Proposal 8: Discuss Editor’s note 8: Signalling of SIB31 in RRCConnectionReconfiguration not for HO with RIL O305 and RIL X501.

Editor’s note 9: Signalling of PUSCH segment in eMTC

Editor’s Note: FFS signalling and field description of ce-PUSCH-TxDuration based on RAN1 revised spreadsheet.

The description in the updated RAN1 spreadsheet [3] is as follows:

| TransmissionDurationPUSCH-r17 | Configuration of UL transmission segment is indicated on SIB and reconfigured by RRC signalling in RRC_CONNECTED UL transmission segment duration with one value X per PUSCH/PUCCH for eMTC may be indicated on SIB. • The value X for eMTC PUSCH applies for full-PRB allocation and should be divided by 2, 4 and 8 for sub-PRB allocation of 6, 3 and 2-out-of-3 tones allocation, respectively. | • Full-PRB allocation (unit: subframes): 2 4 8 16 32 64 128 256 • Sub-PRB allocation (unit: resource units for 6 tones): 1 2 4 8 16 32 64 128 Sub-PRB allocation (unit: resource units for 3 tones): 1 2 4 8 16 32 64 Sub-PRB allocation (unit: resource units for 2-out-of-3 tones): 1 2 4 8 16 32 |
| --- | --- | --- |

Based on the update, it is rapporteur’s understanding, that the value signalled in the SIB can also be used for sub-PRB in connected mode, thus the definition should be updated. Then, there is no more motivation to have different parameters for full PRB and sub PRB in dedicated signalling.

Proposal 9: Editor’s note 9: Signalling of PUSCH segment in eMTC: Modify the signalling of the PUSCH segment duration for eMTC and use a common parameter for full PRB and sub-PRB.

PUSCH-ConfigCommon-v1700 ::=	SEQUENCE {

ce-PUSCH-TxDuration-r17			ENUMERATED {n2, n4, n8, n16, n32, n64, n128, n256}

OPTIONAL	-- Cond NTN

}

CE-PUSCH-TxDurationConfig-r17 ::=	SEQUENCE {

ce-PUSCH-TxDuration-r17			ENUMERATED {n2, n4, n8, n16, n32, n64, n128, n256}

}

ce-PUSCH-TxDuration Duration of PUSCH segment transmission in NTN transmission, see TS 36.213 [23]. Value in number of resource units. Value n2 corresponds to 2 resource units, value n4 corresponds to 4 resource units and so on. The signalled value corresponds to full-PRB allocation and should be divided by 2, 4 and 8 for sub-PRB allocation of 6, 3 and 2-out-of-3 tones allocation, respectively.

Discussion point 9: Do you agree with proposal 9

| Company | Agree (yes/No) | Comments |
| --- | --- | --- |
| Huawei | yes |  |
| OPPO | yes |  |
| Qualcomm | Yes but | The field description for 3 and 2-out-of-3 tones is not correct. Add For 3 tones, value n2 is not applicable (0.5?). For 2-out-of-3 tones, values n2 (0.25?) and n4 (0.5?) are not applicable. We prefer to stick to simply what we currently have in the spec and just add value n64 and n128 are not applicable to 2-out-of-3 tones. Value n128 is not applicable to 3 tones. CE-PUSCH-TxDurationConfig-r17 ::= SEQUENCE { ce-PUSCH-TxDuration-r17 CHOICE { fullPRB ENUMERATED { n2, n4, n8, n16, n32, n64, n128, n256}, subPRB-Six ENUMERATED {n1, n2, n4, n8, n16, n32, n64, n128} } } |
| ZTE | No | RAN1 has assumed that configuration of UL transmission segment is indicated on SIB and reconfigured by RRC signaling in RRC_CONNECTED. Furthermore, based on the previous RRC parameters definition, we have the following understanding: - If ce-PUSCH-TxDuration is present in SIB, it should be clarified (or common understanding) that ce-PUSCH-TxDuration-r17 in PUSCH-ConfigCommon-v1700 is for full PRB configuration. - If ce-PUSCH-TxDuration is absent in SIB, we assume it’s still allowed to configure ce-PUSCH-TxDurationConfig-r17 via dedicated signalling. - If ce-PUSCH-TxDurationConfig-r17 is set to setup (that also means the ce-PUSCH-TxDuration-r17 should be present in ce-PUSCH-TxDurationConfig-r17), we assume the ce-PUSCH-TxDuration-r17 in ce-PUSCH-TxDurationConfig-r17 can override the ce-PUSCH-TxDuration-r17 in SIB (if provided). Moreover, if sub-PRB is configured, the ce-PUSCH-TxDurationConfig-r17 should use the item for sub-PRB and be configured with the value within the corresponding range for the configured sub-PRB. - If ce-PUSCH-TxDurationConfig-r17 is set to release, we assume PUSCH transmission segmentation would not be used for UE in connected mode. So we may have different view from rapporteur. Per our understanding, since there is no case that ce-PUSCH-TxDurationConfig-r17 is set to setup but the ce-PUSCH-TxDuration-r17 is absent from ce-PUSCH-TxDurationConfig-r17, there is no need to think about applying the ce-PUSCH-TxDuration in SIB to connected mode. Therefore, for avoiding any confusion, we suggest to give different parameter names for PUSCH TxDuration in PUSCH-ConfigCommon-v1700 and ce-PUSCH-TxDurationConfig-r17 respectively and give separate field definitions. Moreover, about how to reflect RAN1 agreement, we have similar view as Qualcomm. We think the wording in “R2-22xxxxx Corrections to IOT NTN_v00” suggested by RRC CR rapporteur (mainly copied from RAN1 spreadsheet) is not easy to understand/less readable. Maybe we can just mention the resulted configuration, no need to give the explanation. In a summary, our suggestion is as below: PUSCH-ConfigCommon-v1700 ::= SEQUENCE { ce-PUSCH-TxDurationCommon-r17 ENUMERATED {sf2, sf4, sf8, sf16, sf32, sf64, sf128, sf256} OPTIONAL -- Cond NTN } .......... PUSCH-ConfigDedicated-v1700 ::= SEQUENCE { ce-PUSCH-TxDurationConfig-r17 SetupRelease {CE-PUSCH-TxDurationConfig-r17} OPTIONAL -- Need ON } ........... CE-PUSCH-TxDurationConfig-r17 ::= SEQUENCE { ce-PUSCH-TxDurationDedicated-r17 CHOICE { fullPRB ENUMERATED {sf2, sf4, sf8, sf16, sf32, sf64, sf128, sf256}, subPRB ENUMERATED {n1, n2, n4, n8, n16, n32, n64, n128} } } ce-PUSCH-TxDurationCommon Duration of PUSCH segment transmission in NTN transmission, see TS 36.213 [23]. For value sf2 corresponds to 2 subframes, value sf4 corresponds to 4 subframes and so on.. ce-PUSCH-TxDurationDedicated Duration of PUSCH segment transmission in NTN transmission, see TS 36.213 [23]. The choice value can be set to subPRB and apply the corresponding configuration only when PUSCH sub-PRB allocation is activated. For full PRB allocation, value sf2 corresponds to 2 subframes, value sf4 corresponds to 4 subframes and so on. For sub-PRB allocation, value n1 corresponds to 1 resource unit, value n2 corresponds to 2 resource units and so on. The value cannot be set to n64 or n128 if sub-PRB allocation is 2 tones. The value cannot be set to n128 if sub-PRB allocation is 3 tones.. |
| ZTE2 | Yes | After further check, now we think the following suggested change may be also ok. The main difference is that the configuring way below is to let UE to derive the segment transmission configuration for sub-PRB by itself. Instead, in the suggested way in above ZTE comment, it’s to let the network guarantee a configuration suitable for sub-PRB: PUSCH-ConfigCommon-v1700 ::= SEQUENCE { ce-PUSCH-TxDuration-r17 ENUMERATED {n2, n4, n8, n16, n32, n64, n128, n256} OPTIONAL -- Cond NTN } ........... PUSCH-ConfigDedicated-v1700 ::= SEQUENCE { ce-PUSCH-TxDurationConfig-r17 SetupRelease {CE-PUSCH-TxDurationConfig-r17} OPTIONAL -- Need ON } ........... CE-PUSCH-TxDurationConfig-r17 ::= SEQUENCE { ce-PUSCH-TxDuration-r17 ENUMERATED {n2, n4, n8, n16, n32, n64, n128, n256} } ce-PUSCH-TxDuration Duration of PUSCH segment transmission in NTN transmission, see TS 36.213 [23]. Value in number of resource units. Value n2 corresponds to 2 resource units, value n4 corresponds to 4 resource units and so on. The signalled value corresponds to full-PRB allocation and should be divided by 2, 4 and 8 for sub-PRB allocation of 6, 3 and 2-out-of-3 tones allocation, respectively.. We have the following understanding: - The value in ce-PUSCH-TxDuration is only for full-PRB, no matter it’s provided in SIB or dedicated signalling. - The ce-PUSCH-TxDuration provided via dedicated signalling can override the configuration in SIB. - If ce-PUSCH-TxDurationConfig is set to setup and meanwhile ce-PUSCH-SubPRB-Config is also set to setup, UE will determine how to divide ce-PUSCH-TxDuration, by 2 or 4 or 8? according to the specific configuration in ce-PUSCH-SubPRB-Config. There is only one thing that, the above field description is applied for ce-PUSCH-TxDuration in both SIB and dedicated signalling. We understand the value in SIB cannot be divided. In order to avoid any confusion, we suggest a clarification as below: ce-PUSCH-TxDuration Duration of PUSCH segment transmission in NTN transmission, see TS 36.213 [23]. Value in number of resource units. Value n2 corresponds to 2 resource units, value n4 corresponds to 4 resource units and so on. The signalled value corresponds to full-PRB allocation. If PUSCH sub-PRB allocation is activated, the signalled value should be divided by 2, 4 and 8 for sub-PRB allocation of 6, 3 and 2-out-of-3 tones allocation, respectively.. |

Editor’s note 11: RAN4 inputs for part-EARFCN signalling

Editor’s Note: RAN4 inputs for part-EARFCN signalling

RAN4 has no TU for IOT NTN in Rel-17 so it is very unlikely they will not reply to the LS RAN1 has sent.

As the field description refers to RAN4 specification (see below), rapporteur proposes just to remove the Editor’s note.

earfcn-LSB Indicates the 2 least significant bits of the EARFCN for NTN bands where 100 kHz raster is used, see TS 36.101 [42].

Proposal 10: Editor’s note 11: RAN4 inputs for part-EARFCN signalling: Remove the Editor’s note.

Discussion point 10: Do you agree with proposal 10

| Company | Agree (yes/No) | Comments |
| --- | --- | --- |
| Huawei | yes |  |
| OPPO | yes |  |
| Qualcomm | Yes |  |
| ZTE | Yes |  |

# Conclusion

To be completed.

# Reference

- 3GPP TS 36.331 E-UTRAN Radio Resource Control (RRC) (Release 17)

- R2-2203221 Report of [Pre117-e][012][IOT-NTN] Control Plane Open Issues (Huawei)	Huawei, HiSilicon

- R1-2202540 Consolidated higher layers parameter list for Rel-17 LTE, RAN1