3GPP TSG-RAN WG2 Meeting #119-e                                                                 R2-2206994

Online, Aug 17 – Aug 26, 2022

Source:	CATT

Title:	Discussion on Selective Activation of Cell Groups in NR-DC

Agenda Item:	8.4.3

Document for:	Discussion and Decision

Introduction

The objective in the WID of [1] specifies the NR-DC with selective activation of the cell groups:

To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements: - To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4] Note 4: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

In this contribution, in section 2, we give our initial considerations on scenarios and basic solutions related to the selective activation of cell groups in NR-DC. The proposals are summarized in Section 3.

Discussion

Selective activation of MCG cell groups

In the WID [1], it indicates that “to specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements”, “at least for SCG” means that besides SCG, we could also consider selective activation of cell groups for MCG, i.e. after CHO successful execution, the UE maintains the CHO configuration to allow subsequent MCG cell group change after changing MCG without reconfiguration and re-initiation of CHO.

Different from selective activation of SCG cell groups i.e. MCG is fixed; selective activation of MCG cell groups has bigger challenges as uncertainty of UE movement route. The following figure is more clearly to illustrate this problem.

Figure 1 Selective activation of MCG cell groups

In the figure above, if the UE is configured with candidate PCell 1~ PCell 4, when the CHO execution conditions of candidate PCell 1 is fulfilled, the UE executes PCell handover from source PCell to candidate PCell 1. And then, the UE moves in the direction of the arrow, candidate PCell 2~ PCell 4 will never be selected. And the SCG configuration for each MN is different, so from this point view it will introduce complexity for inter-node interaction. Therefore, we suggest that we only discuss selective activation of cell groups for SCG in this release. We need more time to research the meaning for selective activation of cell groups for MCG.

Proposal 1:  NR-DC with selective activation of the cell groups is only applicable to SCG.

Basic framework for selective activation of cell groups

Based on the description of WID [1], NR-DC with selective activation of the cell groups aims at enabling subsequent CPC/CPA after SCG change, without reconfiguration and re-initialization on the CPC/CPA preparation from the network. The benefit of doing so is a reduction of the signalling overhead and interrupting time for SCG change.

During the WID discussion of RAN meeting, it seems that companies have different understanding on how to trigger the subsequent cell group change. Some companies think it is triggered by UE based on pre-configured CPAC execution conditions (i.e UE based solution), Some other companies think that the SCG change is performed based on NW command (i.e. NW based solution), i.e. the network configures the candidate cell groups configuration to UE in advance, the network makes the decision of SCG change and sends the SCG change command to UE. The following figure illustrates these two solutions in detail.

Figure 2 UE based solution (left) & Network based solution (right)

However, according to our understanding to the WID, it is should be UE-based solution, i.e. the UE performs each SCG change based on the fulfilled CPAC execution condition(s) and the CPAC configuration will be kept after each SCG change until it is re-configured or released.

Therefore, it is proposed,

Proposal 2: RAN2 to confirm the baseline for selective activation of cell groups:

- UE based solution, i.e. SCG change is trigged by UE based on CPAC execution conditions

Scenarios for selective activation of cell groups

The intention of this objective is to allow the UE perform subsequent CPC/CPA without prior CPC/CPA reconfiguration and re-initialization from the network. From this point view, the scenario is applicable for MN kept with SCG change. So the CPC/CPA configuration kept by UE should correspond to the same MN. It is obvious that the candidate SNs of the stored CPC and CPA should be the neighbor cells that have overlapping coverage with the MN. Distinctly, it is MN instead of SN who is the suitable node to make the decision to choose the candidate SNs. Due to the MN have the neighbor cells information of MN based measurement report based on the MN measConfig or deployment, but the SN only have the neighbor cells information of SN not MN based measurement report based on the SN measConfig or deployment. It can’t ensure the candidate cells decided by source SN can be under the coverage of MN i.e. could be configured with the MN as DC.

Figure 3 MN-initiated CPC and SN- initiated CPC

In the figure above, candidate SN 1 ~ SN4 and source SN is the candidate SN/PSCell which is selected by MN based on MN RRM measurement results. Candidate SN5~SN7 is the candidate SN/PSCell which is selected by SN based on SN RRM measurement results. Assuming the UE is configured with candidate SN 5~7, if the UE performs SCG change from source SN to candidate SN 6, it is obvious that the SN6 is out of the coverage of MN which is not applicable for the cell group selective for the source MN.

Observation 1: The selective activation of cell groups is applicable for case when MN is kept and SCG is changed.

Observation 2: It is likely that MN makes the decision of the configuration of target SNs since SN does not have the information of the neighbor cells which may have the overlapping coverage with MN.

Besides, if the UE based solution is the baseline, take the execution condition update into consideration, it is the MN initiated CPAC is feasible. R17 CPAC mainly discussed MN-initiated CPA, MN-initiated inter-SN CPC, SN-initiated inter-SN CPC. For MN-initiated CPA and MN-initiated inter-SN CPC, candidate PSCell and execution conditions are selected and generated by MN based on MN configured measConfig. These candidate PSCell execution conditions do not need to be updated unless MN updates the MN measConfig. MN selects these suitable neighbor cells as candidate PSCell and sends to UE, UE could perform PSCell change among these candidate PSCell without configuration release.

However, for SN-initiated inter-SN CPC, candidate PSCell and execution conditions are generated by SN i.e. measConfig configured by SN. When SCG change, SN configured measConfig will be updated which means the candidate PSCell execution conditions also need to be updated corresponding.

Observation 3: If UE based solution is the taken baseline, the SN-initiated CPC will introduce complexity for execution condition update.

Based on the above analysis, for the objective of selective activation of cell groups, only the case of MN initiated case should be taken into consideration.

Proposal 3: Only support MN-initiated CPA and MN-initiated inter-SN CPC for selective activation of the cell groups.

Conclusion

Based on the previous analysis in section 2, our main contributions are summarized as follows:

Proposal 1:  NR-DC with selective activation of the cell groups is only applicable to SCG.

Proposal 2: RAN2 to confirm the baseline for selective activation of cell groups:

- UE based solution, i.e. SCG change is trigged by UE based on CPAC execution conditions

Observation 1: The selective activation of cell groups is applicable for case when MN is kept and SCG is changed.

Observation 2: It is likely that MN makes the decision of the configuration of target SNs since SN does not have the information of the neighbor cells which may have the overlapping coverage with MN.

Observation 3: If UE based solution is the taken baseline, the SN-initiated CPC will introduce complexity for execution condition update.

Proposal 3: Only support MN-initiated CPA and MN-initiated inter-SN CPC for selective activation of the cell groups.

Reference

[1] RP-221799, Revised WID on Further NR mobility enhancements.