3GPP TSG-RAN WG2 Meeting #119                                                      R2-2207026

Online, 15 Augest–26 Augest 2022

Source:	Qualcomm Incorporated

Title:	QoE collection for IDLE and Inactive state

Agenda Item:	8.14

Document for:	Discussion and Decision

# Introduction & Background

According to the agenda and the WID [1], QoE configuration and collection in Inactive state and IDLE state will be discussed as follow for MBS service.

- Specify for QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE states for MBS, at least for broadcast service [RAN3, RAN2].

- Specify the mechanism to support the alignment of the existing radio related measurement and QoE reporting.

This contribution discusses the QoE configuration and measurement collection for IDLE and Inactive state.

# Discussion

2.1 QoE configuration

2.1.1 Common QoE configuration

QoE configuration and collection for IDLE and Inactive state is mainly for MBS broadcast service. And UE can receive MBS broadcast service in IDLE state, Inactive state and Connected state. The broadcast service configuration can be provided to the UE over MCCH channel using MBSBroadcastConfiguration message, and the configuration for MCCH channel is broadcast in SIB20. UE in IDLE, Inactive and CONNECTED state can receive MBS broadcast service according to the common configuration in SIB20 and MCCH channel.

Observation 1: UE in IDLE, Inactive and Connected state can receive MBS broadcast using MBSBroadcastConfiguration over MCCH channel.

For QoE configuration, if UEs in IDLE state are required to collect QoE measurements, QoE configuration needs to be provided to IDLE state UEs. In order to avoid requesting UE to connected state just for QoE configuration purpose, UE should be able to be configured QoE configuration in IDLE state.

Observation 2: UE should be able to be configured QoE configuration for MBS broadcast service in IDLE state to avoid entering connected state just for QoE configuration purpose.

One possible way to provide UE in IDLE state is to include QoE configuration for MBS broadcast in MCCH channel, and UE in IDLE state can receive QoE configuration.It can be FFS whether existing MBSBroadcastConfiguration or new MCCH message is used.

Proposal 1: Introduce common QoE configuration for MBS broadcast service, the common QoE configuration is included MCCH message.

Propsoal 2: It is FFS whether the existing MBSBroadcastConfiguration message or a new MCCH message will be used.

Since the MCCH can be received for UEs in all RRC states, then the common QoE configuration can be applied for UE in all RRC states.

Proposal 3: The common QoE configuration for MBS broadcast service can be applied for UE in IDLE, Inactive and Connected states.

2.1.2 Dedicated QoE configuration

In Rel-17, QoE configuration is provided to UE using dedicated RRC message. For MBS broadcast service, if the network just intend to request specific UEs to collect QoE measurement or if the network intends to provide specific QoE configurations to some specific UEs, the network should be able to send dedicated RRC message to provide QoE configuration for MBS broadcast servic. AS SA5 defined, for each QoE configuration, there could be area scope configured, so the network can provide area scope to the UE for the QoE configuration.

Proposal 4: The dedicated QoE configuration for MBS broadcast service can be provided to the UE using dedicated RRC message.

Propsoal 5: The dedicated QoE configuration can be used by UE in Idle state, Inactive state or Connected state.

Proposal 6: An area scope can be provided to UE for the dedicated QoE configuration, and UE considers the dedicated QoE configuration is valid within the area scope.

If the UE is configured and has valid dedicated QoE configuration for MBS broadcast service, the UE should ignore the common QoE configuration if configured.

Proposal 7: If the UE is configured and has valid dedicated QoE configuration for MBS broadcast service, the UE should ignore the common QoE configuration.

2.1.3 MBS broadcast QoE configuration content

In Rel-17, each QoE configuration contains service type, RRC ID(measConfigAppLayerId), QoE configuration container, and the QoE configuration defined in Rel-17 can still be applied to MBS service QoE in Connected and Inactive state. The RRC ID is allocated the gNB can included in UE context. However, when the UE is released to IDLE state, the RRC ID is invalid any more. So for QoE configuration applied in IDLE state, the RRC ID(measConfigAppLayerId) should not be allocated and provided to the UE. The usage of the RRC ID is to replace QoE reference for signalling overhead saving purpose and based on the RRC ID the gNB can derive the MCE address for the received QoE report. Hence, QoE configuration container, service type should be included in QoE configuration RRC message. Other information e.g. the information used by gNB to derive the MCE address is FFS.

Then QoE reference should be included for the QoE configuration in IDLE state.

Proposal 8: The QoE configuration applied in IDLE state contains service type, QoE configuration container. It is FFS for other information.

2.2 QoE collection in IDLE and Inactive state

When the UE is configured QoE measurements for MBS broadcast service, UE AS layer forwards the QoE configuration to the application layer, and application layer performs QoE measurement. For the collected QoE measurements, it is should be buffered in the UE, and when the UE enters Connected state, UE can send to the gNB.

Proposal 9: The collected QoE measurements in IDLE and Inactive state should be buffered in the UE and not trigger RRC connection establishment or resume procedure.

In Rel-17, when QoE reporting is paused, UE will buffer the data in AS layer with 64KB buffer size. For IDLE and Inactive state, the AS layer 64KB buffer can be reused. And if the AS layer buffer size is full, the QoE measurement should be buffered in application layer.

Proposal 10: The QoE measurements collected in IDLE and Inactive state can be buffered in AS layer with reusing the 64KB buffer size defined for Connected state in Rel-17.

Proposal 11: If the AS layer buffer (64KB) is full, the QoE data should be buffered in application layer.

2.3 QoE report of data collected in IDLE and Inactive state

In Rel-17, SRB4 was introduced for QoE data reporting and UE include QoE data in the MeasurementReportAppLayer message. For IDLE and Inactive state, the same bearer and RRC message defined in Rel-17 can be reused for IDLE and Inactive state QoE data reporting.

Proposal 12: Reuse existing MeasurementReportAppLayer and SRB4 to transmit QoE data collected in IDLE and Inactive state.

When UE initiates RRC connection establishment or resume procedure, the UE can indicate to gNB whether there is QoE data buffered, based on this indication, the gNB can provide SRB4 configuration to the UE for QoE reporting.

Proposal 13: UE indicates QoE available indication in MSG5, gNB may provide SRB4 configuration based on the available indication.

When the UE reports QoE data, the new gNB may not be provided the QoE configuration assisted information, e.g. MCE address, then UE should include MCE information for the QoE data to the gNB, and the gNB can forward the QoE data to the appropriate MCE based on the MCE information. MCE address can be indicated to gNB in case that the gNB has no QoE configuration.

Proposal 14: UE AS layer indicates MCE information (e.g. MCE address or MCE ID) for each reported QoE container to the gNB, and gNB forwards the QoE data to the appropriate MCE based on the MCE information.

And it is assumed application layer has the MCE information, so UE application layer provides MCE information to AS layer together with the QoE report container. But, this needs to be confirmed by SA4.

If application layer cannot provide MCE information to the AS layer, then the gNB should indicate MCE information to the UE in the QoE configutation.

Proposal 15: It is assumed that UE application layer provides MCE information to AS layer together with the QoE report container. This needs to be confirmed by SA4.

Proposal 16: If application layer cannot provide MCE information to AS layer, gNB should configure MCE information to UE in the QoE configuration.

2.3 Alignment of the existing radio related measurement and QoE reporting

In Rel-17, alignment between immediate MDT and QoE measurements is made possible by UE providing a session start or session stop indication to NG-RAN. To achieve alignment, NG-RAN can configure the immediate MDT only upon receiving session start indication and can deactivate immediate MDT upon the reception of session stop indication.

In Rel-18, we can further discuss how to achieve alignment between logged MDT and QoE configuration of MBS broadcast service.

One solution would be to keep logged MDT suspended at the UE till session start. UE records logged MDT measurements only upon receiving the session start indication from the UE APP layer and terminates the logging upon receiving a session stop indication from UE APP layer.

Proposal 17: Logged MDT is kept suspended at UE till session start. UE records logged MDT measurements only upon receiving the session start indication from the UE APP layer and terminates the logging upon receiving a session stop indication from UE APP layer.

2.4  RVQoE collection in IDLE and Inactive state

From the WID objective, it is unclear whether RVQoE measurement collection needs to be done in IDLE and Inactive state. It should be beneficial to collect RVQoE measurement for MBS broadcast service, then gNB can do real-time radio configuration optimization. Then RAN2 is proposed to clarify whether RVQoE measurement collection is needed in IDLE and Inactive state.

Proposal 18: It is proposed to clarify whether RVQoE measurement collection is needed in IDLE and Inactive state.

# Conclusion

This contribution discusses the MBS broadcast service QoE configuration and report for IDLE and Inactive state.

For QoE configuration,

Observation 1: UE in IDLE, Inactive and Connected state can receive MBS broadcast according to the common configuration MCCH channel.

Observation 2: UE should be able to be configured QoE configuration for MBS broadcast service in IDLE state to avoid entering connected state just for QoE configuration purpose.

Proposal 1: Introduce common QoE configuration for MBS broadcast service, the common QoE configuration is included MCCH message.

Propsoal 2: It is FFS whether the existing MBSBroadcastConfiguration message or a new MCCH message will be used.

Proposal 3: The common QoE configuration for MBS broadcast service can be applied for UE in IDLE, Inactive and Connected states.

Proposal 4: The dedicated QoE configuration for MBS broadcast service can be provided to the UE using dedicated RRC message.

Propsoal 5: The dedicated QoE configuration can be used by UE in Idle state, Inactive state or Connected state.

Proposal 6: An area scope can be provided to UE for the dedicated QoE configuration, and UE considers the dedicated QoE configuration is valid within the area scope.

Proposal 7: If the UE is configured and has valid dedicated QoE configuration for MBS broadcast service, the UE should ignore the common QoE configuration.

Proposal 8: The QoE configuration applied in IDLE state contains service type, QoE configuration container. It is FFS for other information.

QoE collection in IDLE and Inactive state,

Proposal 9: The collected QoE measurements in IDLE and Inactive state should be buffered in the UE and not trigger RRC connection establishment or resume procedure.

Proposal 10: The QoE measurements collected in IDLE and Inactive state can be buffered in AS layer with reusing the 64KB buffer size defined for Connected state in Rel-17.

Proposal 11: If the AS layer buffer (64KB) is full, the QoE data should be buffered in application layer.

QoE report of data collected in IDLE and Inactive state

Proposal 12: Reuse existing MeasurementReportAppLayer and SRB4 to transmit QoE data collected in IDLE and Inactive state.

Proposal 13: UE indicates QoE available indication in MSG5, gNB may provide SRB4 configuration based on the available indication.

Proposal 14: UE AS layer indicates MCE information (e.g. MCE address or MCE ID) for each reported QoE container to the gNB, and gNB forwards the QoE data to the appropriate MCE based on the MCE information.

Proposal 15: It is assumed that UE application layer provides MCE information to AS layer together with the QoE report container. This needs to be confirmed by SA4.

Proposal 16: If application layer cannot provide MCE information to AS layer, gNB should configure MCE information to UE in the QoE configuration.

MDT-QoE alignment for IDLE and Inactive state,

Proposal 17: Logged MDT is kept suspended at UE till session start. UE records logged MDT measurements only upon receiving the session start indication from the UE APP layer and terminates the logging upon receiving a session stop indication from UE APP layer.

RVQoE collection in IDLE and Inactive state,

Proposal 18: It is proposed to clarify whether RVQoE measurement collection is needed in IDLE and Inactive state.

# Reference

[1]    RP-221803: Enhancement on NR QoE management and optimizations for diverse services