3GPP TSG-RAN WG2 Meeting #119-e                                  R2-2207039

Online, August 17-29, 2022

Agenda item:		6.1.2

Source:				Samsung

Title:					RRC Corrections for MBS

Document for:		Discussion & Decision

# 1 Introduction

This contribution discusses the RRC corrections for MBS.

# 2 Discussion

## 2.1 Correction to MBS interest indication

It is expected that UE transmits a MBS interest indication when a radio link failure is detected after a transmission of MBS interest indication. The behaviour is aligned with the legacy LTE MBMS. However, it is missed in NR MBS and should be addressed to enhance the reliability of the MBS interest indication signaling. Text Proposal A is provided in the annex.

Proposal 1: RAN2 agrees that UE transmits a MBS interest indication when a radio link failure is detected after a transmission of MBS interest indication. Adopt Text Proposal A to TS 38.331.

## 2.2 Correction for MBS broadcast retention

MBS broadcast is received by the UE regardless of the RRC state switching and events like T300 expiry, and therefore, broadcast MRBs and configurations should be retained. Particularly, when UE receives RRCSetup, it releases radio resources for all established RBs except SRB0, including release of the RLC entities, of the associated PDCP entities and of SDAP. However, this should also exclude MBS broadcast MRBs. In addition, upon RRCSetup reception, UE releases the RRC configuration except for the default L1 parameter values, default MAC Cell Group configuration and CCCH configuration. However, this should also exclude RRC configuration for MBS broadcast.

Further, upon timer T300 expiry, UE resets MAC, releases the MAC configuration and re-establishes RLC for all RBs that are established. However, this should exclude MBS broadcast MRBs.

Text Proposal B is provided in the annex.

Proposal 2: RAN2 agrees for MBS broadcast MRBs retention when UE receives RRCSetup and when T300 expires. Adopt Text Proposal B to TS 38.331.

# 3 Conclusion

Request RAN2 to discuss and agree to the following proposals:

Proposal 1: RAN2 agrees that UE transmits a MBS interest indication when a radio link failure is detected after a transmission of MBS interest indication. Adopt Text Proposal A to TS 38.331.

Proposal 2: RAN2 agrees for MBS broadcast MRBs retention when UE receives RRCSetup and when T300 expires. Adopt Text Proposal B to TS 38.331.

# 4 Reference

[1] 3GPP TS 38.331 v17.1.0: Radio Resource Control (RRC) protocol specification

# 5 Annex – Text Proposals

Text Proposal A

>>>>>>>>>>>>>>>> Start of Change<<<<<<<<<<<<<<<<<

#### 5.3.7.5	Reception of the RRCReestablishment by the UE

The UE shall:

<Omitted Text>

- submit the RRCReestablishmentComplete message to lower layers for transmission;

1>	if the PCell provides SIB21:

2>	if the UE initiated transmission of a MBSInterestIndication message during the last 1 second preceding detection of radio link failure:

3>	initiate transmission of a MBSInterestIndication message in accordance with clause 5.9.4;

1>	the procedure ends.

>>>>>>>>>>>>>>>> End of Change<<<<<<<<<<<<<<<<<

Text Proposal B

>>>>>>>>>>>>>>>> Start of First Change<<<<<<<<<<<<<<<<<

#### 5.3.3.4	Reception of the RRCSetup by the UE

The UE shall perform the following actions upon reception of the RRCSetup:

1>	if the RRCSetup is received in response to an RRCReestablishmentRequest; or

1>	if the RRCSetup is received in response to an RRCResumeRequest or RRCResumeRequest1:

2>	if sdt-MAC-PHY-CG-Config is configured:

3>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

3>	instruct the MAC entity to start the timeAlignmentTimer associated with the PTAG, if it is not running;

2>	discard any stored UE Inactive AS context and suspendConfig;

2>	discard any current AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key;

2>	release radio resources for all established RBs except SRB0 and except MBS broadcast MRBs, including release of the RLC entities, of the associated PDCP entities and of SDAP;

2>	release the RRC configuration except for the default L1 parameter values, default MAC Cell Group configuration and CCCH configuration and except RRC configuration for MBS broadcast;

2>	indicate to upper layers fallback of the RRC connection;

<Omitted Text>

>>>>>>>>>>>>>>>> End of First Change<<<<<<<<<<<<<<<<<

>>>>>>>>>>>>>>>> Start of Second Change<<<<<<<<<<<<<<<<<

#### 5.3.3.7	T300 expiry

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established (except MBS broadcast MRBs);

<Omitted Text>

>>>>>>>>>>>>>>>> End of Second Change<<<<<<<<<<<<<<<<<