3GPP TSG-RAN WG2 Meeting #119-e	R2-2207044

e-Meeting, August 17~26, 2022

Agenda item:	8.5.2

Source:	Qualcomm Incorporated

Title:	XR-awareness in RAN

WID/SID:	FS_NR_XR_enh

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss possible enhancements to RAN2 procedures, based on specific characteristics and requirements of XR traffic.

# Discussion

## General aspects

In this section, we discuss a few QoS related issues, e.g., PDU discard, deadlines, etc. These discussions are applicable to XR traffic in general, irrespective of whether they are related to flows based on individual PDU or PDU sets.

### PDU Discard

For the discussions in this section, assume that each PDU or PDU set has a clearly defined deadline which can be used by UE in determining whether to discard the PDU or PDU set. In the next section, we then discuss how this deadline should be defined.

If a PDU misses its deadline (whether it is associated with a PDU set or not), it may or may not become obsolete to the application. For example, depend on of individual implementation, an XR application may be able to still use belated PDUs in its decoding. Or if PDU A is late but the decoding of PDU B depends on the content of PDU A, then PDU A is still useful to the application even if it has missed its own deadline. As explained in SA4’s LS [1], such decisions could depend on implementations of individual applications. For these reasons, we think that on downlink, if a PDU is already in UE’s layer-two buffer, then it should be up to UE implementation whether to discard or deliver the PDU after it misses its deadline or becomes redundant.

Proposal 1.	On downlink, it is up to UE implementation whether to discard or deliver a PDU which is already in UE’s layer-two buffer after it has missed its deadline.

On uplink, the situation may be slightly different. From UE’s perspective, it may be beneficial not to transmit a PDU if that PDU has already missed its deadline, because that would save UE power in UL transmission and avoid wasting UL radio resource. The latter especially matters when there is a congestion on UL. On the other hand, for the similar reason explained above for DL, a belated packet may still be useful to the application. Such applications can provide assistance information to CN and CN then informs RAN about applications’ preference. Therefore, we think a more flexible option to leave this decision to network configuration, i.e. network can configure whether UE should transmit or discard a PDU after it has missed its deadline.

Proposal 2.	On uplink, network configures whether UE should discard or transmit a PDU after it has missed its deadline.

PDU discard may have impact on some layer-two procedures. For example,

- When a RLC receiver discards a PDU in an AM DRB, the transmitter should be notified so that the transmission window can be timely updated. Otherwise, the transmission window may be stalled and block subsequent transmissions. Similarly, if a PDU is discarded by its transmitter, it is useful for the transmitter to inform the receiver of the gap.

- Another example is HARQ procedure. On uplink, after a PDU misses its deadline and is subject to discard (per network configuration), it is wasteful for both network and UE to continue its HARQ re-/transmission. Since NR uses async HARQ without configurable maximum number of retransmissions, UE needs a method to inform network to terminate the associated HARQ process.

Proposal 3.	Study impacts of PDU discard on layer-two procedures.

### Deadlines used by UE

In this section, we discuss how to define a deadline for UE to use in its layer-two procedures. We first discuss it for downlink and then uplink.

On downlink, once a PDU or PDU set is received by UE, it is held in a playout buffer until it should be processed for display. Therefore, application client has full knowledge of when the delivery deadline for a PDU or PDU set is. Moreover, as we discussed for Proposal 1, once a PDU or PDU set is in UE’s layer-two buffer (whether it has been successfully received or not), it is up to UE implementation whether to discard or deliver the PDU or PDU set. Since UE and application client are on the same device, it is fairly easy for UE to get delivery deadline for each PDU or PDU set (e.g. via cross-layer indication). Therefore, there is no need for network to provide any additional delay budget or deadline information to UE.

Observation 1.	On downlink, UE can obtain delivery deadline for each PDU or PDU set from application client. No additional delay budget or deadline needs to be configured or signalled for UE’s handling of PDUs or PDU sets.

On uplink, if network configures UE to discard PDUs or PDU sets that have missed their deadlines for crossing Uu interface, then UE needs to know what this deadline is. The current SA2 spec defines both end-to-end packet delay budget (PDB) and PDB for 5G RAN for a QoS flow, as follows:

- The End-to-end PDB is specified in the spec for a 5QI;

- The 5G-AN_PDB is the end-to-end PDB subtracted by a static value of, e.g., 1, 2, 5, 10, 20ms. This static value covers the delay between N6 terminating points of UPF and 5G-AN. Therefore, 5G-AN_PDB covers the delay through gNB, air interface, and UE’s layer-2 procedures.

Since it is not possible for a UE to know the amount of delay through gNB, the 5G-AN_PDB as currently defined cannot be used as a deadline by UE in its layer-two procedures.

Observation 2.	On uplink, UE needs to have a deadline defined for its layer-two procedures. The 5g-AN_PDB defined in the current SA2 spec cannot be used for the purpose.

We think network can configure UE, on a per-DRB basis, with a UE delay budget (denoted UE_PDB) that covers the delay between UE’s layer-two entry points and gNB’s layer-two termination point. It is in fact a component of the existing end-to-end PDB that has not been specified today but needed by UE in its handling of XR data.

On uplink, since PDUs in a PDU set typically can be delivered to UE at the same time, this delay budget can be defined for each individual PDU, regardless of whether it is associated with a PDU set or not.

This UE_PDB can be defined as an upper-bound for the delay from the time when UE first receives a PDU from application to the time when UE receives a positive RLC status report for the PDU in case of an AM DRB or when UE completes the HARQ process for the PDU in case of an UM DRB.

Proposal 4.	Network can configure a per-DRB delay budget for PDU on uplink, regardless of whether it is associated with a PDU set or not. It is the upper-bound for the delay from when UE first receives a PDU from application to the time when UE receives a positive RLC status report for the PDU in case of an AM DRB or when UE completes the HARQ process for the PDU in case of an UM DRB.

### Deadlines used by RAN

RAN2 generally do not discuss or specify gNB’s behaviors. However, in the following we argue that it may be desirable for UE to provide assistance information to gNB in its determination of deadlines for PDUs and PDU sets.

First, we show that the 5G-AN-PDB defined in the current SA2 spec may lead to deadlines earlier than necessary and thus results in loss in system capacity. We show it by using an example on downlink traffic, illustrated in Figure 1. In the figure,

- Nominal arrival time denotes the idealized arrival time of a PDU at RAN if there is no jitter. For a PDU set, it is the idealized arrival time of the first PDU in the PDU set.

- Actual arrival time is the actual time when a PDU arrives at RAN. Actual arrival time can be before, at, or after nominal arrival time, depending on the jitter experienced by the PDU. These three cases are illustrated by PDU A, B and C in Figure 1, respectively.

- Deadline is the target time by when a PDU or a PDU set should be delivered to application. There can be two ways to define a deadline:

  - If one follows the current SA2 convention, a configured deadline for a PDU arriving at RAN equals its actual arrival time + 5G-AN_PDB.

  - Delivery deadline is the absolute time by when a PDU or PDU set needs to be processed by application for display. More specifically, upon reception at UE, a PDU or PDU set is held in a playout buffer until it is time to be processed for display. Therefore, from application’s perspective, there is an absolute deadline for each PDU or PDU set.

- Delay budget is the margin between configured deadline and actual arrival time (to keep the current convention used in SA2 spec). Nominal delay budget is the margin between delivery deadline and nominal arrival time. For PDU sets, these two terms can be extended to PDU set delay budget and nominal PDU set delay budget, respectively.

Figure 1. Deadline, arrival time and delay budget.

As one may see from the example above, because 5G-AN_PDB is a fixed value for all PDUs, it needs to be chosen such that all PDUs can meet their deadlines, even if a PDU has experienced a maximum amount of jitter (e.g. PDU C in Figure 1). Since most PDUs would have jitters shorter than the maximum, there would be PDUs which do not meet this configured delay budget but still have time left before their actual delivery deadlines.

On uplink, similar observations can be made. If nominal arrival times of PDUs are known to gNB, then gNB can estimate the amount of delay a PDU has experienced on uplink and then derive the remaining delay budget that PDU has left until it reaches the application. That information obviously is useful for gNB’s scheduling.

Therefore, If RAN has the knowledge of delivery deadlines of downlink traffic or nominal arrival times of uplink traffic, it can have more delay budget in its scheduling and hence achieve higher system capacity. For UE power saving, the more delay budget a PDU has, the shorter on duration needs to be configured and hence the more UE power savings can be achieved.

Observation 3.	If RAN has the knowledge of delivery deadlines of downlink traffic or nominal arrival times of uplink traffic, it can have more delay budget in its scheduling and hence achieve higher system capacity and enable more UE power savings.

On uplink, we think it would be the simplest if UE can provide nominal arrival times of its uplink traffic to RAN, because application and UE are hosted on the same device.

On downlink, there may be several options available for RAN to obtain delivery deadlines. For example,

- In one option, since delivery deadlines are determined by application client, UE can provide them to RAN. UE first convert delivery deadlines provided by application client, which are defined in real time, to 5G system time and then provide them to gNB. Since UE and application clients are hosted on the same device, they are fully synchronized in time, this conversion should be straightforward. As delivery deadlines for periodic traffic can be defined based on periodicity and start offset of the traffic, this signaling from UE to RAN needs to happen only once, instead of per PDU or PDU set. Hence the signaling overhead is minimal.

- In another option, delivery deadlines can be provided to RAN by application server via 5GC nodes. For example, when 5GC establishes a QoS flow, it configures the flow with nominal delay budget instead of legacy delay budget. This option requires RAN either to be able to infer nominal arrival times and periodicity of the flow based on measurements on traffic arrivals or use some methods to determine the offset between the absolute time used by application server and 5G system time. However, neither of them seems trivial to implement.

Comparing the above two options, we think it is much simpler, from system’s perspective, to have UE provide nominal arrival times (for uplink) and delivery deadlines (for downlink) to gNB.

Observation 4.	It is simpler to have UE than 5GC provide delivery deadlines and nominal arrival times to RAN.

In summary, we believe that RAN can use delivery deadlines (for downlink) and nominal arrival times (for uplink) instead of configured deadlines (i.e. actual arrival time + a fixed delay budget) to have more delay budget in its scheduling of PDUs and PDU sets and hence achieve higher system capability. And that can be enabled by a simple, one-time signaling from UE to RAN. We therefore think it is worthwhile for RAN2 to study such enhancements.

Proposal 5.	RAN can use delivery deadlines (for downlink) and nominal arrival times (for uplink) instead of configured deadlines (i.e. actual arrival time + a fixed delay budget) in its scheduling of PDUs and PDU sets. RAN2 study signaling enhancements that enable UE to provide that information to RAN.

### Delay statistics report to RAN

In some cases (e.g. if a flow has varying frame rate), it is not easy to signal nominal arrival times and delivery deadlines for each PDU or PDU set. Network then has to use a fixed delay budget in its scheduling. As we have explained in Section 2.1.3, that can result in conservative deadlines. Therefore, having UE provide feedback on delay statistics (e.g. average, standard deviations, etc) can help network adapt the delay budgets it configures/applies to compensate the inefficiency. More specifically,

- On downlink, UE can measure the amount of delay budget left before the delivery deadline (aka residual delay budget) of a PDU or PDU set and then provide those statistics to RAN. RAN then can use that information to adjust the delay budget it applies on DL traffic. For example, if residual delay budget is large, RAN can increase the delay budget it applies.

- On uplink, UE can measure the amount of delay experienced by a PDU or PDU set when it is successfully received by RAN (e.g. upon reception of a positive RLC status report). Network then can use the statistics of this delay to estimate the residual delay budget for the rest of the connection, i.e. end-to-end delay budget provisioned for the flow subtracted by the delay reported by UE.

Proposal 6.	Network can configure UE to measure and report DL and/or UL delay statistics for selected DRBs.

## Enhancements for PDU sets

It has become a common understanding that a video frame in XR traffic may arrive at RAN as a set of PDUs (e.g. due to IP segmentation). SA2 has been discussing issues related to PDU sets, e.g., how to define and mark them, PDU set-level instead of PDU-level QoS attributes, etc, and how to support them. Since RAN2 have only three meetings for its study, we do not think RAN2 have to wait for SA2’s progress; instead, RAN2 can start evaluating impacts of PDU sets on RAN2 procedures at the same time.

### PDU set Content criteria

In SA4’s reply LS to SA2 [1], three different encoding methods (aka content criteria) for PDU sets are described: (1) “all or nothing”; (2) “good until the first loss”; (3) FEC with either static or variable code rate. Different content criteria can have different implications on how PDU sets should be handled. For example, with mode 1, a PDU, as well as the entire PDU set, may become obsolete to the application if it has missed its deadline. For the only two modes, in addition to delay, the “position” of a PDU in a PDU set also matters. For example, once UE has received enough number of PDUs in a PDU set for its decoding, then all subsequent PDUs in that PDU set becomes redundant and thus may be obsolete to the application.

We think these three models are all valid models, as suggested by SA4 [1] and hence they all should be included in RAN2’s study on PDU sets.

Proposal 7.	Study on PDU sets should consider at least the following content criteria: “all or nothing”, “good until first loss” and “FEC with either static or variable code rate”.

In Section 2.1.2, it is discussed that an individual PDU may be subject to discard if it misses its delivery deadline or delay budget. In the above, it is explained that for a PDU associated with a PDU set, whether it becomes of no value to the application may be subject to additional criteria, depending on the content criteria of the PDU set. However, we think the same argument for individual PDUs still applies. That is,

- On downlink, it should be up to UE implementation whether to deliver or discard a PDU in a PDU set after it becomes obsolete (content criteria of the associated PDU no longer can be fulfilled) or redundant (content criteria of the associated criteria have already been fulfilled).

- On uplink, the decision can be up to network configuration.

Proposal 8. 	On downlink, it is up to UE implementation whether to deliver or discard a PDU in a PDU set after

- it misses its deadline; or

- the content criteria of its associated PDU set no longer can be fulfilled; or

- the content criteria of its associated PDU set have already been fulfilled.

Proposal 9.	On uplink, network configures whether UE delivers or discards a PDU in a PDU set after

- it misses its deadline; or

- the content criteria of its associated PDU set no longer can be fulfilled; or

- the content criteria of its associated PDU set have already been fulfilled.

### Intra-PDU set handling

Data packets in a PDU set may or may not arrive at RAN at the same time, due to jitter caused by core network. Even if they do, we think it is simpler to map each data packet into an individual PDCP SDU, because aggregating multiple data packets into a single PDCP SDU would create extra unnecessary complexities. For example, header compression and integrity protection procedures may need to be revisited. In addition, the size of a typical PDU set is larger than the maximum size of PDCP SDU for most XR encoding rates. Since a PDU set has to be split between multiple PDCP SDUs in typical use cases, the advantage of PCDP aggregation is not going to be substantial.

Proposal 10. 	SDAP maps each data packet in a PDU set to a single PDCP SDU.

A PDU set is originally generated as a single data unit at application server, and it is played back as a single data unit at the receiver. Therefore, if two PDUs belong to the same PDU set, then they should have the same delivery deadline.

Proposal 11.	All PDUs in the same PDU set share a common deadline.

In general, if one PDU in a PDU set is subject to discard because it has become obsolete (e.g. in case of “all or nothing”) or redundant (in case of FEC), then the subsequent PDUs in the same set will become obsolete or redundant too. However, according to SA4 [1], different PDUs in the same PDU set may have different importance to the application. For example, that may depend on the encoding scheme implemented by application. Therefore, it is desirable to keep the decision on whether to discard a PDU in a PDU set independent from status of other PDUs in that same PDU set, i.e. discard of one obsolete or redundant PDU does not necessarily trigger the discard of all subsequent PDUs in the same PDU set. In other words, decision criteria described in Proposal 9 & 10 for PDU A should not be affected by PDU B.

Proposal 12.	The decision on whether to discard a PDU in a PDU set as described in Proposal 9 & 10 can be made independently from the status of other PDUs in the same PDU set.

Although PDUs in a PDU set share a common set of QoS attributes, it is more efficient to perform scheduling and RLC and HARQ retransmissions based on individual PDU instead of entire PDU set. It is especially the case for applications using FEC, since not all PDUs in a PDU set are needed.

Proposal 13.	HARQ and RLC re-/transmission procedures are based on individual PDU instead of PDU set.

### Inter-PDU set handling

Some video encoding schemes use “delta compression” techniques, e.g. there may be so-called P-frame whose decoding may depend on the content of a prior frame (so called I-frame). There may also be frames (e.g. so called B-frames) whose decoding depend on both prior and subsequent frames. According to SA4’s rely LS to SA2 [1], in more recent encoding schemes, there may also be frames which are more important than other.

For these reasons, SA2 is discussing to introduce a new parameter, PDU set importance, for inter-PDU set handling for DL traffic [2]. From RAN2’s perspective, we think there are limited enhancements for RAN2 to study to support inter-PDU set handling for DL traffic, because most of the differentiated handling are implemented at gNB. Only if SA2 agree to support differentiated inter-PDU handling for UL traffic, RAN2 need to discuss enhancements for supporting it.

Proposal 14.	RAN2 study enhancements for differentiated inter-PDU handling only if SA2 agree to support it for UL traffic.

SA2 are also discussing whether there are impacts due to dependency between PDU sets. In SA4’s reply LS to SA2 [1], it is stated that

“… The handling of dependent PDU Sets once a leading PDU Set is lost is not universally defined and depends on the operation of the application. …” “…a PDU Set may “depend” on previously received PDU Sets. However, such dependencies do not necessarily result in discarding dependent information units, but the user experience may be degraded. …”

As we can see from the above statements, coupled discard of PDU sets are not always necessary and hence should not be mandated. Moreover, such enhancements do introduce extra implementation complexities for UE. For example, to support coupled discard, UE needs to keep additional states of discarded PDUs and check every received PDU set for potential dependency with every previously discarded PDUs.

Proposal 15.	Coupled discard between PDU sets with dependency are not studied by RAN2.

### Signaling aspects

Based on the discussions in Section 2.2.1 and 2.2.2, we can see that intra-PDU set handling has impact on layer-two procedures. Hence information about a PDU set is needed at both transmitter and receiver. And since the information is needed for each PDU in a DRB which is based on PDU sets, the information needs to be sent in band via user plane signaling.

The impact of PDU set at transmitter side is mostly on PDCP layer. At receiver side, the impact can be on both PDCP and RLC layers. Since the interaction between PDCP and RLC layer can be done via cross-layer indications, we think PDCP can be the right layer to include PDU set information.

We think it would enable simpler receiver implementation (e.g., for cross-layer indication between layer-two protocols) if PDU set information is not ciphered and not included in integrity protection, just like SDAP header.

Proposal 16.	PDU set information is sent in band in PDCP header of each PDU in a PDU set. It is not ciphered and not included in integrity protection.

For a receiver to process a PDU according to the content criteria and QoS requirements of a PDU set, the receive must be able to identify which PDU set this PDU is associated with. To that purpose, PDU set must be indexed by a sequence number, and a PDU must be indexed within its associated PDU set. In addition, for the receiver to know when it is done a PDU set (e.g. remove its states), it also has to know the size of the PDU set or which PDU is the last one in the set (e.g. if the transmitter starts streaming packets before it completes encoding the entire set).

Proposal 17.	PDU set information in a PDU header should include at least fields that can help identify the association between a PDU and a PDU set, e.g. sequence number of PDU set, index of PDU within its associated PDU set, size or end of a PDU set, etc.

A receiver also needs to know the content criteria of a PDU set for its reception. If content criteria is static or semi-static (e.g. “all or nothing”), then they can be RRC configured instead of being dynamically signalled. On the other hand, if PDU sets are FEC protected and their redundancy ratio is dynamically adapted, then the content criteria have to be signalled in band via user-plane signaling.

Proposal 18.	PDU set information in a PDU header can optionally include content criteria for an PDU set, if they cannot be statically configured.

As discussed for Proposal 12, all PDUs in a PDU set share a common deadline. If UE is configured to use nominal deadline  for a DRB, then the deadline does not need to signal in band, as both UE and gNB can derive this information. On the other hand, if UE is configured to use configured deadline a DRB, this common deadline needs to be signalled in band, because there is no simple reference point in time for the receiver to derive this information.

Proposal 19.	PDU set information in a PDU header can optionally include the deadline based on legcy PDB of a PDU set, which is common for all PDUs in the PDU set.

## Enhancements for better adaptation by application

Many implementations of XR applications (typically at application source) are capable of adapting their bit rates or frame rates in response to variations in the quality of its connections (e.g. available bandwidth or delay or loss rate). Such adaptations by application certainly can help provide a more robust user experience.

Observation 5.	XR application sources often are capable of adapting their bit/frame rates based on the quality of their connections.

A number of UE configurations depend on traffic patterns of XR application. For example, periodicity of UE’s DRX and SPS/CG configurations likely would be configured based on the frame rate of XR traffic. So would SR configuration if network chooses to match its periodicity with frame rate of UL traffic and rely on subsequent BSRs to schedule bursts. UE’s LCP configuration would be configured in accordance with XR’s bit rate.

Since applications’ adaption typically performed at application level (i.e. aka “over the top”), RAN needs to adapt UE’s configurations in accordance with application’s adaptation in a timely manner. Otherwise, QoS requirements cannot be guaranteed whenever there is a mismatch between traffic pattern and UE’s configurations. Clearly, some coordination between application, RAN and UE is needed.

Observation 6.	RAN/UE need to adapt UE’s configurations to match application’s rate adaptation in a timely manner, to ensure consistent QoS performance.

We think there can be at least two options for RAN/UE to adapt with applications, depending on the direction of signaling flow:

Option 1.

- After application plans to change the bit rate or frame rate (or periodicity) of a QoS flow, it notifies AF. Note that even in the case where traffic is UL centric, application can signal AF over UE’s radio link.

- AF then coordinates with CN to adapt the bit rate or periodicity of the corresponding QoS flow. Currently there is no mechanism for AF to simply adjust a QoS flow’s attributes such as bit rate; instead, AF has to repeat the QoS flow establishment procedure for the purpose. Hence this step can take a long time (e.g. 10s of msec) to complete.

- Once the QoS flow is updated with the new rate or frame rate (or periodicity), CN establishes the new QoS flow with RAN. RAN in turn reconfigures UE’s DRX/SPS/CG/LP configurations.

Option 2.

- After application plans to change the bit rate or frame rate (or periodicity) of a QoS flow, it notifies AS layer of UE. Note that in case the traffic is DL centric, application server can notify application client at UE and the application client then notifies the AS layer of UE.

- UE provides the preferred bit rate or frame rate (or periodicity) to RAN. RAN in turn reconfigures UE’s DRX/SPS/CG/LCP configurations.

- To update bit/frame rate of the flow with CN, RAN may leverage the existing alternative QoS profile (AQP) feature. More specifically, AF establishes a set of AQPs with CN when the QoS flow is initially established. During a session, when RAN finds the current QoS profile is no longer appropriate, it sends a QoS Notification Control to CN to update the flow’s QoS profile (i.e. replace the current one by an AQP). With this option, no QoS flow re-establishment needs to be performed. Hence it can be completed much faster than Option 1.

- After CN completes its adaption of new rate, it in turn notifies the AF.

Comparing the above two options, we can see that rate adaption can be performed with much less latency if UE instead of AF initiates the signaling and hence Option 2 is more preferred.

Observation 7.	It is more efficient for RAN instead of application function to request CN to adapt an existing flow’s QoS profile.

Proposal 20.	RAN2 study enhancements for UE to indicate its preferred bit/frame rate to RAN.

How fast an application can adapt its bit/frame rate determines how robust the user experience can be. That in turn depends on how fast RAN is able to reconfigure UE.

Observation 8.	How fast application can adapt its bit/frame rate affects user experience but depends on how fast RAN is able to reconfigure UE.

Unfortunately, legacy RRC Reconfiguration procedure and UE Assistance Information (UAI) message requires significant delay compared to the typical periodicity of XR traffic. The long delay is due to two factors: propagation delay from DU to CU, which typically is around 10msec, and RRC processing delay requirement, which is also 10msec. Therefore, if UE requests a new bit/frame rate by UAI and network uses RRC Reconfiguration to change UE’s configurations, it would take at least 30msec or longer to complete the procedure. If we take delays in CN signaling also into account, that means it would take at least several XR cycles to effect a change. The call flow below illustrates such an example.

We think an enhancement to enable faster adaptation is to use MAC CE instead RRC message. More specifically,

- RAN can pre-configure a set of candidate bit rates and/or frame rates. UE then uses a MAC CE to indicate which rate it prefers;

- RAN can pre-configure a set of DRX/SPS/CG/LCP configurations and then use MAC CE to indicate to UE which one it should use, instead of using legacy RRC Reconfiguration procedure.

Here we note that for SPS and type-2 CGs, it is already possible for network to configure multiple configurations with different parameters and then use activation DCI to tell UE which configuration to use. There are two limitations with this approach. First, it limits the number of combination of parameters to the maximum allowed SPS/CG configurations, which is 16. That can quickly become inadequate if the number of parameters in a combination increases beyond 3. Second, this approach limits the parameters for adaption to only those which can be signaled within an activation DCI.

Proposal 21.	RAN2 study MAC CE based adaptation of UE configurations. FFS which UE configurations should be included in this enhancement.

Currently applications adapt their bit/frame rate based on their own measurements on end-to-end QoS of their connections, e.g. delay, loss rate, available bit rate (ABR), etc. We’d argue that it is beneficial to applications if RAN can provide similar information about UE’s radio link too.

Although what RAN can provide is not end-to-end QoS information, it can be much more accurate and responsive than what application can estimate on their own about UE’s radio link, which typically is the bottleneck of an end-to-end connection. Take ABR as an example. When RAN reconfigures UE’s cell group, CA configuration or total bandwidth, it is faster and more accurate for RAN to indicate to application how those changes may affect its ABR, and at a granularity finer than what UE can estimate on its own. In fact, Available Bit Rate MAC CE is already available for RAN and UE to use. What may need to be enhanced for XR is that ABR for XR traffic can be a function of its frame rate. Hence it is desirable for network to indicate ABR for different frame rates, so that application can be better informed in its rate adaption. Similarly, on UL, it is desirable for UE to be able to inquire ABR at a frame rate different from its current one.

Delay and loss of traffic going through RAN can depend on the loading at RAN. When there is a fast or large change in a cell’s loading, it is more efficient for RAN to provide an estimate of the new delay and loss to application than having application estimate them by its own measurements. That would be too slow and more error prone.

Observation 9.	RAN can provide faster and better estimates on a flow’s QoS than application itself.

Proposal 22.	RAN2 study RAN assistance information to UE (application) such as enhanced available bit rate, delay and loss that can help enable better adaptations by XR applications.

# Conclusion

Based on the above analysis, we’d recommend RAN2 to discuss and adopt the following proposals:

General aspects

Proposal 1.	On downlink, it is up to UE implementation whether to discard or deliver a PDU which is already in UE’s layer-two buffer after it has missed its deadline.

Proposal 2.	On uplink, network configures whether UE should discard or transmit a PDU after it has missed its deadline.

Proposal 3.	Study impacts of PDU discard on layer-two procedures.

Observation 1.	On downlink, UE can obtain delivery deadline for each PDU or PDU set from application client. No additional delay budget or deadline needs to be configured or signalled for UE’s handling of PDUs or PDU sets.

Observation 2.	On uplink, UE needs to have a deadline defined for its layer-two procedures. The 5g-AN_PDB defined in the current SA2 spec cannot be used for the purpose.

Proposal 4.	Network can configure a per-DRB delay budget for PDU on uplink, regardless of whether it is associated with a PDU set or not. It is the upper-bound for the delay from when UE first receives a PDU from application to the time when UE receives a positive RLC status report for the PDU in case of an AM DRB or when UE completes the HARQ process for the PDU in case of an UM DRB.

Observation 3.	If RAN has the knowledge of delivery deadlines of downlink traffic or nominal arrival times of uplink traffic, it can have more delay budget in its scheduling and hence achieve higher system capacity and enable more UE power savings.

Observation 4.	It is simpler to have UE than 5GC provide delivery deadlines and nominal arrival times to RAN.

Proposal 5.	RAN can use delivery deadlines (for downlink) and nominal arrival times (for uplink) instead of configured deadlines (i.e. actual arrival time + a fixed delay budget) in its scheduling of PDUs and PDU sets. RAN2 study signaling enhancements that enable UE to provide that information to RAN.

Proposal 6.	Network can configure UE to measure and report DL and/or UL delay statistics for selected DRBs.

Enhancements for PDU sets

Proposal 7.	Study on PDU sets should consider at least the following content criteria: “all or nothing”, “good until first loss” and “FEC with either static or variable code rate”.

Proposal 8. 	On downlink, it is up to UE implementation whether to deliver or discard a PDU in a PDU set after

- it misses its deadline; or

- the content criteria of its associated PDU set no longer can be fulfilled; or

- the content criteria of its associated PDU set have already been fulfilled.

Proposal 9.	On uplink, network configures whether UE delivers or discards a PDU in a PDU set after

- it misses its deadline ; or

- the content criteria of its associated PDU set no longer can be fulfilled; or

- the content criteria of its associated PDU set have already been fulfilled.

Proposal 10. 	SDAP maps each data packet in a PDU set to a single PDCP SDU.

Proposal 11.	All PDUs in the same PDU set share a common deadline.

Proposal 12.	The decision on whether to discard a PDU in a PDU set as described in Proposal 9 & 10 can be made independently from the status of other PDUs in the same PDU set.

Proposal 13.	HARQ and RLC re-/transmission procedures are based on individual PDU instead of PDU set.

Proposal 14.	RAN2 study enhancements for differentiated inter-PDU handling only if SA2 agree to support it for UL traffic.

Proposal 15.	Coupled discard between PDU sets with dependency are not studied by RAN2.

Proposal 16.	PDU set information is sent in band in PDCP header of each PDU in a PDU set. It is not ciphered and not included in integrity protection.

Proposal 17.	PDU set information in a PDU header should include at least fields that can help identify the association between a PDU and a PDU set, e.g. sequence number of PDU set, index of PDU within its associated PDU set, size or end of a PDU set, etc.

Proposal 18.	PDU set information in a PDU header can optionally include content criteria for an PDU set if they cannot be statically configured.

Proposal 19.	PDU set information in a PDU header can optionally include the deadline based on legacy PDB of a PDU set, which is common for all PDUs in the PDU set.

Enhancements for better adaptation by applications

Observation 5.	XR application sources often are capable of adapting their bit/frame rates based on the quality of their connections.

Observation 6.	RAN/UE need to adapt UE’s configurations to match application’s rate adaptation in a timely manner, to ensure consistent QoS performance.

Observation 7.	It is more efficient for RAN instead of application function to request CN to adapt an existing flow’s QoS profile.

Proposal 20.	RAN2 study enhancements for UE to indicate its preferred bit/frame rate to RAN.

Observation 8.	How fast application can adapt its bit/frame rate affects user experience but depends on how fast RAN is able to reconfigure UE.

Proposal 21.	RAN2 study MAC CE based adaptation of UE configurations. FFS which UE configurations should be included in this enhancement.

Observation 9.	RAN can provide faster and better estimates on a flow’s QoS than application itself.

Proposal 22.	RAN2 study RAN assistance information to UE (application) such as enhanced available bit rate, delay, loss that can help enable better adaptations by XR applications.

# References

- S4-220505, LS Rely on QoS support with PDU set granularity, May 2022.

- S2-2205074, KI#4 and #5, Baseline for PDU Set based QoS framework, Huawei, CATT, China Mobile, Futurewei, HiSilicon, Lenovo, KDDI, OPPO, Samsung, ZTE, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, May 2022.