3GPP TSG-RAN WG2 Meeting #119 electronic	R2-2207078

Online, August 2022

Agenda:	8.9.3

Source:	OPPO

Title:	 	Discussion on further enhancement of U2N service continuity

Document for:	Discussion, Decision

- Introduction

- Specify mechanisms to enhance service continuity for single-hop Layer-2 UE-to-Network relay for the following scenarios [RAN2, RAN3]: - Inter-gNB indirect-to-direct path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> gNB Y”) - Inter-gNB direct-to-indirect path switching (i.e., “remote UE <-> gNB X” to “remote UE <-> relay UE A <-> gNB Y”) - Intra-gNB indirect-to-indirect path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB X”) - Inter-gNB indirect-to-indirect path switching (i.e., “remote UE<-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB Y”) Note 2A: Scenario D is to be supported by reusing solutions for the other scenarios without specific optimizations.

According to the WID, to enrich the supported scenario, In Rel-18, RAN2 should not only work on the inter-gNB path switch between direct link and indirect link but also work on the indirect to indirect path switch for the inter- and intra-gNB case. In this contribution, we will investigate whether the enhancement in Rel-17 made can be reused for the Rel-18 introduced scenarios and if not, what enhancements can be considered.

- Discussion

  - Measurement Report

In Rel-17 UE-to-Network Relay, when remote UE performs measurement report for path switch, it should include Relay UE ID, a serving cell ID of relay UE, and RSRP information. When it comes to Rel-18, naturally, remote UE should still report the same information for path switch.

In Rel-18 UE-to-Network relay path switch, as in Rel-17, remote UE should also report Relay UE ID, serving cell ID of relay UE, and RSRP information within the measurement report.

  - Measurement quantity

In Rel-17, when remote UE performs an indirect to direct path switch, SL-RSRP of the serving relay is preferably used as the SL measurement quantity. Furthermore, if data is not available, then either SL-RSRP / SD-RSRP can be used. On the other hand, the SD-RSRP of the target relay can be used as the SL measurement quantity in the case of a path switch from a direct to an indirect path switch.

When it comes to Rel-18 service continuity, for scenario A and scenario B, the same principle can be reused. For scenarios C and D, similarly, it is up to remote UE implementation on whether SL-RSRP or SD-RSRP can be used if there is no available data transmission; otherwise, SL-RSRP of serving relay can only be used. On the other hand, the SD-RSRP of the candidate relay can be used as the SL measurement quantity.

In Rel-18 U2N service continuity, for scenario A and Scenario B, SL-RSRP, if data is available, of the serving relay is used as the SL measurement quantity. If data is not available, then up to UE implementation on whether SL-RSRP or SD-RSRP can be used.

In Rel-18 U2N service continuity, for scenario A and scenario B, SD-RSRP of the target relay is used as the SL measurement quantity for the case of path switch from direct to indirect path switch.

In Rel-18 U2N service continuity, for scenario C and scenario D, SL-RSRP, if data is available, of the serving relay is used as the SL measurement quantity, if data is not available, then up to UE implementation on whether SL-RSRP or SD-RSRP can be used.

In Rel-18 U2N service continuity, for scenario C and scenario D, the SD-RSRP of the candidate relay can be used as the SL measurement quantity.

  - Measurement event

In Rel-17 U2N service continuity, to trigger remote UE to perform measurement reports, two additional measurement events were defined accordingly. For remote UE performing indirect to direct path switch, when the radio measurement of serving relay becomes worse than a configured threshold and optionally, the radio measurement of neighbor Uu cell becomes better than another configured threshold, in that case, remote UE should perform measurement report.

On the other hand, for remote UE performing direct to indirect path switch, when the radio measurement of candidate relay becomes better than a configured threshold, and optionally, the radio measurement of serving Uu cell becomes worse than another configured threshold, in that case, remote UE should perform measurement report.

When it comes to Rel-18 service continuity, for scenario A and scenario B, the same principle can be reused to trigger remote UE to perform measurement reports.

In Rel-18 service continuity, for scenario A and scenario B, the legacy measurement event defined for Rel-17 service continuity can be reused.

However, for scenario C and scenario D, since the measurement of the two links are both PC5 links rather than inter-RAT comparison in Rel-17. Therefore, it is suggested to consider the following options to define the measurement event for indirect to indirect path switch:

- Event XX: The radio measurement of the serving relay becomes worse than threshold-XX1, and the radio measurement of the candidate relay becomes better than threshold-XX2

- Event XY: The radio measurement of the serving relay becomes worse than the radio measurement of the candidate relay with a configured offset-XY.

In Rel-18 service continuity, for scenario C and scenario D, RAN2 discusses the following alternative measurement events for indirect to indirect path switch:

- Event XX: The radio measurement of the serving relay becomes worse than threshold-1, and the radio measurement of the candidate relay becomes better than threshold-2.

- Event XY: The radio measurement of the serving relay becomes worse than the radio measurement of the candidate relay with a configured offset

  - T304 timer

In Rel-17 U2N service continuity, a new timer, T420, is introduced for the direct-to-indirect path switch. The remote UE starts the timer upon reception of the RRC reconfiguration message indicating a direct-to-indirect path switch, and the remote UE initiates RRC re-establishment upon timer expiry. The key point to define such a new timer is the new stop condition, i.e., upon successfully sending the RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgment is received from target L2 U2N Relay UE).

Similarly, when it comes to Rel-18, for scenario A and scenario B, the same timer can be reused. But for scenario C and scenario D, since both are targeted at indirect-to-indirect path switch, which aims at different scenarios, whether the timer can be reused should be discussed. If one can reuse the same start/stop condition for T420, there seems no need to define a new timer.

In Rel-18 service continuity, for scenario-A, the same T304 can be reused, and for scenario B, the same T420 can be reused.

In Rel-18 service continuity, for scenario C and scenario D, RAN2 discusses reusing the T420.

  - RRC_IDLE/RRC_INACTIVE Relay UE

In Rel-17 U2N service continuity, the gNB can select a relay UE in any RRC state, i.e., RRC_IDLE/INACTIVE/CONNECTED, as a target relay UE when triggering the direct or indirect path switch procedure for the remote UE. After receiving the path switch command, remote UE establishes a PC5 link with the relay UE and sends HO a complete message via the relay UE, which will trigger the relay UE, if in RRC_IDLE or RRC_INACTIVE, to enter the CONNECTED state.

Similarly, when it comes to Rel-18, for all of the scenarios, the gNB can still select a relay UE in any RRC state. In addition, the remote UE-oriented solution to trigger the relay UE to enter RRC CONNECTED state is still feasible.

In Rel-18 service continuity, for scenario B/C/D, the target relay UE can be in any RRC state.

In Rel-18 service continuity, for scenario B/C/D, for the target relay in RRC_INACTIVE / RRC_IDLE, as in Rel-17, it is suggested to rely on the message from remote UE to trigger the relay UE entering into the RRC_CONNECTED state.

- Conclusion

In this contribution, we have analysed whether the legacy Rel-17 design for service continuity can be reused in Rel-18 newly introduced scenarios, and if not, how to make further enhancements. Thus, a brunch of proposals is given below:

Proposal 1:	In Rel-18 UE-to-Network relay path switch, as in Rel-17, remote UE should also report Relay UE ID, serving cell ID of relay UE, and RSRP information within the measurement report.

Proposal 2:	In Rel-18 U2N service continuity, for scenario A and Scenario B, SL-RSRP, if data is available, of the serving relay is used as the SL measurement quantity. If data is not available, then up to UE implementation on whether SL-RSRP or SD-RSRP can be used.

Proposal 3:	In Rel-18 U2N service continuity, for scenario A and scenario B, SD-RSRP of the target relay is used as the SL measurement quantity for the case of path switch from direct to indirect path switch.

Proposal 4:	In Rel-18 U2N service continuity, for scenario C and scenario D, SL-RSRP, if data is available, of the serving relay is used as the SL measurement quantity, if data is not available, then up to UE implementation on whether SL-RSRP or SD-RSRP can be used.

Proposal 5:	In Rel-18 U2N service continuity, for scenario C and scenario D, the SD-RSRP of the candidate relay can be used as the SL measurement quantity.

Proposal 6:	In Rel-18 service continuity, for scenario A and scenario B, the legacy measurement event defined for Rel-17 service continuity can be reused.

Proposal 7:	In Rel-18 service continuity, for scenario C and scenario D, RAN2 discusses the following alternative measurement events for indirect to indirect path switch:

Event XX: The radio measurement of the serving relay becomes worse than threshold-1, and the radio measurement of the candidate relay becomes better than threshold-2.

Event XY: The radio measurement of the serving relay becomes worse than the radio measurement of the candidate relay with a configured offset

Proposal 8:	In Rel-18 service continuity, for scenario-A, the same T304 can be reused, and for scenario B, the same T420 can be reused.

Proposal 9:	In Rel-18 service continuity, for scenario C and scenario D, RAN2 discusses reusing the T420.

Proposal 10:	In Rel-18 service continuity, for scenario B/C/D, the target relay UE can be in any RRC state.

Proposal 11:	In Rel-18 service continuity, for scenario B/C/D, for target relay in RRC_INACTIVE / RRC_IDLE, as in Rel-17, it is suggested to rely on the message from remote UE to trigger the relay UE entering into RRC_CONNECTED state.